3GPP TSG-RAN4 Meeting # 116-bis	R4-2514051

# Prague, CZ, October 13th – October 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for UEIBM reporting delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UEIBM window based requirements are incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying the value of M for current beam to be equal to M of the new beam. Modification of reporting delay for the window based reporting to cover the windows that do not trigger the report. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.3.4, 9.13.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------Start of Change #1----------------------------------------

#### 9.5.3.4	Event Triggered Reporting for the UE initiated beam management

For a UE configured with a CSI-ReportConfig with eventType-r19 and with dl-OrJointTCI-StateList，with or without eventDetectionTimeWindowLength-r19 configuration, the reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

When eventDetectionTimeWindowLength-r19 is not configured,

The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE transmits first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than TL1-meas_basic + Tfirst_UL_channel, where:

TL1-meas_basic is the maximum of L1-RSRP measurement periods of the reference signals of the beams corresponding to the event

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, UE shall perform L1-RSRP measurement based on the L1-RSRP measurement period of current beam and new beam(s), respectively.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event1’, UE shall perform L1-RSRP measurement based on current beam L1-RSRP measurement period.

If the newBeamResourceSet-r19 is based on SSB, and if the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is based on SSB:

the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-1 or 9.5.4.1-2 where TReport. = 0, M equals to the M of the new beam, and TSSB is the periodicity of the SSB-Index of the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state for the current beam.

the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-1 or 9.5.4.1-2 where TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

If the reference signal in the indicated TCI state and newBeamResourceSet-r19 are all based on CSI-RS:

the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-1 or 9.5.4.2-2 where TReport. = 0, M equals to the M of the new beam, and TCSI-RS is the periodicity of the reference signal in the indicated TCI state for the current beam.

the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-1 or 9.5.4.2-2 where TReport. = 0, and TCSI-RS is the periodicity of the the reference signal configured by the newBeamResourceSet-r19 for new beams.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event7’, UE shall perform L1-RSRP measurement based on L1-RSRP measurement period of the beams in active TCI state list and new beams.

If the newBeamResourceSet-r19 is based on SSB, and if the SS/PBCH block that is QCLed with the reference signal corresponding to the activated TCI states with the highest valueOfQ-r19 is also based on SSB:

the L1-RSRP measurement period of each beam of activated TCI states refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-1 or 9.5.4.1-2 where TReport. = 0, M equals to the M of the new beam, and TSSB is the periodicity of the SSB-Index of the reference signal in the activated TCI states or the SS/PBCH block which is QCLed with the reference signal in the activated TCI states.

the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-1 or 9.5.4.1-2 where TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

If the reference signal in the valueOfQ-r19 highest L1-RSRP out of the reference signals among the activated TCI states and newBeamResourceSet-r19 are all based on CSI-RS:

the L1-RSRP measurement period of each beam of activated TCI states refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-1 or 9.5.4.2-2 where TReport. = 0, M equals to the M of the new beam, and TCSI-RS is the periodicity of the reference signal in the activated TCI states.

the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-1 or 9.5.4.2-2 where TReport. = 0, and TCSI-RS is the periodicity of the the reference signal configured by the newBeamResourceSet-r19 for new beams.

Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next PUCCH transmission occasion

Otherwise, if eventDetectionTimeWindowLength-r19 is provided, the event triggered measurement reporting delay shall be no larger than W * eventDetectionTimeWindowLength-r19 + (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel, where

- W is the number of windows after an event that will trigger a measurement report within which the number of event instances fulfilling reporting condition is less than eventInstanceCount-r19 and a measurement report is not triggered.

- eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 are configured by the network

- TL1-meas_basic is the measurment period above based on the configuration,

- L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started for the window that will trigger the measurement report and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindowLength-r19).

- The above requriement is applicable when eventDetectionTimeWindowLength-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic . The requirement does not apply when eventDetectionTimeWindowLength-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic.

----------------------------------------Start of Change #2----------------------------------------

#### 9.13.3.4	Event triggered reporting for UE initiated beam management

For a UE configured with a CSI-ReportConfig with eventType-r19 and with dl-OrJointTCI-StateList, with or without eventDetectionTimeWindowLength-r19 configuration the reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

When eventDetectionTimeWindowLength-r19 is not configured,

The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report after condition is met over the air and the point when the UE transmits first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than TL1-meas_basic + Tfirst_UL_channel.

TL1-meas_basic is the maximum of L1-RSRP measurement periods of the reference signals of beams corresponding to the event

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, UE shall perform L1-RSRP measurement based on the L1-RSRP measurement period of current beam and new beam(s) respectively.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event1’, UE shall perform L1-RSRP measurement based on the L1-RSRP measurement period of current beam.

If the SS/PBCH block is QCLed with the reference signal in the indicated TCI state, the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_SSB_CDP in Table 9.13.4.1-1 or 9.13.4.1-2 where TReport. = 0, M equals to the M of the new beam, and TSSB_CDP is the periodicity of the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state for the current beam

If the reference signals in newBeamResourceSet-r19 are based on SSB for a cell with additional PCI, the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB_CDP in Table 9.13.4.1-1 or 9.13.4.1-2 where TReport. = 0, and TSSB_CDP is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event7’, UE shall perform L1-RSRP measurement based on L1-RSRP measurement period of current beam and new beam respectively.

If the SS/PBCH block is QCLed with the reference signal corresponding to the activated TCI states with the highest valueOfQ-r19  the L1-RSRP measurement period of the beam refers to TL1-RSRP_Measurement_Period_SSB_CDP as specified in table 9.13.4.1-1 or 9.13.4.1-2 where TReport. = 0, M equals to the M of the new beam, and TSSB is the periodicity of the SSB-Index of the reference signal corresponding to the activated TCI states with the highest valueOfQ-r19

If the reference signals in newBeamResourceSet-r19 are based on SSB for a cell with additional PCI, the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB_CDP in Table 9.13.4.1-1 or 9.13.4.1-2 where TReport. = 0, and TSSB_CDP is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

Tfirst UL channel is the time to first PUCCH after L1 reporting condition is triggered

Otherwise, if eventDetectionTimeWindowLength-r19 is provided, the event triggered measurement reporting delay shall be no larger than W * eventDetectionTimeWindowLength-r19 + (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel, where

- W is the number of windows after an event that will trigger a measurement report within which the number of event instances fulfilling reporting condition is less than eventInstanceCount-r19 and a measurement report is not triggered.

- eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 are configured by the network

- TL1-meas_basic is the measurement period above based on the configuration

- L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started for the window that will trigger the measurement report and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindowLength-r19).

- The above requriement is applicable when eventDetectionTimeWindowLength-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic . The requirement does not apply when eventDetectionTimeWindowLength-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic.

----------------------------------------End of Changes ----------------------------------------