3GPP TSG-RAN WG4 Meeting #116bis	R4-2514056

Prague, Czech Republic, 13-17 October, 2025

Source:	Nokia

Title:	Discussion on NR_MIMO_Ph6 RRM requirements

Agenda item:	7.10.3

Document for:	Discussion

Introduction

- On enhancing UL capacity and coverage, specify the following enhancements:

  - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification:

    - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding

      - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible.

    - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set

      - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

- On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements:

  - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively)

    - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking

    - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively

  - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs

    - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

Rel-20 MIMO phase 6 [1] aims at introducing enhancements for UL coverage and capacity and DL CSI acquisition. More specifically, the objectives of the WID [1] are given below:

In this paper, we present our views on the RRM requirements for early CSI/CSI-RS/SRS triggering.

SCell transition from deactivation to activation

Agreement:

For early triggering of SRS-AS when SCell transition from deactivation to activation, support at least aperiodic SRS-AS transmission on a SCell, triggered based on legacy SCell activation command activating the SCell.

- FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4)

- FFS: How to trigger the aperiodic SRS-AS transmission based on legacy SCell activation command

- FFS: Whether/what new information is provided in SCell activation command?

Agreement:

For early triggering of CSI/CSI-RS when SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell triggered based on legacy SCell activation command activating the SCell.

- The aperiodic CSI reporting is assosicated with at least aperiodic CSI-RS for CSI on the SCell

- FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4)

- FFS: How to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI based on legacy SCell activation command?

- FFS: Whether/what new information is provided in SCell activation command?

In RAN1#122 meeting, the following was agreed regarding SRS transmission and triggering of CSI/CSI-RS during transition of a deactivated SCell to activated state.

Early triggering of aperiodic CSI-RS and CSI report

Currently, SCell activation delay requirements have been defined for various scenarios. The WID states that early SRS/CSI/CSI-RS triggering for SCell activation is based on legacy SCell activation MAC CE, which implies that direct SCell activation is not in the scope. Regarding SCell activation delay requirements that are based on SCell activation MAC-CE, RAN4 needs to discuss which requirements are in the scope.

Early SRS/CSI/CSI-RS triggering for SCell activation is based on legacy SCell activation MAC CE, so direct SCell activation is not in the scope.

For early triggering of CSI/CSI-RS, the discussion can in our view start from the following scenarios for SCell activation:

- SCell Activation Delay Requirement for Deactivated SCell (8.3.2)

- SCell Activation Delay Requirement for Deactivated PUCCH SCell (8.3.12)

Start discussing early triggering of CSI/CSI-RS based on the following requirements:

  - SCell Activation Delay Requirement for Deactivated SCell (8.3.2)

  - SCell Activation Delay Requirement for Deactivated PUCCH SCell (8.3.12)

The existing SCell activation delay has been defined for known and unknown SCell activation. In the case of early triggering of CSI/CSI-RS, when SCell transitions from deactivated to activated, we propose that RAN4 first discuss requirements for known SCell and wait for RAN1 discussions on how to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI to progress before discussing whether and how to define requirements for unknown SCell.

RAN1 is yet to discuss and agree on how to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI.

RAN4 can start to discuss based on known SCell activation and wait for RAN1 progress on CSI-RS triggering and reporting timeline before discussing unknown SCell activation.

Regarding other requirements, RAN4 should discuss which of the following requirements are in the scope of this WI.

RAN4 to discuss which of the following SCell activation delay requirements are in the scope of the NR_MIMO_Ph6 work item:

  - SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells (8.3.7)

  - SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells (8.3.13)

  - SCell Activation Delay Requirement for Deactivated SCell based on measurement in IDLE/INACTIVE mode (8.3.2A)

  - Fast SCell Activation Delay Requirement for Deactivated SCell (8.3.16)

  - SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation (8.3.17)

  - SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting (8.3.18)

  - OD-SSB based SCell Activation Delay Requirement for Deactivated SCell (8.3.19)

  - OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells (8.3.22)

  - SCell Activation Delay Requirement for Deactivated PUCCH SCell based on OD-SSB (8.3.24)

  - OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation (8.3.26)

Furthermore, there are features whose impact to these sections is captured within some existing section, such as shortMeasInterval-r18. RAN4 needs to clarify the scope regarding all such features.

In the case of SCell activation, the existing delay requirements take into account the delay to acquire the CSI-RS and the UE processing time, when needed, and the time to the first available uplink resource to transmit the CSI report. These delay requirements are based on periodic and semi-persistent CSI-RS resources. CSI reporting delay is part of the delay regardless of if the SCell is known or unknown, and the delay is defined as an uncertainty parameter. The components related to CSI-RS triggering and measurement may be part of the delay if the SCell is unknown.

SCell activation delay requirements have been defined for periodic and semi-persistent CSI-RS resources.

The existing SCell activation delay requirements also factor in the time for the UE to transmit a CSI report.

In the case of early CSI reporting, RAN1 has agreed that the aperiodic CSI reporting is associated with at least aperiodic CSI-RS resources for CSI on the SCell.

The discussion about the timeline of the aperiodic CSI reporting and associated aperiodic CSI-RS resources is still open in RAN1, and it has been discussed that RAN4 can discuss and define the related requirements.

In the case of periodic and semi-persistent CSI-RS resources/CSI reporting, the UE has more than one opportunity to measure and report the CSI, which is not the case in aperiodic CSI reporting using aperiodic CSI-RS resources. Hence, even though the resources (CSI-RS and UL resources for aperiodic CSI reporting) are assigned by the network and are therefore to be considered as uncertainty factors on UE side, it is important that the transmission of the aperiodic CSI-RS resource and UL resources for the ensuing aperiodic CSI report are assigned such that the UE is actually in a position to measure and report.

UE has only one opportunity to measure and report when the CSI-RS resources and the CSI report are aperiodic in nature.

RAN4 to discuss and agree on a timeline in relation to SCell activation delay when the UE is ready to receive the aperiodic CSI-RS resources and transmit the aperiodic CSI report.

Early triggering of SRS-AS

SRS transmission is not part of the current SCell activation delay. However, RAN1 has discussed that RAN4 would need to discuss the timeline of early triggering of SRS-AS. When SCell transitions from deactivated to activated state, the SCell activation delay requirements need to be studied to determine when the UE shall be ready for the aperiodic SRS transmissions. As the SRS is transmitted on the SCell, PUCCH SCell activation requirements shall be studied. RAN4 can start by discussing single PUCCH SCell activation delay requirement.

RAN4 to discuss the timeline when the UE shall be ready to transmit the aperiodic SRS in relation to PUCCH SCell activation delay.

UE transition from IDLE/INACTIVE to CONNECTED mode

Agreement:

For a UE transition from IDLE to CONNECTED mode, support the following procedure at least for early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting):

- Step-1: The UE receives the resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering provided in the system information before MSG3.

- FFS: Which SIB is used to carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering

- Step-2: The UE reports its capability on early SRS/CSI-RS/CSI triggering through MSG3

- Step-3: The UE receives MSG4 that triggers early SRS-AS/CSI-RS/CSI based on  the capability reported by the UE.

- Step-4: The UE performs aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting.

- FFS: Timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting

Note: The term “capability” above does not mean legacy RRC based UE capability.

FFS: For a UE transition from INACTIVE to CONNECTED mode, whether above procedure can be reused at least for early aperiodic SRS-AS/CSI-RS/CSI triggering

Note: Whether the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting can be configured/triggered simultaneously will be discussed separately

In RAN1#122, the procedure for early aperiodic SRS-AS/CSI-RS triggering/CSI reporting when a UE transitions from IDLE to CONNECTED mode was agreed.

Currently, there are no RAN4 requirements for the UE transition from idle/inactive to connected mode, and consequently, for CSI reporting or SRS transmission specifically associated to this transition. The timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting is still open for discussion in RAN1. In our view RAN4 should wait for more RAN1 agreements for this topic before discussing whether early triggering of CSI reporting or SRS transmission at UE transition from idle/inactive to connected mode has RAN4 impact.

There are currently no RAN4 requirements for UE transition from idle/inactive to connected mode or CSI reporting or SRS transmission related to this transition.

RAN4 shall conclude about any possible RAN4 impact due to CSI reporting and SRS transmission for UE transition from idle/inactive mode to connected mode, when RAN1 has made sufficient progress on this topic.

SCell transition from dormant to non-dormant BWP

Proposal 1.3.1A:

For early triggering of SRS-AS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic SRS-AS transmission on the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy.

- Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field and resizing the existing DCI field

- FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported

- FFS: Triggering mechanism(s)

- FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4)

Proposal 1.3.1B:

For early triggering of CSI/CSI-RS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI on the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy.

- Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field and resizing the existing DCI field

- FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported

- FFS: Triggering mechanism(s)

- FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4)

RAN1 has made the following agreements for early CSI triggering and SRS transmission when SCell transitions  from dormant to non-dormant BWP.

RAN1 has discussed that for SCell transition from dormant to non-dormant BWP, the requirements for the timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI as well as the timeline of the aperiodic SRS-AS transmission are to be discussed and defined by RAN4. The requirement that is related to this transition is the BWP switching delay in RAN4 specification. Separate delay requirements have been defined for BWP switch delay on single CC and BWP switch delay on multiple CC.

RAN4 should discuss the possible impact on these sections, but since there are no relevant RAN1 agreements on this scenario, we propose that RAN4 should wait for further progress on this topic in RAN1 before deciding on the impact on RAN4 requirements.

Discussions on timeline of aperiodic CSI reporting and aperiodic SRS-AS transmission when a SCell transitions from a dormant to non-dormant BWP are to be defined by RAN4 according to RAN1 discussion.

RAN4 shall wait for RAN1 to conclude on the timelines of the aperiodic CSI reporting and aperiodic SRS-AS transmission when a SCell transitions from a dormant to non-dormant BWP before discussing the any possible RAN4 requirement impact.

Conclusion

In this paper, the following observations and proposals were made:

- Early SRS/CSI/CSI-RS triggering for SCell activation is based on legacy SCell activation MAC CE, so direct SCell activation is not in the scope.

Proposal 1: Start discussing early triggering of CSI/CSI-RS based on the following requirements:

a. SCell Activation Delay Requirement for Deactivated SCell (8.3.2)

b. SCell Activation Delay Requirement for Deactivated PUCCH SCell (8.3.12)

Observation 1: RAN1 is yet to discuss and agree on how to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI.

Proposal 2: RAN4 can start to discuss based on known SCell activation and wait for RAN1 progress on CSI-RS triggering and reporting timeline before discussing unknown SCell activation.

Proposal 3: RAN4 to discuss which of the following SCell activation delay requirements are in the scope of the NR_MIMO_Ph6 work item:

a. SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells (8.3.7)

b. SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells (8.3.13)

c. SCell Activation Delay Requirement for Deactivated SCell based on measurement in IDLE/INACTIVE mode (8.3.2A)

d. Fast SCell Activation Delay Requirement for Deactivated SCell (8.3.16)

e. SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation (8.3.17)

f. SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting (8.3.18)

g. OD-SSB based SCell Activation Delay Requirement for Deactivated SCell (8.3.19)

h. OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells (8.3.22)

i. SCell Activation Delay Requirement for Deactivated PUCCH SCell based on OD-SSB (8.3.24)

j. OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation (8.3.26)

Observation 2: SCell activation delay requirements have been defined for periodic and semi-persistent CSI-RS resources.

Observation 3: The existing SCell activation delay requirements also factor in the time for the UE to transmit a CSI report.

Observation 4: In the case of early CSI reporting, RAN1 has agreed that the aperiodic CSI reporting is associated with at least aperiodic CSI-RS resources for CSI on the SCell.

Observation 5: UE has only one opportunity to measure and report when the CSI-RS resources and the CSI report are aperiodic in nature.

Proposal 4: RAN4 to discuss and agree on a timeline in relation to SCell activation delay when the UE is ready to receive the aperiodic CSI-RS resources and transmit the aperiodic CSI report.

Proposal 5: RAN4 to discuss the timeline when the UE shall be ready to transmit the aperiodic SRS in relation to PUCCH SCell activation delay.

Observation 6: There are currently no RAN4 requirements for UE transition from idle/inactive to connected mode or CSI reporting or SRS transmission related to this transition.

Proposal 6: RAN4 shall conclude about any possible RAN4 impact due to CSI reporting and SRS transmission for UE transition from idle/inactive mode to connected mode, when RAN1 has made sufficient progress on this topic.

Observation 7: Discussions on timeline of aperiodic CSI reporting and aperiodic SRS-AS transmission when a SCell transitions from a dormant to non-dormant BWP are to be defined by RAN4 according to RAN1 discussion.

Proposal 7: RAN4 shall wait for RAN1 to conclude on the timelines of the aperiodic CSI reporting and aperiodic SRS-AS transmission when a SCell transitions from a dormant to non-dormant BWP before discussing the any possible RAN4 requirement impact.

- References

- RP-251856 	New WID: NR MIMO Phase 6

- R1-2506597 	Moderator summary on enhancing DL CSI acquisition