# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514095

# Prague, Czech, 13th – 17th October, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements on Ku band NTN

Agenda item:	5.10.2

Document for:	Approval

Introduction

RRM core requirements for Rel-19 Ku band NTN are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. In this paper we will provide our views on RRM performance requirements on Ku band NTN.

Discussion

In previous meetings, we have already defined several core requirements, we shall recognize defined requirements and discuss which test cases shall be confirmed.

In core part discussion, we reached several agreements as below:

Agreement: - For SCS, consider SCS=30kHz in FR1-NTN numerology and 120kHz in FR2-NTN numerology, following the discussion in BDaT session. - Mobile VSAT UE shall be considered from RAN4 RRM requirement perspective. There are four cases shall be considered for mobile VSAT UE: - Case 1: mobile VSAT UE served by GSO in FR1-NTN numerology - Case 2: mobile VSAT UE served by NGSO in FR1-NTN numerology - Case 3: mobile VSAT UE served by GSO in FR2-NTN numerology - Case 4: mobile VSAT UE served by NGSO in FR2-NTN numerology Agreement: - Define RRM requirements for added 15kHz SCS in FR1-NTN numerology for GSO in Ku-band Agreement: - Clarify in clause 3.6.12 that “Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN”.

It is evident that the requirements defined for FR2-NTN apply for VSAT UE working in FR1-NTN, except for timing requirements. Since the timing requirements are not related to UE antenna but different SCS. Based on this point, we would like to define measurement accuracy requirements and test procedures.

2.1.1 Measurement Accuracy Requirements

From our perspective, measurement accuracy requirements for Ka band NTN shall be baseline when defining measurement accuracy requirements for Ku band NTN. The following bullets shall be considered:

- Intra-frequency RSRP accuracy requirements

- Inter-frequency RSRP accuracy requirements

- Intra-frequency RSRQ accuracy requirements

- Inter-frequency RSRQ accuracy requirements

- Intra-frequency SINR accuracy requirements

- Inter-frequency SINR accuracy requirements

- L1-RSRP accuracy requirements

In previous meeting, we have agreed that the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN including accuracy requirements. Thus, the legacy measurement accuracy for Ka band NTN shall be reused for Ku band FR1-NTN and FR2-NTN, even if SCS is different.

We shall also clarify how and where to capture the accuracy requirements for Ku band NTN. Actually, we think the existing clause shall not be changed. Since the accuracy requirements for Ka band could be directly reused, and also apply to VSAT working in FR1-NTN. Hence, we shall not separate the case with different numerology. On the contrary, we suggest to add additional clarification on FR1 numerology in FR2-NTN clause. However, considering the different SCS, RAN4 shall only add FR1 numerology without changing legacy FR2-NTN accuracy requirements. For example,

- Unless otherwise specified, the requirements for absolute SS-RSRQ accuracy in this clause apply to a cell on the same frequency as that of the serving cell in FR2-NTN or VSAT working in FR1-NTN. Accuracy; Conditions. Normal condition; Extreme condition; SSB Ês/Iot; Io Note 2 range. Minimum Io; Maximum Io. dB; dB; dB; dBm / SCSSSB Note 1; dBm/BWChannel; dBm/BWChannel. SCSSSB = 120 kHz; SCSSSB = 240 kHz; SCSSSB = 15 kHz; SCSSSB = 30 kHz. 7; 10; ≥-6; Same value as SSB_RP derived from annex B.2.17, according to UE VSAT type; Same value as SSB_RP derived from annex B.2.17, according to UE VSAT type; -121; -118; N/A; -70. 9; 12; N/A; -70; -50. NOTE 1: Values based on EIS as defined in clause 10.3 of TS 38.101-5 [42]. Applicable side condition selected depending on angle of arrival. NOTE 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table..

Proposal 1: The following measurement accuracy requirements shall be considered:

- Intra-frequency RSRP accuracy requirements

- Inter-frequency RSRP accuracy requirements

- Intra-frequency RSRQ accuracy requirements

- Inter-frequency RSRQ accuracy requirements

- Intra-frequency SINR accuracy requirements

- Inter-frequency SINR accuracy requirements

- L1-RSRP accuracy requirements

Observation 1: The requirements defined for FR2-NTN apply for VSAT working in FR1-NTN including accuracy requirements.

Proposal 2: The legacy measurement accuracy requirements for Ka band NTN shall be reused for Ku band FR1-NTN and FR2-NTN with different numerologies agreed in core part..

Proposal 3:

For how to capture the measurement accuracy requirements, RAN4 shall add FR1 numerology in FR2-NTN accuracy requirements table.

For where to capture the measurement accuracy requirements, RAN4 shall use the existing measurement accuracy requirements for FR2-NTN instead of introducing new clause.

2.1.2 Test principle and setups

We observe that clause A.3.36 is test principle and setups refer to Satellite access. However, FR1-NTN and FR2-NTN have both been defined. To our understanding, only FR2-NTN shall be a reference.

Proposal 4: For test principle and setups, the testing related to satellite access defined in A.3.36 with FR2 can be a reference.

2.1.3 Test configuration and parameters for Ku band NTN

We do think the legacy FR2-NTN testing procedure and test configuration could be the baseline for Ku abnd NTN. We shall consider that how to capture different SCS in one test configuration table or different test configuration tables.

Option 1: If add different SCS in one test configuration table, if mobile VSAT UE supports both GSO and NGSO, it is at least to pass two test config with FR1 numerology and FR numerology, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

Option 2: If use different test configuration tables, if mobile VSAT UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

Proposal 5: For test configuration,

  - Option 1: If add different SCS in one test configuration table, if mobile VSAT UE supports both GSO and NGSO, it is at least to pass two test config with FR1 numerology and FR numerology, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

  - Option 2: If use different test configuration tables, if mobile VSAT UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

Conclusion

In this paper we provided our views on RRM performance requirements on Ku band NTN.

Proposal 1: The following measurement accuracy requirements shall be considered:

- Intra-frequency RSRP accuracy requirements

- Inter-frequency RSRP accuracy requirements

- Intra-frequency RSRQ accuracy requirements

- Inter-frequency RSRQ accuracy requirements

- Intra-frequency SINR accuracy requirements

- Inter-frequency SINR accuracy requirements

- L1-RSRP accuracy requirements

Observation 1: The requirements defined for FR2-NTN apply for VSAT working in FR1-NTN including accuracy requirements.

Proposal 2: The legacy measurement accuracy requirements for Ka band NTN shall be reused for Ku band FR1-NTN and FR2-NTN with different numerologies agreed in core part..

Proposal 3:

For how to capture the measurement accuracy requirements, RAN4 shall add FR1 numerology in FR2-NTN accuracy requirements table.

For where to capture the measurement accuracy requirements, RAN4 shall use the existing measurement accuracy requirements for FR2-NTN instead of introducing new clause.

Proposal 4: For test principle and setups, the testing related to satellite access defined in A.3.36 with FR2 can be a reference.

Proposal 5: For test configuration,

  - Option 1: If add different SCS in one test configuration table, if mobile VSAT UE supports both GSO and NGSO, it is at least to pass two test config with FR1 numerology and FR numerology, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

  - Option 2: If use different test configuration tables, if mobile VSAT UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

- 4 References

- TS38.133