# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514096

# Prague, Czech, 13th – 17th October, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for Ku band NTN

Agenda item:	4.2.7.4

Document for:	Approval

Introduction

At RAN #107 meeting the revised WI“Introduction of Ku bands for NR NTN”[1] was approved. In the WID, Ku bands have been introduced for NR NTN with the mobile VSAT UE. In #116 meeting we will specify the remaining issues on the topics. In current WID description, RAN4’s work could be seen as below:

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

Discussion

  - 2.1 Soft satellite switching with re-sync

In the last meeting, RAN4 has discussed the satellite switch delay requirements including hard satellite switch and soft satellite switch, the agreements could be seen as below:

Issue 1-2-1: Hard satellite switching with re-sync Agreement: - The components in hard satellite switching delay requirements refer to legacy hard satellite switching delay and inter satellite handover delay. Issue 1-2-2: soft satellite switching with re-sync Agreement: - Agree to introduce RRM requirements for soft satellite switching with re-sync for the VSAT UE with electronic beam steering: Tsoft_switch = max(t-service-t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing - The requirements in this clause do not apply if the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-ServiceStart-r18 [and ssb-TimeOffset-r18] is acquired by the UE for the first time to the first slot corresponding to t-ServiceStart-r18 is is less than [1] s. - Further refinement on the wording is not precluced. - Further discussion on the value of [1] is not precluded. - Extend the period of the scheduling restriction by two sided TRX_switch, TRX_switch = 1 symbol.

We observe that RAN4 still has two points to check. The first one is whether ssb-TimeOffset-r18 shall be added in additional condition, the second one is whether the preparing time 1s is reasonable. First of all, we know that the additional condition is proposed since the company deems that VSAT has a large number of antenna elements, there are also a vast number of beamforming weights. The UE typically does not pre-load the beamforming weights for all neighbor satellites into memory, it only loads the weights for the target satellite after receiving the switching indication. From our perspective, we would like to study them one by one.

- Whether to preclude [ssb-TimeOffset-r18]

- We do think the parameter ssb-TimeOffset-r18 shall be added. We could observe the concrete details in TS38.331 as below:

SatSwitchWithReSync field descriptions ssb-TimeOffset Indicates the time offset of the SSB from target satellite at its uplink time synchronization reference point with respect to the SSB from source satellite at its uplink time synchronization reference point. It is given in number of subframes. t-ServiceStart Indicates the time information on when the target satellite is going to start serving the area currently covered by the serving satellite. The field indicates a time in multiples of 10ms after 00:00:00 on Gregorian calendar date 1st January 1900 (midnight between Sunday, December 31, 1899, and Monday, January 1, 1900). The exact start time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-ServiceStart is the uplink time synchronization reference point of the serving satellite.

- t-ServiceStart-r18 informs the UE when to start switching to the target satellite, whereas ssb-TimeOffset-r18 tells the UE where to find the target satellite's synchronization signal block. If ssb-TimeOffset is absent, UE will not distinguish whether SSB is from serving satellite or target satellite. Consequently, we deem ssb-TimeOffset is necessary.

- Less than [1s]

For this time, we do have strong view. Actually, this time mainly refer to beamforming weight preparing time including storage media access and margin. To our understanding, 1s could be the baseline.

Observation 1: The time mainly refer to beamforming weight preparing time including storage media access and margin.

Observation 2: t-ServiceStart-r18 informs the UE when to start switching to the target satellite, whereas ssb-TimeOffset-r18 tells the UE where to find the target satellite's synchronization signal block.

Proposal 1: ssb-TimeOffset is necessary and 1s could be the baseline.

  - 2.2 Terminology update

In previous meeting, RF session has already defined the new frequency range for Ku band NTN. We confirm the range of FR2 is extended and starting from 10GHz. The range of FR1 is also extended. In previous meeting, we only confirmed FR1 terminology as below ans we shall complete the whole story:

Agreement: - For VSAT UE in Ku-band with FR1-NTN numerology, use the terminology of ‘VSAT UE in FR1-NTN’.

Proposal 2: For Ku-band in FR2-NTN numerology, not to change the existing terminologies: ‘VSAT UE in FR2-NTN’ and ‘FR2-NTN’.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR Ku band NTN enhancement.

Observation 1: The time mainly refer to beamforming weight preparing time including storage media access and margin.

Observation 2: t-ServiceStart-r18 informs the UE when to start switching to the target satellite, whereas ssb-TimeOffset-r18 tells the UE where to find the target satellite's synchronization signal block.

Proposal 1: ssb-TimeOffset is necessary and 1s could be the baseline.

Proposal 2: For Ku-band in FR2-NTN numerology, not to change the existing terminologies: ‘VSAT UE in FR2-NTN’ and ‘FR2-NTN’.

- References

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN.

- R4-2512121, WF on RRM requirements for NR_NTN_Ku_bands, Apple.