3GPP TSG-RAN WG4 Meeting # 116bis 	    R4-2514118

Prague, CZ, 13 – 17 October 2025

Agenda Item:	6.11.6

Source: 	Ericsson

Title:	On testing of UE-based positioning case 1

Document for:	Approval

# Introduction

This contribution discusses the performance requirements for UE-based positioning case 1 in the revised WID for AI/ML for NR air interface [1].

# Discussion

In UE-based positioning case 1 the AI/ML model is deployed at the UE to support the AI/ML based positioning and concerns the scenario where the UE is configured by the network node to infer its location and report it to the network. To ensure that the inferred location information is reported by the UE to the network in a timely manner, reporting delay requirement for UE-based positioning case 1 was agreed to be defined in RAN4#116. The agreement is copied below for reference and the aspects related to the reporting delay requirement are discussed in detail in our companion paper R4-2514117 [2]. To validate if this requirement can be met by the UE a test case needs to be specified by RAN4.

Agreement from RAN4#116 on reporting delay requirement for UE-based positioning case 1 [3]:

- Take the framework of the existing reporting delay requirements

- Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report

- Check until next meeting on the number of samples needed for the measurements

Observation 1: To ensure that the inferred location information is reported by UE to the network in a timely manner, reporting delay requirement for UE-based positioning case 1 was agreed to be defined in RAN4#116.

To define the test case for UE-based positioning case 1 in FR1, test configurations in Table A.6.6.12.1.1-1, general test parameters in Table A.6.6.12.1.1-2, and cell specific test parameters in Table A.6.6.12.1.1-3, for NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in the existing specification can be reused. And to define the test case for UE-based positioning case 1 in FR2, test configurations in Table A.7.6.9.1.1-1, general test parameters in Table A.7.6.9.1.1-2, and cell specific test parameters in Table A.7.6.9.1.1-3, for NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in the existing specification can be reused. The number of cells in the tests shall however be updated to 4 and the configuration that are not relevant for UE-based positioning case 1 in Table A.6.6.12.1.1-2, Table A.6.6.12.1.1-3, Table A.7.6.9.1.1-2, and Table A.7.6.9.1.1-3 shall be removed. It should also be noted that depending on the signaling designed by RAN2, new parameters that are relevant for testing need to be considered. Moreover, TDL-A and TDL-C channel models shall be considered for FR1 and FR2 testing respectively.

Proposal 1: RAN4 to define two test cases for testing UE-based positioning case 1 reporting delay requirement. One test case for FR1 and another for FR2.

Proposal 2: 4 cell setup is considered for testing UE-based positioning case 1 reporting delay requirement in FR1 and FR2.

Proposal 3: To define the test case for UE-based positioning case 1 in FR1, test configurations in Table A.6.6.12.1.1-1, general test parameters in Table A.6.6.12.1.1-2, and cell specific test parameters in Table A.6.6.12.1.1-3, for NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in the existing specification is reused as a baseline keeping the following aspects in mind:

- configuration that are not relevant for UE-based positioning case 1 in Table A.6.6.12.1.1-2 and Table A.6.6.12.1.1-3 shall be removed.

- depending on the signalling designed by RAN2, new parameters (not existing in A.6.6.12.1.1-2 and Table A.6.6.12.1.1-3) may need to be considered.

Proposal 4: To define the test case for UE-based positioning case 1 in FR2, test configurations in Table A.7.6.9.1.1-1, general test parameters in Table A.7.6.9.1.1-2, and cell specific test parameters in Table A.7.6.9.1-3, for NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in the existing specification is reused as a baseline keeping the following aspects in mind:

- configuration that are not relevant for UE-based positioning case 1 in Table A.7.6.9.1.1-2 and Table A.7.6.9.1.1-3 shall be removed.

- depending on the signalling designed by RAN2, new parameters (not existing in A.7.6.9.1.1-2 and Table A.7.6.9.1.1-3) may need to be considered.

Proposal 5: FR1 testing for UE-based positioning case 1 reporting delay requirement is done under TDL-A propagation condition.

Proposal 6: FR2 testing for UE-based positioning case 1 reporting delay requirement is done under TDL-C propagation condition.

# Summary

In this contribution, testing of UE-based positioning case 1 is discussed. The observations and proposals below summarize the discussion presented in this paper.

Observation 1: To ensure that the inferred location information is reported by UE to the network in a timely manner, reporting delay requirement for UE-based positioning case 1 was agreed to be defined in RAN4#116.

Proposal 1: RAN4 to define two test cases for testing UE-based positioning case 1 reporting delay requirement. One test case for FR1 and another for FR2.

Proposal 2: 4 cell setup is considered for testing UE-based positioning case 1 reporting delay requirement in FR1 and FR2.

Proposal 3: To define the test case for UE-based positioning case 1 in FR1, test configurations in Table A.6.6.12.1.1-1, general test parameters in Table A.6.6.12.1.1-2, and cell specific test parameters in Table A.6.6.12.1.1-3, for NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in the existing specification is reused as a baseline keeping the following aspects in mind:

- configuration that are not relevant for UE-based positioning case 1 in Table A.6.6.12.1.1-2 and Table A.6.6.12.1.1-3 shall be removed.

- depending on the signalling designed by RAN2, new parameters (not existing in A.6.6.12.1.1-2 and Table A.6.6.12.1.1-3) may need to be considered.

Proposal 4: To define the test case for UE-based positioning case 1 in FR1, test configurations in Table A.7.6.9.1.1-1, general test parameters in Table A.7.6.9.1.1-2, and cell specific test parameters in Table A.7.6.9.1-3, for NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in the existing specification is reused as a baseline keeping the following aspects in mind:

- configuration that are not relevant for UE-based positioning case 1 in Table A.7.6.9.1.1-2 and Table A.7.6.9.1.1-3 shall be removed.

- depending on the signalling designed by RAN2, new parameters (not existing in A.7.6.9.1.1-2 and Table A.7.6.9.1.1-3) may need to be considered.

Proposal 5: FR1 testing for UE-based positioning case 1 reporting delay requirement is done under TDL-A propagation condition.

Proposal 6: FR2 testing for UE-based positioning case 1 reporting delay requirement is done under TDL-C propagation condition.

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

- R4-2514117, On core requirement for AI/ML based positioning, Ericsson.

- R4-2511890, WF on NR_AIML_air_part1, Qualcomm Incorporated.