3GPP TSG-RAN WG4 Meeting #116bis	R4-2514121

Prague, CZ, 13 – 17 October 2025

Agenda Item:	7.12.3

Source:	Ericsson

Title:	On requirements for RRM measurement prediction

Document for:	Approval

# Introduction

The WID for AI/ML aided mobility concerns scenarios where the AI/ML model for RRM measurement prediction is deployed at UE or at the network node. From the scenario point of view, temporal domain prediction case A, temporal domain prediction case B, and frequency domain prediction are within the scope of the WI. From the RRM measurement prediction point of view cell level L3-RSRP measurement for UE sided model and beam level and cell level L3-RSRP measurement for network sided model are within the scope of the WI. Moreover, LCM of the model deployed at the UE by the network node, data collection for UE sided and network sided model are also within the WI scope. For all these aspects, corresponding requirement needs to be specified by RAN4. Though both UE sided, and network sided models are within the WI scope, the discussions in the sections to follow focus on the requirements for UE sided model as per the agenda item set by RAN4 chair.

# Discussion

## Scenarios

### Temporal domain prediction

Temporal domain prediction is one of the use cases for RRM measurement prediction for mobility purposes. Flavors of temporal domain prediction that are within the scope of Rel. 20 WI are shown in Figure 1and Figure 2. In temporal domain prediction case-A, the UE performs multiple measurements within skipping any measurement occasion within observation window (OW) to predict RRM measurements to be valid for the time instances within the prediction window (PW). Different to this, in temporal domain prediction case-B the UE may skip performing measurements on some of the measurement occasion within the OW to predict RRM measurements to be valid for the time instances within the PW.

Figure 1. Temporal domain prediction case-A.

From the core requirement point of view, for the scenario where the number of measurement occasions within OW and the number of time instances for which the measurements are to be predicted by the UE within the PW remains same, temporal domain prediction case-B becomes one of the special cases of temporal domain prediction case-A. In this regard, from the core requirement point of view there is no need to differentiate between temporal domain prediction case-A and temporal domain prediction case-B.

Figure 2. Temporal domain prediction case-B.

From the core requirement point of view there is no need to differentiate between temporal domain prediction case-A and temporal domain prediction case-B.

### Frequency domain prediction

Frequency domain prediction of RRM measurement for mobility is also within the scope of WI and concerns a scenario where the measurement performed on one cell is used by UE to predict RRM measurement for another cell by the UE. An extension to the frequency domain prediction is the scenario where the UE performs measurement on a group of cells to predict RRM measurements for another group of cells. The measured and predicted cells in frequency domain prediction scenario are considered co-located.

According to the WID, frequency domain in general is a part of the Rel. 20 WI. This in our understanding, means that both cluster-based and non-cluster-based approaches are within the WI scope and shall be considered in RAN4 discussions. If a prioritization needs to be done to streamline the discussion, non-cluster-based approach shall be prioritized over cluster-based approach.

RAN4 to identify impact on RAN4 specification by non-cluster-based approach and cluster-based approach for frequency domain prediction. If a prioritization needs to be done to streamline the discussion, non-cluster-based approach shall be prioritized over cluster-based approach.

## Performance monitoring

For performance monitoring of UE-sided model, the WID highlights two key aspects. First, the model related choices for mobility purposes can be up to the UE’s implementation. Second, the performance monitoring of AI/ML model at UE can be performed at the network side. For performance monitoring to be performed at the network, performance metric reported by the UE to the network is vital. Given that the model related choices can be up to the UE’s implementation, determining the right performance metric to be reported to network, reported metric arrive in a timely manner to the network, and ensuring that the metric reported by different implementation choices made by UE vendors are consistent with each fall within the RAN4 scope. Therefore, RAN4 should identify the performance metric to be reported for performance monitoring, specify delay requirement for performance metric reporting, and discuss the accuracy of the reported metric during the WI. The accuracy of the reported metric can be defined in the performance part of the WI, but the other aspects highlighted above can already be discussed as a part of the core requirement.

For UE sided model for AI/ML mobility, RAN4 to identify performance metric to be reported for performance monitoring at network side.

RAN4 to define delay requirement for performance metric reporting for performance monitoring of UE sided model at the network node.

## Data collection

Data collection is another important aspect when it comes to training or retraining the model deployed at UE. To perform data collection UE may need some assistance data from the network. Depending on the assistance data and data collection configuration provisioned by the network UE can perform the data collection for which corresponding requirement needs to be specified. One of the important aspects that needs to be considered for data collection is the impact of data collection on the any ongoing other RRM procedures at UE. When UE is configured to perform other RRM measurements, data collection procedure is down prioritized by the UE.

Depending on the assistance data and data collection configuration provisioned by the network UE can perform the data collection for which corresponding requirement needs to be specified.

When UE is configured to perform other RRM measurements, data collection procedure is down prioritized by the UE.

# Summary

Aspects related to UE-sided model for AI/ML aided mobility are discussed in this contribution and the following are proposed.

Proposal 1	From the core requirement point of view there is no need to differentiate between temporal domain prediction case-A and temporal domain prediction case-B.

Proposal 2	RAN4 to identify impact on RAN4 specification by non-cluster-based approach and cluster-based approach for frequency domain prediction. If a prioritization needs to be done to streamline the discussion, non-cluster-based approach shall be prioritized over cluster-based approach.

Proposal 3	For UE sided model for AI/ML mobility, RAN4 to identify performance metric to be reported for performance monitoring at network side.

Proposal 4	RAN4 to define delay requirement for performance metric reporting for performance monitoring of UE sided model at the network node.

Proposal 5	Depending on the assistance data and data collection configuration provisioned by the network UE can perform the data collection for which corresponding requirement needs to be specified.

Proposal 6	When UE is configured to perform other RRM measurements, data collection procedure is down prioritized by the UE.

# References

RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital.