3GPP TSG-RAN WG4 Meeting #116bis	R4-2514127

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	Discussion on UE demodulation performance and CSI reporting requirements for MIMO_Ph5

Agenda item:	6.12.3

Document for:	Discussion

Introduction

In RAN4#116 good progress was made with relation to enabling companies providing simulation results for alignment for 128 port extension.

In addition, the question related to introducing reporting requirements for the new CJT reporting was discussed without agreements reached.

In the following we will provide Nokia’s view on the remaining topics and provide our observations and proposals were relevant.

UE demodulation/CSI reporting performance requirements

General scope

Clarify if PMI reporting requirements should be specified for eTypeII-r19

In RAN4#116, no decision was made on whether to introduce reporting requirements for the eTypeII-r19 codebook [1]:

Issue 1-1-1: Clarify if PMI reporting requirements should be specified for eTypeII-r19 Way forward: - FFS on RAN4 should specify PMI reporting requirements for eTypeII-r19 - Interested companies are welcomed to provide simulation results for next meeting

As we discussed in our previous contribution [3], the codebook structures for the extended eTypeII codebook and the eTypeII Doppler codebook have not been modified.  The extensions to these codebooks, as required by the work item objective, have been implemented by adding support for larger port counts without structural changes to the codebooks.  Therefore, the methods used by UEs to implement the extended Type II codebook search will likely be extended versions of algorithms used for the legacy codebooks.  
Also, legacy codebooks are prerequisite features for the extended codebooks in the UE capabilities being developed for Rel-19.  Introducing additional requirements for the extended Type II codebooks would test the extended search algorithms as well as extended prediction algorithms for the eTypeII Doppler codebook.  In addition, requirements would test the increased computational complexity due to the additional antenna ports or areas where the implementation is modified, rather than simply extended, to control the computational complexity.  For the extended eTypeII codebook, we prefer to have a requirement to address these issues.  We consider the eTypeII Doppler codebook in the next section.

Introducing requirements for the extended Type II codebooks would test the increased computational complexity and any implementation modifications introduced to control this complexity.

Introduce PMI reporting requirements for eTypeII-r19.

Clarify if PMI reporting requirements should be specified for typeII-Doppler-r19

After RAN4#116 it was still open if PMI reporting requirements should be introduced for typeII-Doppler-r19 (see [1]):

Issue 1-1-2: Clarify if PMI reporting requirements should be specified for typeII-Doppler-r19 Way forward: - FFS on RAN4 should specify PMI reporting requirements for eTypeII-Doppler-r19

The structures of the eTypeII and eTypeII Doppler codebooks are similar especially when N4=1 in the Doppler codebook, plus N4=1 is the only case with requirements in the legacy codebook.  Assuming that N4=1 would be the only case considered for the Rel-19 Doppler codebook, requirements for both of these Rel-19 TypeII codebooks will be testing similar mechanisms for deriving the codebook element, except for the prediction algorithm and the additional CSI-RS overhead required for the Doppler codebook.  Satisfying a legacy Doppler requirement and a Rel-19 eTypeII requirement should be sufficient for the eTypeII Doppler codebook, though we understand that the Rel-19 eTypeII codebook is not a prerequisite for supporting the eTypeII Doppler codebook.  Therefore, we propose to only introduce requirements for the eTypeII codebook.

Do not introduce PMI reporting requirements for typeII-Doppler-r19.

Clarify if CSI reporting requirements should be specified for UE reporting enhancements for CJT

In RAN4#116 Nokia provided a clearer view and possible way forward to the question of introduction of CSI reporting requirements. The issue was not further discussed online and is therefore still open for RAN4#116bis with the following WF (see [1]):

Issue 1-1-5: Clarify if CSI reporting requirements should be specified for UE reporting enhancements for CJT Proposals: For new CSI reporting for time/frequency/phase offsets: - Option 1: RAN4 should not introduce CSI reporting requirements for UE reporting enhancements for CJT - Option 2: RAN4 to further discuss the necessary details for a new test framework of performance requirements for the CJT calibration offsets, i.e., TO, CFO and PO. Part of the work would be to determine whether random PMI can be used as viable/reliable option, or the follow PMI is required during the CJT test. For PMI reporting: - Option 1: Further discussion on introducing new PMI reporting test for CJTC Dd and Rel-18 eType-II CJT CSI reports with delay offset compensation. Way forward: - Encourage supporters for introducing CJT related CSI reporting test requirements to provide testable test setup.

Based on what was discussed offline in the RAN4#116 meeting, we keep our view from RAN4#116 (Option 2) to further discuss a new RAN4 test framework of performance requirements for the CJT calibration offsets. On the other hand, we are, in the interest of saving time, open to reduce the scope of our initial proposal to focus only on TO and CFO, which could be tested with an FDD setup. PO may make the things a bit more complicated because also a TDD setup would be needed. Therefore, we suggest PO should not be considered at this point to avoid the additional needs of a TDD a setup including UL signals for channel acquisition (i.e., SRS).

Therefore, we will still focus on TO and CFO, which is also in line with the current plan for RRM where also PO is not further discussed for similar reasons (i.e., additional complications of the test, time constraints, etc).

It is however our view, that the work developed in RRM has a limited scope primarily focused on the accuracy of the offset estimation. The effect of the offset compensation in PDSCH cannot be inferred from the quality of the CJT offset estimates but it requires the actual evaluation of the impact on the throughput performance. That is why this work in RAN4 Demod is important.

TO and CFO may be both studied with a FDD setup in a reasonable time in the context of RAN4 Demod. In contrast, PO brings further complications because it needs a special TDD setup which may extend the necessary time for a full study of all the CJT calibration features.
The work carried out in RRM is focused on TO and CFO, whereas PO is now out of scope. The work from RRM with TO and CFO only focuses on the CJT offset estimates accuracy. Demod however, can go further by properly studying and evaluating the impact of the CJT offset compensation using the usual way of determining the gain achieved when following the UE CSI CJT reporting.

Regarding the proposal about PMI reporting requirements as an extension of CJT Rel-18 eType II with CJT Dd (i.e., CJT TO reporting), this shall not be considered as a typical PMI requirements test but is instead a new type of test, which require not only PMI but also CJT offsets reporting as part of the test. In addition, both CJT precoding and offsets need to be properly applied for correct PDSCH transmission. An extended justification of the latter is provided in the next subsection.

The test to be proposed cannot be regarded only as an extension of PMI requirements for CJT eType II but as new different type of test dealing with CJT PMI and calibration offsets reporting for PDSCH compensation.

In addition, we think that we may make a more solid test framework if TO and CFO are both considered, not only TO as in the previously mentioned PMI requirements proposal. This would require a bit more effort, but it is feasible under a same framework for CJT offset performance requirements. This test may use follow PMI and/or random PMI if desired, however it is worth noticing that using follow PMI allows also to test the PMI requirements as well as CJT calibration requirements in the same framework.
From our previous simulation results provided in RAN4#116, we have proven that having tests with random and/or follow PMI is for both feasible. Proposed simulation parameters are included in the annexes of this document.

If follow PMI is used the new proposed test will cover PMI requirements in addition to the CJT calibration offset compensation in the same framework.

Motivation for a new type of test instead of PMI requirement test

When the offset and CSI report are measured and calculated at the UE, there are some specific details that must be highlighted for the CJT Dd + Rel-18 CJT eType II case. In this case the 3GPP specification TS38214 for Rel-19 [4] contemplates the linkage of the CJT TO measurements with CJT eType II PMI. This means that the CJT CSI report is calculated with a pre-compensation of the CSI-RS resources measured at the UE side in order to calculate the CSI report. There are two cases:

- Jointly triggered case, which happens when the NW triggers both the CSI calculation and the CJT delay offset measurement at the same time. Then UE measures the delay offset and compensates it in the same transmission occasion of the CSI-RS received to be used for CJT eType II calculation.

- Separately triggered case, the CJT delay offset measurement happens in a time occasion before to the DCI triggering of the CJT CSI report. However, still the CSI report is compensated with the delay offset measurement even if it was not measured at the same time.

In any of the cases here, internally the UE needs to pre-compensate the CSI-RS for CJT eType II considering the effect of TO. This may at first glimpse justify an extension of the PMI requirements assuming the PMI calculation for Rel-18 has changed. However, this is just one aspect of the full picture. The full test requires the following components:

- PMI calculation and reporting.

- Offset calculation and reporting

- precoding based on PMI at PDSCH

- Offset compensation at PDSCH.

As seen, PMI calculation/reporting must come accompanied by several other components that make of this test something new. Moreover, focusing only on TO with CJT eType II may bring an incomplete framework. For that reason, we propose using the same framework for both TO and CFO. Any pre-compensation aspects at the UE as those mentioned for TO are not the main key aspect, but the full procedure that ranges from PMI and offsets reporting up to the CJT precoding and offset compensation at the NW side for PDSCH.

The PMI requirement tests for CJT Dd + Rel-18 CJT eType II are justified at first glimpse based on the pre-compensation of the CSI-RS for PMI calculation for jointly and separately triggered CJT Dd + Rel-18 CJT eType II cases. However, the full test includes more than that, namely PMI and CJT offsets calculation and reporting, as well as CJT precoding based on PMI and offset compensation at PDSCH. For that reason, we are having a new test proposal in this case to be considered.

Introduce a new test for CJT calibration offset performance and create a new subsection in TS38-101-4. The test shall be focused on TO and CFO with a FDD setup.

    - Test metric for CJT

Issue 1-5-3: Test metric for CJT Proposals: - Option 1: Decide the type of CSI acquisition (follow PMI, random PMI, UL CSI acquisition) before deciding the type of baseline used for the test metric. - Other options are not precluded

Regarding the utilization of follow PMI and random PMI, we are fine to use any of those two methods. However, we prefer to use the follow PMI, because it provides a test that covers not only the CJT calibration offset performance test but also is indeed an extension of the PMI requirements for Rel-18 CJT eType II. If random PMI is used, we are failing to test the PMI requirements in the same framework. However, random PMI may be at the same time easier to implement and adopt in a focused framework for CJT calibration offset performance test, i.e., if we decide the PMI requirements should not being tested in the same test but only CJT offset compensation should be evaluated.

- As a starting point, use follow PMI when defining the new requirements.

Test assumptions for typeI-SinglePanel-r19

NZP-CSI-RS configurations for 64 CSI-RS ports

In RAN4#116 it NZP-CSI-RS configuration was still not agreed (see [1]):

Issue 1-2-6: NZP-CSI-RS configurations for 64 CSI-RS ports Proposals: - Option 1: Proposed NZP-CSI-RS configuration NZP CSI-RS; CSI-RS resource 1 and 2; CSI-RS resource Type; Aperiodic. Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); Row 17,(2, 4, 6, 8). CSI-RS resource 1; First OFDM symbol in the PRB used for CSI-RS (l0, l1); Row 17, (1, 7). CSI-RS resource 2; First OFDM symbol in the PRB used for CSI-RS (l0, l1); Row 17, (5, 12). CSI-RS resource 1 and 2; CSI-RS interval and offset; slot; Not configured. aperiodicTriggeringOffset; 0. - Other options are not precluded

Option 1 adapts the NZP-CSI-RS configuration used in the legacy 32-port requirements for 64 ports.  Resource 2 uses the 32-port configuration from the legacy requirements and resource 1 extends the configuration to 64 ports with a similar resource shifted in time and avoiding a conflict with the ZP-CSI-RS.  We support this option.

Use the configuration from the table below for NZP-CSI-RS configuration for 64 CSI-RS ports (option 1)

| NZP CSI-RS | CSI-RS resource 1 and 2 | CSI-RS resource Type |  | Aperiodic |
| --- | --- | --- | --- | --- |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) |
|  | CSI-RS resource 1 | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (1, 7) |
|  | CSI-RS resource 2 | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (5, 12) |
|  | CSI-RS resource 1 and 2 | CSI-RS interval and offset | slot | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |

Scaling factors for MIMO correlation matrices

In RAN4#116 there were several proposals for the scaling factors for the MIMO correlation matrices including a proposal to not use scaling factors at all (see [1]):

Issue 1-2-10: Scaling factors for MIMO correlation matrices Proposals: - Option 1: 0.00026 for 64 (8,4,2) × 2, 0.00050 for 64 (8,4,2) × 4, 0.00053 for 128 (16,4,2) × 2, 0.00101 for 128 (16,4,2) × 4 - Option 2: RAN4 to use scaling factor a=0.00010 for both 64 CSI-RS ports - Option 3: do not use scaling factors - Other options are not precluded

In our last RAN4 contribution [3], we proposed diagonal loading factors a to be chosen to ensure that the correlation matrices are positive semi-definite (PSD).  Our approach to determine these scaling factors included checking both the minimum eigenvalue of the matrix as well as the condition number of the matrix.  The values obtained are given in Option 1.  According to TS 38.101-4 [5], condition numbers for existing correlation matrices were determined to ensure that the matrices are PSD “after round-off to 4 digit precision.”  Here, we incorporate this round-off more carefully into our procedure for calculating the scaling factors.

The first procedure used to calculate scaling factors is shown here:

- Initialize a=0

- Calculate R=Rspat+aI1+a

- Round R to 4 decimal places as R.

- If the minimum eigenvalue of R is greater than or equal to zero and the condition number of R is less than TCond, save the value of a and stop.

- Otherwise, set a=a+δ.

where Rspat is the raw correlation matrix defined in TS 38.101-4, a is the scaling factor, I is the identify matrix, δ=10-5 is the scaling factor step size, and TCond is the condition number threshold.  The approach in our previous contribution did not include Step 3, so the termination condition in Step 4 was applied to the matrix R.  To observe the effect of the condition number threshold, we calculate the scaling factors for two different thresholds -TCond=100,000 and TCond=50,000.  The results from applying this procedure using these two thresholds will be labeled as “Eig/Condition 100,000” and “Eig/Condition 50,000”.

In addition to the above procedure, we also employ a simplified procedure where the termination condition only depends on the minimum eigenvalue of the rounded correlation matrix:

- Initialize a=0

- Calculate R=Rspat+aI1+a

- Round R to 4 decimal places as R.

- If the minimum eigenvalue of R is greater than or equal to zero, save the value of a and stop.

- Otherwise, set a=a+δ.

where Rspat is the raw correlation matrix defined in TS 38.101-4 [5], a is the scaling factor, I is the identify matrix, and δ=10-5 is the scaling factor step size.  The results from applying this simplified procedure will be labeled as “Eigenvalue”.

Table 1 below provides the resulting scaling factors for the 64 and 128 port cross-polarized antenna configurations with 2 and 4 Rx antennas.  The agreed antenna configuration for both cases are used: (8,4,2) for 64 Tx ports and (16,4,2) for 128 Tx ports.  Both high and medium correlation levels have been calculated.  We observe that the calculated scaling factors can be substantially larger when applying the condition number criterion in addition to the eigenvalue criterion.  However, the purpose of the condition number criterion is to provide better numerical stability in calculations using the correlation matrices.  The values of the scaling factor can also vary substantially depending on the selected condition number threshold.  The eigenvalue-only criterion can be considered to have a condition number threshold of infinity.

Calculated values of the scaling factor increase when the condition number criterion is applied in addition to the minimum eigenvalue criterion.

Table 1:  Proposed Scaling Factors for Antenna Correlation Matrices for More than 32 Tx Ports

| Array Type | Antenna Configuration | Correlation Level | Eig/Condition 100,000 | Eig/Condition 50,000 | Eigenvalue |
| --- | --- | --- | --- | --- | --- |
| Cross-polarized | 64 (8,4,2)×2 | High | 0.00073 | 0.00111 | 0.00032 |
| Cross-polarized | 64 (8,4,2)×4 | High | 0.00145 | 0.00215 | 0.00065 |
| Cross-polarized | 64 (8,4,2)×2 | Medium | 0.00065 | 0.00075 | 0.00032 |
| Cross-polarized | 64 (8,4,2)×4 | Medium | 0.00092 | 0.00146 | 0.00050 |
| Cross-polarized | 128 (16,4,2)×2 | High | 0.00143 | 0.00223 | 0.00057 |
| Cross-polarized | 128 (16,4,2)×4 | High | 0.00249 | 0.00398 | 0.00090 |
| Cross-polarized | 128 (16,4,2)×2 | Medium | 0.00124 | 0.00164 | 0.00059 |
| Cross-polarized | 128 (16,4,2)×4 | Medium | 0.00196 | 0.00300 | 0.00094 |

We agree that the most important condition to place on the correlation matrices is that they are PSD.  Both procedures achieve this objective.  For a measure of numerical stability, we tend to prefer the approach with a condition number threshold of 100,000.  We propose to use this approach and adopt the corresponding values given in the table above.  Values for additional antenna configurations can be determined as needed for other array types, antenna configurations, and correlation levels.

Both shown methods for determining the scaling factors (with and without the condition number criterion) achieve the primary objective of ensure that the correlation matrices are PSD, but the condition number criterion is meant to add a measure of numerical stability.

Adopt the following scaling factors for 64- and 128-port High and Medium correlation level matrices to ensure PSD matrices with a measure of numerical stability:

| Array Type | Antenna Configuration | Correlation Level | Scaling Factor |
| --- | --- | --- | --- |
| Cross-polarized | 64 (8,4,2)×2 | High | 0.00073 |
| Cross-polarized | 64 (8,4,2)×4 | High | 0.00145 |
| Cross-polarized | 64 (8,4,2)×2 | Medium | 0.00065 |
| Cross-polarized | 64 (8,4,2)×4 | Medium | 0.00092 |
| Cross-polarized | 128 (16,4,2)×2 | High | 0.00143 |
| Cross-polarized | 128 (16,4,2)×4 | High | 0.00249 |
| Cross-polarized | 128 (16,4,2)×2 | Medium | 0.00124 |
| Cross-polarized | 128 (16,4,2)×4 | Medium | 0.00196 |

Whether to specify requirements for 128 CSI-RS ports

In RAN4#116 it was still not decided if requirements for 128 CSI-RS ports are to be defined (see [1]):

Issue 1-2-12: Whether to specify requirements for 128 CSI-RS ports? Way forward: - If introduced, 128 ports for TDD only. Introduce applicability rules as if 128 ports cases could pass, UE can skip corresponding 64 ports test.

As proposed in our previous contribution [3], we continue to support specifying requirements for 128 CSI-RS ports.  Our reasoning focuses on demonstrating that a UE which supports 128 ports can meet the desired performance target with the increased computational complexity required to support 128 ports.  In our previous contribution, we supported developing these requirements using CSI-RS spanning two slots.  We will address this issue again in the next section.

For Rel-19 typeI-SinglePanel-r19, introduce PMI reporting requirements for 128 CSI-RS ports.

Since support for 64 CSI-RS ports is a prerequisite for supporting 128 CSI-RS ports, we also consider the possibility of applying only the 128-port requirement to a UE which supports 128 ports, avoiding the need to test both 64 and 128 ports.  The 64-port requirement can be better guaranteed in this case when there is consistency between the 64 and 128 port configurations.  We also address this issue in the next section for the CSI-RS resource configuration.

Applicability rules for 64 and 128 CSI-RS port requirements can be supported with consistency between the configurations.

Row configuration of CSI-RS resources for 128 CSI-RS ports

In RAN4#116 the row configuration discussion of CSI-RS resources for 128 CSI-RS ports provided one option (see [1]):

Issue 1-2-16: Row configuration of CSI-RS resources for 128 CSI-RS ports Proposals: - Option 1: RAN4 to use row#18 of Table 7.4.1.5.3-1 for 128 port CSI-RS transmission.

In our previous contribution [3], as noted above, we have supported configuring the 128 CSI-RS ports over two slots.  We have reconsidered this support for two reasons.  First, the need for the UE to support two-slot aggregation of the CSI-RS resources means that the requirements will not be applied to UE’s which don’t support two-slot aggregation.  Second, as indicated in the last meeting, the two-slot CSI-RS configuration with density 1 has a very large overhead.  In order to control the CSI-RS overhead and have a requirement which applies to all UEs which support 128 CSI-RS ports, we can support a single-slot configuration of the CSI-RS resources.  Rather than supporting Option 1, however, for consistency between the 64 and 128 port requirements, we propose to extend the 64-port CSI-RS configuration to 128 ports.  The proposed configuration is shown in the table below as part of our proposal.

Use following CSI-RS configuration for 128 ports, which is an extension of the 64-port configuration proposal with density 1/2:

| NZP CSI-RS | CSI-RS resources 1-4 | CSI-RS resource Type |  | Aperiodic |
| --- | --- | --- | --- | --- |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  | CSI-RS resource 1 | Density (ρ) |  | 1/2 (even) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (1, 7) |
|  | CSI-RS resource 2 | Density (ρ) |  | 1/2 (even) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (5, 12) |
|  | CSI-RS resource 3 | Density (ρ) |  | 1/2 (odd) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (1, 7) |
|  | CSI-RS resource 4 | Density (ρ) |  | 1/2 (odd) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (5, 12) |
|  | CSI-RS resources 1-4 | CSI-RS interval and offset | slot | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |

typeI-codebookMode-r19 for 128 CSI-RS ports

In RAN4#116 it was still open which Mode to use for the typeI-codebookMode-r19 for 128 CSI-IS ports (see [1]):

Issue 1-2-17: typeI-codebookMode-r19 for 128 CSI-RS ports Proposals: - Option 1: Mode A

As we have indicated in a previous contribution [6], we support prioritizing Type I codebook Mode A due to lower feedback overhead than Scheme B.  Therefore, we propose to adopt Option 1 if requirements are introduced for 128 ports.

For the codebook mode for 128 CSI-RS ports, adopt Mode A (Option 1).

Test assumptions for eTypeII-r19 (if introduced)

The following parameters were agreed in RAN4#116 as assumptions for eTypeII-r19 [1]:

| Configuration | Value |
| --- | --- |
| Test metric | Reuse the test metric for eType-II PMI reporting requirements, throughput ratio between the following PMI and random Rel-19 refined Type-I PMI Mode A |
| Duplex mode | Both FDD and TDD |
| Number of Rx antennas | Both 2Rx and 4Rx |
| Propagation channel | TDLA30-5 |
| Correlation configuration | XP-Medium |
| typeII-ri-restriction-r19 | Rank2 |
| Number of CSI-RS ports | 64 ports |
| N1 and N2 configuration for 64 CSI-RS ports | (8,4) |
| (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports | K=2 |
| MCS level | Use the same MCS value(s) as Type I codebook/ |
| numberOfPMI-SubbandsPerCQI-subband-r19 | 1 |
| paramCombination-r19 | 6 (starting point) |
| CQI/RI/PMI delay | Based on capability 2 |
| Other parameters | Reuse the test parameters of the existing Rel-16 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline |

As we have proposed above, we support introducing requirements for eTypeII-r19 based on these parameters.

We see no reason to change the agreements for the eTypeII-r19 test assumptions from RAN4#116.

Test assumptions for typeII-Doppler-r19 (if introduced)

In RAN4#116, the following proposals for the test assumptions for the typeII-Doppler-r19 codebook were presented for consideration [1]:

Issue 1-4-1: Duplex mode Way forward: - Option 1: Both FDD and TDD - Other options are not precluded Issue 1-4-2: Number of Rx antennas Way forward: - Option 1: Both 2Rx and 4Rx - Other options are not precluded Issue 1-4-3: Propagation channel Way forward: - Option 1: TDLA30-20 - Other options are not precluded Issue 1-4-4: Correlation configuration Way forward: - Option 1: XP-Medium - Other options are not precluded Issue 1-4-5: Number of CSI-RS ports Way forward: - Option 1: 64 ports - Other options are not precluded Issue 1-4-6: N1 and N2 configuration for 64 CSI-RS ports Way forward: - Option 1: (8,4) - Other options are not precluded Issue 1-4-7: (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports Way forward: - Option 1: K=2 - Other options are not precluded Issue 1-4-8: typeII-ri-restriction-r19 Way forward: - Option 1: Rank2 - Other options are not precluded Issue 1-4-9: numberOfPMI-SubbandsPerCQI-subband-r19 Way forward: - Option 1: set as 1 - Other options are not precluded Issue 1-4-10: number of configured resource groups Kdopp Way forward: - Option 1: The number of configured resource groups Kdopp set as 4 as the starting point. - Other options are not precluded Issue 1-4-11: Other test parameters Way forward: - Option 1: Reuse the test parameters of the existing Rel-18 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline. - Other options are not precluded

As we have proposed above, we do not support introducing requirements for typeII-Doppler-r19.  Nevertheless, if these requirements are introduced, we would support the parameters in the following proposal, which accepts the proposed options from RAN4#116.

If requirements for typeII-Doppler-r19 are decided to be introduced, use the following configuration as starting point:

| Configuration | Value |
| --- | --- |
| Duplex mode | Both FDD and TDD |
| Number of Rx antennas | Both 2Rx and 4Rx |
| Propagation channel | TDLA30-20 |
| Correlation configuration | XP-Medium |
| Number of CSI-RS ports | 64 ports |
| N1 and N2 configuration for 64 CSI-RS ports | (8,4) |
| (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports | K=2 |
| typeII-ri-restriction-r19 | Rank2 |
| numberOfPMI-SubbandsPerCQI-subband-r19 | 1 |
| number of configured resource groups Kdopp | 4 |
| Other parameters | Reuse the test parameters of the existing Rel-18 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline. |

Test assumptions for CJT calibration (if introduced)

How to define CJT calibration offset reporting requirements

In RAN4#116 we provided a proposal on how to define CJT calibration offset reporting requirements which was captured in the WF (see [1])

Issue 1-5-1: How to define CJT calibration offset reporting requirements Proposals: - Option 1: As a starting point consider testcases with FDD. Consider follow PMI as first priority and include random PMI in the analysis as second priority. - Other options are not precluded

The steps needed for the FDD case of the proposed test are shown in Figure 1 and listed below. The TDD setup is only necessary for the PO testing and as mentioned we consider we must focus only on TO and CFO due to time constraints. We may consider for the test having two TRPs, one of the TRP being the anchor TRP and another TRP with an offset with respect to the anchor TRP. For completeness sake, we do not fully re-iterate our proposal from RAN4#116 here, but focus on the key aspects that we want to propose related to the FDD setup:

Step 1:	The stimuli generator is initialized with input conditions. For example, the range in which offsets are going to be measured and the way values are going to be swept, for instance, using a ramp sequence.

Step 2:	A first set of DL reference signals (e.g., TRS) is used to measure the offset. For that, the DL reference signals of one of the TRPs (i.e., the one with an offset with respect to the anchor) are modified by adding a given offset value on the top. The offset could be represented generically by a term Oat that multiplies the reference signal rt to get as a result  rnct=rt*Oat.

Step 3:	UE measures the offset and reports the offset based on a defined number of quantization steps and quantization range (initial configuration steps for the UE reporting are omitted). We may call the reported offset as Oqt.

Step 4:	A second set of DL reference signals (i.e., CSI-RS) are used to calculate the CSI for the plurality of CSI-RS ports of the TRP set (i.e., for the 2 TRPs). Notice that in this second set of reference signals, for the TRP that is not the anchor or reference TRP (same one from step 2), this TRP must carry the same offset as in step 2.

Step 5:	UE reports the CSI with CJT eType II to the testing equipment for the two measured TRPs. For the specific TO case, the PMI is also pre-compensated with the offset measured in step 3. For CFO, this part is an implementation aspect.

Step 6:	Based on the received CSI report (from step 5) and the reported offset (from step 3), the testing equipment may compensate the offset and apply the precoding obtained from the CSI report on PDSCH. Compensate the offset means removing the offset by multiplying by its own conjugate. Very generically speaking, if the actual transmitted signal (without compensation) in PDSCH is rnct=rt*Oa(t) for a time index t, the compensation consists of generating a compensated signal by multiplying by the conjugate of the reported offset, i.e., Oqt*, i.e. rct=rt*Oat*Oqt*. If Oat=Oq(t) there is a perfect compensation of the offset. However, the compensation is not perfect due to quantization and measurement/estimation issues at the UE side, which may lead to some compensation error during PDSCH transmission.

Step 7:	The baseline CSI-RS reference signal for comparison is transmitted to compare with the previous case (step 6) in which PDSCH had the offset compensated based on the UE report. The baseline reference signal could be perfectly compensated (no offset), randomly compensated or no compensated at all (with an offset). From our previous results in RAN 1 # 116, we learned that the perfectly compensated case is not a good baseline as the gap with the offset compensation under test case is very little. However, we can still work with no compensation or random compensation of the CJT calibration offset as baselines.

Step 8:	UE reports the CSI based on the baseline reference signals received in step 7.

Step 9:	With the received CSI report for the baseline reference signal, the testing equipment precodes the PDSCH based only on this CSI report with the conditions of the baseline reference signal in step 7.

Step 10:	Finally, the comparison consists of comparing the PDSCH performance from the CJT offset compensated case vs. the PDSCH as configured for the baseline (i.e. non-compensated/randomly compensated offset).

Figure 1. Flowchart for the CJT offset performance test with FDD scheme (and PMI calculation).

Consider a FDD setup for the test, including PMI calculation and offset estimation at the UE, as well as PDSCH precoding and offset compensation at the testing equipment side. Baseline may be obtained from non-compensated/random compensation of the CJT offsets and their respective resulting PMI.

Whether the PMI calculation is necessary for the CJT calibration performance requirements

In RAN4#116 it was brought up if the PMI calculation is necessary for the CJT calibration performance (see [1]):

Issue 1-5-2: Whether the PMI calculation is necessary for the CJT calibration performance requirements Proposals: - Option 1: Companies to provide simulations to study the behaviour of random and follow PMI. As starting point use 2 TRPs with 8 transmission ports each, MCS level 13 (other values may be also discussed), 2Rx/4Rx antennas at the UE. Remaining PMI parameters shall be reused from the PMI requirement for Rel. 18 eType II CJT. CJT calibration parameters shall be reused from the accuracy studies carried out in RRM core. - Other options are not precluded

We provide again our simulations from our contribution for RAN4#116. At the moment, we provided some initial simulations by comparing the follow and random PMI precoding with different cases for TO, CFO with offsets being compensated/not-compensated for PDSCH. We use MCS level 13, 4 Rx antenna at the UE with transmission limited to single rank (rank 1), 2 TRP transmission each TRP with 8 transmission ports. For CSI we use Rel. 18 eType II mode 2. Other parameters correspond in this simulation to the Rel. 18 assumptions used for CJT PMI requirements.

Regarding the CJT calibration parameters they were reused from the current studies carried out in RRM core. For CFO we consider a central carrier frequency at 3.5GHz with a fixed CFO corresponding to 0.01ppm (350Hz). CFO is quantized with maximum quantization range AFO=0.02ppm and quantization resolution equal to MFO= 256 steps (8 bits). On the other hand, for TO the delay is fixed to 0.5CP = 2.345μs (microseconds). The maximum quantization range is  ATO=CP and quantization resolution equal to MTO= 256 steps.

Follow and Random PMI precoding case with separate and joint TO and CFO calibration offsets reporting:

For TO and CFO, we analysed both offsets simulated and measured separately (Figure 2 with TO and Figure 3 with CFO, respectively) and also the case for joint measurement with TO/CFO (Figure 4) was considered. We notice that both random and follow PMI when the TO and/or CFO are compensated, they have little difference with our predefined baseline in those initial simulations, i.e., upper bound with no offset(s) or ideal compensation. This result lead us to think that using the ideal case as baseline is not a good option for the designed test, as the gain gap is quite reduced. In contrast, the gain is quite clear and noticeable when then comparison is made against the non-compensated case. Therefore, the non-compensated case could be a good candidate for being the actual baseline.

On the other hand, when follow PMI is compared vs. random PMI in terms of feasibility as mechanism for CSI acquisition, we may notice that both provides gains with respect to the TO, CFO and TO/CFO non-compensated cases, then both may fit well as part of the test. In our view, we would prefer to use the follow PMI because it provides realistic throughput results. Moreover, it provides a test for the CJT offsets and PMI requirements, all in one single framework.

| (a) | (b) |
| --- | --- |

Figure 2. Follow (a) and Random (b) PMI precoding case with TO calibration offsets reporting and with/without TO compensation (TOC)

| (a) | (b) |
| --- | --- |

Figure 3. Follow (a) and Random (b) PMI precoding case with CFO calibration offsets reporting and with/without FO compensation (FOC)

Figure 4. Follow (a) and Random (b) PMI precoding case with joint TO/CFO calibration offsets reporting and with/without TO/FO compensation (TOC/FOC)

- Based on our results the upper bound baseline (no offset or ideal compensation) is not a good candidate for being baseline of the CJT offset calibration reporting test. The gain gap is quite narrow with respect to the compensated case under test, for both cases with follow and random PMI. On the other hand, the comparison of the compensated case vs. non-compensated for both random and follow PMI provides a good gain gap and is a good baseline for the CJT offset calibration reporting test.

Use the non-compensated CJT offset case with resulting non-compensated follow PMI as comparison baseline.

BS demodulation performance requirements

General scope

Clarify if demodulation requirements should be specified for 3-antenna-port codebook-based UL transmissions

Issue 2-1-1: Clarify if demodulation requirements should be specified for 3-antenna-port codebook-based UL transmissions Way forward: - FFS on RAN4 should specify PUSCH demodulation requirements for 3-antenna-port non-coherence codebook-based UL transmissions - Interested companies are welcomed to provide simulation results next meeting

During RAN4#116 all companies except for an individual company felt that it would be worthwhile to define performance requirements for 3Tx UL codebooks, especially as the performance is expected to differ from 4Tx and 2Tx

During RAN4#116 the majority of companies supported defining performance requirements

RAN4 shall define performance requirements for 3Tx UL codebooks

Conclusion

The following Observations and Proposals were made:

UE demodulation/CSI reporting performance requirements

General scope

Clarify if PMI reporting requirements should be specified for eTypeII-r19

- Introducing requirements for the extended Type II codebooks would test the increased computational complexity and any implementation modifications introduced to control this complexity.

- Introduce PMI reporting requirements for eTypeII-r19.

Clarify if PMI reporting requirements should be specified for typeII-Doppler-r19

Do not introduce PMI reporting requirements for typeII-Doppler-r19.

Clarify if CSI reporting requirements should be specified for UE reporting enhancements for CJT

TO and CFO may be both studied with a FDD setup in a reasonable time in the context of RAN4 Demod. In contrast, PO brings further complications because it needs a special TDD setup which may extend the necessary time for a full study of all the CJT calibration features.
The work carried out in RRM is focused on TO and CFO, whereas PO is now out of scope. The work from RRM with TO and CFO only focuses on the CJT offset estimates accuracy. Demod however, can go further by properly studying and evaluating the impact of the CJT offset compensation using the usual way of determining the gain achieved when following the UE CSI CJT reporting.

The test to be proposed cannot be regarded only as an extension of PMI requirements for CJT eType II but as new different type of test dealing with CJT PMI and calibration offsets reporting for PDSCH compensation.

If follow PMI is used the new proposed test will cover PMI requirements in addition to the CJT calibration offset compensation in the same framework.

Motivation for a new type of test instead of PMI requirement test

The PMI requirement tests for CJT Dd + Rel-18 CJT eType II are justified at first glimpse based on the pre-compensation of the CSI-RS for PMI calculation for jointly and separately triggered CJT Dd + Rel-18 CJT eType II cases. However, the full test includes more than that, namely PMI and CJT offsets calculation and reporting, as well as CJT precoding based on PMI and offset compensation at PDSCH. For that reason, we are having a new test proposal in this case to be considered.

Introduce a new test for CJT calibration offset performance and create a new subsection in TS38-101-4. The test shall be focused on TO and CFO with a FDD setup.

Test metric for CJT

- As a starting point, use follow PMI when defining the new requirements.

Test assumptions for typeI-SinglePanel-r19

NZP-CSI-RS configurations for 64 CSI-RS ports

Use the configuration from the table below for NZP-CSI-RS configuration for 64 CSI-RS ports (option 1)

| NZP CSI-RS | CSI-RS resource 1 and 2 | CSI-RS resource Type |  | Aperiodic |
| --- | --- | --- | --- | --- |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) |
|  | CSI-RS resource 1 | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (1, 7) |
|  | CSI-RS resource 2 | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (5, 12) |
|  | CSI-RS resource 1 and 2 | CSI-RS interval and offset | slot | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |

Scaling factors for MIMO correlation matrices

Calculated values of the scaling factor increase when the condition number criterion is applied in addition to the minimum eigenvalue criterion.

Both shown methods for determining the scaling factors (with and without the condition number criterion) achieve the primary objective of ensure that the correlation matrices are PSD, but the condition number criterion is meant to add a measure of numerical stability.

Adopt the following scaling factors for 64- and 128-port High and Medium correlation level matrices to ensure PSD matrices with a measure of numerical stability:

| Array Type | Antenna Configuration | Correlation Level | Scaling Factor |
| --- | --- | --- | --- |
| Cross-polarized | 64 (8,4,2)×2 | High | 0.00073 |
| Cross-polarized | 64 (8,4,2)×4 | High | 0.00145 |
| Cross-polarized | 64 (8,4,2)×2 | Medium | 0.00065 |
| Cross-polarized | 64 (8,4,2)×4 | Medium | 0.00092 |
| Cross-polarized | 128 (16,4,2)×2 | High | 0.00143 |
| Cross-polarized | 128 (16,4,2)×4 | High | 0.00249 |
| Cross-polarized | 128 (16,4,2)×2 | Medium | 0.00124 |
| Cross-polarized | 128 (16,4,2)×4 | Medium | 0.00196 |

Whether to specify requirements for 128 CSI-RS ports

For Rel-19 typeI-SinglePanel-r19, introduce PMI reporting requirements for 128 CSI-RS ports.

Applicability rules for 64 and 128 CSI-RS port requirements can be supported with consistency between the configurations.

Row configuration of CSI-RS resources for 128 CSI-RS ports

Use the following CSI-RS configuration for 128 ports, which is an extension of the 64-port configuration proposal with density 1/2:

| NZP CSI-RS | CSI-RS resources 1-4 | CSI-RS resource Type |  | Aperiodic |
| --- | --- | --- | --- | --- |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  | CSI-RS resource 1 | Density (ρ) |  | 1/2 (even) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (1, 7) |
|  | CSI-RS resource 2 | Density (ρ) |  | 1/2 (even) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (5, 12) |
|  | CSI-RS resource 3 | Density (ρ) |  | 1/2 (odd) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (1, 7) |
|  | CSI-RS resource 4 | Density (ρ) |  | 1/2 (odd) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (5, 12) |
|  | CSI-RS resources 1-4 | CSI-RS interval and offset | slot | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |

typeI-codebookMode-r19 for 128 CSI-RS ports

For the codebook mode for 128 CSI-RS ports, adopt Mode A (Option 1).

Test assumptions for eTypeII-r19 (if introduced)

We see no reason to change the agreements for the eTypeII-r19 test assumptions from RAN4#116.

Test assumptions for typeII-Doppler-r19 (if introduced)

If requirements for typeII-Doppler-r19 are decided to be introduced, use the following configuration as starting point:

| Configuration | Value |
| --- | --- |
| Duplex mode | Both FDD and TDD |
| Number of Rx antennas | Both 2Rx and 4Rx |
| Propagation channel | TDLA30-20 |
| Correlation configuration | XP-Medium |
| Number of CSI-RS ports | 64 ports |
| N1 and N2 configuration for 64 CSI-RS ports | (8,4) |
| (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports | K=2 |
| typeII-ri-restriction-r19 | Rank2 |
| numberOfPMI-SubbandsPerCQI-subband-r19 | 1 |
| number of configured resource groups Kdopp | 4 |
| Other parameters | Reuse the test parameters of the existing Rel-18 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline. |

Test assumptions for CJT calibration (if introduced)

How to define CJT calibration offset reporting requirements

Consider a FDD setup for the test, including PMI calculation and offset estimation at the UE, as well as PDSCH precoding and offset compensation at the testing equipment side. Baseline may be obtained from non-compensated/random compensation of the CJT offsets and their respective resulting PMI.

Whether the PMI calculation is necessary for the CJT calibration performance requirements

- Based on our results the upper bound baseline (no offset or ideal compensation) is not a good candidate for being baseline of the CJT offset calibration reporting test. The gain gap is quite narrow with respect to the compensated case under test, for both cases with follow and random PMI. On the other hand, the comparison of the compensated case vs. non-compensated for both random and follow PMI provides a good gain gap and is a good baseline for the CJT offset calibration reporting test.

Use the non-compensated CJT offset case with resulting non-compensated follow PMI as comparison baseline.

BS demodulation performance requirements

General scope

Clarify if demodulation requirements should be specified for 3-antenna-port codebook-based UL transmissions

During RAN4#116 the majority of companies supported defining performance requirements

RAN4 shall define performance requirements for 3Tx UL codebooks

- Annex

Following simulation parameters in Table 2 were used for the CJT calibration offset simulations presented here based on current RRM core work for TO/CFO and Rel. 18 CJT eType-II PMI requirements parameters. Some observations were added in the table regarding other parameters that could be also tested in further simulations.

Table 2. Simulation assumptions used for TO/CFO reporting simulations.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | FDD, 15kHz SCS, TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| UE antenna configuration | 1X4 (1X2 could be also considered) |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| CSI CJT codebook | Rel-18 eType II mode 2 |
| Number of CSI-RS ports per TRP | 8 |
| Beams configuration | {4,4} |
| Beta, Pv | ½, ¼ |
| Rank | 1 (in further simulations rank=2 could be considered) |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP |
| Number of samples (TRS resource sets) per report | 1 sample (i.e. 1 report for each TRS resource set measurement) |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (CSI-RS for tracking) |
| CSI-RS config per TRP | 4 TRSs per TRP |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |

- References

- R4-2512527,  Way Forward for [116][327] NR_MIMO_demod

- R4-2508627,  Way Forward for [115][329] NR_MIMO_demod

- R4-2510425, “Discussion on UE demodulation performance and CSI reporting requirements for MIMO_Ph5,”, Nokia, RAN4#116, Bengaluru, India, 25-29 August 2025.

- TS38.214, NR; Physical layer procedures for data

- TS 38.101-4, NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

- R4-2506442, “Discussion on UE demodulation performance and CSI reporting requirements for MIMO_Ph5,”, Nokia, RAN4#115, Valletta, Malta, 19-23 May 2025.