3GPP TSG-RAN WG4 Meeting #116bis	R4-2514128

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	Discussion on UE demodulation performance requirements for LPWUS

Agenda item:	6.16.4

Document for:	Discussion

Introduction

In RAN4#116 several agreements were made enabling companies to provide initial simulation results. Still open is the issues about introducing requirements for FAR as well as the question about the need to introduce a dedicated test mode.

Regarding introduction of a dedicated test mode, it was asked online that companies who did not support introducing a dedicated test mode would provide evidence into why it is not required.

In the following we will provide our view on the remaining issues as well as our observations and proposals for how to continue.

Discussion

Testability Aspects

Whether to define a test mode for LP-WUS/WUR

In RAN4#116 it was discussed, if should be defined a test mode for LP-WUS/WUR (see [1]):

Issue 1-1-1: Whether to define a test mode for LP-WUS/WUR - Agreement: Interested companies can provide an analysis of the expected test durations for the different modes next meeting. Further discuss if a test mode is required based on the provided analysis.

As indicated in RAN4#116 we have no specific interest to force introducing a dedicated test mode without reasonable arguments to do so. We do see it is possible to run the tests without a dedicated testmode, however we have concerns that the test duration will be quite long. This is especially the case for the idle mode test which needs to be defined to ensure UEs only supporting idle mode LPWUS is also tested.

As discussed in RAN4#116 it will be up to the companies that is against introducing a dedicated test mode to provide further analysis into how long the test duration will be and it can then be further discussed if a dedicated test mode is required or not.

The test setup for LP-WUS/WUR can be as illustrated in Figure 1.

Figure 1 Illustration of a typical test setup for a device included LP-WUR. The downlink signal providing info to both the LP-WUR and the main radio receiver. Uplink is only connected to the main radio transmitter. For TDD it can be a single I/O bi-directional interface supporting both up-link and down-link.

Test time in idle based on msg. 1/ 3 to test equipment

To verify the performance of the LP-WUR in idle based on 1% MDR the number of LP-WUS detections/ responses should be in the range of 1000. (factor of 10x to achieve good accuracy of the MDR). The steps for responding to a received LP-WUS dedicated to the device is as follows:

- Perform decoding of the LP-WUS

- Wakeup the Main Radio (MR)

- Send MSG. 1(PRACH), receive msg. 2 for time sync, transmit msg3, establish connection

- Terminate connection

- Enter LP-WUS/WUR connection

- Goto sleep

The time for this procedure can be in the range of app. 10 sec and should be repeated app. 1000 times. This gives a test time of almost 3 hours for each test configuration. For 2 info bits (4 groups) the number of iterations should be 4x1000 and for 5 info bits (32 groups) number of iterations should be 32x1000.

The similar procedure can be followed when the device is under test for FAR. In this case a failing scenario is when the device is responding when not expected.

Assuming a FAR of max 1% as with MDR, this gives a total test time of app. 6 hour. As this test is containing both LR and MR activity, any given error can be due to other actions than the actual test purpose – test and verification of the demodulation of the LP-WUS it might be beneficial to have a test mode and thereby minimizing the test time and reduce the activity and errors there could come from MR activity.

MDR tests if done based on RF agreement can be implemented without introducing a specific test mode, however with the current RF agreement, such a test will likely take a long time. It is however RAN4 objective to define the tests but not necessarily decide how the test is implemented.

Further analyse the test time based on companies’ feedback with the focus on deciding if a test mode is to be introduced for LP-WUS/WUR testing.

Whether to define separate test cases for idle/inactive and connected modes

In RAN4#116 it was discussed if separate test cases should be defined for idle/inactive and connected modes.

Issue 1-1-3: Whether to define separate test cases for idle/inactive and connected modes - Proposals - Option 1: Define separate test cases procedure for the modulator in idle and connected modes.

The way of operation is quite different whether the device is in idle or connected mode with respect to the usages and decoding of the LP-WUS. In connected mode the length of the sequence (L) and number of info bits is assumed to be smaller than in idle mode. In idle mode the number of info bits will be high in dense areas to minimize the wakeup of larger groups of devices.

We see significant differences in usage and decoding of LP-WUS between idle and connected mode, which makes it relevant to define separate test cases. In addition, a UE might only support idle mode, hence dedicated test for idle mode is required.

Define separate test cases procedure for the modulator in idle and connected mode.

Demodulation Requirements

Whether to introduce FAR test

In RAN4#116 it was discussed if FAR tests should be introduced (see [1]):

Issue 2-1-2: Whether to introduce FAR test - Proposals - Option 1: Yes - Option 2: No

In addition, as also highlighted in RAN4#116, the following agreement was made in RAN4#111 RF session (see [3])

Agreements: - MDR is used as the metric for LP_WUR Rx RF requirements - FFS MDR values. - FAR will be defined as demodulation requirements - FFS FAR values.

Based on the agreement in RAN4 RF, it is expected by RAN4 RF that RAN4 demod will introduce demodulation requirements for FAR. In case RAN4 does not introduced requirements for FAR, it can be expected that companies will bring the issue up again in RAN4 RF as the agreement in RAN4 RF will not be fulfilled.

Since all companies already agreed that false alarm (FAR) requirements are needed as demodulation requirements in RAN4 RF session, we assume there has been no change in companies view since then.

It has already been agreed in RAN4#114 to introduce LP-WUS requirements for MDR, however MDR requirements only secures, that the UE wakes up when needed, it does not measure if the UE wakes up by a false detection which matches the UE assigned group. The FAR requirements are therefore needed to ensure, that the threshold of the LP-WUS detection is not set too low which will ensure MDR tests are passed without any failed detection of valid LP-WUS but due to the low threshold setting the UE will possible also wake up on false detection even in cases where no LP-WUS is send at all.

Introduction of LP-WUS is a very costly feature when looking at the spectrum lost to LP-WUS. It is therefore in our view critical, that the UEs are optimized to not have any/low number of FAR as well as not have too high MDR.

As the MDR level can be optimized by increasing the FAR level to 1% as is agreed assumption for MDR test in RAN4#116 [1] and thereby minimizing the missed detections with the downside of more false wakeup of the MR it is important to secure the agreed level of FAR by a dedicated test.

Issue 2-1-1: Assumption of FAR for the MDR test - Agreement: 1% FAR

Previous agreement in RAN4#111 RF session is that FAR is expected to be defined as demodulation requirements, hence no FAR requirements were introduced by RAN4 RF. In addition, it was agreed in RAN4#116 that for MDR tests the UE can assume a FAR of 1%. Based on the agreement, we expect the UE will be implemented such that FAR will be close to 1%, hence it will be needed to introduce requirements for 1% FAR to ensure the assumption not exceeded.

RAN4 shall introduce requirements for FAR for each requirement introduced for MDR. The FAR test shall run with the same parameters as the related MDR test.

Test metric for FAR test (if introduced)

In RAN4#116 the test metric for FAR test was discussed (see [1]):

Issue 2-1-3: Test metric for FAR test (if introduced) - Proposals - Option 1: 1% FAR - Option 2: 0.1% FAR

Since the agreement from RAN4#116 dictates that for MDR test it can be assumed that there is 1% FAR, we see it reasonable to also use 1% FAR as the requirement for FAR tests [1].

Issue 2-1-1: Assumption of FAR for the MDR test - Agreement: 1% FAR

Set test metric for FAR test to 1% FAR (Option 1)

If FAR tests are introduced, the overall idea with the test will be to ensure, that the UE does not wake-up in situations where no LP-WUS is sent by the gNB (this would in some situations be the majority of time in normal operation) or the group ID in the LP-WUS sent by the gNB does not belong to the UE itself (FAR based on signal). Such a wake up can only happen in the case where the UE detect a LP-WUS and at the same time decode the LP-WUS to refer to the group ID of the UE.

At least two options can be considered when discussing how to implement a FAR test:

- Introduce AWGN from the TE and monitor how often the UE wakes up by UE responding to PDCCH.

- Introduce AWGN from the TE and monitor many times the UE has woken up by UE reporting the number of LP-WUS it has detected.

- Introduce a setup where the TE send valid LP-WUS with different group identifiers randomly and monitor how often the UE wakes up by UE responding to PDCCH.

- Introduce a setup where the TE send valid LP-WUS with different group identifiers randomly and monitor how many times the UE has woken up by UE reporting the number of LP-WUS it has detected.

Carrier frequency

In RAN4#116 the following was proposed as carrier frequency as starting point for alignment (see [1]):

Issue 2-1-12: Carrier frequency - Proposals - Option 1: Use fc=900MHz for FR1 FDD and fc=2.6GHz for FR1 TDD as starting point - FFS for FR2 TDD

In our provided simulation contribution [2] we have used the proposed values from option 1 above for our FR1 results. Provided results shows SNR values which are inside acceptable ranges for the two suggested channel models for FR1.

For FR2 we have selected to use 28GHz in our simulations, which also seems to provide acceptable SNR values.

Our simulation results show that the proposed values for FR1 from RAN4#116 can be used for requirement definition. In addition, we have used 28GHz for our FR2 simulation results which also shows acceptable results for requirement definition.

Use fc=900MHz for FR1 FDD and fc=2.6GHz for FR1 TDD. Use fc=28GHz for FR2 TDD.

Channel model

In RAN4#116 two channel models were agreed for evaluation of what to use in requirement definition (see [1]).

Issue 2-1-14: Channel model - Agreement: - Consider both TDLA30-10 and TDLC300-100 for FR1 evaluation. Further discuss down-selection based on the simulation results. - Consider TDLA30-10 only for FR2

The proposed channel models are the same models as is mostly used for requirement definition. In above agreement it is also stated that further down-selection shall be based on simulation results. We do not see down-selection as resulting in only one channel model, however more in the spirit of only selecting one channel model for each requirement configuration.

We have provided simulation results in our simulation contribution [2] using the channel models from the agreement above and have the following observation.

There is a significant difference in SNR between AWGN and TDLC/TDLA channel models. In addition, there is also significant difference between TLDA and TDLC models, hence it is reasonable to define requirements for both TDLA30-10 and TDLC300-100.

For FR1 cover both TDLA30-10 and TDLC300-100 for requirement definition across the final agreed requirements. For FR2 cover TDLA30-10 for requirement definition.

Time offsets

In RAN4#116 the following offsets were agreed (see [1]):

Issue 2-1-17: Time offsets - Agreement: - T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4 - Note: The selection of time offsets will depend on the agreed LP-WUS waveform.

In our simulation setup and as can be seen in our simulation contribution [2] we have used 0.5us as time offset which have provided reasonable SNR values for 1% MDR.

For FR2 use T=0.5us as time offset.

Test to be considered for requirement definition

Based on previous sections observations we see no reason to propose additional channel models. In addition, we believe that TDLA will match best to scenarios with low number of devices and TLDC is a better match in scenarios with high number of devices. In case of high number of devices, it is likely that there is a need for 5 infobits as well as L=32, hence we see TDLC valid in such cases.

In addition, we see it reasonable to reduce down to only one value of L for each info bit selection when looking at the same receiver/detector.

This leads us to the following proposal which covers both FR1 and FR2:

Consider defining requirements using the following testcases covering both MDR and FAR:

| Test # | Freq. range | Duplex | Receiver type | Detector | Channel model | M | L | Info bits |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | FR1 | FDD | OOK4 | Envelope detector | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 1-2 |  |  |  |  |  | M=4 |  |  |
| Test 1-3 |  |  |  |  | TDLC300-100 | M=1 | L=32 | Infobits=5 |
| Test 1-4 |  |  |  |  |  | M=4 |  |  |
| Test 1-5 |  |  |  | Sequence detector | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 1-6 |  |  |  |  |  | M=4 |  |  |
| Test 1-7 |  |  |  |  | TDLC300-100 | M=1 | L=32 | Infobits=5 |
| Test 1-8 |  |  |  |  |  | M=4 |  |  |
| Test 2-1 | FR1 | TDD | OOK4 | Envelope detector | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 2-2 |  |  |  |  |  | M=4 |  |  |
| Test 2-3 |  |  |  |  | TDLC300-100 | M=1 | L=32 | Infobits=5 |
| Test 2-4 |  |  |  |  |  | M=4 |  |  |
| Test 2-5 |  |  |  | Sequence detector | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 2-6 |  |  |  |  |  | M=4 |  |  |
| Test 2-7 |  |  |  |  | TDLC300-100 | M=1 | L=32 | Infobits=5 |
| Test 2-8 |  |  |  |  |  | M=4 |  |  |
| Test 3-1 | FR1 | FDD | OFDM | N/A | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 3-2 |  |  |  |  | TDLC300-100 |  | L=32 | Infobits=5 |
| Test 4-1 | FR1 | TDD | OFDM | N/A | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 4-2 |  |  |  |  | TDLC300-100 |  | L=32 | Infobits=5 |
| Test 5-1 | FR2 | TDD | OOK1 | Envelope detector | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 5-2 |  |  |  |  |  |  | L=32 | Infobits=5 |

We are open to discuss if down selection to additionally reduce the test load after alignment is achieved.

Whether to define single requirement for Type 1 and Type 2 receivers

In RAN4#116 it was brought up, whether to define single requirement for Type1 and Type2 receivers (see [1]):

Issue 2-1-21: Whether to define single requirement for Type 1 and Type 2 receivers - Proposals - Option 1: RAN4 to study the performance for both receiver types (Type 1: OOK and Type 2: OFDM) and further discuss based on simulation results if one or two sets of requirements are to be defined. - Option 2: Do not define separate demodulation requirements for Type-1 and Type-2 LP-WUR capable UEs if those are not tied to Envelope vs Sequence Detection.

In our simulation results contribution [2] we see significant difference between OOK and OFDM receiver performance, hence it is reasonable to define two sets of requirements.

Our simulation results show high enough difference between the two receiver types to differentiate between them, hence two sets of requirements can be defined.

Define two sets of requirements (one for each receiver type: OOK and OFDM).

Whether to define capability rules for Type 1 and Type 2 receivers

In RAN4#116 it was brought up, whether to define capability rules for Type1 and Type2 receivers (see [1]):

Issue 2-1-22: Whether to define capability rules for Type 1 and Type 2 receivers - Proposals - Option 1: Don’t define any capability rules related to Type1/2 receivers for demodulation requirements.

We do not see any specific reason at this time to define capability rules related to the two receiver types for demodulation requirements.

Do not define any capability rules related to the two receiver types (OOK and OFDM).

Whether to re-use the definition from RAN4 RF for the receiver types (if separate requirements are introduced)

In RAN4#116 it was brought up, whether to re-use the definition from RAN4 RF for the receiver types (see [1]):

Issue 2-1-23: Whether to re-use the definition from RAN4 RF for the receiver types (if separate requirements are introduced) - Proposals - Option 1: If agreed to introduce two sets of requirements, then re-use the definition from RAN4 RF for the receiver types.

If two sets of requirements are introduced, we are fine with re-using the definition from RAN4 RF for the receiver types.

If agreed to introduce two sets of requirements, then re-use the definition from RAN4 RF for the receiver types (Option 1).

Number of coded bits after rate-matching

In RAN4#116 it was discussed how many coded bits to use after rate-matching (see [1]):

Issue 2-1-25: Number of coded bits after rate-matching - Agreement: - [12, 16] for 2 info bits - 32 for 5 info bits - Other options are not precluded

If the LP-WUS sequence is generated with OOK4 M=4 and sequence length of 24 the LP-WUS would fit into a NR slot as illustrated in Figure 2. As this would be a “practical” and optimized solution it could be considered selecting “LRM” matching a configuration for 1 slot.

Figure 2 Illustration of a possible LP-WUS setup. In this configuration can LP-WUS with OOK4 M=2 LRM=12 and OOK4 M=4 LRM=24

For our simulation we have conducted test with info-bit=5 and LRM=24 to compare with the LRM=32 agreed test. As the test with LRM=24 showed a bit worse performance than for the test with LRM=32 we suggest continuing the performance validation based on L=32.

Based on our simulation results in [2] we see no significant difference between LRM=12 or LRM=16 for 2 info bits, hence we support using LRM=12 for requirement definition. In addition, we have simulation LRM=24 for 5 info bits and also here we see no difference between LRM=24 or LRM=32 for 5 info bits.

Set the number of coded bits after rate matching as follows (LRM):
- for 2 info bits set LRM=12
- for 5 info bits set LRM=32

Choice of M for OFDM receiver

In RAN4#116 it was agreed to use M=1 and M=4 for OFDM receiver configuration as a starting point (see [1]):

Issue 2-1-26: Choice of M for ORFM receiver - Agreement: - M=1 and M=4 as a starting point - Interested companies can also bring results with M=2

As OFDM is overlayed the OOK signal the assumption is that by using M=1 and M=4 will have quite similar performance. As starting point we have used M=1.

Select only one M value for OFDM receiver verification. Since we expect no significant difference in results with M=1 and M=4 both can be selected for requirement definition.

Inclusion of requirements in the specification

In RAN4#116 it was brought up where to include new requirements in the specification (see [1]):

Issue 2-1-27: Inclusion of requirements in the specification - Proposals - Option 1: - RAN4 to add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR1. - RAN4 to add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR2.

Since the demodulation requirements for LPWUS are a new type of requirements, we support adding new subclauses for the new requirements as is proposed on option 1.

Introduce new subclauses for the LP-WUS requirements:
- RAN4 to add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR1.
- RAN4 to add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR2.

Conclusion

The following Observations and Proposals were made:

Testability Aspects

Whether to define a test mode for LP-WUS/WUR

- MDR tests if done based on RF agreement can be implemented without introducing a specific test mode, however with the current RF agreement, such a test will likely take a long time. It is however RAN4 objective to define the tests but not necessarily decide how the test is implemented.

- Further analyse the test time based on companies’ feedback with the focus on deciding if a test mode is to be introduced for LP-WUS/WUR testing

Whether to define separate test cases for idle/inactive and connected modes

We see significant differences in usage and decoding of LP-WUS between idle and connected mode, which makes it relevant to define separate test cases. In addition a UE might only support idle mode, hence dedicated test for idle mode is required.

Define separate test cases procedure for the modulator in idle and connected mode.

Demodulation Requirements

Whether to introduce FAR test

Previous agreement in RAN4#111 RF session is that FAR is expected to be defined as demodulation requirements, hence no FAR requirements were introduced by RAN4 RF. In addition, it was agreed in RAN4#116 that for MDR tests the UE can assume a FAR of 1%. Based on the agreement, we expect the UE will be implemented such that FAR will be close to 1%, hence it will be needed to introduce requirements for 1% FAR to ensure the assumption is not exceeded.

RAN4 shall introduce requirements for FAR for each requirement introduced for MDR. The FAR test shall run with the same parameters as the related MDR test.

Test metric for FAR test (if introduced)

Set test metric for FAR test to 1% FAR (Option 1)

Carrier frequency

Our simulation results show that the proposed values for FR1 from RAN4#116 can be used for requirement definition. In addition, we have used 28GHz for our FR2 simulation results which also shows acceptable results for requirement definition.

Use fc=900MHz for FR1 FDD and fc=2.6GHz for FR1 TDD. Use fc=28GHz for FR2 TDD.

Channel model

There is a significant difference in SNR between AWGN and TDLC/TDLA channel models. In addition, there is also significant difference between TLDA and TDLC models, hence it is reasonable to define requirements for both TDLA30-10 and TDLC300-100.

For FR1 cover both TDLA30-10 and TDLC300-100 for requirement definition across the final agreed requirements. For FR2 cover TDLA30-10 for requirement definition.

Time offsets

For FR2 use T=0.5us as time offset.

Test to be considered for requirement definition

Consider defining requirements using the following testcases covering both MDR and FAR:

| Test # | Freq. range | Duplex | Receiver type | Detector | Channel model | M | L | Info bits |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | FR1 | FDD | OOK4 | Envelope detector | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 1-2 |  |  |  |  |  | M=4 |  |  |
| Test 1-3 |  |  |  |  | TDLC300-100 | M=1 | L=32 | Infobits=5 |
| Test 1-4 |  |  |  |  |  | M=4 |  |  |
| Test 1-5 |  |  |  | Sequence detector | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 1-6 |  |  |  |  |  | M=4 |  |  |
| Test 1-7 |  |  |  |  | TDLC300-100 | M=1 | L=32 | Infobits=5 |
| Test 1-8 |  |  |  |  |  | M=4 |  |  |
| Test 2-1 | FR1 | TDD | OOK4 | Envelope detector | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 2-2 |  |  |  |  |  | M=4 |  |  |
| Test 2-3 |  |  |  |  | TDLC300-100 | M=1 | L=32 | Infobits=5 |
| Test 2-4 |  |  |  |  |  | M=4 |  |  |
| Test 2-5 |  |  |  | Sequence detector | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 2-6 |  |  |  |  |  | M=4 |  |  |
| Test 2-7 |  |  |  |  | TDLC300-100 | M=1 | L=32 | Infobits=5 |
| Test 2-8 |  |  |  |  |  | M=4 |  |  |
| Test 3-1 | FR1 | FDD | OFDM | N/A | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 3-2 |  |  |  |  | TDLC300-100 |  | L=32 | Infobits=5 |
| Test 4-1 | FR1 | TDD | OFDM | N/A | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 4-2 |  |  |  |  | TDLC300-100 |  | L=32 | Infobits=5 |
| Test 5-1 | FR2 | TDD | OOK1 | Envelope detector | TDLA30-10 | M=1 | L=12 | Infobits=2 |
| Test 5-2 |  |  |  |  |  |  | L=32 | Infobits=5 |

We are open to discuss if down selection to additionally reduce the test load after alignment is achieved.

Whether to define single requirement for Type 1 and Type 2 receivers

Our simulation results show high enough difference between the two receiver types to differentiate between them, hence two sets of requirements can be defined.

Define two sets of requirements (one for each receiver type: OOK and OFDM).

Whether to define capability rules for Type 1 and Type 2 receivers

Do not define any capability rules related to the two receiver types (OOK and OFDM).

Whether to re-use the definition from RAN4 RF for the receiver types (if separate requirements are introduced)

If agreed to introduce two sets of requirements, then re-use the definition from RAN4 RF for the receiver types (Option 1).

Number of coded bits after rate-matching

Set the number of coded bits after rate matching as follows (LRM):
- for 2 info bits set LRM=12
- for 5 info bits set LRM=32

Choice of M for OFDM receiver

Select only one M value for OFDM receiver verification. Since we expect no significant difference in results with M=1 and M=4 both can be selected for requirement definition.

Inclusion of requirements in the specification

Introduce new subclauses for the LP-WUS requirements:
- RAN4 to add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR1.
- RAN4 to add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR2.

- References

- R4-2512526_WF for [116][330] NR_LPWUS_Demod

- R4-2514129 - Simulations for UE demodulation performance requirements for LPWUS

- R4-2410569 - WF for [111][136] NR_LPWUS_UERF