3GPP TSG-RAN WG4 Meeting #116bis                                        R4-2514131 Prague, Czech Republic, October 13 - 17, 2025

Source: 	Huawei, HiSilicon

Title: 	On HPUE for NTN in Rel-20

Agenda Item:	7.6.3

Document for:	Approval

Introduction

According to the Rel-20 WID on enhanced requirements for NR NTN and IoT NTN Phase 2 [1], the objective related to NTN HPUE is reproduced here.

HPUE for NTN Core Part - Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types - Example bands are Band 256 and 255 - Necessary Tx requirements, at least including - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives - The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

In this contribution, we would like to provide our views regarding this topic.

Discussion

- 2.1 Initial considerations on IoT-NTN PC1.5

Based on the fruitful discussion in Rel-19, RF requirements for IoT-NTN HPUE are determined. By CR [2], eMTC-based PC2 and NB-IoT-based PC1/PC2 are available for IoT-NTN. Specifically, they are:

- For category M1, PC2

  - MOP

  - MPR, A-MPR and additional OOB requirements

  - Configured transmitted power

  - REFSENS for FDD duplex mode

- For category NB1 and NB2, PC1 and PC2

  - MOP

  - MPR, A-MPR and additional OOB requirements

  - Configured transmitted power and PC2 power boosting

Note that all those requirements are also based on RAN4 co-existence study and feasibility analysis. It was merely a RAN4 workload trade-off that led to not having PC1.5 at the end of Rel-19. Firstly, we think it would be helpful to get a clarification on whether RAN4 can focus on category NB1 and NB2 for PC1.5. If RAN4 can focus on category NB1 and NB2, then it must be easier for PC1.5 discussion because previous co-existence and feasibility study have already covered PC1.

Proposal 1: Clarify whether RAN4 can focus on category NB1 and NB2 for the introduction of PC1.5 for IoT-NTN.

Since ACLR and SEM are basic requirements for MPR evaluation, we propose to reuse existing requirements.

Proposal 2: Check if existing ACLR and SEM requirements can be reused for PC1.5.

- ACLR as defined in clause 6.5B.3.4 in TS 36.102,

- SEM as defined in clause 6.5B.3.2 in TS 36.102.

Another issue that should be noted is additional spectrum emission mask was revisited for accommodating higher power classes, the corresponding requirements are reproduced as below.

It can be observed that for the first two defined regions the spectrum emission limit increases by the same amount as the difference between any two power classes. Thus RAN4 can check if the same rule can still apply to PC1.5.

Proposal 3: Check if following requirements can be adopted as the additional spectrum emission mask for NS_02N for PC1.5:

- 4dBm when ΔfOOB is ± 0.09–0.28 MHz,

- -1dBm when ΔfOOB is ± 0.28–0.85 MHz, and

- -13dBm when ΔfOOB is larger than ± 0.85 MHz.

Regarding power boosting, the adopted boosting level is as follows.

While feasibility analysis is needed for determining whether to extend this feature to PC1.5, we notice that the gap between PC1.5 and PC1 is only 2dB so the necessity for such extension is questionable as the expected gain is about 1 or 2 dB at most. Therefore we have the following proposal.

Proposal 4: Check if RAN4 can only focus on the RF requirements and not consider power boosting for PC1.5.

Conclusion

In this contribution we share our initial considerations on Rel-20 HPUE for NTN. We have following proposals:

Proposal 1: Clarify whether RAN4 can focus on category NB1 and NB2 for the introduction of PC1.5 for IoT-NTN.

Proposal 2: Check if existing ACLR and SEM requirements can be reused for PC1.5.

- ACLR as defined in clause 6.5B.3.4 in TS 36.102,

- SEM as defined in clause 6.5B.3.2 in TS 36.102.

Proposal 3: Check if following requirements can be adopted as the additional spectrum emission mask for NS_02N for PC1.5:

- 4dBm when ΔfOOB is ± 0.09–0.28 MHz,

- -1dBm when ΔfOOB is ± 0.28–0.85 MHz, and

- -13dBm when ΔfOOB is larger than ± 0.85 MHz.

Proposal 4: Check if RAN4 can only focus on the RF requirements and not consider power boosting for PC1.5.

- References

- RP-252958, “New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2”, Moderator (RAN4 Vice Chair), 3GPP TSG-RAN Meeting #109, Beijing, China, September 15-18, 2025.

- R4-2510268, “Big CR for 36.102 for IoT-NTN HPUE”, vivo, Apple, CATT, China Telecom, Echostar, Ericsson, Globalstar, Hisilicon, Huawei, Inmarsat, LGE, Mediatek, Nokia, OPPO, Qualcomm, Samsung, Skyworks, Thales, viasat, Xiaomi, ZTE, 3GPP TSG-RAN4 Meeting #116, Bengaluru, India, August 25-29, 2025.