3GPP TSG-RAN WG4 Meeting #116bis	R4-2514140

# Prague, Czech, October 13th – 17th 2025

Source:	Ericsson

Title:	Discussion on Rel-19 Ku band impact on NTN demodulation requirements

Agenda item:	5.10.3

Document for:	Discussion

# Introduction

In Rel-19 WID “Introduction of Ku band for NR NTN” [1], following objectives are mentioned for performance part:

The objective of the performance part is to update as appropriate for Ku band:

- RRM performance requirements and test cases

- UE demodulation and CSI reporting requirements

  - For 15 kHz subcarrier spacing, the requirements are only applicable for GSO and low mobility scenarios.

- Satellite access node demodulation requirements

- Satellite access node conformance tests

In this contribution, general view on demodulation requirements for NR NTN Ku bands.

# Discussion

In Rel-19 RF discussion, following frequency bands are introduced for NTN Ku band.

- FR1-NTN: n248 (UL: 14000MHz – 14500MHz, DL: 10700MHz – 12750MHz) and n247 (UL: 13750MHz – 14000MHz, DL: 10700MHz – 12750MHz).

- FR2-NTN: n509 (UL: 14000MHz – 14500MHz, DL: 10700MHz – 12750MHz) and n508 (UL: 13750MHz – 14000MHz, DL: 10700MHz – 12750MHz).

It can be seen that both FR1 and FR2 configurations would be supported on same frequency range. For FR1-NTN Ku bands, both 15kHz and 30kHz SCS would be applied. For FR2-NTN Ku bands, only 120kHz SCS would be applied.

NTN Ku band include both FR1 and FR2 configurations.

By checking current NTN demodulation requirements, both FR1-NTN and FR2-NTN requirements have been defined. For Ku bands, it is higher than legacy FR1-NTN bands but lower than legacy FR2-NTN bands, following issues could be considered for whether introduce new demodulation requirements:

- Channel model. Generally, different band would lead to different Doppler shift. But Doppler shift value in current channel models for NTN requirements is residual Doppler after pre-compensation on both SAN and UE side. More practical channel models with timing and Doppler drift are introduced for UE conformance testing, but it is agreed that the existing demodulation requirements won’t be impacted.

In previous discussions on FR2-NTN demodulation, companies agree to take artificial value for Doppler shift rather than 0.1ppm restriction. In that case, Doppler value in NTN channel model could be considered as independent of frequency bands. Reuse existing channel models in demodulation requirements is feasible for NTN Ku band.

Channel models including Doppler shift in existing demodulation requirements can be reused for Ku band if new requirement is considered.

- Bandwidth and SCS. More channel bandwidths are introduced for FR1-NTN.

Existing SAN demodulation requirements include 5MHz CBW 15kHz SCS and 10MHz CBW 30kHz SCS for FR1-NTN and 50MHz CBW 120kHz SCS for FR2-NTN. They could be applied for Ku band according to applicability rule.

Existing UE demodulation requirements include 10MHz CBW 15kHz SCS for FR1-NTN and 200MHz CBW 120kHz SCS for FR2-NTN. They can also be applied to Ku band. But the question is if requirement for 30kHz SCS of FR1-NTN Ku band should be introduced.

For FR1-NTN S band, typical UE implementation only supports 15kHz SCS, but it might not be true for FR1-NTN Ku band regarding typical UE implementation might be 30kHz SCS in this frequency range. Based on RF discussion, FR1-NTN Ku band UE would only support either 15kHz or 30kHz SCS. To avoid no requirement for this implementation, it is reasonable to introduce new requirements for 30kHz SCS. But it would need confirmation from operator and UE vendors. The channel bandwidth could choose typical configuration or the maximum value (e.g., 100MHz) which depends on the discussion.

FR1-NTN Ku band UE would only support either 15kHz or 30kHz SCS.

- Antenna configuration. The typical UE type in Ku band discussion is VSAT with parabolic and phase array antennas. It is assumed that both 1Rx and 2Rx could be supported by VSAT in RF discussion which is the same as the condition for Ka band.

Current UE demodulation requirements only consider 2Rx for FR1-NTN and 1Rx for FR2-NTN. If Ka band approach is followed, there would be no necessary to consider 2Rx for FR2-NTN Ku band. Then the question is if 1Rx would be typical implementation for FR1-NTN Ku band UE. If it is confirmed by operators and UE vendors, new requirement might be needed, otherwise it is enough to reuse existing requirements.

Existing SAN demodulation requirements include both 1Rx and 2Rx which is sufficient for Ku band implementations.

Existing SAN and UE demodulation requirements could be reused for NTN Ku band.

FR2-NTN Ku band UE antenna implementation is the same as for Ka band.

Proposal 1	Do not introduce new SAN demodulation requirements for both FR1-NTN and FR2-NTN Ku band.

Proposal 2	Do not introduce new UE demodulation requirements for FR2-NTN Ku band.

Proposal 4	Introduce 30kHz SCS UE demodulation requirement for FR1-NTN Ku band if it is confirmed as typical implementation.

Proposal 5	Introduce 1Rx UE demodulation requirement for FR1-NTN Ku band if 1Rx is confirmed as typical implementation.

If 1Rx is agreed to be introduced for FR1-NTN Ku band UE demodulation requirements, the UE type would be dedicated to NTN deployment (e.g., VSAT) which is similar as in Ka band situation, so the new requirement for PDSCH mapping type A and PDCCH would be needed.

If 30kHz SCS is agreed to be introduced for FR1-NTN Ku band UE demodulation requirements, it would impact both 1Rx and 2Rx UE configurations. Regarding there is no 30kHz SCS FDD UE demodulation requirements even in TN specifications, both PDSCH and PDCCH requirements are also needed if introduced.

Based on analysis above, there would be following potential new requirements for FR1-NTN Ku band UE demodulation.

- PDSCH mapping type A and PDCCH

  - 1Rx

    - 15kHz SCS

    - 30kHz SCS

  - 2Rx

    - 30kHz SCS

Following potential new UE demodulation requirements are needed for FR1-NTN Ku band.

- PDSCH mapping type A and PDCCH

  - 1Rx

    - 15kHz SCS

    - 30kHz SCS

  - 2Rx

    - 30kHz SCS

# Conclusions

In the previous sections we made the following observations:

Observation 1	NTN Ku band include both FR1 and FR2 configurations.

Observation 2	Channel models including Doppler shift in existing demodulation requirements can be reused for Ku band if new requirement is considered.

Observation 3	FR1-NTN Ku band UE would only support either 15kHz or 30kHz SCS.

Observation 4	Existing SAN and UE demodulation requirements could be reused for NTN Ku band.

Observation 5	FR2-NTN Ku band UE antenna implementation is the same as for Ka band.

Observation 6	Following potential new UE demodulation requirements are needed for FR1-NTN Ku band.

	PDSCH mapping type A and PDCCH

o	1Rx

	15kHz SCS

	30kHz SCS

o	2Rx

	30kHz SCS

Based on the discussion in the previous sections we propose the following:

Proposal 1	Do not introduce new SAN demodulation requirements for both FR1-NTN and FR2-NTN Ku band.

Proposal 2	Do not introduce new UE demodulation requirements for FR2-NTN Ku band.

Proposal 4	Introduce 30kHz SCS UE demodulation requirement for FR1-NTN Ku band if it is confirmed as typical implementation.

Proposal 5	Introduce 1Rx UE demodulation requirement for FR1-NTN Ku band if 1Rx is confirmed as typical implementation.

# References

[1] 	RP-252911, Revised WID: Introduction of Ku bands for NR NTN, SES, CHTTL, JSAT