# 3GPP TSG-RAN WG4 Meeting # 116bis                                 R4-2514148

Prague, CZ, 13rd – 17th, Oct. 2025

# Title: 	Discussion on demodulation performance requirements for 6Rx UE with interference

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.5.4

# Document for:	Approval

# 1 Introduction

In RAN #109 meeting, the new WID of UE performance requirements for 6Rx UE devices (both handheld and FWA), which was approved in [1], wherein the objectives related to 6Rx UE are listed as below:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

In this contribution, we give some initial discussions on the UE performance requirements for 6Rx devices (both handheld and FWA), which include inter-cell interference cancellation and intra-cell inter-user interference cancellation.

# 2 Discussion

As we all know, co-channel inter-cell and intra-cell inter-user interference cancellation performance requirements were introduced in Rel-17. And in Rel-19, RAN4 introduced 6Rx demodulation requirements for PDSCH and CSI performance. With 6Rx, larger performance gain can be verified, due to larger spatial domain freedom compared to 2/4Rx. As a consequence, from demodulation perspective, there are more benefits to define UE performance requirements for 6Rx UE devices under interference scenarios.

  - Sub-topic 2-1 Inter-cell interference cancellation scenario

Regarding the inter cell interference cancellation scenario, since RAN4 discussed receiver structure in Rel-17, and MMSE-IRC receiver type was captured in [2], since we don’t need to discuss more details anymore. Also, the WID noticed that Rel-17 interference model could be reused. From the TS 38.101-4[3], INR1= 7.77dB and INR2=2.29dB were assumed for the first strong interference cell and the second strong interference cell, or INR=7.58dB only for the interference cell.

Proposal 1. Rel-17 INR values can be as a start point, e.g. INR1= 7.77dB and INR2=2.29dB or INR=7.58dB.

Regarding the other simulation parameters. rank, MCS, antenna configuration, we have the following discussion.

With 6Rx for UE devices, which is a high processing capability. The number of layers could be up to 6 layers. As mentioned in the WID [1], only up to 4 MIMO layers for target cell. Thus, we propose to consider 2 layers, 4layers for serving cell with inter-cell interference.

Proposal 2. Propose to consider 2layers, 4layers for serving cell with inter-cell interference.

Regarding modulation order, RAN4 introduced MCS 17 and MCS 13 for rank 2 and rank 4 for PDSCH demodulation requirements in Rel-19 for 6Rx. From capability perspective, we also would like to consider high modulation order. Even if high modulation order is more sensitive to interference caused by more receiver antennas.

Proposal 3. Propose to consider MCS 17 for rank 2 and MCS 13 for rank 4 as starting point.

With handheld and FWA devices, if RAN4 wants to define one set of requirements for both handheld and FWA devices, we prefer to consider TDLA channel model. If RAN4 wants to define separate requirements for both handheld and FWA devices, we prefer to consider TDLA channel model for FWA device, and TDLC for handheld UE. However, in Rel-19 discussion, only one propagation model TDLA30-10 is considered.

Proposal 4. Propose to consider TDLA30-10 propagation model for both handheld and FWA devices.

And for other parameters, e.g. duplex model, bandwidth and subcarrier spacing, we have following proposal.

Proposal 5. Propose to consider FDD/TDD, 10M/15kHz and 40M/30kHz for demodulation requirements.

Further CQI requirements, it is clearly pointed that the CQI reporting requirements need to be wait for the conclusion of Rel-19. In the last meeting, the CQi requirements for 6Rx UE still FFS. Therefore, we propose to postpone the discussion of this issue.

Proposal 6. Propose to postpone the discussion of CQI requirements. Wait for the conclusion of Rel-19.

  - Sub-topic 2-2 Intra-cell inter-users interference cancellation scenario

Regarding the intra-cell inter-users interference cancellation scenario, since RAN4 defined requirements for 2/4Rx in Rel-17 for MMSE-IRC receiver. Since RAN4 can follow the approach of Rel-17 as starting point, e.g. duplex model, bandwidth and subcarrier spacing, etc.

Proposal 7. RAN4 can follow the approach of Rel-17 as start point, e.g. duplex model, bandwidth and subcarrier spacing, etc.

From our understanding, following simulation parameters may need to be considered again, e.g. rank, MCS, antenna configuration and propagation condition. With 6Rx UE, only rank 2+2 is specified in the WID. Therefore, only rank 2+2 should be considered.

Regarding modulation order, we prefer considering MCS 13 and MCS 17 as starting point.

Proposal 8. Propose to only consider rank 2+2 to define requirements.

Proposal 9. Propose to consider MCS 13 and MCS 17 as starting point.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for 6Rx UE with interference, the conclusions are:

Proposal 1. Rel-17 INR values can be as a start point, e.g. INR1= 7.77dB and INR2=2.29dB or INR=7.58dB.

Proposal 2. Propose to consider 2layers, 4layers for serving cell with inter-cell interference.

Proposal 3. Propose to consider MCS 17 for rank 2 and MCS 13 for rank 4 as starting point.

Proposal 4. Propose to consider TDLA30-10 propagation model for both handheld and FWA devices.

Proposal 5. Propose to consider FDD/TDD, 10M/15kHz and 40M/30kHz for demodulation requirements.

Proposal 6. Propose to postpone the discussion of CQI requirements. Wait for the conclusion of Rel-19.

Proposal 7. RAN4 can follow the approach of Rel-17 as start point, e.g. duplex model, bandwidth and subcarrier spacing, etc.

Proposal 8. Propose to only consider rank 2+2 to define requirements.

Proposal 9. Propose to consider MCS 13 and MCS 17 as starting point.

# 4 References

- RP- 252957, WID: NR demodulation performance: Phase 6.