# 3GPP TSG-RAN WG4 Meeting # 116bis                                 R4-2514150

Prague, CZ, 13rd – 17th, Oct. 2025

# Title: 	Discussion on demodulation performance requirements for FR1 SU-MIMO with spatial channel model

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.5.3

# Document for:	Approval

# 1 Introduction

In RAN #109 meeting, the new WID of UE performance requirements for FR1 SU-MIMO with spatial channel model, which was approved in [1], wherein the objectives related to FR1 SU-MIMO with spatial channel model are listed as below:

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

In this contribution, we give some initial discussions on the UE performance requirements for SU-MIMO with spatial channel model.

# 2 Discussion

To date, all demodulation performance requirements defined by RAN4 uniformly adopt TDL based channel models. This model defines two types of antenna arrays—Uniform Linear Array (ULA) and Cross-Polarized (X-pol)—and generates spatial correlation levels and directions by configuring different correlation factors and beam control methods. Even when correlation matrices are introduced into such TDL models, their application remains limited: all multipath components are subjected to the same correlation matrix. This approach fails to reflect the spatial characteristics of different scattering clusters in real-world scenarios. Based on those reasons, RAN4 discussed the spatial channel model based on CDL and TDL channel models in Rel-19 in order to better simulate real-world signal propagation and verify the benefits in some scenarios.

After analyzing and studying specifications TS38.901 and 38.827, RAN4 finally develop a new channel model, named rCDL-C1 channel model. Compared to the CDL-C channel model in 38.901, the number of clusters and values such as power, AOA, AOD, ZOD, and ZOA have been modified.

Furthermore, some companies have proposed introducing spatial directivity by applying spatial angles to MIMO channels. Compared to the traditional TDL channel model, the xTDL-C1 channel model allows different taps to have different angles of AOA/AOD, thus achieving different spatial characteristics.

In the terms of performance evaluation, RAN4 focused on two aspects, alignment and comparison. For alignment part, FR1 SU-MIMO PDSCH 4T4R with 4 layers and 8T8R with 8 layers, SU-MIMO PMI 8Tx4Rx with 4 layers and Type-I codebook and 8Tx4Rx with 4 layers and eType II codebook are evaluated for both rCDL-C1 and xTDL-C1 channel models. For comparison part, SU-MIMO PDSCH 4T4R 4 layers and 8T8R 8 layers, PMI 8Tx 4Rx 4 layers with Type I and eType II, 8Tx 4Rx 2 layers with Type I and eType II are evaluated for rCDL-C1 and xTDL-C1, and legacy TDL channel models.

In general, RAN4 created a truncated a channel model, named xCDL-C1 in TR 38.753, and the specific values, such as delay, power, AOD, AOA, ZOD, and ZOA has been achieved. Thus, for Rel-20 work, RAN4 can directly use the parameters in xCDL-C1 to define requirements. And regarding the test cases, it is clear pointed that the only 8T8R 2 codeword use case and the 4T4R case are agreed to be defined. Therefore, RAN4 can follow this conclusion to define such test cases requirements.

Proposal 1. RAN4 can follow the conclusion from RAN plenary to define 8T8R 2 codeword use case and the 4T4R case.

And regarding the PMI test cases, based the simulation results of Rel-19 in the following [2]. Currently, the test cases of eType-II codebook with 70% and 90% throughput have a larger span among all companies. And for Type-I codebook with 70% throughput also larger than 2.5dB span. From the view of requirements definition, the simulation results for R19 are insufficient to support the demodulation requirements defined by RAN4. Therefore, we think that RAN4 needs more discussions on the performance alignment of PMI test cases.

Simulation result summary for FR1 SU-MIMO PMI 8Tx4Rx with 4 layers, follow PMI

| SNR at Norm. Throughput [dB] |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Source #9 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type-I | 70% | 12.4 | 11.0 | 9.7 | 10.3 | 10.2 | 9.7 | 9.5 | 10.2 | 9.8 | 10.4 | 2.9 |
|  | 90% | 14.4 | 13.1 | 12.5 | 12.8 | 12.6 | 12.6 | 12.0 | 12.5 | 12.3 | 12.7 | 2.4 |
| eType-II | 70% | 14.0 | 14.3 | 8.4 | 11.6 | 10.1 | 10.8 | 8.2 | 13.8 | 10.8 | 11.5 | 6.1 |
|  | 90% | 15.9 | 17.1 | 12.3 | 14.0 | 12.6 | 13.9 | 10.6 | 16.8 | 12.9 | 14.0 | 6.5 |

Observation 1. The simulation results for R19 are insufficient to support the demodulation requirements defined by RAN4. Because the results of PMI test cases can’t alignment well.

Proposal 2. RAN4 needs more discussions on PMI test cases on how to align the simulation results.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for 6Rx UE with interference, the conclusions are:

Observation 1. The simulation results for R19 are insufficient to support the demodulation requirements defined by RAN4. Because the results of PMI test cases can’t alignment well.

Proposal 1. RAN4 can follow the conclusion from RAN plenary to define 8T8R 2 codeword use case and the 4T4R case.

Proposal 2. RAN4 needs more discussions on PMI test cases on how to align the simulation results.

# 4 References

- RP- 252957, WID: NR demodulation performance: Phase 6.

- 3GPP TR 38.753 V1.1.0 (2025-09).