3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514184

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on RRM impacts for CA_n5_n8

Source: 	Huawei, HiSilicon

Agenda item:	5.9.3

Document for:	Discussion

Introduction

For WI “LTE_NR_R19_Simult_RxTx”, main session reached the following agreement in RAN4#116.

For CA_n5-n8, in RAN4#116bis, an agenda item in RRM session will be introduced to discuss and determine if there is RRM impact. If RRM impact is identified and agreed, this topic will not be further pursued. If not, the discussion will continue in main session under R19 maintenance. - When n5DL is operated, n8UL RF path needs to be switched off and the same for the other way around. - Proponent companies are encouraged to bring detailed analysis, including the possible gain/loss. Before the conclusion on RRM impact is made, no further discussion is needed in the main session.

In this paper, we will provide our views on RRM impacts for CA_n5_n8.

Discussion

Background

In band combination CA_n5_n8, the UL of one band (n8) partially overlaps with the DL of the other band (n5), as illustrated in Figure 1.

Figure 1: Illustration of frequency location of band n5 and band n8

Main session has studied different options for implementing the CA_n5-n8, and in RAN4#112-bis [1] it was agreed to support option 2 which is defined in TR 38.872 and copied below.

1) Full band n5 and n8 RF filters implementation with option 1 and option2: - Option 1: Only support 1UL/2DL CA. Single UL in n5 - Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL 2) Dedicated RF filters implementation with partial frequency range - Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL without any scheduling restriction on both bands.

The detailed procedure for supporting option 2 has been discussed for some meetings, and in RAN4#116 the WF [2] suggested a solution in detail but the WF was noted. In the next sections, we will provide our analysis on the gain and RRM impacts based on the solution in [2] as copied below for convenience.

Step 1: Access n5 as PCell Step 2: RRC reconfiguration to Case 2: Add n8 with only n8-DL as SCell UL Tx in SCell via n5-UL - RFFE configuration is #A: n5-UL: Tx, n5-DL:Rx, n8-DL:Rx Step 3: RRC reconfiguration to Case 1: PCell remains at n5, reconfigure SCell n8 by adding n8-UL - Freedom of RFFE configuration - Alt.#1: Not changed, remaining as #A: (n5-UL: Tx, n5-DL:Rx, n8-DL:Rx) - Alt#2: Or reconfigured to #B: (n5-UL:Tx, n8-UL:Tx, n8-DL:Rx) Step 4: When n8-UL needs to be scheduled from Slot #N to Slot #N+m, - NW to choose a slot #N-k, where DCI issued to the UE that n8-UL transmission happens at slot #N - k is chosen - to be large enough so the UE can make successful RFFE reconfiguration to Alt. #2 - There will be no SSB/PDCCH/CSI-RS on n5-DL to be received by the UE between Slot #N-k and #N - m is chosen - No SSB/PDCCH/CSI-RS on n5-DL to be received by the UE between Slot #N to #N+m - The DCI also schedules n5-UL between Slot#N to #N+m via 1 TB mapping to multiple slots via features: - Repetitive (Rel-15) - Non-repetitive (Rel-17) Hence n5-UL transmission during this period is not impacted. - UE to complete reconfiguration of RFFE between Slot #N-k and #N (k is chosen to guarantee that no n5-DL reception is required) - UE to make UL transmission at n8-UL at Slot #N until Slot #N+m - Once UE complete n8-UL transmission, reconfigure RFFE from #B to #A immediately - NW do not schedule any PDSCH before Slot #N+m+q - q is chosen - There is no SSB/PDCCH/CSI-RS on n5-DL to be received by the UE between Slot #N+m and Slot #N+m+q - There is no uplink transmission on n8-UL for the UE between Slot #N+m and Slot #N+m+q Step 5: Repeat Step 4 if new n8-UL transmission is required. NOTES: - PCell is always n5, and n8 always serves as SCell. - SCell is active - No SCell activation issue - PCell and SCell is collocated, and have the same frame timing - No UL timing issue - Only RFFE Alt.#1 works (i.e., when RRC reconfigured from Case 2 to Case 1, RFFE configuration do not change), otherwise, NW do not configure the UE with 2UL/2DL (scheduling restriction) - UE capability required - NW needs to know the UE capability : (1) No RFFE reconfiguration when adding n8-UL to SCell; (2) RFFE reconfigured back to 1UL/2DL when finishing UL transmission on n8-UL. - Besides the UE capability, UE just follows NW instructions, no need for change of behaviors, signaling, etc., and no additional requirements.

Gain of the proposed solution

Our understanding about the gain of the proposed solution is illustrated in Figure 2.

Figure 2: Available UL and DL resource with and without proposed solution

In Figure 2a, the UL and DL resource is static allocated without the proposed solution. As UE can only be scheduled with 1UL/2DL, UL resource is half of the DL resource, assuming same BW on the two bands.

In Figure 2b, with the proposed solution, the ratio between UL and DL resource can be flexibly determined based on NW scheduling. Of course, there would be some loss in the overall resource due to the switching. However, for UL heavy service there is clear gain in Figure 2b as there is much more UL resources available for scheduling. In the figure, the example shows around 50-50 split between n5 DL and n8 UL, which means there is around 50% more UL resources in Figure 2b compared to Figure 2a.

Observation 1: The proposed solution in [2] allows flexible UL and DL resource allocation, and can provide over 50% gain in UL throughput for UL heavy service.

Of course, proposed solution in [2] is not the only solution to support option 2 thus achieving flexible UL and DL resource allocation in CA_n5_n8. Another solution is to adapt the switching pattern based solution from the R19 WI of “NR_LBCA_Sw”. However, in RAN#109 it was agreed to not include FDD/FDD band combination in the R20 WI of “NR_LBCA_Sw_enh”. It means if we do not support the proposed solution in [2] or similar, option 2 will not be supported in either R19 or R20. It is noted that here FDD/FDD doesn’t intend the same switching between 1UL/2DL and 2UL/1DL as mentioned in [3], given that the proposed switching in [3] is not clear regarding whether 2UL1DL configuration is proposed or not.

Observation 2: CA_n5_n8 is not included in the scope of R20 WI of “NR_LBCA_Sw_enh”, which means without the proposed solution in [2], only 1UL/2DL will be supported in R19 and R20.

RRM impacts of the proposed solution

In Figure 3, we show an example of scheduling pattern for the proposed solution. In our view, there would be no impact to RRM requirements if NW makes scheduling such that UE is not scheduled to transmit on n8_UL in slots where UE is configured or scheduled to receive on n5_DL. This restriction is straightforward given the spectrum overlap between n5_DL and n8_UL. In addition, due to reconfiguration of RF front end, UE is not scheduled in the one slot immediate before and after the transmission on n8_UL.

For example, in Figure 3, a periodic signal of SSB is transmitted every 20ms in n5_DL, so NW ensures that n8_UL is not scheduled in SSB slots of n5_DL (slot #0 and #1). In Figure 3, the search space on n5_DL is configured with periodicity of 20 slots, offset 0 and duration 9, so the UE is scheduled on n8_UL from slot #10 to #18, where UE is not configured to monitor PDCCH or scheduled with PDSCH on n5_DL. In Figure 3 slot #9 and #19 are reserved for UE to reconfigure its RF front end, so UE is not scheduled on during those slots.

Figure 3: Example of scheduling pattern for the proposed solution

Measurement and mobility requirements

Measurement and mobility requirements are defined based on the assumption that UE can receive periodic reference signal, in particular SSB and CSI-RS for L1 measurement, and SMTC for L3 measurement. As UE is not expected to be scheduled on n8_UL during the slots containing SSB/SMTC and CSI-RS, UE reception of the reference signal is not impacted, so the existing measurement and mobility requirements should apply.

UL timing requirements

As PCell and SCell are co-located, UL timing for both n5_UL and n8_UL are based on sync from n5_DL (PCell DL). As UE is not expected to be scheduled on n8_UL during the slots containing SSB on n5_DL, DL sync is not impacted, so the existing UL timing requirements should apply.

SCell related requirements

As SCell stays in activated state during all steps of the proposed solution, there is no impact to requirements for SCell activation or measurement for deactivated SCell. At step 3, there is an RRC reconfiguration to add n8_UL. Since UE is not expected to reconfigure its RF front end at the RRC reconfiguration, no requirement needs to be defined related to this RRC reconfiguration.

Scheduling restriction

There is scheduling restriction as described in the beginning of current section. Our view is that they should be defined as a NW restriction (with the wording “UE is not expected to the scheduled …”) rather than a UE requirement (with the wording “UE is not required to transmit or receive …”). As to in which spec to capture the restriction, we suggest to capture it in RF spec or UE capability description as it is straightforward given the spectrum overlapping and it is very specific for this band combination of CA_n5_n8.

Proposal 1: RAN4 to confirm there is no RRM impact to support the proposed solution in [2].

Proposal 2: RAN4 to capture the following NW scheduling restriction or similar in RF spec: UE is not scheduled to transmit on n8_UL in slots where UE is configured or scheduled to receive on n5_DL.

Conclusions

In this paper we provided our views on RRM impacts for CA_n5_n8.

Observation 1: The proposed solution in [2] allows flexible UL and DL resource allocation, and can provide over 50% gain in UL throughput for UL heavy service.

Observation 2: CA_n5_n8 is not included in the scope of R20 WI of “NR_LBCA_Sw_enh”, which means without the proposed solution in [2], only 1UL/2DL will be supported in R19 and R20.

Proposal 1: RAN4 to confirm there is no RRM impact to support the proposed solution in [2].

Proposal 2: RAN4 to capture the following NW scheduling restriction or similar in RF spec: UE is not scheduled to transmit on n8_UL in slots where UE is configured or scheduled to receive on n5_DL.

Reference

- R4-2417125, WF on UL CA_n5-n8, CATT, China Telecom, Huawei, HiSilicon

- R4-2511850, WF on UL CA_n5-n8, CATT, Huawei, China Telecom, China Unicom, Anterix

- RP-252841, Apple’s view on Rel-20 cross-area topics, Apple