3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2514205

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	Discussion on AI-enabled RRM measurement prediction

Agenda item:			7.12.3

Document for:	Discussion

- Brief observation/background on scenarios/use cases in Rel-19

  - Prediction deployment

- UE-side model deployment

Based on the WID agreed in RANP #109, the following consensus on AI/ML model deployment was achieved:

- For UE sided model, spatial domain prediction is not supported

- For UE sided model, L3 beam level prediction is not supported

- For other use cases:

| Use cases | UE sided model | Network sided model |
| --- | --- | --- |
| Temporal domain case A | Yes | Yes |
| Temporal domain case B | Yes | Yes |
| Frequency domain | Yes | Yes |

* Inter-frequency prediction: RAN2 start evaluation from co-located scenario and start from the case that cell A and cell B are in the same sector

- Scenario/Sub-use case priorities in Rel-19

RAN4 to start the discussion for scenario 2, 3, 4

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

* 1st goal: To study AI/ML measurement prediction algorithm that can enable measurement overhead reduction; 2nd goal: To study AI/ML measurement prediction algorithm that can enhance HO performance

- Sub-use case 1/2/3:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated.

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s).

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

  - Report mechanism

The report mechanisms for temporal domain Case A/Case B and frequency domain prediction were agreed in RAN2 #131, below

- For temporal domain Case A, NW can indicate the list of target cells for which it expects results (if available) as part of inference configuration. This list is optional. - For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message - For temporal domain Case B, the inference report can include the latest measurement results (regardless of actual results or predicted results). - For frequency domain prediction, the inference report can include the latest measurement results of the predicted cell. (No change to the existing measurement report)

From the agreement, we have the following observations:

Observation 1: For the report mechanism of Case A, sequence of predicted measurement results for multiple future time instances in PW per one cell and one predicted measurement result in PW per one cell are supported

Observation 2: For the report mechanism of Case B, the latest measurement results reported can be actual measurement results or predicted measurement results as long as accuracy requirements satisfied.

- Discussion

  - Temporal prediction scenario

    - Impacts from observation windows/MRRT

Impacts from OW for Case A based on ideal simulation assumption agreed in RAN2 is shown below

Figure 1 L3 RSRP difference with different OW lengths of 1/5/10/20 samples corresponding to 80/400/800/1600 ms  with fixed PWs for the UE speed 30km/h, sliding-filtering option

From the figure 1, we can observe that increase the OW length can enhance prediction accuracy if the length of PW is fixed, while when the length of OW exceeds a certain value (e.g., 5 samples), the accuracy increment becomes marginal. Besides, the latest RSRP result at the end of the PW has the maximum RSRP difference, and increase the PW length will decrease the prediction accuracy due to the deterioration of the correlation between historical sample (s) and future sample (s). However, to fully exploit the advantage of AI/ML approaches, the case with more long length of PW is preferred. From this,

Proposal 1: For FR2 to FR2 intra-frequency temporal domain case A, OW length and/or PW length should be specified in Rel-20 AI mobility, the necessity of setting threshold (s) for OW length and/or PW length can be discussed.

For case B, based on our preliminary simulation results, we can observe that there is a trade-off between MRRT and prediction accuracy performance. When MRRT increases, the prediction accuracy gets worse although the number of skipped measurement samples grows and the measurement overhead decreases. One reason is that, as the MRRT increases, the length of PW becomes large, and the gap between the last predicted value in PW and the last actual measured value in OW increases, the corresponding correlation declines. In this sense, MRRT can be taken as a side condition when to define prediction accuracy requirement. At the same time, in order to maintain the overall HO performance no matter actual results or predicted results are reported as measurement results, and to balance the prediction performance and RRM measurement reduction under fast fading channel modelling, RAN4 could start drafting the corresponding simulations for the subsequent prediction accuracy evaluation.

Proposal 2: For FR1 to FR1 intra-frequency temporal domain case B, MRRT can be considered as a side condition when defining prediction accuracy requirement

- RAN4 could start drafting simulations for the subsequent prediction accuracy evaluation

    - Measurement relaxation

In the current spec., when the corresponding relaxed measurement criteria satisfies, measurement relaxation can be performed in several cases, for example SSB based RLM/BFD relaxation for UE fulfilling relaxed measurement criteria in connected mode, intra-frequency NR cells measurement relaxation for UE fulfilling not-at-cell edge criterion/ low mobility criterion and not-at-cell edge criterion/ low mobility criterion for idle mode mobility, etc. In this sense, for FR1 to FR1 intra-frequency temporal domain case B prediction, if UE can perform inference and predict RSRP/RSRQ measurement results of time instances (i.e., measurement occasions) in a future time period based on AI/ML supported temporal domain L3 RSRP prediction function, when relaxation criterion is met, measurement reduction based relaxed measurement/evaluation delay can be considered. On the other hand, based on the preliminary simulation results, AI based approach is superior to non-AI approach, in this sense, at least from mobility performance enhancement perspective, measurement relaxation can also be accounted in AI/ML-enabled mobility. While, with the same MRRT value, the measurement skipping pattern can vary depending on the UE’s implementation, whether skipping pattern is considered as an influence factor when to define measurement relaxation requirement (s) needs to be discussed.

Proposal 3: For FR1 to FR1 intra-frequency temporal domain case B, RAN4 to discuss the measurement relaxation (e.g., measurement period relaxation and/or evaluation period relaxation) requirements for UE fulfilling the certain relaxation criteria in Rel-20 AI mobility.

- The relationship of measurement relaxation factor and the skipping pattern/MRRT parameter should be involved

  - Inter-frequency prediction scenario

    - Impact on CSSF

Based on the current RRM measurement framework, L3 measurement is based on the measConfig, which uses measIDs to link measObjects and reportConfigs, where measObject provides ssbFrequency to measure, and based on the legacy measurement mechanism, if UE is configured with a MO, and the MO is associated with reporting, UE shall keep measuring all cells on this frequency layer. Based on such configuration mechanism, when UE is configured to measure multiple frequency layers or multiple cells, CSSF scaling is needed. In the current Spec., CSSFintra for intra-frequency measurements and CSSFinter for inter-frequency measurements are considered, separately. Taking inter-frequency measurements as example, the requirements are scaled by the number of competing MOs in the gap, so if UE can perform inference and predict RSRP measurement results of other competing MOs of frequency layer i in the same gap based on AI/ML supported frequency domain L3 RSRP prediction function, CSSFinter can be reduced, based on which the measurement delay can be shorten, accordingly. From this,

Proposal 4: For FR1 to FR1 inter-frequency (frequency domain) prediction, RAN4 to specify the inter-frequency measurement delay requirement considering CSSF (CSSFinter) reduction in Rel-20 AI mobility

    - Impact on MG

Gap based measurements typically has longer delays than non-gap based measurements, depending on the gap periodicity and the number of measurement objects configured to be measured in gaps. When the UE to perform the measurement of a frequency, it first receives the RSs of the frequency via RF receiver, and then processes the RS to obtain the measurement result (e.g., RSRP) of all the cells in the frequency via BB, wherein, RF retuning is needed in the first step. In this sense, throughput degradation will be incurred due to the unavoidable scheduling restriction and interruption, the basic reason is that when UE use MG to perform measurement, UE cannot receive or transmit on downlink/uplink shared channels of the serving cell during such MG. Considering that the intention corresponding to scenarios 3 is to reduce the measurement overhead, especially the MG needed for the measurements on other frequency, if UE can perform inference and predict RSRP measurement results of frequency j based on AI/ML supported frequency domain L3 RSRP prediction function, MG can be pruned or some MG occasion can be skipped for reduction during a time period for certain frequency j. Naturally, based on such MG reduction, the current gap related measurement delay requirement and scheduling restriction requirement will be impacted.

Proposal 5: For FR1 to FR1 inter-frequency (frequency domain) prediction, RAN4 to specify the inter-frequency with gap measurement delay requirement and scheduling restriction requirement considering MG reduction in Rel-20 AI mobility.

- The MG reduction can at least include: MG pruning and/or MG occasion skipping for reduction during a time period

  - Requirements for model monitoring

From our understanding, an AI/ML inference feature supported UE is able to predict a period/some occasions configured to measure, while the applicable inference AI/ML model related to key inference parameters and associated ID  currently in use is not always suitable for the expected implementations especially when the radio environment and UE trajectories change, in this sense, to maintain the robust mobility performance, model monitoring is needed.  The involved aspects can be GT for monitoring establishment and monitoring related conditions (e.g., criterion and the corresponding threshold) definition.

Proposal 6: For AI/ML-enabled mobility, RAN4 to specify requirements for model monitoring including monitoring related conditions (e.g., criterion and the corresponding threshold) definition

  - Requirements for SCell operation cycle

If we to combine mobility and CA scenario together, more requirements shall be impacted. Regarding the SCell operation cycle, the following SCell statuses are considered: non-configured, configured but deactivated, activation and activated, and the DL CC operational states are summarized as below

Figure 2 DL CC operational states

Measurements are considered in each stable state and during some of the transition between states, in this sense, since measurement prediction results can be inferred by AI/ML model, the current SCell addition, timeline of known/unknown SCell activation and known condition of the SCell during SCell activation procedure, and SCell activation condition shall be impacted theoretically. In this sense, we have the following proposal:

Proposal 7: For AI/ML-enabled mobility under CA scenario, RAN4 to discuss requirements for SCell operation cycle involving SCell addition and SCell activation.

- The SCell activation requirement can at least include: known/unknown SCell activation timeline and known condition of the SCell

  - Cell prediction requirements

As we discussed in AI PHY, the discussion on metrics/KPIs for cell-level prediction is also needed. In SI, we agreed that the prediction accuracy for RRM measurement cell level prediction is “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases. However, different interpretations on average RSRP can be obtained, at least: 1) Average L3 cell level RSRP difference across prediction window (dB); 2) Top (%) RSRP across all predicted instances in the prediction window; 3) Average L3 cell level RSRP difference between each predicted result and the corresponding GT value separately; 4) Average L3 cell level RSRP difference at prediction window edge (dB). From such interpretations, the prediction accuracy Metrics/KPIs for cell prediction can be defined from the following two perspectives at least, which align the logic discussed in AI BM:

- Option 1: Cell prediction accuracy: Top-1(%), Top-K(%)

- Option 2: Maximum RSRP among Top-K predicted cells is larger than the RSRP of the strongest cell – x dB

Proposal 8: For AI/ML-enabled mobility, RAN4 to discuss the accuracy metrics/KPIs for cell prediction requirements, the following two options of the accuracy KPI can be taken as the candidate

- Cell prediction accuracy: Top-1(%), Top-K(%)

- Maximum RSRP among Top-K predicted cells is larger than the RSRP of the strongest cell – x dB

- Conclusion

In this contribution, we provided our initial viewpoints on AI-enabled RRM measurement prediction requirement. The following observations and proposals are obtained:

Observation 1: For the report mechanism of Case A, sequence of predicted measurement results for multiple future time instances in PW per one cell and one predicted measurement result in PW per one cell are supported

Observation 2: For the report mechanism of Case B, the latest measurement results reported can be actual measurement results or predicted measurement results as long as accuracy requirements satisfied.

Proposal 1: For FR2 to FR2 intra-frequency temporal domain case A, OW length and/or PW length should be specified in Rel-20 AI mobility, the necessity of setting threshold (s) for OW length and/or PW length can be discussed.

Proposal 2: For FR1 to FR1 intra-frequency temporal domain case B, MRRT can be considered as a side condition when defining prediction accuracy requirement

- RAN4 could start drafting simulations for the subsequent prediction accuracy evaluation

Proposal 3: For FR1 to FR1 intra-frequency temporal domain case B, RAN4 to discuss the measurement relaxation (e.g., measurement period relaxation and/or evaluation period relaxation) requirements for UE fulfilling the certain relaxation criteria in Rel-20 AI mobility.

- The relationship of measurement relaxation factor and the skipping pattern/MRRT parameter should be involved

Proposal 4: For FR1 to FR1 inter-frequency (frequency domain) prediction, RAN4 to specify the inter-frequency measurement delay requirement considering CSSF (CSSFinter) reduction in Rel-20 AI mobility

Proposal 5: For FR1 to FR1 inter-frequency (frequency domain) prediction, RAN4 to specify the inter-frequency with gap measurement delay requirement and scheduling restriction requirement considering MG reduction in Rel-20 AI mobility.

- The MG reduction can at least include: MG pruning and/or MG occasion skipping for reduction during a time period

Proposal 6: For AI/ML-enabled mobility, RAN4 to specify requirements for model monitoring including monitoring related conditions (e.g., criterion and the corresponding threshold) definition

Proposal 7: For AI/ML-enabled mobility under CA scenario, RAN4 to discuss requirements for SCell operation cycle involving SCell addition and SCell activation.

- The SCell activation requirement can at least include: known/unknown SCell activation timeline and known condition of the SCell

Proposal 8: For AI/ML-enabled mobility, RAN4 to discuss the accuracy metrics/KPIs for cell prediction requirements, the following two options of the accuracy KPI can be taken as the candidate

- Cell prediction accuracy: Top-1(%), Top-K(%)

- Maximum RSRP among Top-K predicted cells is larger than the RSRP of the strongest cell – x dB

4. Reference