3GPP TSG-RAN WG4 Meeting #116bis	R4-2514264

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	Discussion on XR ph3 RRM performance requirements

Agenda item:	6.19.2

Document for:	Discussion

Introduction

This contribution provides Nokia’s views on open issues after RAN4#116 on RRM performance requirements for XR enhancements [1].

Discussion

List of Test Cases

In the RAN4#116 meeting, a comprehensive list of potential test cases for XR measurement gap cancelation was provided in the moderator summary [3]. As the number of TUs mentioned in work item description on XR enhancements are limited, RAN4 should focus on the most important test cases to complete this work item in time. Therefore, we provide a carefully selected list of test cases to ensure enough test coverage for XR measurement gap cancelation while keeping a reasonable RAN4 workload.

RAN4 to agree on developing XR measurement gap cancelation test cases provided in Table 1 below cases to ensure enough test coverage while keeping a reasonable RAN4 workload.

Table 1 List of recommended RRM test cases for XR measurement gap cancelation

| Gap type | Intra/Inter/Inter-RAT | Frequency Range | DRX | Test type |
| --- | --- | --- | --- | --- |
| Type 1 gap per UE | Intra-frequency | FR1 | OFF | Meas procedure |
|  |  | FR2-1 | OFF | Meas procedure |
|  | Inter-frequency | FR1 | ON | Meas procedure |
|  |  |  | OFF | Meas procedure + accuracy |
|  |  | FR2-1 | ON | Meas procedure |
|  |  |  | OFF | Meas procedure + accuracy |
|  | Inter-RAT E-UTRAN TDD | FR1 (serving) | ON | Meas procedure |
|  |  |  | OFF | Meas procedure + accuracy |
| Type 2 gaps per UE | Intra-frequency | FR1 | OFF | Meas procedure |
|  |  | FR2-1 | OFF | Meas procedure |
|  | Inter-frequency | FR1 | OFF | Meas procedure |
|  |  | FR2-1 | OFF | Meas procedure |
| L1 RSRP measurements with gaps | Inter-frequency | FR1 | OFF | Meas procedure |
|  |  | FR2-1 | OFF | Meas procedure |

Furthermore, RAN4 should select NR SA test cases as baseline and extend inter-RAT measurement test cases defined in Annexure 6.6.3 of TS 38.133 [4] for XR MG cancelation scenarios.

One open issue on test case list is [2]:

Sub-topic 2-2 – Test case list Issue 2-2-1: Scenarios for the definition of the test cases <Way forward> Discuss the proposals - Proposal 1: it is proposed to define test cases for NR-SA. - Proposal 2: it is proposed to define test cases for FR1 and FR2-1. - Other proposals not precluded.

Considering the agreements regarding deployment scenarios, Proposals 1 and 2 should be agreeable.

Agreement from RAN4 #112bis R4-2416864 [5]

Issue 1-1: Deployment scenarios < Agreement>: - Frequency ranges - Agree on FR1 and FR2-1

Agreement from RAN4 #113 R4-2420103 [6]

Issue 1-1: Deployment scenarios < Agreement>: - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID

Agreement from RAN4 #114bis R4-2504911 [7]

## Issue 1-1: Deployment scenarios < Agreement>: - Skipping requirements applies for NR-SA

Considering the agreements above, we propose that test cases are develop for NR-SA in FR1 and FR2-1.

RAN4 to define test cases for NR-SA.

RAN4 to define test cases for FR1 and FR2-1.

One open issue on type of gaps is [2]:

Issue 2-2-3: Per UE and per FR gaps <Way forward> Discuss the following options - Option 1: Test cases are specified only for per-UE MGs - Option 2: Test cases are specified for both per-UE and per-FR MGs

Regarding the open issue on per UE and per FR gaps, we recommend RAN4 to prioritize per-UE MG test cases and deprioritize per-FR MG test cases. This is because per-FR MG test cases depend on optional UE capability and hence these test cases are not applicable for all UEs.

Test cases are specified only for per-UE MGs.

Another open issue on test case list is [2]:

Issue 2-2-4: Type 1 and Type 2 gaps <Way forward> Discuss the following options - Option 1: Test cases are defined for both Type 1 and Type 2 gaps - Option 2: Test cases are defined only for Type 1 gaps

Regarding the discussion on Type 1 and Type 2 gaps, we recommend RAN4 to define the test cases for both Type 1 and Type 2 gaps. However, the test coverage for Type 2 gaps need not be as broad as Type 1 gaps and RAN4 should focus on only non-DRX test cases for Type 2 gaps.

Test cases are defined for both Type 1 and Type 2 gaps.

RAN4 to define the test cases for both Type 1 and Type 2 gaps but should focus on only non-DRX test cases for Type 2 MGs.

The open issue about DRX is shown below [2]:

Issue 2-2-5: DRX proposals <Way forward> Discuss the following proposals: - Proposal 1: Define the test cases for the cases without DRX and with DRX. - Proposal 2: Test cases without DRX should have higher priority than with DRX. - Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG. - Inter-RAT with DRX and with Type 1 MG can be developed with lower (second) priority. - Focus on non-DRX test cases for Type 2 MGs. - Proposal 3: Each type of test case is specified for either DRX or non-DRX but not both.

On the DRX aspects in finalizing the test case list, we need to consider that RAN4 is defining XR core requirements for both with and without DRX configurations. Since DRX configuration increases the XR data latency, higher priority should be assigned to test cases without DRX configuration. However, test cases with DRX should be considered at least for inter-frequency with Type 1 gaps whereas inter-RAT with Type 1 gaps can be considered as second priority. Considering the proposals of issue 2-2-5, we prefer Proposal 2. We are fine with Proposal 1, as we think it is necessary to have test cases with and without DRX, but broader test case coverage for non-DRX cases is needed.

Test cases without DRX configuration should have higher priority than with DRX configuration.

      - Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG

      - Develop TCs for inter-RAT with DRX and with Type 1 MG

      - Focus on non-DRX test cases for Type 2 MG

An open issue on target of what should be verified in RRM test is shown below [2]:

Issue 2-2-6: Test target <Way forward> - RRM test cases should verify - Proposal 1: accuracy requirements - Proposal 2: delay extension with gap cancelation - Proposal 3: Reception/transmission without scheduling restrictions during cancelled gap occasions - Proposal 4: The UE processing timeline of receiving the DCI-based MG skipping indication

To finalize the test target, way forward [2] captures four aspects which XR RRM test cases should verify. We think that the accuracy requirements can be tested in accuracy specific tests for which a single test case for each FR1 and FR2 should be sufficient. On the other hand, test cases for measurement procedure can cover the remaining testing aspects, i.e., the ability of the UE to adapt the measurement delay in accordance with the number of cancelled measurement gap occasions, UE transmitting or receiving data without scheduling restrictions during cancelled gap occasions, and UE processing timeline of receiving the DCI-based MG skipping indication.

RRM test cases should verify

      - Accuracy requirements

      - Delay extension with gap cancelation

      - Reception/transmission without scheduling restrictions during cancelled gap occasions

      - The UE processing timeline of receiving the DCI-based MG skipping indication

The accuracy requirements are tested in limited number of accuracy specific test, including

      - Inter-frequency measurements with Type 1 gaps, without DRX in FR1

      - Inter-frequency measurements with Type 1 gaps, without DRX in FR2-1

      - Inter-RAT E-UTRAN TDD with Type 1 gaps, without DRX

Issue 2-2-7: List of test cases <Way forward> - Define test cases for - Proposal 1: Intra-frequency measurements with gaps - Proposal 2: Inter-frequency measurements with gaps - Proposal 3: Inter-RAT E-UTRAN FDD measurements - Proposal 4: Inter-RAT E-UTRAN TDD measurements - Proposal 5: Inter-frequency L1-RSRP with MG - Proposal 6: L3 and L1 measurements without gaps

Regarding the proposals on defining the test cases in issue 2-2-7 of [2], we think RAN4 should define test cases for: (a) Intra-frequency measurements with gaps, (b) Inter-frequency measurements with gaps, (c) Inter-RAT E-UTRAN FDD or TDD measurements, and (d) Inter-frequency L1-RSRP with MG.

RAN4 should define test cases for

      - Intra-frequency measurements with gaps,

      - Inter-frequency measurements with gaps,

      - Inter-RAT E-UTRAN TDD measurements,

      - Inter-frequency L1-RSRP with MG.

The following is open for discussion regarding test cases configuration [2]:

Sub-topic 2-3 – Test case configuration Issue 2-3-1: Type 2 test configurations <Way forward> - Proposal 1: For Type 2 MGs test cases, include 2 gaps with partial overlap. - Proposal 2: For Type 2 MGs test cases, reuse measurement gap configuration of A.6.6.18.2.1, i.e. - Low priority MG pattern #1 with MGRP=40ms, offset=39ms - High priority MG pattern #2 with MGRP=80ms, offset=4ms - Other options are not precluded

To finalize the Type 2 test configuration, it is important to develop test cases which cover the combination of concurrent gap dropping rules together with XR measurement gap cancelation. For example, if the test case is considering 2 non-overlapping gap patterns, the expected behaviour would be similar to skipping of type 1 gap occasions. Therefore, we propose that 2 partially overlapping MG patterns are used for testing Type 2 gaps. One example can be to reuse the configuration of A.6.6.18.2.1 for the partially overlapping scenario, i.e. MGRP=40ms, offset=39ms for the MG with low priority, and MGRP=80ms and offset = 4ms for the MG with high priority.

For Type 2 MGs test cases, include 2 gaps with partial overlap

For Type 2 MGs test cases, measurement gap configuration of A.6.6.18.2.1 can be reused, i.e.,

      - Low priority MG pattern #1 with MGRP=40ms, offset=39ms

      - High priority MG pattern #2 with MGRP=80ms, offset=4ms

Measurement Gap Cancelation Model

The following is open for discussion regarding gap cancellation model [2]:

Sub-topic 2-4 – MG cancelation model Issue 2-4-1: Cancelation model for XR test cases <Way forward> - Proposal 1: Measurement gap cancelation is determined randomly by the test equipment. - Proposal 2: For Type 2 gaps, the measurement gap cancelation ratio is configured independently for each configured gap pattern. - Other options are not precluded.

As part of the Rel-16 NR-U performance work, a model for CCA failure was developed for the test cases. This model was developed because NR-U core requirements included limitation on the number of samples during a measurement cycle over which the requirements don’t apply.

The discussion on the conditions for measurement gap cancelation is still open for the RRM core requirements. In case the limitation on the maximum number of skipped occasions is not specified by RAN4, then the measurement cancelation model can be very simplified in comparison to the NR-U model.

NR-U CCA model for RRM test cases includes limitation on the maximum number of samples with non-successful CCA.

Measurement gap cancelation is determined randomly by the test equipment.

Furthermore, since 2 gap patterns can be configured with Type 2 MGs, and the measurement delay is different for each MG pattern, it is proposed that the cancelation model can apply a different cancelation ratio for each gap pattern. In that manner, we can verify that the cancelation of one gap pattern is not influencing the measurement delay of the MOs related to the other gap pattern.

For Type 2 gaps, the measurement gap cancelation ratio is configured independently for each configured gap pattern.

Issue 2-4-2: Cancelation ratio for Type 1 gaps <Way forward> - Option 1: - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset - Option 2: Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests. Issue 2-4-2: Cancelation ratio for Type 2 gaps <Way forward> - Option 1: For Type 2 MGs test cases, configure cancelation ratio as: - For MG pattern #1 with MGRP=40ms, offset=39ms, cancelation ratio of 0% - For MG pattern #2 with MGRP=80ms, offset=4 ms, cancelation ratio of 50% for UE supporting 5ms Toffset and 40% for UE supporting 3ms Toffset - Other options are not precluded

In [8] we have provided simulation results of ratio of satisfied UEs vrs gap cancellation ratio. As part of this analysis, we have shown that in order to achieve a good XR capacity, the cancellation ratio is in the order of 40% to 50%. Additionally, the a larger XR cancelation ratio is needed for UEs that supports 5 ms Toffset in comparison to UE supporting 3 ms Toffset. Therefore, we propose as a baseline, considering our previous simulation results, to adopt a cancelation ratio of 50% for UEs supporting 5 ms Toffset and 40% for UEs supporting 3 ms Toffset.

For Type 1 MGs test cases, RAN4 should agree on Option 1, i.e.,

- Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset

- Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

Figure 5 – Static scenario: Ratio of satisfied UEs vs skipping ratio for Alt 1-1 when TSO={1, 3, 5} ms. Ratio of satisfied UEs has been measured when cell load is equal to 7 UEs/cell (i.e., the XR capacity of ‘w/o MG’ when PDB=10ms).

The second issue regarding cancellation ratio is about Type 2 gaps. Since it is expected that the measurement delay of concurrent gaps is extended individually for each gap pattern, we should verify that the measurement delay of one gap pattern is not affected by measurement cancellation of gap occasion of another pattern. One each way of making this modification is by applying 0% cancellation for one concurrent gap pattern, and 50%, or 40% cancellation ratio for the second gap pattern.

For Type 2 MGs test cases, RAN4 should agree on Option 1, i.e., configure cancelation ratio as:

      - For MG pattern #1 with MGRP=40ms, offset=39ms, cancelation ratio of 0%.

      - For MG pattern #2 with MGRP=80ms, offset=4 ms, cancelation ratio of 50% for UE supporting 5ms Toffset and 40% for UE supporting 3ms Toffset.

DCI Aspects

Sub-topic 2-5 – DCI aspects Issue 2-5-1: Toffset between DCI and start of MG <Way forward> - Option 1: Define test cases for both 3ms offset and 5ms offset. - Option 2: Define test cases for 5ms offset

Regarding the several open issues related to DCI aspects, the first issue is to decide the Toffset between DCI and start of MG during the test for which there are two proposals. The first proposal is to define test cases for both 3ms offset and 5ms offset whereas the other proposal is to define test cases for 5ms offset only.

In our opinion, the workload to define test cases for both 3ms offset and 5ms offset is the same as that for defining test cases for 5ms offset only, it is just a matter of configuration of the test case. Furthermore, to ensure that UEs declaring 3ms Toffset capability are respecting their declared processing timeline of receiving the DCI-based MG skipping indication, such UEs need to be tested with 3ms offset only. Otherwise, there will be no guaranteed minimum performance corresponding to 3ms capability.

Define test cases for both 3ms offset and 5ms offset.

Since test case can cover both 3ms offset and 5ms offset with different configuration depending on UE capability.

Issue 2-5-2: Types of DCI with cancelation indication <Way forward> - Proposal 1: In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancelation. - Other options not precluded

Considering the types of DCI for test case definition, it is not clear why we need to cover more than 1 DCI type. From RRM functionality perspective we don’t define different UE behaviour depending on the DCI format. The most important aspect to be considered in this case is regarding UE capabilities. As shown below, the XR gap canceling feature includes a field for the UE to declare support of additional DCI, where formats 0_1 and 1_1 are mandatory.

enableTx-RxDuringMeasGap-r19 Indicates whether the UE supports reception/transmission during measurement gaps/scheduling restrictions is enabled based on explicit indication by DCI format 0_1 and DCI format 1_1. The capability signalling of this feature includes the following: - additionalDCI-r19 indicates additional supported DCI formats for enabling reception/transmission in gap/restriction. Value ‘dci0-1and1-2’ indicates DCI format 0_2 and DCI format 1_2, value ‘dci0-3’ indicates DCI format 0_3, value ‘dci1-3’ indicates DCI format 1_3; - indicationField-r19 indicates UE behaviour for indication field value of “0”. Value ‘option1’ means that, for explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. Value ‘option2’ means that, for explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 behaviour; - minimumTimeOffset-r19 indicates the minimum time offset from the end of PDCCH carrying the gap/restriction cancellation to the start of cancelled gap/restriction. Value ‘ms5’ indicates 5ms, value ‘ms3’ indicates 3ms. If a UE indicates support for the reception of the indication with DCI format 0_3 and/or 1_3, the UE supports reception of the indication on a band with that DCI format if the UE also indicates support for that DCI format on that band.

Considering the RAN2 UE capability, it is proposed that only 1 mandatory DCI format is tested, and for simplification we propose to UE DCI 1-1.

Define RAN4 test cases using DCI 1-1 for the canceling indication.

Issue 2-5-3: Behaviour of Option 1 and Option 2 for indication of value ‘0’ <Way forward> - Proposal 1: RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Other options not precluded

Regarding the open issue of testing the behaviour of Option 1 and Option 2 for indication of value ‘0’, we think that the testing of Option 2 is expected in every test case. However, for testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability. This will ensure that test coverage is broad while keeping the similar workload.

RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows:

- Test Option 2 in every test case.

- For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability.

Conclusion

The following Observations and Proposals were made:

- NR-U CCA model for RRM test cases includes limitation on the maximum number of samples with non-successful CCA.

- RAN4 to agree on developing XR measurement gap cancelation test cases provided in Table 1 below cases to ensure enough test coverage while keeping a reasonable RAN4 workload.

| Gap type | Intra/Inter/Inter-RAT | Frequency Range | DRX | Test type |
| --- | --- | --- | --- | --- |
| Type 1 gap per UE | Intra-frequency | FR1 | OFF | Meas procedure |
|  |  | FR2-1 | OFF | Meas procedure |
|  | Inter-frequency | FR1 | ON | Meas procedure |
|  |  |  | OFF | Meas procedure + accuracy |
|  |  | FR2-1 | ON | Meas procedure |
|  |  |  | OFF | Meas procedure + accuracy |
|  | Inter-RAT E-UTRAN TDD | FR1 (serving) | ON | Meas procedure |
|  |  |  | OFF | Meas procedure + accuracy |
| Type 2 gaps per UE | Intra-frequency | FR1 | OFF | Meas procedure |
|  |  | FR2-1 | OFF | Meas procedure |
|  | Inter-frequency | FR1 | OFF | Meas procedure |
|  |  | FR2-1 | OFF | Meas procedure |
| L1 RSRP measurements with gaps | Inter-frequency | FR1 | OFF | Meas procedure |
|  |  | FR2-1 | OFF | Meas procedure |

RAN4 to define test cases for NR-SA.

RAN4 to define test cases for FR1 and FR2-1.

Test cases are specified only for per-UE MGs.

Test cases are defined for both Type 1 and Type 2 gaps.

RAN4 to define the test cases for both Type 1 and Type 2 gaps but should focus on only non-DRX test cases for Type 2 MGs.

Test cases without DRX configuration should have higher priority than with DRX configuration.

  - Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG

  - Develop TCs for inter-RAT with DRX and with Type 1 MG

  - Focus on non-DRX test cases for Type 2 MG

RRM test cases should verify

  - Accuracy requirements

  - Delay extension with gap cancelation

  - Reception/transmission without scheduling restrictions during cancelled gap occasions

  - The UE processing timeline of receiving the DCI-based MG skipping indication

The accuracy requirements are tested in limited number of accuracy specific test, including

  - Inter-frequency measurements with Type 1 gaps, without DRX in FR1

  - Inter-frequency measurements with Type 1 gaps, without DRX in FR2-1

  - Inter-RAT E-UTRAN TDD with Type 1 gaps, without DRX

RAN4 should define test cases for

  - Intra-frequency measurements with gaps,

  - Inter-frequency measurements with gaps,

  - Inter-RAT E-UTRAN TDD measurements,

  - Inter-frequency L1-RSRP with MG.

For Type 2 MGs test cases, include 2 gaps with partial overlap

For Type 2 MGs test cases, measurement gap configuration of A.6.6.18.2.1 can be reused, i.e.,

  - Low priority MG pattern #1 with MGRP=40ms, offset=39ms

  - High priority MG pattern #2 with MGRP=80ms, offset=4ms

Measurement gap cancelation is determined randomly by the test equipment.

For Type 2 gaps, the measurement gap cancelation ratio is configured independently for each configured gap pattern.

For Type 1 MGs test cases, RAN4 should agree on Option 1, i.e.,

- Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset

- Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

For Type 2 MGs test cases, RAN4 should agree on Option 1, i.e., configure cancelation ratio as:

  - For MG pattern #1 with MGRP=40ms, offset=39ms, cancelation ratio of 0%.

  - For MG pattern #2 with MGRP=80ms, offset=4 ms, cancelation ratio of 50% for UE supporting 5ms Toffset and 40% for UE supporting 3ms Toffset.

Define test cases for both 3ms offset and 5ms offset.

Since test case can cover both 3ms offset and 5ms offset with different configuration depending on UE capability.

Define RAN4 test cases using DCI 1-1 for the canceling indication.

RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows:

- Test Option 2 in every test case.

- For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability.

- References

- RP-250107, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia, Qualcomm

- R4-2512144, WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2509068, Topic summary for [116][225] NR_XR_Ph3, Moderator (Nokia)

- 3GPP TS 38.133, Requirements for support of radio resource management (Release 19).

- R4-2416864, WF on RRM requirements for NR_XR_Ph3, RAN4#112-bis, Nokia

- R4-2420103, WF on RRM requirements for NR_XR_Ph3, RAN4#113, Nokia

- R4-2504911, WF on RRM requirements for NR_XR_Ph3, RAN4#114-bis, Nokia

- R4-2504505, Analysis of skip ratio on XR capacity and mobility performance, RAN4#114bis, Nokia