3GPP TSG-RAN WG4 Meeting #116bis									                     R4- 2514265

Prague, Czech, 13th – 17th Oct. 2025

Agenda item:			7.7.4

Source:	Ericsson

Title:	Discussion on UAV UE RF requirements

Document for:	Discussion

# Introduction

A new WI on enhancement of NR RF and RRM requirements for UAV was approved in RAN#109 meeting. The objectives of the core part of the WI are shown below [1]:

- Scenario: - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km. - A direct radio link between BS on the ground and UAV UE. - Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM. - Specify RRM requirements, if needed Note 1: No RAN1 impact are expected

This paper presents some preliminary views and observations on UAV UE RF requirements.

# Discussion

## Tx Requirements

According to the stated objectives, the maximum conductive output power to be considered in this WI is up to 33 dBm, which exceeds any existing any power class defined in the current specification. Current UE maximum output power of PC1 is 31 dBm with +2/-3 dB tolerance. There is an ongoing discussion of the high-power UE for NR, and maximum output power is up to 32 dBm [2]. RAN4 is yet to determine whether to introduce new power class.

For ATG UEs, the maximum output power of ATG UE is vendor-declared via UE capability signaling, ranging from 23 dBm to 40 dBm. This flexibility suits ATG’s sparse deployment, high altitude (Vertical distribution between 3 km and 10 km) where interference is less of a concern due to low density.

Compared to the ATG network, UAV deployment scenario involves lower coverage height (up to 600 – 1000 m), higher deployment density, potentially many drones in urban and suburban areas. Though, the assumption of the number of UAV UEs served from the UAV BS is yet to be defined (e.g. maybe 5 or more per cell), such elevated densities amplify aggregate interference risks, including uplink noise rise from multiple UAVs transmitting at higher powers (up to 33 dBm), and inter-drone sidelobe effects in antenna array configurations. These factors highlight key questions in defining the UAV’s maximum output power, out of band emission and other Tx parameters.

- Compared to the ATG network, UAV scenario involves lower coverage height, higher deployment density.

- Consider further how to specify the maximum output power for UAV UE.

An increase in output power level generally leads to higher adjacent channel leakage. Therefore, additional measures and evaluation are needed to ensure the coexistence of the UAV operated through ground BS with other services in-band and in adjacent bands. Such services are FSS, RAS, etc.

The ACLR requirements should be determined according to the outcome of co-existence analysis.

- Further evaluate the ACLR, SEM and spurious emissions, considering the co-existence and regulations.

## Rx Requirements

The situation of the out-of-band blocking differs from a ground-based UE, and it may not be similar to ATG UE either, as overall shape, weight for the UAV under study is unknow. Currently, there is no one standard of classification used in the industry. According to the US DoD, the UAV with service height below 3500 ft (1067 m) belongs to Group 2, medium size UAV weight between 9.5 -25 kg [4]. EASA divides UAS (unmanned aerial system) operation into three main categories: open (low risk), specific (medium risk) and certified (high risk). The category is determined by factors such as operation environment, drone weight, proximity to people, flight altitude, etc. Furthermore, UAV may contain other type of radio transmitters that are yet to know. Therefore, OOBB requirement needs further investigation.

As stated in the previous section, the UAV deployment scenario differs from that of the ATG network. Therefore, several receiver requirements require further study, a few examples listed below:

- OOBB requirements

- The maximum input level considering the worst-case scenario yet to be defined.

- Adjacent channel selectivity

- The OOBB requirements, maximum input level needs further investigation and the adjacent channel selectivity should be specified based on the co-existence analysis.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	Compared to the ATG network, UAV scenario involves lower coverage height, higher deployment density.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Consider further how to specify the maximum output power for UAV UE.

Proposal 2	Further evaluate the ACLR, SEM and spurious emissions, considering the co-existence and regulations.

Proposal 3	The OOBB requirements, maximum input level needs further investigation and the adjacent channel selectivity should be specified based on the co-existence analysis.

# References

- RP-252908, New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), Moderator (Huawei).

- RP-252952, New WID: UE RF enhancements for NR FR1, Phase 5, Moderator (vivo).

- TR 38.876, Air-to-ground network for NR.

- U.S Army Unmanned aircraft system roadmap 2010-2035.