3GPP TSG-RAN WG4 Meeting#116                                R4-2514271

Prague, CZ, 13th – 17th, Oct. 2025

Agenda item:	6.8.4.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on the OTA testing scope reduction

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. For the OTA testing scope reduction, we would like to share the further views based on the latest agreement in [4].

Issue 1-2-1: Out of band blocking - Agreement on step size of OOBB requirement, keep the following options for further discussion: - Option 1: 1/3 of the minimum supported bandwidth - Option 1a: Frequency range (MHz); Minimum supported BS channel bandwidth (MHz); Measurement step size (MHz). FR1; 5; 1. 10; 3. 20; 6. - Option 2: use 2/3 of the test signal configuration channel bandwidths Frequency range (MHz); Minimum supported BS channel bandwidth (MHz); Measurement step size (MHz). FR1; 5; 3. 10; 6. 20; 13.

Discussion

- 2.1. RX out of band blocking

It is well known that RX out of blocking requirement is another most time consuming requirement from receiver perspective in addition to transmitter spurious emission requirement. We also have similar discussion for FR2 BS OOBB requirement in terms of step size for CW signal. The corresponding technical justification is captured in TR 38.817-02. For FR1 BS OOBB requirement testing, the same principle could be reused to reduce the amount of time used for the time-consuming OTA conformance testing, while continue to ensure that any spurious response within the receive bandwidth will still be verified with sufficient granularity.

Extracted from TR38.817-02 The base station out-of-band blocking requirement is tested with a CW interferer and the wanted uplink signal. A spurious response occurs when the m-th harmonic of the interferer frequency mixes with the n-th harmonic of an internal signal, like a clock or a local oscillator signal, and this mixing product falls within the receive bandwidth of the wanted signal (or near the receive bandwidth, if the receiver selectivity is insufficient to suppress the spurious mixing product). The frequency of the unwanted mixing product equals ±m·fCW±n·finternal, where fCW is the interferer frequency and finternal is the frequency of the internal signal involved in the mixing product. m can assume the values 1, 2, 3, … and n can assume the values 0, 1, 2, …. Spurious responses for which the harmonic number m of the interferer signal equals 4 or more are usually not significant. Hence, the frequency step size should be chosen such that any spurious response due to a maximum third harmonic of the interferer will be captured within the receive bandwidth of the wanted signal. To reduce the amount of time used for the time-consuming OTA conformance testing, while continue to ensure that any spurious response within the receive bandwidth will still be verified with sufficient granularity, the measurement step size for interfering signal at frequency above 6 GHz can be set according to the minimum supported BS channel bandwidth as shown in Table 10.6.2-1 in the NR BS type 2-O OTA out-of-band blocking conformance testing. For interfering signal at frequency below 6 GHz, the traditionally measurement step size of 1 MHz can be kept. Table 10.6.2-1: Measurement step size for out-of-band blocking conformance testing Minimum supported BS Channel bandwidth of wanted signal [MHz]; 50; 100; 200; 400. Measurement step size for interfering signal [MHz]; 15; 30; 60; 60.

As shown in the following figure 6, if we want to ensure the ±3·fCW±n·finternal falling within receiver bandwidth, the step size of CW signal should be less than 1/3 of BW.

Figure 6. the illustration of relationship between step size for CW signal and receiver bandwidth for testing

If we go with option 2 as captured in the WF [8], it is possible that IMD between 3rd harmonic signal of testing CW signal and internal clock cannot been falling within the carrier bandwidth.

Proposals 1: propose to reuse the principle design for Rel-15 FR2 BS OOBB CW step size e.g. 1/3 of BW as captured in TR 38.817-02.

Conclusions

In this contribution, we shared some initial views on OTA test scope cases in RAN4 and proposals are made as following:

Proposals 1: propose to reuse the principle design for Rel-15 FR2 BS OOBB CW step size e.g. 1/3 of BW as captured in TR 38.817-02.

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 1 |
|  | 10 | 3 |
|  | 20 | 6 |

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- RP-240430,Views on BS RF topics for Rel-19, Huawei, approved.

- R4-2419827	Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP
[4] R4-2419828	Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP

- R4-2419243	Test case reduction for TX IM, Huawei

- R4-1709451   Conducted transmit intermodulation for NR BS range 1, ZTE

- R4-2504740	Way Forward for [114bis][304] NR_BS_RF_Part3_OTA_TRP, ZTE, approved.

- R4-2508658   Way Forward for [115][304] NR_BS_RF_Part3_OTA_TRP, approved.

- R4-2512580  Way Forward for [115][304] NR_BS_RF_Part3_OTA_TRP_final