3GPP TSG-RAN WG4 Meeting #116bis	R4-2514290

Prague, Czech Republic, 13-17 October, 2025

Source: 	Qualcomm Incorporated

Title: 	Views on Intra-band non-collocated NR-CA Phase 2 UE Demod requirements

Agenda item:	 6.2.3

Document for:	Agreement

- Introduction

According to the WID in RP-242843, the objectives of the WI on the Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2 relevant to the performance demodulation part are the following:

- Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed.

  - RF power imbalance requirements of up to 25dB

[…]

- Specify the corresponding performance requirements for type 4a/4b capable FWA UE.

  - PDSCH demodulation performance requirements

According to the objectives above, we submit with this document our company’s proposals for the work to be done in RAN4.

# Demodulation Requirements scope for Intra-Band non-collocated NR CA UE Type 4

Existing requirements for this feature in are defined in TS 38.101-4, Section 5.2A.2.6, and they verify the ability to demodulate the signal transmitted by the PCell and SCell of an FR1 intra-band non-contiguous NR-CA UE in non-colocated scenarios. Existing requirements are measured on both PCell and SCell simultaneously. The “stronger” CC is configured with Rank 2 and the “weaker” CC is configured with Rank 1, resulting in an SNR difference of 24.2dB.

Observation 1: Existing requirements for NR UE supporting non-colocated intra-band NR CA are defined with rank equal [2,1] for the strong and weak CC respectively, The SNR imbalance between the two CCs is 24.2dB.

According to the objectives included in the WID, we propose to introduce PDSCH demodulation requirements for NR UEs Type 4 which support Rank up to 4 Layers. for the higher power CC.

Proposal 1: RAN4 to introduce NR-CA PDSCH demodulation simulation for Type 4 UEs supporting intra-band non-collocated.

Proposal 2: RAN4 not to introduce EN-DC PDSCH demodulation simulation for Type 4 UEs supporting intra-band non-collocated.

Proposal 3: If new PDSCH demodulation requirements for Type 4 UEs supporting intra-band non-collocated NR-CA are introduced, they should be applicable only if the UE indicates support for the intraBandNR-CA-non-collocated-r19 features and providing nonCollocatedTypeNR-CA-v19xy with value “type4”.

Furthermore, it is our understanding tha existing requirements are tested only for CCs spaced by at least 80 MHz. This should be included in the RAN5 test conditions for this requirement, and it is aligned with the same condition for other UE RF requirements for this feature. For reference, the LS from RAN4 to RAN5 in R4-2316951 contains the following test condition:

- Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC in R18.

Proposal 4: RAN4 should confirm with RAN5 that the existing test condition for intraBandNonColocatedCA-r18 which defines a minimum gap of 80MHz for the two CCs is applicable to new requirements in this WI, if introduced.

## Simulation Assumptions

With respect to the assumptions for the simulation results to be collected with this test, we believe that the test setup can be inherited from the existing requirements, with the introduction of the higher maximum rank as supported by the Type 4 UEs introduced in this WI.

Proposal 5: Simulation assumptions should align with the existing requirements in TS 38.101-4, Section 5.2A.2.6 where applicable.

Proposal 6: The Rank to be considered for initial evaluation should be Rank 4 for the “stronger” CC, and Rank 2 for the “weaker” CC.

Proposal 7: The final choice of MCS and Rank should consider a maximum SNR difference of 25dB.

- Conclusions

Observation 1: Existing requirements for NR UE supporting non-colocated intra-band NR CA are defined with rank equal [2,1] for the strong and weak CC respectively, The SNR imbalance between the two CCs is 24.2dB.

Proposal 1: RAN4 to introsduce NR-CA PDSCH demodulation simulation for Type 4 UEs supporting intra-band non-collocated.

Proposal 2: RAN4 not to introduce EN-DC PDSCH demodulation simulation for Type 4 UEs supporting intra-band non-collocated.

Proposal 3: If new PDSCH demodulation requirements for Type 4 UEs supporting intra-band non-collocated NR-CA are introduced, they should be applicable only if the UE indicates support for the intraBandNR-CA-non-collocated-r19 features and providing nonCollocatedTypeNR-CA-v19xy with value “type4”.

Proposal 4: RAN4 should confirm with RAN5 that the existing test condition for intraBandNonColocatedCA-r18 which defines a minimum gap of 80MHz for the two CCs is applicable to new requirements in this WI, if introduced.

Proposal 5: Simulation assumptions should align with the existing requirements in TS 38.101-4, Section 5.2A.2.6 where applicable.

Proposal 6: The Rank to be considered for initial evaluation should be Rank 4 for the “stronger” CC, and Rank 2 for the “weaker” CC.

Proposal 7: The final choice of MCS and Rank should consider a maximum SNR difference of 25dB.