3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514291

Prague, CZ, 13th – 17th October 2025

Source:	Qualcomm Incorporated

Title:	RF specifications for PC2 and PC1.5 FRMCS devices

Agenda item:	5.5.2

Document for:	Approval

# 1.	Introduction

In RAN#108 a new WID [1] to define RF specifications for fixed-wireless/vehicle-mounted (FWVM) devices was created. In RAN4#116 a WF was agreed [2]. In this paper we further present our views on this topic.

# 2. 	Discussion

This WID [1] stipulates the creation of RF specifications for handheld UE, FWA and FWVM (fixed-wireless/vehicle mounted) applications in bands n100 and n101 as shown below.

The core objectives of the WI are to develop RF requirements that are applicable to different UE power classes (PC), for different UE devices types (handheld, FWA (Fixed Wireless Access), FWVM (fixed-wireless/vehicle-mounted)) for a single TDD or a single FDD band as described below.

- Objective 1.  For TDD band: PC1.5 and PC2 for handheld UE and FWA, and PC2, PC1.5 and PC1 for FWVM

- Objective 2.  For FDD band: PC2 for handheld UE and FWA, and PC2 and PC1 for FWVM

Note: The bands with the default power class shall be introduced before the high power UE is requested for this band in this present WI.

Redcap UEs are out-of-scope of this WI (there is a separate REL-19 WI NR_PC2_RedCap_UE addressing this case).

According to the WI noted above and the WF [2] the major objectives of this work are to create RF specifications for PC2 in n100 and both PC1.5 and PC2 in n101 for FRMCS cab-radios.

Based on the ECC decision (20)02 [3] the FRMCS minimum ACLR requirement for PC2 and PC1.5 is 37dB. This means that the FRMCS ACLR requirement needs to be 6dB higher than that for the equivalent NR power classes. In NR the ACLR specification for PC1 is 37dBm. Due to the 37dB ACLR requirement it was decided in the WF [2] to use a PC1 PA as a reference design.

Observation 1: FRMCS has a 6dB larger ACLR requirement compared to NR for PC2 and PC1.5

In the last RAN4 meeting 2 options for MPR requirements were discussed. It was decided that either PC1 MPR would be used for FRMCS PC1.5 and PC2 or the UE PC1.5 and PC2 MPR values would be used for the corresponding FRMCS power classes.

However, further thought on this reveals that it may not be possible to directly use the UE PC1.5 and PC2 MPR values as the UE ACLR is only 31dB which is 6dB lower than for FRMCS.

Observation 2: May not be possible to use the UE PC1.5 and PC2 MPR values as the UE ACLR is only 31dB which is 6dB lower than for FRMCS.

For FRMCS applications the PA bias is fixed at the PC1 values, and the output power is reduced to achieve PC1.5 and PC2 power levels. For this type of operation, it is not necessary to define MPR requirements for PC1.5 and PC2 as the PA is operating as a PC1 PA. Also, any attempt to attempt to define PC1.5 and PC2 MPRs using this larger more linear device may lead to overly optimistic results as PAs tailored for working at either PC1.5 or PC2 would generally be less linear than a PC1 device which in turn may lead to smaller MPRs.

Proposal 1: For the FRMCS application where a PC1 PA is used there is no need to define MPRs for PC1.5 and PC2.

Observation 3: MPRs defined for PC1.5 and PC2 using a PC1 PA may not represent the actual MPRs of PAs tailored specifically for PC1.5 or PC2 operation.

It is understood that FRMCS inter-band operation at PC1.5 or PC2 is desired. This can be achieved by defining CA inter-band combinations with power levels of PC1.5 or PC2 and reducing the PC1 power of each PA so that the sum of the powers from both CCs equals the required inter-band power.

Observation 4: For FRMCS inter-band operation at PC1.5 and PC2 the PC1 power levels of each PA can be reduced so that the sum of the powers from each CC equals the desired inter-band power value.

# Conclusion

In this paper we present further thoughts on defining Tx RF specifications for fixed-wireless/vehicle-mounted (FWVM) devices operating in bands n100 and n101 and make the following observations and proposals.

Observation 1: FRMCS has a 6dB larger ACLR requirement compared to NR for PC2 and PC1.5

Observation 2: May not be possible to use the UE PC1.5 and PC2 MPR values as the UE ACLR is only 31dB which is 6dB lower than for FRMCS.

Proposal 1: For the FRMCS application where a PC1 PA is used there is no need to define MPRs for PC1.5 and PC2.

Observation 3: MPRs defined for PC1.5 and PC2 using a PC1 PA may not represent the actual MPRs of PAs tailored specifically for PC1.5 or PC2 operation.

Observation 4: For FRMCS inter-band operation at PC1.5 and PC2 the PC1 power levels of each PA can be reduced so that the sum of the powers from each CC equals the desired inter-band power value.

# References

[1] RP-251402, Revised WID: Rel-19 High power UE (power class 1.5 and 2) for NR FR1 TDD/FDD single band for handheld/FWA UEs, and high-power UE operation (power class2, 1.5 and 1) for FWVM (fixed-wireless/vehicle-mounted) use cases in a single NR band, June 9th – 13th , 2025

[2] R4-2511718, WF on introduction of PC1.5 and 2 in band n100 and n101, August 25th-29th , 2025

[3] Harmonised use of the paired frequency bands 874.4- 880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR), ECC Decision (20)02, 28th June 2024