3GPP TSG-RAN WG4 Meeting #116bis	R4-2514308

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	RRM core requirements for AI/ML Based Beam Management

Agenda item:	6.11.4

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#116 in the scope of Rel-19 WI NR_AIML_air ‎[1]. Agreements achieved during RAN4#116 meeting are summarized in the WF ‎[2].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion. In this paper, we provide some additional views on issues related to RRM core requirements, as listed below:

- Performance metric for AI/ML BM

- Prediction reporting delay

- LCM and performance monitoring related requirements

- Conditions for known TCI state

Discussion

Performance metric for AI/ML BM

Measurement error impact and the ground truth

During RAN4#112 and RAN4#112bis meetings, RAN4 agreed to study the impact of measurement errors to AI/ML BM functionality on the performance of prediction of AI/ML BM. The agreements of RAN4 #113 are provided below:

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training (set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. - impact on different prediction accuracy metrics should be taken into account - studies already performance by other groups (RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

Agreement: ​ RAN4 to perform study to assess the impact of measurement errors on prediction accuracy​ Perform LLS to evaluate the RSRP prediction accuracy and other KPIs​ - UE Model trained using ideal measurements​ Simulation assumptions in R4-2417211 can be used as starting point.

The updated simulation assumptions [3] were agreed in RAN4#116 meeting. Based on these simulation assumptions, the simulation results for KPI1 are provided in Table 1. These simulations consider independent RF errors for each Tx beam. As shown in the table, KPI1 is evaluated for up to two highest ground-truth RSRP beams, i.e. for N=1 and 2, from the predicted Top-K beam set. KPI1 for N = 2 in the table is obtained by simple aggregation of the correct predictions of the two highest ground-truth RSRP beams among the predicted Top-K beam set. Specifically, the computation follows a complete step-by-step procedure for calculating beam ID prediction accuracy metric detailed in section 2.1.2.

| K | N | X = 0 |  |  |  | X = 1 |  |  |  | X = 2 |  |  |  | X = 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Case 1 | Case 3 |  |  | Case 1 | Case 3 |  |  | Case 1 | Case 3 |  |  | Case 1 | Case 3 |  |  |
|  |  |  | Alt1 | Alt2 | Alt3 |  | Alt1 | Alt2 | Alt3 |  | Alt1 | Alt2 | Alt3 |  | Alt1 | Alt2 | Alt3 |
| 1 | 1 | 75.64 | 65.88 | 54.37 | 53.06 | 84.05 | 75.27 | 62.36 | 60.94 | 88.36 | 81.88 | 69.26 | 67.84 | 90.80 | 86.31 | 75.27 | 73.93 |
| 2 | 1 | 90.80 | 84.82 | 76.74 | 75.70 | 93.86 | 88.91 | 81.66 | 80.76 | 95.37 | 91.41 | 85.56 | 84.79 | 96.40 | 93.10 | 88.50 | 87.97 |
|  | 2 | 70.39 | 78.45 | 69.24 | 60.54 | 75.79 | 83.69 | 75.18 | 67.50 | 79.44 | 87.20 | 80.05 | 73.37 | 82.24 | 89.67 | 83.81 | 78.27 |
| 3 | 1 | 94.82 | 91.23 | 85.62 | 84.83 | 96.65 | 93.78 | 89.13 | 88.48 | 97.52 | 95.33 | 91.84 | 91.34 | 98.09 | 96.34 | 93.75 | 93.41 |
|  | 2 | 80.98 | 87.64 | 80.63 | 74.67 | 85.47 | 91.09 | 85.27 | 80.49 | 88.52 | 93.23 | 88.80 | 84.99 | 90.61 | 94.64 | 91.34 | 88.39 |
| 4 | 1 | 96.67 | 94.70 | 90.50 | 89.92 | 97.84 | 96.35 | 93.12 | 92.61 | 98.44 | 97.28 | 95.01 | 94.61 | 98.78 | 97.95 | 96.35 | 96.00 |
|  | 2 | 86.39 | 92.47 | 86.93 | 82.58 | 89.69 | 94.82 | 90.60 | 87.31 | 91.98 | 96.16 | 93.18 | 90.72 | 93.48 | 97.08 | 94.98 | 93.18 |

Table 1: Evaluation of KPI1 for N =1, 2 in Scenario1

From the results, we observe a clear decrease in prediction accuracy when more than one best beam is considered in KPI1 evaluation. This means, for N=2, the accuracy of predicting second best beam is lower in comparison to predicting only the best beam, and that brings down the aggregate success rate in comparison to N=1. Since RAN4 conformance testing must ensure good UE performance in the field, we think a reasonable beam Id prediction accuracy of around 90 percentage can be used to narrow down on the agreeable values for K, N and X.

- RAN4 to consider using K = 4, N = 2 and 0≤X<2 dB as agreeable parameter values for KPI1 of beam prediction accuracy of AI/ML BM use case.

Performance metrics

The following agreement related to RSRP prediction accuracy was reached in RAN4#116 meeting:

Agreement (RAN4 #116) - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided.

The above agreement, from RAN4#116, clearly confirms the applicability of RSRP accuracy requirements to the top-K beams whenever RSRP prediction is reported. Importantly, it highlights the relevance of verifying the accuracy of all the top-K reported beams. Hence the prediction performance of any AI/ML BM model, irrespective of whether it predicts RSRPs or ONLY Beam Ids, must inherently consider all the reported top-K beams in its performance evaluation.

- Performance evaluation of beam prediction functionality considers all the reported top-K beams.

Since this agreement captures the applicability of accuracy requirements for one of the two cases where RSRPs are reported, as mentioned in the below agreement from RAN4#114, RAN4 still needs to agree on the complete description of the performance metric and its accuracy requirements for the other case when only beam Ids are reported. Here, Observation1 ideally should lead to similar agreement where the prediction accuracy evaluation considers all the top-K predicted beams even when the BM functionality reports only beam IDs.

The following agreements related to beam ID prediction accuracy were reached in the past RAN4 meetings:

Agreement (RAN4 #114) - If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options - If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

Agreement (RAN4 #114bis) - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values - Note: if x=0 and N=1, it means Top-K/1 (%): the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

As explained in our previous TDoc [4], scheduling and load-balancing constraints in the field, in general, mandate the NW to configure a UE for reporting more than one best beam. Hence, the minimum meaningful value of the number of reported predicted beams is K=2.

As per the agreement in RAN4#114bis, presented above, the description of Beam Id Prediction metric considers strongest N beams among the top-K predicted beams while evaluating the success rate for the correct prediction. Considering this along with Observation 1 and 2, it can be inferred that 2≤N≤K

Also, the RAN4#114bis agreement describes the condition for correct prediction only for the maximum ground-truth RSRP beam among the top-K predicted beams. Because the agreement further includes top-N (N</=K) predicted beams to be considered in the success rate evaluation, defining a clear condition to determine the correct prediction of other beams within the predicted top-K beam set is necessary for completing the description of this metric. At this, we identify that a fair evaluation requires comparing ground-truth RSRPs of equal ranked beams among both the predicted and measured top-K beam sets, where the ranking of beams in both the sets is based on ground-truth RSRPs.

Elaborating further, in order to determine if the ith ranked predicted beam is indeed a correct prediction, RSRP of ith ranked predicted beam, yi is compared with the RSRP of the corresponding ith ranked genie-aided beam, yi relaxed by x dB. Here, x remains fixed for all i.  In addition to using a fair evaluation criterion, having a constant threshold, x for each of these N highest ground-truth RSRP beams within the predicted top-K beam set, is also attributed to the fact that measurement errors impact all beams uniformly. Finally, the condition that needs to be checked for the ith ranked beam is Ci=lyi≥yi-x.

- Prediction of ith ranked predicted beam is deemed a successful event, if RSRP of ith ranked predicted beam, yi is greater than or equal to the RSRP of the corresponding ith ranked genie-aided beam, yi-x

- A complete step-by-step procedure to compute the beam ID prediction accuracy metric is described as follows:

- Sort the predicted set of Top-K beams in the order of measured RSRPs. So, as per this, rank-1 beam among the predicted Top-K beam set is the one with highest measured RSRP

- Sort Top-K measured beam set in the order of measured RSRPs

- Initialize aggregate success rate, s=0

- for i=1, 2,…,N, where N≤K

  - Compute success rate of ith ranked beam, si

    - Compare measured RSRP of rank-i predicted beam, yi with the measured RSRP of Top-i measured beam yi, where a successful correct prediction has this difference contained within a tolerance margin of x dB.

- si=lyi-yi≤x

  - Compute the aggregate success rate, s

    - s=s+ si

- s=sN (N is averaging factor)

In summary, aggregate success rate for an inference instance is computed as s=1Ni=1Nlyi-yi≤x

From the above observations and the detailed procedure for computing the beam ID prediction metric, we present the following proposals for discussing it during the upcoming RAN4 meeting:

- RAN4 to discuss and agree on the FFS components of the RAN4#114bis agreement on the beam Id prediction metric considering the following:

- At least N strongest ground-truth RSRP beams among the top-K predicted beams must be included in the success rate evaluation, where 2≤N≤K

  - RAN4 should consider N=2, if there is no consensus in RAN4 for N>2

- Correct prediction of these N beams is evaluated using a fixed value of x

- RAN4 to discuss and incorporate the following changes to complete the description of the beam Id prediction metric:

- The correct prediction of ith ranked predicted beam, si, is considered as the ground truth RSRP of the ith ranked predicted beam larger than or equal to the ground truth RSRP of the ith ranked genie-aided beam – x dB, where i=1, 2,…,N. The ranking of beams within both the top-K predicted and genie-aided beam sets is based on ground-truth RSRPs

- si=lyi-yi≤x

- Compute the success rate,  s, by aggregating the correct prediction of the strongest N beams among the top-K predicted beam set

s=1Ni=1Nsi

Prediction reporting delay

The following agreement was reached in RAN4#116 meeting for prediction reporting delay:

- Agreement: - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay - The reporting delay includes the inference delay - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion

Furthermore, following agreements were reached in RAN4#116 meeting for measurement period for inference:

Agreement: observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report) For case 1 reuse M,N,P from legacy measurement requirements For case 2, use T*M, N P (M,N,P same as legacy measurement requirement) FFS on T value, T can also be 1. T can also be based on capability This observation period is to be used for the prediction delay requirement Agreement: - UE has to be ready to start measurements for inference after sending RRC reconfiguration complete - Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay - This applies to all reporting schemes

The below table shows the measurement period for L1-RSRP reporting in legacy.

Table 9.5.4.2-2: Measurement period TL1-RSRP_Measurement_Period_CSI-RS_BP_Rel19 for FR2

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS_BP_Rel19 (ms) |
| --- | --- |
| non-DRX | max( TReport, ceilM*P*N*TCSI-RS ) |
| DRX cycle ≤ 320ms | max( TReport, ceil 1.5*M*P*N)*max⁡( TDRX, TCSI-RS) |
| DRX cycle > 320ms | ceilM*P*N* TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. Treport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

Since, clause 9.5.4.2 of TS 38.133 includes both measurement and reporting delay, our view is that no separate requirements are needed for prediction reporting delay and measurement period for inference.

- There is no need to have separate requirements for prediction reporting delay and measurement period for inference for AI/ML BM use cases.

We would like to address as well the FFS on the value of T in the last meeting. We think that the value of T can be considered 1 for all types of reports i.e. periodic, semi-persistent and aperiodic.

- For periodic and semi-persistent:

- When timeRestrictionForChannelMeasurement is configured, the parameters M=1 and T can also be 1.

- However, when timeRestrictionForChannelMeasurement is not configured, the parameter M=3, T=1

- For aperiodic: CSI-RS resources M=1, T=1.

Regarding the value of T in the delay requirement for predicted L1-RSRP, RAN4 should consider T=1 for all types of reports i.e. periodic, semi-persistent and aperiodic.

Therefore, we suggest to not consider T in the requirements. The proposed delay requirement for predicted L1-RSRP TL1-RSRP_Measurement_Period_Prediction_CSI-RS is given in the Table below:

Table 9.5.4.X: Measurement period for prediction TL1-RSRP_Measurement_Period_Prediction_CSI-RS for FR2

| Configuration | TL1-RSRP_Measurement_Period_Prediction_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N)* max(TCSI-RS, Tproc)+Tinference |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P*N)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. NOTE 4: [Tinference is the inference delay]. |  |

Regarding inference delay (Tinference), following two options may be considered:

Option 1: The inference delay (Tinference) is negligible. In this case, the legacy requirements will be valid for predicted L1-RSRP as well.

Option 2: If the inference delay (Tinference) is not negligible. In this case, RAN4 needs to further discuss the value of Tinference .

In case if the inference delay is negligible, RAN4 should consider legacy delay requirements for the predicted L1-RSRP as well.

In case if the inference delay is not negligible for the definition of the delay requirement for predicted L1-RSRP, RAN4 needs to further discuss and agree on the value of the inference delay.

LCM and performance monitoring related requirements

Performance monitoring related requirements

The following agreement was reached in RAN4#116 meeting:

Agreement: - Do not define performance monitoring delay requirements for LCM purpose - If the monitoring delay is identificated as critical in the future, this issue can be revisited. No agreements on performance monitoring accuracy.

Agreement (RAN 1 #120b)

For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |  |
| SP report | Not support | Support | Support |  |
| P report | Support | Support | Support |  |

Agreement (RAN 1 #120bis)

- For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB

Agreement (RAN1 #119)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

RAN1 achieved following agreement for performance monitoring mechanism and metric:

RAN1#121 Agreement

For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N)

- Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring

  - condition:

    - for the transmssion occasion of monitoring resources, it has a linked inference report

    - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference

  - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance.

- Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig

- the size of CSI field associated with the RS-PAI is log2(N+1)

RAN1 has agreed on the definition of report quantity of beam prediction accuracy, RS-PAI, which can be defined as the total count of accurate reference signal prediction instance(s), Np, that meets the conditions and Np is calculated over N valid performance monitoring instances. The need to determine the N latest transmission occasion(s) of monitoring resources is considered.

- Accordingly, RAN4 will need to define new accuracy requirements for the performance monitoring metric for AI/ML BM, RS-PAI, corresponding to number of N latest transmission occasion(s) of monitoring resources. It is critical to verify the accuracy of this performance monitoring metric in RAN4, since many network sided decision regarding life-cycle management of AI/ML configuration depend on this metric. Furthermore, performance degradation can be caused if the accuracy of the performance monitoring metric is not guaranteed since the UE may continue using a badly performing AI/ML configuration.

It is clear from the RAN1 definition of the report quantity of the beam prediction accuracy report for monitoring that the performance monitoring metric is different from the beam prediction accuracy metrics considered in RAN4 as part of the performance requirements. Therefore, a separate accuracy requirement is needed in RAN4 to verify the accuracy of performance monitoring metric, RS-PAI.

RAN4 needs to define accuracy requirements for UE-assisted monitoring where the UE reports the beam prediction accuracy, RS-PAI, in BM-Case1 and BM-Case2. The accuracy of RS-PAI should be tested at least in static radio conditions.

Conditions for known TCI state

The following agreement was reached in RAN4#116 meeting:

Agreement: - On Detectability and SNR conditions: - The UE has sent at least 1 L1-RSRP inference report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB - Time conditions: - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement - Agreement on QCL relationship - If the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known. If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability.

Based on the above agreement, we suggest the following changes in clause 8.10.2 (Known conditions for TCI state). These changes are also captured in our proposed draft CR.

8.10.2	Known conditions for TCI state

The TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state

-	TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or beam prediction reporting or measurement

-	The UE has sent at least 1 L1-RSRP report or 1 L1-RSRP inference report for the target TCI state before the TCI state switch command

-	The TCI state remains detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state ≥ -3 dB

- The predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B.

Otherwise, the TCI state is unknown.

Note: [If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability].

The suggested changes in the draft CR (R4-2514311) should be endorsed to update known conditions for TCI state in clause 8.10.2 of TS 38.133.

Conclusion

In this paper we share our views on RAN4 impacts from issues related to RRM core requirements for AI/ML enabled BM-Case1 and BM-Case2. Specifically, we cover following aspects:

- Performance metric for AI/ML BM

- Prediction reporting delay

- LCM and performance monitoring related requirements

- Conditions for known TCI state

In the paper, the following Observations and Proposals were made:

Observation 1:	Performance evaluation of beam prediction functionality considers all the reported top-K beams.

Observation 2: Prediction of ith ranked predicted beam is deemed a successful event, if RSRP of ith ranked predicted beam, yi is greater than or equal to the RSRP of the corresponding ith ranked genie-aided beam, yi-x

Proposal 2: RAN4 to discuss and agree on the FFS components of the RAN4#114bis agreement on the beam Id prediction metric considering the following:

- At least N strongest ground-truth RSRP beams among the top-K predicted beams must be included in the success rate evaluation, where 2≤N≤K

  - RAN4 should consider N=2, if there is no consensus in RAN4 for N>2

- Correct prediction of these N beams is evaluated using a fixed value of x

Proposal 3: RAN4 to discuss and incorporate the following changes to complete the description of the beam Id prediction metric:

- The correct prediction of ith ranked predicted beam, si, is considered as the ground truth RSRP of the ith ranked predicted beam larger than or equal to the ground truth RSRP of the ith ranked genie-aided beam – x dB, where i=1, 2,…,N. The ranking of beams within both the top-K predicted and genie-aided beam sets is based on ground-truth RSRPs

- si=lyi-yi≤x

- Compute the success rate,  s, by aggregating the correct prediction of the strongest N beams among the top-K predicted beam set

s=1Ni=1Nsi

Observation 3:	There is no need to have separate requirements for prediction reporting delay and measurement period for inference for AI/ML BM use cases.

Proposal 4: Regarding the value of T in the delay requirement for predicted L1-RSRP, RAN4 should consider T=1 for all types of reports i.e. periodic, semi-persistent and aperiodic.

Proposal 5: In case if the inference delay is negligible, RAN4 should consider legacy delay requirements for the predicted L1-RSRP as well.

Proposal 6: In case if the inference delay is not negligible for the definition of the delay requirement for predicted L1-RSRP, RAN4 needs to further discuss and agree on the value of the inference delay.

Proposal 7: RAN4 needs to define accuracy requirements for UE-assisted monitoring where the UE reports the beam prediction accuracy, RS-PAI, in BM-Case1 and BM-Case2. The accuracy of RS-PAI should be tested at least in static radio conditions.

Proposal 8: The suggested changes in the draft CR (R4-2514311) should be endorsed to update known conditions for TCI state in clause 8.10.2 of TS 38.133.

- References

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2511890, “WF on NR_AIML_air_part1”, Qualcomm Incorporated, RAN4#116, Bengaluru, India, 25th ‒ 29th August, 2025.

- R4-2511797, “WF on updated simulation assumptions for AIML BM”, vivo et al., RAN4#116, Bengaluru, India, 25th ‒ 29th August, 2025.

- R4-2509932, “RRM performance requirements for beam management and positioning accuracy”, Nokia., RAN4#116, Bengaluru, India, 25th ‒ 29th August, 2025.