3GPP TSG-RAN4 Meeting #116-bis	R4-2514328

# Prague, Czechia, 13rd - 17th October, 2025

Agenda item:		7.3.3

Source:				Keysight Technologies UK Ltd

Title:					On NTN OTA Testability Aspects for Ka/Ku Bands

Document for:		Approval

# Introduction

This contribution is discussing various testability topics for NTN OTA testing for Ka/Ku Bands. This topic was agreed to be studied within the new Rel-20 WI [1], i.e.,

Objectives for this core part work item are as follows - Define enhanced test methodologies to cover NTN Ka and Ku bands - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority - FR1 OTA test method extension is not precluded, if also applicable - Study whether and how to support circular polarization measurements - Linear polarization is the baseline - Specify the required quiet zone to cover VSAT device type based on industry needs - 60cm QZ as a starting point - Specify preliminary MU assessment for test methodologies

# Discussion

In the following, the undefined testability aspects for radiated Tx/Rx characteristics from 38.101-5 [2] are highlighted with observations and proposals presented.

#### OTA Test Condition

The applicability of requirements to conducted vs radiated testing is defined in Table 4.2-1 of [2] which clearly indicates that any NTN testing beyond 10 GHz is performed in a radiated environment.

Table 4.2-1: Requirement set applicability Requirement; Requirement set. FR1-NTN bands; FR2-NTN bands. Below 10 GHz with antenna connector; Above 10 GHz. Conducted transmitter characteristics; Clause 6; NA; NA. Conducted receiver characteristics; Clause 7; NA; NA. Conducted performance requirements; Clause 8; NA; NA. Radiated transmitter characteristics; NA; Clause 9; Clause 9. Radiated receiver characteristics; NA; Clause 10; Clause 10. Demodulation performance requirements (Radiated requirements); NA; Clause 11; Clause 11.

However, it has not been defined anywhere in [2] that measurements shall be performed in the far field as it was done for FR2, e.g., Clause 3.1 of TS 38.101-2 [3]

TRP(Link=TX beam peak direction, Meas=TRP grid): measurement of the TRP of the UE such that the measurement angles are aligned with the directions of the TRP grid points within an acceptable measurement uncertainty while the link angle is aligned with the TX beam peak direction NOTE: For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region

Observation 1: While NTN testing beyond 10 GHz is defined in a radiated environment, the need for far-field conditions has not been specified yet

Since the same principles should apply to NTN as to TN testing, it is proposed to include in [2] that the measurements shall be performed with FF conditions.

Proposal 1: Include a statement in TS 38.101-5 [2] that for radiated requirements, the radiated interface boundary is associated to the far-field region as stated in TS 38.101-2 [3]

#### Test Methodology: IFF vs DFF

From a test methodology perspective, the definition of permitted systems needs to be addressed similar to what was done originally for FR2 in the testability SI [4] and later captured in [5].

While the size of VSAT antennas has not explicitly been mentioned anywhere in [2], it is mentioned that NTN VSATs commonly leverage electronic or mechanical beam steering, e.g., Clause 9.2.1.0 of [2]

For the hybrid beam steering capable NTN VSAT, which can adjust its antenna(s) or beam(s) in both electronic steering and mechanical steering ways, the applicable requirements shall follow either electronic or mechanical beam steering requirements depending on the NTN VSAT type it declared. The NTN VSAT types are specified in Table 9.2.1.0-1 below. Table 9.2.1.0-1: The definitions of NTN VSAT Types NTN VSAT class; NTN VSAT type; Type description. Fixed VSAT; 1; Fixed VSAT communicating with GSO and LEO with mechanical steering antenna.. 22; Fixed VSAT communicating with GSO and LEO with electronic steering antenna.. 3; Fixed VSAT communicating with LEO only with electronic steering antenna.. Mobile VSAT; 43; Mobile VSAT communicating with GSO and LEO with mechanical steering antenna.. 52, 3; Mobile VSAT communicating with GSO and LEO with electronic steering antenna.. 64; Mobile VSAT communicating with LEO only with electronic steering antenna.. NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to bands n512, n511, n510, n509, n508 and n248, n247; LEO support is only applicable to bands n509, n508 and n248, n247. NOTE 4: The NTN VSAT type 6 is only applicable to bands n509, n508 and n248, n247..

It can therefore be assumed that these antennas are significantly larger in size than those integrated in FR2 PC1 FWA UEs for which only the IFF methodology is permissible/feasible.

While the antenna size has not been mentioned explicitly, the antenna aperture size has been implicitly mentioned in Clause 10.8 of [2]

Table 10.8.1-1: Off-axis Co-polarized gain limit φ value (degree); gain (dBi). φmin ≤ φ ≤ 48°; 32 – 25log10(φ). 48° ≤ φ ≤ 85°; -10. 85° ≤ φ ≤ 180°; 0. NOTE: φmin = 1o or 100λ/D (degrees) whichever is the greater, for D/λ≥ 50. φmin = 2o or 114(D/ λ)-1.09 (degrees) whichever is the greater, for D/λ 50. where D is the nominal diameter of the antenna. The receiver antenna off-axis gain of each cross-polarized components in any direction φ degrees from the antenna main beam shall not exceed the levels specified in Table 10.8.1-2. Table 10.8.1-2: Off-axis Cross-polarized gain limit φ value (degree); gain (dBi). φr ≤ φ ≤ 7°; 23 – 20log10(φ). NOTE: φr = 1o or 100λ/D (degrees) whichever is the greater where D is the nominal diameter of the antenna.

Using the D/λ=50 condition, the corresponding FF condition based on the Fraunhofer distance of DFF=2D2/λ can be calculated which is presented in Table 1.

Table 1: Fraunhofer distance requirements for VSAT antenna sizes of D/λ=50

| D/l | Band | f [GHz] | l [m] | D [m] | DFF=2D2/l [m] |
| --- | --- | --- | --- | --- | --- |
| 50 | Ku | 10.7 | 0.028 | 1.40 | 140 |
| 50 | Ku | 14.5 | 0.021 | 1.03 | 103 |
| 50 | Ka | 17.3 | 0.017 | 0.87 | 87 |
| 50 | Ka | 30 | 0.010 | 0.50 | 50 |

Clearly, the antenna aperture estimates in Table 1 indicate that a direct FF (DFF) system is simply not feasible, i.e., the NTN VSAT OTA testing must be based on indirect FF (IFF) methodologies as the baseline similar to what was decided in FR2 [4][5].

Proposal 2: Define the IFF methodology as the baseline for NTN VSAT OTA testing

An example IFF test system based on the Compact Antenna Test Range (CATR), the de-facto industry standard for FR2 UE RF, demodulation, and RRM testing [4][5], is illustrated in Figure 1 for large(r) DUTs. Conformance tests which assess the end-to-end performance of UEs, e.g., EIRP/EIS/EVM/timing, are commonly using signalling tests with a communication tester/gNB. Here, the DUT is mounted on a 2-axis positioner within the quiet zone opposite the CATR reflector which transforms spherical waveforms into plane waves. The test equipment for mm-wave operation such as >10 GHz NTN VSAT primarily consists of a communication tester and remote radio head(s), connected to the feed probe, together with an interface unit that converts IF to RF and vice versa.

Figure 1: Example IFF test system based on CATR for end-to-end measurements

#### Positioner Handling and Quiet Zone Size

While the 60 cm QZ has been agreed as starting point in the WI [1], it is assumed that larger QZs are not precluded for some NTN VSAT device types. Unfortunately, the QZ size does not sufficiently specify important design aspects of NTN VSAT OTA test systems.

In order to determine the positioner requirements for OTA test systems, it is important to know the VSAT overall device size and weight for each of the NTN VSAT types defined in Clause 9.2.1.0 of [2].

Proposal 3: VSAT vendors/NTN operators to provide the maximum VSAT device size and weight limits for each NTN VSAT type

To determine the minimum required reflector sizes of Compact Antenna Test Ranges (CATRs), the most prominent implementation of IFF methodologies, for NTN VSAT testing it is furthermore important to know the maximum antenna aperture, i.e., the diameter of a sphere required to fully enclose the radiating elements of the VSAT. Since the antenna system of the VSAT must be contained entirely within the QZ and the reflector size is directly proportional to the QZ size, the maximum antenna aperture is required to properly design suitable NTN VSAT OTA systems.

Proposal 4: VSAT vendors/NTN operators to provide maximum antenna aperture for each NTN VSAT type

If there is a large discrepancy between the maximum antenna aperture and the maximum VSAT device size, the question of black box, white box, and grey box testing might have to be discussed as well [8], [9].

Observation 2: If there is a large discrepancy between the maximum antenna aperture and the maximum VSAT device size, the question of black box, white box, and grey box testing might have to be discussed

#### Measurement Grids for BP Searches, TRP, etc.

The transmitter power requirements clearly define the need for a Tx beam peak (BP) direction search, i.e., Clause 9.2.1.1 of [2], within a partial spherical area centred around the top pole.

The requirement should be verified with test metrics of EIRP (Link=Tx beam peak direction, Meas=Link angle).The peak EIRP of Tx beam peak direction should be verified within the declared minimum elevation angle supported for transmitting. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type. Where the supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 9.2.1.1-1 below is taken as an example. Figure 9.2.1.1-1 Example measurement grid for min peak EIRP with the declared supported minimum elevation angle

The minimum and maximum partial spherical areas within which the Tx BP must be verified, based on the range of supported minimum elevation angles above, are visualized in Figure 2, respectively.

Figure 2: Minimum (±15°) and maximum (±87°) partial surface area for Tx BP search

Similar to what was done for FR2 UE BP searches, the worst-case antenna assumptions, e.g., pattern, HPBW, array configuration, etc., are required for each of the NTN VSAT Types in Table 9.2.1.0-1 of [2] to assess the minimum required measurement grid spacing as well as the measurement uncertainty.

The requirements in Clause 9.2.1 of [2] also include TRPMAX. Its assessment also requires the knowledge of the antenna assumptions to define the required measurement grids, integration ranges, e.g., how far beyond the declared minimum elevation angle EIRPs must be recorded, and MU.

The off-axis EIRP emission density limits, defined in Clause 9.2.2 [2], require sampling of EIRP on the planes tangent or perpendicular to the GSO arc. Additionally, the antenna performance requirements in Clause 9.6.2 of [2] require gain evaluations in the plane tangent and perpendicular to the GSO arc. The measurement grid/angular step size for these measurements will have to be assessed based on antenna assumptions including beam steering capabilities of the antenna system.

Proposal 5: VSAT vendors to provide the worst-case antenna assumptions and beam steering assumptions for each of the NTN VSAT Types in Table 9.2.1.0-1 of [2] to assess the minimum required measurement angular grid spacing as well as the measurement uncertainty for BP searches, TRP, and off-axis EIRP emission density limits/gain measurements.

For Spurious Emissions, defined in Clause 9.5.3 [2], TRP measurements must be performed with the UL beam directed towards the in-band TX beam peak (BP) direction. To assess the required TRP measurement grid and the corresponding MUs, the worst-case assumption of the pattern of the spur must be defined as well, as done in Clause M.4.5 of [7] for FR2 UE RF.

Proposal 6: Define the worst-case antenna assumptions of the spurs for each of the NTN VSAT Types in Table 9.2.1.0-1 of [2] to assess the minimum required measurement grid spacing as well as the measurement uncertainty for the spurious emissions TRP.

#### Automatic Beam Steering/Beam Lock Test Function

As discussed earlier, the expectation is that VSATs employ electronic or mechanical beam steering for normal operation. It is not clear whether these VSATs are expected to automatically steer the UL&DL beams towards the DL direction inside the OTA test system where the DL direction corresponds to the IFF probe (reflector direction) connected to the DL port of the communication tester. Clarification from VSAT vendors/NTN operators is requested.

Proposal 7: VSAT vendors/NTN operators to clarify whether VSATs automatically steer the UL&DL beams towards the DL direction inside the OTA test system where the DL direction corresponds to the IFF probe (reflector direction) connected to the DL port of the communication tester.

If VSATs indeed support automatic beam steering inside OTA test systems, the test procedures for EIRP/EIS measurements could be closely aligned with those currently used for FR2 UE RF testing. In that case, a test mode would be required for TRP testing to lock the beam in the beam peak direction to avoid the beam continuously changing its UL beam, similar to the UE Beamlock test Function (UBF), defined in Clause 5.4 of [6].

Proposal 8: If VSATs support automatic beam steering towards the DL probe in the OTA test system, a beam lock test function required for TRP measurements must be defined and supported by VSAT vendors.

If VSATs do not support automatic beam steering inside OTA test systems, the OTA test system, specifically the SW test executive, will require constant communication with the VSAT to make sure that the UL&DL beam directions are aligned with the measurement probe test direction for EIRP/EIS measurements. In that case, no beam lock test function is required. However, to reduce the development complexities and to optimize test times of OTA test systems, a standardized beam steering interface between VSATs and test system integrators is highly desirable.

Observation 3: If VSATs do not support automatic beam steering inside OTA test systems, a standardized beam steering interface between VSATs and test system integrators is highly desirable to reduce the development complexities and to optimize test times

#### Untestable Requirements in OTA Test System

The following requirements are untestable in conventional OTA test systems:

- Pointing Stability, see Clause 9.6.1.2.2 [2]: the following environmental conditions (“Under the condition of 100 km/h maximum wind speed, with gusts of 130 km/h lasting 3 seconds”) cannot be emulated in an OTA test system. This requirement would require a separate environmental test environment and is not commonly part of radiated conformance test requirements

Proposal 9: Define the pointing stability as a system-level requirement rather than a radiated conformance test requirement as it cannot be tested in conventional OTA test systems

#### Antenna vs end-to-end Performance Testing: Non-Signalling vs Signalling

While most performance metrics in Clauses 9 and 10 are end-to-end performance metrics that rely on antenna, radio, and baseband, Clause 9.6.2 defines antenna performance metrics

### 9.6.2 Antenna performance #### 9.6.2.1 Minimum requirements for bands n510 and n511 The following requirements are applicable to NTN VSAT type 1, type 2, type 4 or type 5 operating in band n511 or in band n510 and communicating with Geostationary Satellite Orbit (GSO) SAN. The co-polarization gain in the plane tangent to the GSO arc shall not exceed the levels specified in Table 9.6.2.1-1. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy. Table 9.6.2.1-1: Co-polarization gain limit in the plane tangent to the GSO arc θ value; Co-polarization gain (dBi). 2° ≤ θ ≤ 7°; 29 – 25log10(θ). 7° ≤ θ ≤ 9.2°; 8. 9.2° ≤ θ ≤ 19.1°; 32 – 25log10(θ). 19.1° < θ ≤ 180°; 0. The co-polarization gain in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2.1-2. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy. Table 9.6.2.1-2: Co-polarization gain limit in the plane perpendicular to the GSO arc θ value; Co-polarization gain (dBi). 3.5° ≤ θ ≤ 7°; 32 – 25log10(θ). 7° ≤ θ ≤ 9.2°; 10.9. 9.2° ≤ θ ≤ 19.1°; 35 – 25log10(θ). 19.1° < θ ≤ 180°; 3.

that are generally independent of radio and baseband. It should also be highlighted that the antenna only performance is generally not part of conformance test requirements as the UE performance is highly dependent on the end-to-end performance, e.g., a high-performance antenna coupled with a poorly performing radio/BB can fail system level requirements.

Antenna gain has traditionally been an internal OEM design requirement to meet the conformance test requirements and has not been specified as part of certification/conformance requirements in 3GPP previously. Defining the antenna gain as done in Clause 9.6.2 [2] is therefore rather unconventional and the justification for these measurements is not clear. From a test system vendor perspective, these measurements can certainly be supported but the question is whether these requirements are merely system level requirements that do not have to be verified or whether they shall be verified as part of the conformance assessment of NTN VSAT devices.

Proposal 10: NTN Operators/VSAT vendors to clarify whether the antenna gain requirements in Clause 9.6.2 are merely system level requirements or whether they shall be verified as part of the conformance assessment of NTN VSAT devices

It should furthermore be noted that the test frequencies of the antenna gain measurements are missing; it should also be clarified whether they are applicable only to the UL (assumed) or UL&DL.

Proposal 11: Clarify the test frequencies for the antenna gain measurements and whether they are applicable only to the UL or UL&DL

The antenna performance is commonly tested using a different test setup than the end-to-end conformance test, see Figure 1, i.e., for the gain measurement, a non-signalling test is performed with a coaxial cable connected to the input/output ports of the antenna system and the gain is measured using either a network analyser (default) or signal generator/spectrum analyser. An example non-signalling test setup, based on the CATR IFF setup in Figure 1, is shown in Figure 3. Here, the vector network analyser is connected to both the feed probe and the RF port of the antenna under test. Fortunately, common OTA test setups can readily accommodate both signalling and non-signalling testing, e.g., see Figure 4 for an example setup, where all the necessary test equipment is integrated inside an equipment rack which commonly also hosts a PC to control the positioning equipment as well as the test executive(s).

Figure 3: Example IFF test system based on CATR for antenna gain measurements

Figure 4: Example IFF test system based on CATR for conformance and antenna performance testing

Observation 4: Common OTA test setups can readily accommodate signalling (used for end-to-end performance metrics such as EIRP, EIS, EVM, timing, etc.) and non-signalling testing (using for antenna performance metrics such as gain)

It is desirable for VSAT vendors/NTN operators to confirm that the intention of the antenna performance gain metric is to assess the isolated antenna performance with a non-signalling test. Additionally, it should be confirmed that the conducted port(s) of the antenna system can and will be made available with the beam steering states controllable using a test interface, especially for antennas with electrical beam steering such as antenna arrays.

Proposal 12: VSAT vendors/NTN operators to confirm that the intention of the antenna performance gain metric is to assess the isolated antenna performance with a non-signalling test

Proposal 13: VSAT vendors/NTN operators to confirm that the conducted port(s) of the antenna system can and will be made available with the beam steering states controllable using a test interface for all NTN VSAT types

In Clause 9.6.2.2, an antenna performance requirement is defined for a specific signalling value, i.e.,

#### 9.6.2.2 Requirement for network signalling value “NS_203N” When "NS_203N" is indicated in the cell, the following requirements shall apply to NTN VSAT.

For a non-signalling test, this would require the test interface with the NTN VSAT antenna to control the antenna tuning as a function of signalling values.

Observation 5: An antenna non-signalling performance test for specific network signalling values would require the test interface with the NTN VSAT antenna to control the antenna tuning as a function of signalling values

Given the various observations and requests for clarification above, it is not absolutely clear whether antenna gain/gain patterns are sufficient for these spectrum mask measurements or whether the correct approach would be to measure EIRPs instead, e.g., were the antenna gain measurements in Clause 9.6.2 defined by mistake?!

Proposal 14: NTN Operators/VSAT vendors to clarify whether these spectrum mask measurements should be based on antenna gain patterns or rather EIRP

#### Linear vs Circular Polarization Testing

The technical specification does not specifically address the need for specific polarization state measurements for the signalling or non-signalling test cases. It merely discusses polarization in Clauses 9.1

## 9.1 General Unless otherwise stated, the transmitter characteristics are specified over the air (OTA) with a single or multiple transmit chains under either LHCP (Left Hand Circular Polarization) or RHCP (Right Hand Circular Polarization) or Linear Polarization.

and 10.2

## 10.2 Polarization characteristics The minimum requirements on the receiver characteristics apply under either LHCP (Left Hand Circular Polarization) or RHCP (Right Hand Circular Polarization) or Linear Polarization.

Unfortunately, the requirements for signalling and non-signalling test cases do not specify whether they apply to a specific CP state (RHCP, LHCP) or LP. Without any polarization mentioned/specified anywhere, the assumption is that these requirements apply to LP measurements such as all other FR2 or FR1 OTA requirements. Given the lack of polarization requirements, it is suggested to clearly define the applicability of the conformance test requirements with respect to polarization.

Observation 6: The lack of a specific applicability definition of the NTN VSAT radiated requirements to CP vs LP leads to the assumption that these requirements apply to LP like all other OTA requirements

Proposal 15: It is proposed to clarify the applicability of the conformance test requirements with respect to polarization.

The topic of the use of linear vs circular polarization for OTA conformance testing has been discussed over the years for FR2 TN and specifically FR1 NTN (below 7.125 GHz) testing [10], [11]. Given the test system complexities, linear polarization for OTA signalling test cases has been used exclusively.

Observation 7: For all existing OTA signalling conformance TN and NTN test cases, linear polarization has been defined exclusively

As outlined in our previous contributions [10], [11], the use of CP for NTN VSAT test cases typically increases the complexity of the underlying test systems and delays the availability of such systems which might be undesirable since this SI has been initiated after the requirements were defined.

Observation 8: The use of CP for NTN VSAT test cases typically increases the complexity of the underlying test systems and delays the availability of such system

However, especially for FR1 NTN below 7.125 GHz, the use of circular polarization has been emphasized for OTA test cases by NTN operators as highly desirable in the past [12]. However, for classical FR1 NTN below 7.125 GHz, it was decided to continue OTA testing with LP mainly due to majority of UEs with LP antennas and the test system concerns

Observation 9: For classical FR1 NTN below 7.125 GHz, it was decided to continue OTA testing with LP

Based on Keysight’s internal investigations over the last few months, we believe that the use of CP for NTN VSAT conformance test cases could be feasible with various testability aspects left to be discussed. However, the WID [1] already provides a clear guidance to use linear polarization as the baseline for conformance testing of NTN VSATs, i.e.,

- Study whether and how to support circular polarization measurements - Linear polarization is the baseline

It is therefore proposed to study whether select conformance metrics, e.g., peak EIRP/EIS, should be assessed using CP before formalizing that NTN VSAT OTA conformance testing shall leverage linear polarization exclusively.

Proposal 16: Study whether select conformance metrics, e.g., peak EIRP/EIS, should be assessed using CP before formalizing that NTN VSAT OTA conformance testing (signalling testing) shall leverage linear polarization exclusively

As outlined above, we realize NTN operators have a vested interest in the assessment of the circular polarization performance. We believe that the antenna gain assessment discussed in the previous section readily lends itself for CP performance evaluations (non-signalling) and thus allows NTN operators to determine the polarization characteristics of the antenna system. It is therefore proposed to perform the antenna gain test cases (non-signalling testing) using CP measurements.

Proposal 17: Perform the antenna gain test cases (non-signalling testing) using CP measurements.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: While NTN testing beyond 10 GHz is defined in a radiated environment, the need for far-field conditions has not been specified yet

Observation 2: If there is a large discrepancy between the maximum antenna aperture and the maximum VSAT device size, the question of black box, white box, and grey box testing might have to be discussed

Observation 3: If VSATs do not support automatic beam steering inside OTA test systems, a standardized beam steering interface between VSATs and test system integrators is highly desirable to reduce the development complexities and to optimize test times

Observation 4: Common OTA test setups can readily accommodate signalling (used for end-to-end performance metrics such as EIRP, EIS, EVM, timing, etc.) and non-signalling testing (using for antenna performance metrics such as gain)

Observation 5: An antenna non-signalling performance test for specific network signalling values would require the test interface with the NTN VSAT antenna to control the antenna tuning as a function of signalling values

Observation 6: The lack of a specific applicability definition of the NTN VSAT radiated requirements to CP vs LP leads to the assumption that these requirements apply to LP like all other OTA requirements

Observation 7: For all existing OTA signalling conformance TN and NTN test cases, linear polarization has been defined exclusively

Observation 8: The use of CP for NTN VSAT test cases typically increases the complexity of the underlying test systems and delays the availability of such system

Observation 9: For classical FR1 NTN below 7.125 GHz, it was decided to continue OTA testing with LP

Proposal 1: Include a statement in TS 38.101-5 [2] that for radiated requirements, the radiated interface boundary is associated to the far-field region as stated in TS 38.101-2 [3]

Proposal 2: Define the IFF methodology as the baseline for NTN VSAT OTA testing

Proposal 3: VSAT vendors/NTN operators to provide the maximum VSAT device size and weight limits for each NTN VSAT type

Proposal 4: VSAT vendors/NTN operators to provide maximum antenna aperture for each NTN VSAT type

Proposal 5: VSAT vendors to provide the worst-case antenna assumptions and beam steering assumptions for each of the NTN VSAT Types in Table 9.2.1.0-1 of [2] to assess the minimum required measurement angular grid spacing as well as the measurement uncertainty for BP searches, TRP, and off-axis EIRP emission density limits/gain measurements.

Proposal 6: Define the worst-case antenna assumptions of the spurs for each of the NTN VSAT Types in Table 9.2.1.0-1 of [2] to assess the minimum required measurement grid spacing as well as the measurement uncertainty for the spurious emissions TRP.

Proposal 7: VSAT vendors/NTN operators to clarify whether VSATs automatically steer the UL&DL beams towards the DL direction inside the OTA test system where the DL direction corresponds to the IFF probe (reflector direction) connected to the DL port of the communication tester.

Proposal 8: If VSATs support automatic beam steering towards the DL probe in the OTA test system, a beam lock test function required for TRP measurements must be defined and supported by VSAT vendors.

Proposal 9: Define the pointing stability as a system-level requirement rather than a radiated conformance test requirement as it cannot be tested in conventional OTA test systems

Proposal 10: NTN Operators/VSAT vendors to clarify whether the antenna gain requirements in Clause 9.6.2 are merely system level requirements or whether they shall be verified as part of the conformance assessment of NTN VSAT devices

Proposal 11: Clarify the test frequencies for the antenna gain measurements and whether they are applicable only to the UL or UL&DL

Proposal 12: VSAT vendors/NTN operators to confirm that the intention of the antenna performance gain metric is to assess the isolated antenna performance with a non-signalling test

Proposal 13: VSAT vendors/NTN operators to confirm that the conducted port(s) of the antenna system can and will be made available with the beam steering states controllable using a test interface for all NTN VSAT types

Proposal 14: NTN Operators/VSAT vendors to clarify whether these spectrum mask measurements should be based on antenna gain patterns or rather EIRP

Proposal 15: It is proposed to clarify the applicability of the conformance test requirements with respect to polarization.

Proposal 16: Study whether select conformance metrics, e.g., peak EIRP/EIS, should be assessed using CP before formalizing that NTN VSAT OTA conformance testing (signalling testing) shall leverage linear polarization exclusively

Proposal 17: Perform the antenna gain test cases (non-signalling testing) using CP measurements.

# References

- RP-252903, Enhancement of UE OTA test method and requirements for NR, RAN VC (vivo), 3GPP TSG RAN Meeting #109, September 2025

- TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements

- TS 38.101-2, User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone

- TR 38.810, Study on test methods

- TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment

- TS 38.509, Special conformance testing functions for User Equipment (UE)

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- R4-1708200, Black Box vs White Box Testing for NR, Rohde & Schwarz, 3GPP TSG-RAN4 Meeting #84, August 2017

- R5-211949, On Larger Quiet Zone Sizes with Grey Box, Keysight Technologies, 3GPP TSG-RAN5 Meeting #90-e, March 2021

- R4-2502177, On Circular Polarization for FR1 NTN OTA Conformance Testing, Keysight Technologies, Samsung, Nokia, Verizon Wireless, AT&T, Mediatek, ZTE, Ericsson, ETS-Lindgren, Rohde&Schwarz, OPPO, Xiaomi, CAICT, 3GPP TSG-RAN4 Meeting #114, February 2025

- R4-1904192, On Polarization Mismatch Impact, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #90bis, April 2019

- R4-2418826, OTA Conformance Testing with Circular Polarisation, Inmarsat, Viasat, 3GPP TSG RAN WG4 #113, November 2024

The QZ is the volume within which the radiating elements of the VSAT have to be placed and within which the amplitude and phase variations of the electromagnetic fields are optimized.