# 3GPP TSG-RAN WG4 Meeting # 116-bis					R4-2514366

# Prague, Czech Republic, 13 October – 17 October 2025

Source:		Ericsson

Title:			RedCap UE RF requirment impact

Agenda item:		7.4.3.1

Document for:		Approval

# Introduction

In this paper, we present our general overview on (e)RedCap RF impact analysis.

# Discussion

In WID[1], the below RF objective is stated:

- Specify the following requirements for (e)RedCap UEs [RAN4]

  - Relaxed RLM/BFD RRM requirements

    - Use R17 RLM/BFD relaxation for legacy UE as a baseline

  - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB)

    - RF requirements, considering example bands: n100, n106, n26, n28 and n85

    - RRM requirements including relaxed RLM/BFD RRM requirements

    - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline

  - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

The 3MHz channel bandwidth should be added and below clause is a summary on the impacted requirements:

Table 1:The impact clause to adding 3MHz on (e)RedCap UEs

| Impacted clause number | Suggest change |
| --- | --- |
| 5.3I Channel bandwidth for (e)RedCap | Remove the restriction on the 3MHz CBW applicability on (e)RedCap Consider if the note 4 should not be applied to (e)RedCap UE as this is Rel-20 feature and should be mandatory from Rel-20. “NOTE 4: This UE channel bandwidth is optional in this release of the specification.” |
| 5.4I Channel arrangement for (e)RedCap | No change |
| 6.2I Transmitter power for (e)RedCap | No change |
| 7.1I General | No change |
| 7.2 Diversity characteristics | No change |
| 7.3.2 Reference sensitivity power level | No Change , same with legacy on no support for four Rx antenna ports for (e)RedCap UE for some band |
| 7.3I.2 Reference sensitivity power level for RedCap | Example band in WID : n100, n106, n26, n28 and n85, the additional band supporting the 3MHz, n31, n72, all are FDD bands. Relaxing factor for 1 Rx for 3MHz for FDD band should be added in Table 7.3I.2-1. Also add 3MHz CBW in HD-FDD REFSENS table Table 7.3I.2-2 and Table 7.3I.2-3. Uplink table Table 7.3I.2-4 should be updated with adding 3MHz |
| 7.3I.3 Reference sensitivity power level for eRedCap | Add the 3MHz, can reuse the scaling factor in 7.3I.2. Only SCS 15kHz for example bands. |

From the table summary above, it can be observed that the discussion on the RF impact focus on the REFSENS clause for 3MHz CBW.

For REFSENS of 3MHz CBW, the 2 Rx REFSENS of for NR UE can be reused for 2 Rx (e)RedCap UE .  For 1 Rx(e)RedCap UE, the 1 Rx relaxing factor of 2.5 dB for 5MHz can be reused for 3MHz in FDD band.  For HD-FDD tightening factor, the WF [2] can be referenced and the same tightening with 5MHz REFSENS for 3MHz REFSENS can be applied assuming the same hardware. The related agreements are quoted as below:

Issue 3-2: 1 RX HD-FDD REFSENS scaling factor

- Agreement: Compared to 1Rx FDD REFSENS, apply 0.8 dB tightening for REFSENS requirement for 1Rx HD-FDD REFSENS as baseline in general, and check band by band whether the additional tightening or relaxation is needed for a certain band.

Issue 3-3: 2 RX HD-FDD REFSENS scaling factor

- Agreement: Compared to 2Rx FDD REFSENS, apply 0.8 dB tightening for REFSENS requirement for 2Rx HD-FDD REFSENS as baseline in general, and check band by band whether the additional tightening or relaxation is needed for a certain band.

1 Rx relaxing factor is 2.5 dB for 3MHz for FDD and same principle of HD-FDD tightening with Rel-17 RedCap.

| FD-FDD 3MHz/5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -99 dBm | 0 dB | n1, n18, n24, n30, n65, n66, n70, n74 |
| > -99 dBm and ≤ -98 dBm | 0.5 dB | n2, n5, n7, n28, |
| > -98 dBm | 0.8 dB | n3, n8, n12, n13, n14, n20, n25, n26, n71 |

During the last meeting maintenance discussion, there are some companies proposing the relaxation of the Rel-17 REFSENS for 1 Rx relaxing factor for CBW of 15MHz and 20MHz due to Tx leakage may be more prominent for the FDD band with shorter duplex distance. We think such further relaxation is not relevant for 3MHz as the CBW less so no further relaxation would be needed for 3MHz.

As the 3MHz CBW introduced in Rel-20, and there is no RAN1/2 impact, we think such feature can be release independent from Rel-17 for RedCap UE and Rel-18 for eRedCap UE for band supporting the 3MHz.

3MHz feature can be release independent from Rel-17 for RedCap UE and Rel-18 for eRedCap UE for band supporting the 3MHz

# Conclusions

In this contribution, we present our general overview for 3Mhz (e)RedCap RF impact observations and proposals:

Proposal-1: 1 Rx relaxing factor is 2.5 dB for 3MHz for FDD and same principle of HD-FDD tightening with Rel-17 RedCap.

Proposal-2: 3MHz feature can be release independent from Rel-17 for RedCap UE and Rel-18 for eRedCap UE for band supporting the 3MHz

# References

- RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple

- R4-2120067, WF on the RedCap RF, Ericsson