3GPP TSG-RAN WG4 Meeting #116bis	R4-2514372

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	Nokia

Title:	Discussion on remaining issues of Rel-19 Mobility Enhancements core part

Agenda item:	4.23.1

Document for:	Discussion

Introduction

In this contribution, we discuss the remaining RRM core requirements for LTM CSI-RS measurements and CLTM cell switch delay.

Discussion

CSI-RS based L1 measurements

CSI-RS acquisition

In the previous meeting, RAN4 agreed with the following guideline on CSI acquisition.

Agreement: - No consensus on defining new RAN4 requirements for early CSI acquisition. - Clarification on the applicability of the existing requirement is not precluded if consensus on the exact clarification can be reached. - It is RAN4 common understanding that for baseline early CSI acquisition, - If fine T/F tracking is included in cell switch delay, UE shall start CSI acquisition at least after UE finishes fine T/F tracking of target cell. - Otherwise, UE shall start CSI acquisition at least after Tprocessing. - No need to capture above agreement in RAN4 spec.

Our understanding is that the above agreement is quite important for the network as it relates to when to transmit the CSI acquisition RS in relation to the cell switch command. We understand that companies do not prefer to capture requirements related to the procedure in this release, but there should be at least a note clarifying the following:

Capture the following as a note in the specification: “Note: If fine T/F tracking is included in cell switch delay, UE is not expected to start CSI acquisition before the UE finishes fine T/F tracking of the target cell. Otherwise, UE is not expected to start CSI acquisition before the end of TLTM-processing.”

RAN1 sent an LS to RAN4 about CSI-RS frequency location regarding the CSI-Acquisition. The main message of the LS is following:

R4-2509007 / R1-2504828 Conclusion: From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for CSI acquisition for candidate cells

From the network point of view, it is ok to have no configuration restrictions. However, to avoid any surprises on the network side, it would be good to discuss in RAN4 whether the CSI acquisition procedure can be performed with partial reference signal or if full RS for CSI acquisition is needed.

In the case where optional CSI-RS acquisition is supported, the UE is expected to receive the CSI-acquisition RS before the cell switch command. In this case the UE has retuned to the serving cell. In case the CSI-RS is not UE specific, the RS may not fall fully within the active BWP from the serving cell point of view.

When the CSI-acquisition RS is after the cell switch command, the UE is tuned into initial BWP when the acquisition is after the TLTM-processing, in this case the RS may not be fully within the initial BWP.

Discuss whether there is need to define applicability condition for CSI acquisition depending on RS frequency location (serving cell active BWP / initial target cell BWP)

CLTM cell switch delay

CLTM requirements for candidate cells without active TCI states in FR2

The RAN4#114bis agreement about CLTM requirements for candidate cells without active TCI states in FR2 was revisited in RAN4#116 meeting, as quoted below, to make sure that at least interruption time requirements apply for the case where UE triggers a conditional cell switch to a candidate cell without active TCI state based on an L1 condition in FR2.

Issue 1-2-3: CLTM requirements for candidate cells without active TCI states in FR2 < Agreement > - Capture the following in the WF. - As starting point: L1 measurement behavior, which was introduced in R18, is not changed. CLTM measurement time requirements (Tmeasure) do not apply for the candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2. Other CLTM requirements (cell switch interruption, etc.) apply. - FFS whether and how to capture in the CR

The remaining open point regarding this agreement is whether and how to capture it in the specification. As the revisited agreement mainly affects the CLTM measurement time requirements, we propose to capture it under the measurement time section (6.3.2.2.1).

RAN4 to capture the agreement about CLTM requirements for candidate cells without active TCI states in FR2 under the measurement time section (6.3.2.2.1). The following wording can be used:

- “The requirements in this clause apply under the following conditions:

- 1> The target cell is an FR1 cell, or

- 1> The target cell is an FR2 cell, and

- 2> there is at least one TCI state on the active TCI state list of the target cell, or

- 2> there are no TCI states on the active TCI state list of any of the configured candidate cells.”

Tmeasure for SSB-based L1-RSRP measurement

In the last meeting, RAN4 agreed to cover only the known target cell case for CSI-RS-based L1-RSRP measurements. For SSB-based L1-RSRP measurements, the understanding is that both known and unknown target cell cases should be covered. However, in the latest version of the specification, only unknown target cell case is reflected in the measurement time requirements when an SSB-based L1-RSRP measurement is used in the event.

##### 6.3.2.2.1 Measurement time The requirements in this clause apply under the following conditions: 1> The target cell is known stated in clause 6.3.1 for CSI-RS L1-RSRP triggered CLTM. The measurement delay is defined from the end of TEvent_DU until UE starts conditional cell switch execution to a target cell. 1> If SSB based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of L1-RSRP measurement periods of the serving cell and target cell. 2> serving cell measurement period TL1-RSRP_Measurement_Period_SSB is defined in clause 9.5.4.1 assuming TReport = 0, and 2> if the target cell is an intra-frequency cell: 3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell, or 3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra + TSSB_time_index_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, otherwise. 2> if the target cell is an inter-frequency cell: 3> target cell measurement period is TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TL1-RSRP_Measurement_Period_SSB_inter as defined in clause 9.3.4 and clause 9.15.5 or 9.15.6 assuming TReport = 0, if deriveSSB-IndexFromCellInter-r17 is enabled or UE has reported SSB index in L3 measurement report of the same cell, or 3> target cell measurement period is TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TL1-RSRP_Measurement_Period_SSB_inter + TSSB_time_index_inter as defined clause 9.3.4 and in clause 9.15.5 or 9.15.6 assuming TReport = 0, otherwise.

We believe that both known and unknown target cell cases should be covered in the CLTM measurement time requirements (Section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event. This is not visible in the current requirement, which includes TPSS/SSS_sync_intra + TSSB_measurement_period_intra. This seems to always assume that the target cell is unknown. These parameters should be zero if the target cell is known.

RAN4 to distinguish between known and unknown target cell cases in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event.

Tfirst-RS/TRS-proc in the CLTM cell switch delay

The conditions for Tfirst-RS = 0 and TRS-proc = 0 were agreed in RAN4#116 meeting and reflected in the ongoing section 6.3.2.2.3. One remaining point is about an additional condition for Tfirst-RS = 0 and TRS-proc = 0, which is related to successful preamble transmission and TA timer in PDCCH-ordered RACH.

#### Issue 1-2-2: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0 - Candidate solutions: - Proposal 1: … - Proposal 2: If UE has received a PDCCH order and consequently a TA MAC-CE, Tfirst-RS = 0 and TRS-proc = 0 based on successful preamble transmission and while the TA timer is running. < Agreement > - Proposal 1 is agreed. - Proposal 2 can be discussed during maintenance phase without impact on core part completion.

The reason behind Proposal 2 is the Rel-18 agreement about applicability of DL sync based on TCI state activation.

Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. TRS-proc is the time for SSB processing. Tfirst-RS = 0 and TRS-proc= 0 under the following conditions: - The target TCI state indicated in the LTM cell switch command is in the serving cell active TCI state list, or - The UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8.25.3 and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or - the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the LTM candidate cell TCI state activation MAC-CE is received, or - The target cell is an FR1 cell, and the UE is not configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8. 25.3, and not more than - TCI state activation delay stated in section 8. 25.3 + 480 ms for an intra-frequency target cell, or - NOTE: longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3. - TCI state activation delay stated in section 8. 25.3 + 160 ms for an inter-frequency target cell, or - The time between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than 160 ms. Otherwise, Tfirst-RS is the time to the first SSB transmission on the target cell after Tcmd + TLTM-RRC-processing + TLTM-processing. TRS-proc = 2 ms.

When L1 measurements are configured, UE is required to keep track of DL sync for 160 ms after TCI state activation, unless the measurement period is up to 160 ms.

When L1 measurements are not configured (LTM is L3-based), UE is required to keep track of DL sync for 480 ms after TCI state activation.

There is no problem if L1 measurements are configured and measurement period is up to 160 ms as in this scenario, the UE is expected to keep track of the DL sync as long as the TCI state remains on the active TCI state list. However, this is only one of the cases, and for the scenario with L1 measurement period larger than 160 ms, which is the case basically always in FR2, and for the L3 measurement scenario, UE is allowed to stop keeping track of DL sync after a short time. As the moment when the condition for cell switch becomes met is not known to the network, there is no way for the network to be able to send the TCI state activation command at the correct time to ensure that UE still has DL sync at the time of cell switch.

In comparison, the agreed values for the TA timer are:

TimeAlignmentTimer ::= ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity}

Values for TA timer are: 500 ms, 750 ms, 1280 ms, 1920 ms, 2560 ms, 5120 ms, 10240 ms, infinity.

This means that if the intention is for UE to have DL sync and a valid TA at the time of conditional cell switch and get the related interruption reduction, network would need to resend the TCI state activation command every 160 ms or 480 ms while the TA timer is running. Otherwise, network might as well not send early TCI state activation command at all as it has almost never an impact on the conditional cell switch delay.

To make the early DL sync feature more reasonable for CLTM, we propose that, for a UE supporting early DL sync through ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18, when the network sends a PDCCH order and the UE sends the preamble to the CLTM target cell and the network responds with a TA MAC-CE with a timer, the DL sync should be considered applicable while the timer is running.

As shown in the timeline below, UE configured with CLTM can expect to receive a TA MAC-CE following the PDCCH order, so the DL sync can apply when the UE has received the TA MAC-CE until the TA timer expires. While the TA timer is running, it is beneficial that the UE keeps track of the DL sync and hence Tfirst-RS and TRS-proc should be zero.

TA timer expires

TA timer expires

UE receives TA MAC-CE

UE receives TA MAC-CE

UE receives PDCCH order

UE receives PDCCH order

UE keeps track of the DL sync with the target cell during this time interval  Tfirst-RS and TRS-proc should be zero

UE keeps track of the DL sync with the target cell during this time interval  Tfirst-RS and TRS-proc should be zero

Tfirst-RS = 0 and TRS-proc = 0 if the UE, supporting early DL synchronization through ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18, has received a TA MAC-CE for the target cell based on PDCCH-ordered RACH preamble transmission, and the associated TA timer is running.

Conclusion

The following proposals were made in this contribution:

Proposal 1: Capture the following as a note in the specification: “Note: If fine T/F tracking is included in cell switch delay, UE is not expected to start CSI acquisition before the UE finishes fine T/F tracking of the target cell. Otherwise, UE is not expected to start CSI acquisition before the end of TLTM-processing.”

Proposal 2: Discuss whether there is need to define applicability condition for CSI acquisition depending on RS frequency location (serving cell active BWP / initial target cell BWP)

Proposal 3: RAN4 to capture the agreement about CLTM requirements for candidate cells without active TCI states in FR2 under the measurement time section (6.3.2.2.1). The following wording can be used:

“The requirements in this clause apply under the following conditions:

1> The target cell is an FR1 cell, or

1> The target cell is an FR2 cell, and

2> there is at least one TCI state on the active TCI state list of the target cell, or

2> there are no TCI states on the active TCI state list of any of the configured candidate cells.”

Proposal 4: RAN4 to distinguish between known and unknown target cell cases in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event.

Observation 1: When L1 measurements are configured, UE is required to keep track of DL sync for 160 ms after TCI state activation, unless the measurement period is up to 160 ms.

Observation 2: When L1 measurements are not configured (LTM is L3-based), UE is required to keep track of DL sync for 480 ms after TCI state activation.

Observation 3: Values for TA timer are: 500 ms, 750 ms, 1280 ms, 1920 ms, 2560 ms, 5120 ms, 10240 ms, infinity.

Proposal 5: Tfirst-RS = 0 and TRS-proc = 0 if the UE, supporting early DL synchronization through ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18, has received a TA MAC-CE for the target cell based on PDCCH-ordered RACH preamble transmission, and the associated TA timer is running.