3GPP TSG-RAN WG4 Meeting #116bis	R4-2514374

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	Nokia

Title:	Discussion on performance part for LTM L1 CSI-RS measurements

Agenda item:	6.17.2.2

Document for:	Discussion

Introduction

This contribution discusses about the details of necessary test needed cases to complete the performance part of Rel-19 mobility enhancements work item.

Discussion

CSI-RS based L1 measurement

CSI-RS based L1 measurement accuracy

The following agreement was made previously regarding CSI-RS L1-RSRP measurements accuracy:

RAN4#90bis Agreement: The L1-RSRP accuracy requirements are defined based on 48PRB. - Clarify in the spec that the performance with larger bandwidth is equal to or better than the accuracy based on 48PRB. - The requirement for larger or lower bandwidth will be further discussed in the future release.

In Rel-19 RAN4 agreed to have requirements for intra-frequency measurements only with refreshed intra-frequency definition. The definition requires that at least 48 PRBs are within the active BWP. The legacy accuracy requirements have been based already on 48 PRB assumption. Therefore, we think it is ok to reuse them but place them into a new section as they are for LTM candidate cells.

Intra-frequency CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbouring cell. I.e., reuse CSI-RS L1-RSRP accuracy requirements with 48 PRB assumption.

Define CSI-RS accuracy requirements in a new clause (similar approach to Rel-18 LTM).

Test configuration

- Issue 4-2-3: test configuration for CSI-RS based L1 measurement - BW

As the legacy L1-RSRP accuracy requirements are already based on 48 PRB and the current minimum requirement defines that 48 PRBs need to be within the active BWP, the test configuration should have similar assumption. In other words, if UE is configured with 48 PRB CSI-RS resource within active BWP, the UE shall pass the test case.

Test configuration is set to have 48 PRBs within the active BWP.

CSI-RS measurement test cases

Issue 4-2-2: test case list for CSI-RS based L1 measurement

CSI-RS measurements are not supported with gaps in this release. Therefore, RAN4 should define test cases for intra-frequency without gaps.

Define CSI-RS intra-frequency test cases without gaps in rel-19

From the test coverage point of view, we should make sure that there is test coverage for features specified in the core requirements and those that are supported by RAN1. RAN4 can define test cases according to the following RAN1 table

| CSI-RS L1-RSRP measurements | CSI-RS L1-RSRP reporting | Reporting format | FR1 / FR2 | FR2 SSB / CSI-RS beam sweeping |
| --- | --- | --- | --- | --- |
| Periodic | Periodic | UCI | FR1 and FR2 | SSB, CSI-RS |
| Periodic | Semi-persistent | UCI | FR1 and FR2 | SSB, CSI-RS |
| Periodic | Aperiodic | UCI | FR1 and FR2 | SSB, CSI-RS |
| Semi-persistent | Semi-persistent | UCI | FR1 and FR2 | SSB, CSI-RS |
| Semi-persistent | Aperiodic | UCI | FR1 and FR2 | SSB, CSI-RS |
| Periodic | Event-triggered | MAC CE | FR1 and FR2 | SSB, CSI-RS |

Define at least the following test cases and align with event triggered test cases to avoid overlap

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR1

  - Intra-frequency Semi-persistent CSI-RS L1-RSRP measurements with semi-persistent reporting on FR1

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with SSB-based beam sweeping

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with CSI-RS-based beam sweeping

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with CSI-RS based beam sweeping

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with SSB-based beam sweeping

Conclusion

The following Proposals were made:

Proposal 1: Intra-frequency CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbouring cell. I.e., reuse CSI-RS L1-RSRP accuracy requirements with 48 PRB assumption.

Proposal 2: Define CSI-RS accuracy requirements in a new clause (similar approach to Rel-18 LTM).

Issue 4-2-3: test configuration for CSI-RS based L1 measurement - BW

Proposal 3: Test configuration is set to have 48 PRBs within the active BWP.

Proposal 4: Define CSI-RS intra-frequency test cases without gaps in rel-19

Proposal 5: Define at least the following test cases and align with event triggered test cases to avoid overlap

1.	Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR1

2.	Intra-frequency Semi-persistent CSI-RS L1-RSRP measurements with semi-persistent reporting on FR1

3.	Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with SSB-based beam sweeping

4.	Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with CSI-RS-based beam sweeping

5.	Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with CSI-RS based beam sweeping

6.	Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with SSB-based beam sweeping