3GPP TSG-RAN4 Meeting #116-bis	R4-2514393

Prague, CZ, Oct. 13 – 17, 2025

Source:	Nokia

Title:	Coexistence (UAV)

Agenda item:	7.7.3

Document for:	Discussion

Introduction

In the last RAN plenary meeting #109 in Prague, the new work items for UAV were approved [1]. In this paper, we present our view on UAV aspects.

### Objective of SI or Core part WI or Testing part WI

The core part of WI:

- Scenario:

  - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km.

  - A direct radio link between BS on the ground and UAV UE.

- Specify RF requirements for UAV UE to support higher UL output power.

  - Example bands:

    - TDD with same TDD pattern, including n41, n77, n78, n79,

    - FDD: n3

  - Consider UAV UE with omni-direction antenna or with antenna array

  - Consider UAV UE with up to 33dBm max total conductive output power

  - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed.

  - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM.

- Specify RRM requirements, if needed

Note 1: No RAN1 impact are expected

### 4.2	Objective of Performance part WI

The performance part of WI:

- Specify the RRM test cases corresponding to the RRM core requirements.

Discussion

UAV max output power and co-existence

Based on the description in the WID, the main UAV application scenarios include low-altitude logistics, travel, tourism, control and supervision, medical rescue, and others, where UL throughput and UL coverage are more critical than DL. To improve coverage for UAVs, one option is to increase the number of gNBs, but this would significantly raise network deployment costs, which the WID aims to avoid. A more straightforward solution is to increase the UAV UE maximum output power (MOP).

According to the WID, MIIT in China has issued a trial upper power limit of 33 dBm for ATG UE, while NR_UAV UEs have supported only 23 dBm since Rel-18, without simulations. The gap from 23 dBm to 33 dBm is substantial. However, given UAV operating altitudes of 600–1000 m, and in some cases 2–4 km, channel conditions are generally much better than for ground UEs, with only high buildings and other UAVs causing significant blockage or interference between gNB and UAV UEs.

The WID also references ATG RF specifications, which allow increasing ATG UE MOP up to 40 dBm (from 23 dBm, with 1 dB granularity) to accommodate variations across aircraft types and frequency ranges. Nonetheless, ATG use cases, i.e., aircraft/airplanes, are fundamentally different from UAVs, as ATG UEs operate at much higher altitudes.

Therefore, to mitigate coexistence issues, a 33 dBm MOP may not be necessary before a full study of 31 dBm (the current maximum for PC1), and it is preferable to reuse the 31 dBm PC1 RF specification as much as possible.

The channel condition in the air “The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km,” is a much better condition compared with the channel conditions of UEs on the ground. The only obstacles to distort the signals in the air are the high buildings and other UAV-like devices.

The ATG use case, i.e., aircraft/airplanes, is clearly different from UAV, as ATG UEs operate at much higher altitudes than UAV UEs.

Based on the WID, in Rel-18 NR that UAV UE's maximum output power is 23dBm without simulations. From 23 dBm to 33dBm is a very huge power difference.

To mitigate coexistence issues, a 33 dBm MOP may not be necessary before a full study of 31 dBm (the current maximum for PC1), and it is preferable to reuse the 31 dBm PC1 RF specification as much as possible.

The WID also notes that, to avoid severe interference to ground networks from the increased uplink power limit of UAV UEs, beamforming or directional antennas should be supported. For ATG UEs, two antenna types, i.e., omnidirectional and antenna arrays, are introduced to provide implementation flexibility, and these are also referenced to guide RF requirement design for UAV RF specifications.

For UAV UEs, due to their altitude, the beamwidth expands significantly when projected from the UAV to the ground, meaning that a single beam can still cause wide-area interference. Therefore, beamforming may not necessarily be effective in reducing interference, but it remains a valid option to enhance uplink coverage, and further study is needed.

Due to the altitude of UAV UEs, the beamwidth expands significantly when projected from the sky to the ground, so a single beam can still cause significant wide-area interference.

Beamforming may not be used to reduce the interference, but rather that it is a valid consideration to use beamforming to enhance the UL coverage. It should be studied.

Conclusion

The following observations and proposals were made:

- The channel condition in the air “The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km,” is a much better condition compared with the channel conditions of UEs on the ground. The only obstacles to distort the signals in the air are the high buildings and other UAV-like devices.

The ATG use case, i.e., aircraft/airplanes, is clearly different from UAV, as ATG UEs operate at much higher altitudes than UAV UEs.

Based on the WID, in Rel-18 NR that UAV UE's maximum output power is 23dBm without simulations. From 23 dBm to 33dBm is a very huge power difference.

- To mitigate coexistence issues, a 33 dBm MOP may not be necessary before a full study of 31 dBm (the current maximum for PC1), and it is preferable to reuse the 31 dBm PC1 RF specification as much as possible.

Due to the altitude of UAV UEs, the beamwidth expands significantly when projected from the sky to the ground, so a single beam can still cause significant wide-area interference.

Beamforming may not be used to reduce the interference, but rather that it is a valid consideration to use beamforming to enhance the UL coverage. It should be studied.

- References

- RP-252954; New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV)