3GPP TSG-RAN WG4 Meeting # 116-bis	R4-2514400

Prague City, CZ, Oct 13 – Oct 17, 2025

Agenda Item: 6.12.2.2

Source: MediaTek Inc.

Title: Discussion on the performance part for R19 MIMO for other RRM requirements

Document for: Discussion

# Introduction

In this paper we discuss the performance part for R19 MIMO phase 5, specifically for part 2, which is related to other RRM requirements. RAN4 had some open issues in WF [1][2] in the last meeting without conclusion. We provide our view on the performance requirements for CJTC reporting and asymmetric DL sTRP/UL mTRP.

# Discussion

# CJTC calibration reporting.

For CJT calibration, RAN4 agreed in the last meeting to define core requirements based on 1 sample only and further discuss the accuracy and SNR in the performance part as captured below from the WF [2].

Issue 1-1: How to define measurement period and accuracy requirements for delay and frequency offset reporting for CJT calibration? Agreement: Define core requirements based on 1 sample only and further discuss the accuracy and SNR in the performance part. Companies are encouraged to discuss how to derive the accuracy performance based on the simulation results.

Based on the simulation results provided by all companies in the last meeting in (R4-2512159), it is observed that the accuracy of frequency offset (FO) and delay offset (DO) is highly degraded when the side condition is -3dB. This degradation is also presented in terms of throughput performance in Nokia paper (R4-2510692) in the last meeting. Therefore, to define the accuracy requirement for FO and DO, we propose to focus on SNR side condition higher than or equal to 0dB since it is challenging to meet acceptable accuracy for -3dB based on 1 sample. Given our simulation results in Appendix A, the accuracy for FO and DO based on 1 sample can be defined as shown in the Table 1 and 2, respectively. Based on our analysis with respect to the results in (R4-2510692), it seems that the accuracy level provided in the tables can still achieve >90% of the throughput performance for both FO and DO.

Table 1: FO accuracy

| Frequency offset |  |  |
| --- | --- | --- |
| Accuracy based on 1 sample | SNR | SCS |
| Level | dB | kHz |
| 39 | 0 | 15 |
| 26 | 3 |  |
| 83 | 0 | 30 |
| 52 | 3 |  |
| Note: accuracy level refers to quantization level based on the report resolution of MFO = 256 and AFO=0.2ppm |  |  |

Table 2: DO accuracy

| Delay offset |  |  |
| --- | --- | --- |
| Accuracy based on 1 sample | SNR | SCS |
| Level | dB | kHz |
| 17 | 0 | 15 |
| 10 | 3 |  |
| 16 | 0 | 30 |
| 14 | 3 |  |
| Note: accuracy level refers to quantization level based on the report resolution of MD = 256 and AD=CP |  |  |

Proposal 1: Define accuracy requirements for FO and DO for CJT calibration based on SNR ≥ 0dB.

Proposal 2: Accuracy requirements for FO and DO for CJT calibration can be defined as in the tables below:

Table 3: FO accuracy

| Frequency offset |  |  |
| --- | --- | --- |
| Accuracy based on 1 sample | SNR | SCS |
| Level | dB | kHz |
| 39 | 0 | 15 |
| 26 | 3 |  |
| 83 | 0 | 30 |
| 52 | 3 |  |
| Note: accuracy level refers to quantization level based on the report resolution of MFO = 256 and AFO=0.2ppm |  |  |

Table 4: DO accuracy

| Delay offset |  |  |
| --- | --- | --- |
| Accuracy based on 1 sample | SNR | SCS |
| Level | dB | kHz |
| 17 | 0 | 15 |
| 10 | 3 |  |
| 16 | 0 | 30 |
| 14 | 3 |  |
| Note: accuracy level refers to quantization level based on the report resolution of MD = 256 and AD=CP |  |  |

In previous discussion in RAN4, it was agreed to define report mapping tables for FO/DO for CJT calibration. Therefore, RAN4 need to discuss the details of the report mapping table. We propose to define the following tables for FO and DO report mapping.

Proposal 3: Report mapping table for frequency/delay offset can be defined based on report resolution 255 as:

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC-F_0 | 0 ≤ CJTC-F < 0.2ppm255 | Hz |
| CJTC-F_1 | 0.2ppm255 ≤ CJTC-F < 0.4ppm255 | Hz |
| … | … | Hz |
| CJTC-F_255 | 0.2ppm ≤ CJTC-F | Hz |

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC-Dd_0 | 0 ≤ CJTC-Dd < CP255 | us |
| CJTC-Dd_1 | CP255 ≤ CJTC-Dd < 2CP255 | us |
| … | … | us |
| CJTC-Dd_255 | CP ≤ CJTC-Dd | us |

In the last meeting, RAN4 also discussed the test cases for CJT as shown below from the WF [1]:

#### Issue 2-2-3: Test scope <Way forward > High level:; QC; Apple; Samsung; CMCC; Nokia; Ericsson; MTK. Whether to define TCs to verify CJTC-Dd, measurement period; Y; FFS; Y; Y; Y; Y; Y. Whether to define TCs to verify CJTC-f，measurement period; Y; FFS; Y; Y; Y; Y; Y. Whether to define TCs to verify CJTC-P， (To be verify?); N; N; N. Whether to define TCs to verify scheduling restriction; Y (if core req.). Whether to define TCs to verify CJTC-Dd, CJTC-f，accuracy; Y; Y.

RAN4 needs to decide on the TCs list to be defined for FO and DO reporting. In our view, TCs for testing the measurement period for FO and DO are needed, and we do not see the need to define the a test case for scheduling restriction.

Proposal 4: RAN4 to consider testing CJTC measurement period for delay offset and frequency offset.

# Asymmetric DL sTRP / UL mTRP

#### Issue 2-3-1: Test scope <Way forward > High level:; QC; Apple; Samsung; CMCC; Huawei; Nokia; MTK. Whether to define TCs to verify timing requirements?; Y; Y; Y; Y; Y. Whether to define TCs to verify TCI state switching requirements?; Y; N. FR1 and/or FR2?; both. Whether to cover Scenario#1 and Scenario#2?; Both for FR1. Scenario#2 for FR2; Both; Both; Scenarios#1; both; Scenarios#1.

For Asymmetric DL sTRP / UL mTRP, on the other hand, RAN4 to discuss whether testing both Scenario#1 and Scenario#2 is required. In our view Scenario#2 is not very different from the legacy multi-DCI based multi-TRP operation. Therefore, RAN4 can focus on testing Scenario#1 in the performance part.

Proposal 5: RAN4 to prioritize testing Scenario#1 for asymmetric DL sTRP / UL mTRP in the performance part.

# Summary

In this paper, the discussion of R19 MIMO is provided. The following observations and proposals are concluded:

Proposal 1: Define accuracy requirements for FO and DO for CJT calibration based on SNR ≥ 0dB.

Proposal 2: Accuracy requirements for FO and DO for CJT calibration can be defined based on Table 1 and 2:

Table 1: FO accuracy

| Frequency offset |  |  |
| --- | --- | --- |
| Accuracy based on 1 sample | SNR | SCS |
| Level | dB | kHz |
| 39 | 0 | 15 |
| 26 | 3 |  |
| 83 | 0 | 30 |
| 52 | 3 |  |
| Note: accuracy level refers to quantization level based on the report resolution of MFO = 256 and AFO=0.2ppm |  |  |

Table 2: DO accuracy

| Delay offset |  |  |
| --- | --- | --- |
| Accuracy based on 1 sample | SNR | SCS |
| Level | dB | kHz |
| 17 | 0 | 15 |
| 10 | 3 |  |
| 16 | 0 | 30 |
| 14 | 3 |  |
| Note: accuracy level refers to quantization level based on the report resolution of MD = 256 and AD=CP |  |  |

Proposal 3: Report mapping table for frequency/delay offset can be defined based on report resolution 255 as in Table 3 and 4:

Table 3: FO report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC-F_0 | 0 ≤ CJTC-F < 0.2ppm255 | Hz |
| CJTC-F_1 | 0.2ppm255 ≤ CJTC-F < 0.4ppm255 | Hz |
| … | … | Hz |
| CJTC-F_255 | 0.2ppm ≤ CJTC-F | Hz |

Table 4: DO report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC-Dd_0 | 0 ≤ CJTC-Dd < CP255 | us |
| CJTC-Dd_1 | CP255 ≤ CJTC-Dd < 2CP255 | us |
| … | … | us |
| CJTC-Dd_255 | CP ≤ CJTC-Dd | us |

Proposal 4: RAN4 to consider testing CJTC measurement period for delay offset and frequency offset.

Proposal 5: RAN4 to prioritize testing Scenario#1 for asymmetric DL sTRP / UL mTRP in the performance part.

# Reference

- R4-2512130, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung.

- R4-2512131, WF on RRM requirements for NR_MIMO_Ph5_Part2, MediaTek.

# Appendix A

Simulation assumption and results for CJT calibration:

Table 1. Simulation parameters for timing and frequency offset estimation

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | - FDD, 15kHz SCS - TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm, 0.2ppm 2nd priority: 0.02 ppm, 0.05ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples (TRS resource sets) for measurement | 1, 3, 5 2nd priority: 2, 4, 10 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |

- Timing offset simulation results:

Table 2. Simulation result for timing offset error estimation.

| SCS | SNR | Timing offset (TO) b/w two TRPs | TO estimation error in AD = CP and MD = 256 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max(CDF5%, CDF95%) #levels 256 |  |  |
|  |  |  | #samples |  |  |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 0.5CP | 29 | 19 | 18 |
| 15KHz | 0 | 0.5CP | 17 | 12 | 10 |
| 15KHz | 3 | 0.5CP | 10 | 7 | 6 |
| 30KHz | -3 | 0.5CP | 30 | 19 | 17 |
| 30KHz | 0 | 0.5CP | 16 | 11 | 10 |
| 30KHz | 3 | 0.5CP | 14 | 9 | 9 |

- Frequency offset simulation results:

Table 3. Simulation result for frequency offset error estimation.

| SCS | SNR | Frequency offset (FO) b/w two TRPs | FO estimation error in AFO = 0.2 ppm and MFO = 256 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max(CDF5%, CDF95%) #levels 256 |  |  |
|  |  |  | #samples |  |  |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 350 Hz | 65 | 39 | 29 |
| 15KHz | 0 | 350 Hz | 39 | 24 | 18 |
| 15KHz | 3 | 350 Hz | 26 | 16 | 12 |
| 30KHz | -3 | 350 Hz | 133 | 71 | 60 |
| 30KHz | 0 | 350 Hz | 83 | 48 | 37 |
| 30KHz | 3 | 350 Hz | 52 | 31 | 24 |