3GPP TSG-RAN5 Meeting #116bis	R4-2514407

# Prague, Czech Republic, 13th Oct 2025 – 17th Oct 2025

Agenda item:	7.6.3

Source: 	Viasat, Space 42, Aalyria

Title: 	Views and proposals related to MPR and A-MPR for bands 255 and 256 for IoT-NTN PC1.5 single Tx

Document for:	Approval

## 1.	Introduction

During the RAN4#116 meeting, as part of NTN WI [1], IoT-NTN single Tx PC1 UE for both handheld and non-handheld UEs was introduced in [2]. During the RAN#109 meeting, as part of the Rel-20 package for NTN, development of specifications for IoT-NTN PC1.5 single Tx UE was approved [3].

This document discusses implications on the MPR and A-MPR specifications as well as consideration of additional emissions requirements based on ETSI harmonized standards for the uplink frequency range of the L-bands b255/n255.

## 2.	Discussion

Given that MOP for PC1.5 (29 dBm) falls in the middle of the values for PC2 (26 dBm) and PC1 (31 dBm), one would expect that the single tone and multi-tone MPR values for PC1.5 will be less than that specified for PC1.

Proposal 1: The single and multi-tone MPR values for PC1.5 single Tx shall be progressively lower than those specified for PC 1 NB-IoT NTN UE.

The A-MPR associated with NS_24N for band 256 is specified for the entire band irrespective of the location of NB-IoT carrier [2]. It is expected that the A-MPR will be necessary only if the NB-IoT carrier is configured towards the upper edge of band 256. Furthermore, given the MOP difference between PC1 and PC1.5, one would expect that the A-MPR associated with band 256 will be proportionately lower than that specified for PC1 NB-IoT NTN UE.

Proposal 2: While specifying A-MPR for NS_24N for band 256, RAN4 shall evaluate the carrier frequency range for which the A-MPR is applicable. It is understood that NB-IoT carriers farther away the upper edge of band 256 will require little to no A-MPR.

Proposal 3: PC1.5 NB-IoT NTN UE A-MPR for NS_24N for band 256 shall be progressively lower than that specified for the PC 1 NB-IoT NTN UE.

In RAN4#114, as part of discussions related to the introduction of the combined L-Band, it was agreed that there exist ETSI requirements for unwanted emissions [4] that the NTN UE is required to comply with for the UL frequency ranges associated with Band 255. As a result, as part of introduction of n251, which has the same UL frequency range as Band 255, additional emission requirements were specified in TS 38.101-5 to comply with the ETSI requirements (listed in Annex A). The two sets of additional emissions requirements specified in TS 38.101-5 [5] are listed in Annex B of this paper.

As discussed in R4-2513841, it is anticipated that as part of maintenance, the additional emission requirements associated with ETSI requirements will be specified for Band 255 in TS 36.102.

Observation 1: As part of the introduction of Combined L-Bands, n251/n252/n253, additional emission requirements per ETSI harmonized standard were specified in Rel-19.

Observation 2: As part of maintenance, additional emission requirements associated with ETSI requirements are anticipated to be specified for Band 255 in TS 36.102.

Proposal 4: In addition to NS_02N, companies shall take into consideration the two sets of ETSI requirements associated with NS_10N and NS_14N for Band 255 specified in Annex B while evaluating the A-MPR for Band 255 PC1.5 single Tx NB-IoT NTN UE.

## 3.	Conclusion

Proposal 1: The single and multi-tone MPR values for PC1.5 single Tx shall be progressively lower than those specified for PC 1 NB-IoT NTN UE.

Proposal 2: While specifying A-MPR for NS_24N for band 256, RAN4 shall evaluate the carrier frequency range for which the A-MPR is applicable. It is understood that NB-IoT carriers farther away the upper edge of band 256 will require little to no A-MPR.

Proposal 3: PC1.5 NB-IoT NTN UE A-MPR for NS_24N for band 256 shall be progressively lower than that specified for the PC 1 NB-IoT NTN UE.

Observation 1: As part of the introduction of Combined L-Bands, n251/n252/n253, additional emission requirements per ETSI harmonized standard were specified in Rel-19.

Observation 2: As part of maintenance, additional emission requirements associated with ETSI requirements are anticipated to be specified for Band 255 in TS 36.102.

Proposal 4: In addition to NS_02N, companies shall take into consideration the two sets of ETSI requirements associated with NS_10N and NS_14N for Band 255 specified in Annex B while evaluating the A-MPR for Band 255 PC1.5 single Tx NB-IoT NTN UE.

## 4.		References

[1]	RP-242901, "Revised WID on Enhance requirements and conductive test methodology for NR NTN and IoT NTN ", Apple, Thales, Inmarsat, Viasat, Globalstar

[2]	TS 36.102, ver 19.1.0

[3]	RP-252962, Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)

[4]	ETSI EN 301 681, Harmonised Standard for Mobile Earth Stations (MES) ofGeostationary mobile satellite systems, including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) under the Mobile Satellite Service (MSS), operating in the 1,5 GHz and 1,6 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU

[5]	TS 38.101-5, version 19.2.0

- Annex A: Current ETSI requirements for L-band [4]

Table A-1: In-band emission requirements for a NTN UE with UL operation in 1626.5-1660.5MHz frequency range

Table A-4: Out-of-band emission requirements for a NTN UE with UL operation in 1668-1675MHz frequency range

- Annex B: RAN4 specifications for n250 and n251 [5]

## B-1: Two NS Values for the two sets of emission requirements for two L-band uplink frequency ranges: 1626.5 to 1660.5 MHz (NS_10N, NS_14N)

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_09N | 6.5.2.3.3 6.5.3.3.8 | n250, n253 | 5 |  | Clause 6.2.3.7 |
| NS_10N | 6.5.2.3.3 6.5.3.3.9 | n251 | 5, 10, 15, 20 |  | Clause 6.2.3.8 |
| NS_13N5 | 6.5.3.3.10 |  | 5 |  |  |
| NS_14N5 | 6.5.3.3.10 |  | 5, 10, 15, 20 |  | Clause 6.2.3.9 |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. NOTE 4: The NS_07N and NS_08N is signalled based on coordination between operators and subject to regional/national regulation NOTE 5: NS_09N (n253, n250), NS_10N (n251) are currently in force. NS_13N (n253, n250), NS_14N (n251) may supersede NS_09N, NS_10N when the new ETSI requirements get ratified. |  |  |  |  |  |

## B-2: Emission requirements for two L-band uplink frequency ranges: 1626.5 to 1660.5 MHz

##### 6.5.2.3.3	Requirements for network signalling value “NS_09N" and “NS_10N"

When "NS_09N" or "NS_10N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3-1 for any channel bandwidth configured within the corresponding band.

Table 6.5.2.3.3-1: Additional requirements for "NS_09N" and "NS_10N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Note (measurement method) |
| --- | --- | --- | --- |
| ± 0-25 | 30 to 15 | 3 kHz | Average |
| ± 25-125 | 15 to -20 |  |  |
| ± 125-425 | -20 |  |  |
| ± 425-1500 | -20 to -35 |  |  |
| ± 1500-36000 | -25 | 30 kHz |  |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

##### 6.5.3.3.9	Requirement for network signalling value "NS_10N"

When "NS_10N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.9-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.9-1: Additional out-of-band requirements for "NS_10N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1660.525 | 30 to 15 | 3 kHz | Average |
| 1660.525 to 1660.625 | 15 to - 20 | 3 kHz | Average |
| 1660.625 to 1660.925 | -20 | 3 kHz | Average |
| 1660.925 to 1662 | -20 to -35 | 3 kHz | Average |
| 1662 to 1662.5 | -25 | 30 kHz | Average |
| 1662.5 to 1665.5 | -30 | 30 kHz | Average |
| 1665.5 to 1670.5 | -30 | 100 kHz | Average |
| 1670.5 to 1680.5 | -30 | 300 kHz | Average |
| 1680.5 to 1690.5 | -30 | 1 MHz | Average |
| 1690.5 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

##### 6.5.3.3.10	Requirement for network signalling value "NS_14N"

When "NS_14" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.10-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.10-1: Additional out-of-band requirements for "NS_14N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |