3GPP TSG-RAN WG4 Meeting #116-bis    	  R4-2514423

Prague, Czech Republic, 13th – 17th of October, 2025

Title: 	General discussion on IoT_NTN_TDD demodulation performance requirements

Agenda Item:	6.21.3 – UE and SAN demodulation performance requirements

Release:	Rel-19

Source: 	THALES, Iridium Satellite LLC, Cambridge Consultants LTD

Document for:	Discussion

# WID information

A new WID Introduction of IoT-NTN TDD mode (RP-242415, New WID on introduction of IoT-NTN TDD mode, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024) was approved in RAN#105. That WID has been revised in RAN#106, as the study phase had been completed. Then, a work plan for Revised WID on Introduction of IoT-NTN TDD mode (RP-243293, Revised WID on introduction of IoT-NTN TDD mode, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024) is proposed for RAN 1, 2, and 4.

## Core part WI

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands.

The work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band.

This work item includes the following objectives:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2]

      - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band.

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

- Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4].

  - Specify band numbering

  - Specify SAN and UE RF characteristics

  - Specify DL and UL channelization.

  - Specify channel bandwidth as 200 kHz

  - Note1: No NTN-NTN coexistence study needed.

  - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

## Performance part WI

Specify the following:

- RRM performance requirements, as needed [RAN4]

- Necessary SAN and UE demodulation requirements [RAN4]

- SAN conformance testing requirements [RAN4]

## Remaining work from current Work Plan

The following updated & adapted WP from R4-2502291/RP-252935 is:

- 3GPPRAN4#116-bis (2025-10-13 - 2025-10-17)

  - Start discussion on demodulation performance and specific configuration related to the new TDD pattern;

  - Start discussion of conformance testing for core part, endorse CRs if any.

- 3GPPRAN4#117 (2025-11-17 - 2025-11-21)

  - Continue discussion and drafting CRs for demodulation performance;

  - Start drafting performance requirements CRs (TS 36.102, TS 36.108, TS 36.133), endorse CRs if any;

  - Start discussion of conformance testing for demodulation performance part (TS 36.181), endorse CRs if any.

- 3GPPRAN4#118 (2026-02-09 - 2026-02-13)

  - Finish demodulation performance CR;

  - Finish conformance testing CRs (if any left).

For the conformance testing aspects, please also recall:

Issue 1-5-2: Work Split for NB-IoT TDD NTN in TS 36.181

Agreement:

| Clause/Requirement | Company (other companies are encouraged to contribute) |
| --- | --- |
| Definitions/Abbreviations | Iridium |
| Conducted/Radiated REFSENS update | THALES |
| Conducted/Radiated Dynamic range update | Iridium |
| Conducted ON/OFF requirements | Iridium |
| ACS requirement (double-check) | Iridium |
| EVM Annex?, Other.. |  |

Note: Other identified Clauses are not precluded

For further information please also check:

- R4-2502292, Way Forward for [114][318] IoT_NTN_TDD_UE_SAN_RF, THALES;

- R4-2504719, Way Forward for [114bis][316] IoT_NTN_TDD_UE_SAN_RF, THALES;

- R4-2508781, Way Forward for [115][316] IoT_NTN_TDD_UE_SAN_RF, THALES;

- R4-2512666, Way Forward for [116][316] IoT_NTN_TDD_UE_SAN_RF, THALES.

Please also check the approved list of documents at RAN#109:

| T-Doc | Title | Company | Type | Decision |
| --- | --- | --- | --- | --- |
| RP-252329 | Status Report - WID on introduction of IoT NTN TDD mode | Iridium Satellite LLC | WI status report | Noted |
| RP-252624 | Corrections to Rel-19 IoT-NTN TDD mode (36.211/213) | RAN1 | CR pack | Approved |
| RP-252795 | Introduction of IoT-NTN TDD mode | RAN2 | CR pack | Approved |
| RP-252407 | RAN4 CRs to IoT_NTN_TDD | RAN4 | CR pack | Approved |
| RP-252935 | Revised WID for introduction of IoT-NTN TDD mode | Iridium Satellite LLC | WID revised | Approved |
| RP-252872 | WI summary for WI Core part: Introduction of IoT-NTN TDD mode | Iridium | WI summary | Approved |

# Generalities for IoT_NTN_TDD demodulation performance requirements

Please also see the Annex from R4-2508781 (Way Forward for [115][316] IoT_NTN_TDD_UE_SAN_RF, THALES) - for information purpose only (since no allocated TUs). The proposals were submitted for demodulation and performance not discussed under [114][318] and [114][317] IoT_NTN_TDD.

Issue 2-2-1: General discussion for NPDSCH minimum performance requirements

- Proposals (P3/R4-2502050):

  - Proposal 1. Introduce a new clause for TDD NB-IoT category for 249 band (currently only FDD configuration is defined) for NPDSCH minimum performance requirements.

- Recommended WF

  - Agree if no controversial discussion.

Issue 2-2-2: General discussion for repetition number under performance requirements

- Proposals (P4/R4-2502050):

  - Proposal 1. Further discuss repetition number to be considered for demodulation and performance aspects. Provide SNR values for less repetition numbers to align with U/L=8 and N=9 as per TDD NB-IOT NTN band 249 configuration.

- Recommended WF

  - Agree if no controversial discussion.

Issue 2-2-3: NPDCCH and NPBCH performance requirements

- Proposals (P6/R4-2502050):

  - Proposal 1. Following FDD process for NB-IoT NTN, propose not to define NPDCCH and NPBCH performance requirements in TS 36.102 for TDD NB-IoT NTN definition.

- Recommended WF

  - Agree if no controversial discussion.

Issue 2-2-4: NPDSCH performance requirements

- Proposals (P7/R4-2502050):

  - Proposal 1. Following FDD process for NB-IoT NTN, propose to define only NPDSCH performance requirements in TS 36.102 for TDD NB-IoT NTN definition.

    - Note: Further discuss on the reference channel to be used for NPDSCH performance requirements.

- Recommended WF

  - Agree if no controversial discussion.

Issue 2-2-5: Specific TDD demodulation performance requirements

- Proposals (P5/R4-2502050):

  - Proposal 1. RAN4 to revisit/consider introduction of new specific (highlighted) TDD clauses. RAN4 to consider at least the introduction of the following performance requirements in TS 36.102:

## 8.3	Demodulation performance requirements for UE category NB1 and NB2

### 8.3.1	Half-duplex FDD

#### 8.3.1.1	NPDSCH demodulation requirements

##### 8.3.1.1.1	Single-antenna port performance

8.3.1.1.1.1	Minimum Requirements for standalone

### 8.3.2	TDD

#### 8.3.2.1	NPDSCH demodulation requirements

The parameters specified in Table 8.3.2.1-1 and Table 8.3.2.1-2 are valid for all half-duplex FDD tests unless otherwise stated.

Table 8.3.2.1-1: Common Test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Uplink downlink configuration (Note 1) |  | 1 or 4 - TBD |
| Special subframe configuration (Note 2) |  | 4 |
| Number of HARQ processes per component carrier (Note 4) | Processes | 1 or 2 |
| Maximum number of HARQ transmission |  | 4 |
| Cyclic Prefix |  | Normal |
| eutraControlRegionSize-r13 |  | 2 for In-band, N/A for Standalone/Guard-band unless otherwise stated |
| downlinkBitmap-r13 and dl-Gap-r13 |  | Not configured |
| dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13 |  | Not configured |
| Unused REs or RB (Note 3) |  | OCNG |
| OCNG pattern |  | NB.OP.1 |
| Note 1: As specified in Table 4.2-2 in TS 36.211 [4] Note 2: As specified in Table 4.2-1 in TS 36.211 [4] Note 3: For in-band mode, the REs for transmission of LTE signals including PDCCH, CRS should be filled by OCNG. Note 4: 1 is applicable for both UE Category NB1 and NB2; 2 is only applicable for UE Category NB2 if UE supports it. |  |  |

Table 8.3.2.1-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| scheduling delay field () |  | 1 for the first HARQ process; 2 for the second HARQ process if configured |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDCCH |  | R.NB.3 TDD for one NRS antenna port; R.NB.4 TDD for two NRS antenna ports |
| (npdcch-Offset-USS-r13) |  | 0 |
| K_offset | ms | 8 (TBD) |

##### 8.3.2.1.1	Single-antenna port performance

8.3.2.1.1.1   Minimum Requirements for standalone

The requirements are specified in Table 8.3.2.1.1.1-2, with the addition of the parameters in Table 8.3.2.1.1.1-1 and the downlink physical channel setup according to [Annex C.3.6]. The purpose of these tests is to verify the performance.

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | TBD (currently 32 for Test 1; 256 for Test 2.) |
| (npdcch-NumRepetitions-r13) |  | subframe | TBD (currently 32 for Test 1; 256 for Test 2.) |
| (nPDCCH-startSF-USS-r13) |  |  | 4 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.3.2.1.1.1-1: Test Parameters for NPDSCH under Standalone

Table 8.3.2.1.1.1-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | TBD (currently R.NB.6 TDD) | TBD | NTN-TDLC5-200 | 1 | 1x1 | 70% | TBD | NB1, NB2 |
| 2 | 200kHz | Non-anchor | TBD (currently R.NB.6-1 TDD) | TBD | NTN-TDLC5-200 | 1 | 1x1 | 70% | TBD | NB1, NB2 |

- Recommended WF

  - To be further discussed.

Issue 3-5-1: FRC parameters for NTN NB-IoT NPUSCH format 1

- Proposals (P10/R4-2502027):

  - Proposal 1. RAN4 to further discuss FRC for NB-IoT NPUSCH format 1 in SAN 36.108 with respect to band 249 for better adaptation to 8 ms TDD Tx time (corresponding to D/U=8). Reference channel A7-2 could be used (A7-1 does not seem applicable).

Table A.7.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A7-1 | A7-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 |
| Number of allocated subcarriers | 1 | 12 |
| Diversity | No | No |
| Modulation | BPSK | QPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 |
| Payload size (bits) | 32 | 136 |
| Allocated resource unit | 2 | 1 |
| Code rate (target) | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.56 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 |
| Total symbols per resource unit | 96 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 2 (when repetition = 2) 4 (when repetition > 2) |

- Recommended WF

  - Agree with Proposal 1 if no controversial discussion.

  - Consider only A7-2 (and not A7-1) for demodulation performance requirements for NTN band 249 TDD NB-IoT. In other words, Consider only NPUSCH format 1 with 15 kHz and 12 sub-carriers for demodulation performance requirements for band 249 TDD NB-IoT. (P15/R4-2502027)

Issue 3-5-2: NTN NPUSCH format 1 demodulation requirements in TS 36.108

- Proposals (P16/R4-2502027):

  - Proposal 1. Revisit demodulation requirements by taking into account specific NTN NB-IoT TDD system constraints with D/U=8 in TS 36.108:

Table 8.5.1.1.1-3: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx, for TDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |
| 1 | 1 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |
| 1 | 2 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |
| 1 | 2 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |

- Recommended WF

  - Agree with Proposal 1 if no controversial discussion.

Issue 3-5-3: FRC parameters for NTN NB-IoT NPUSCH format 2

- Proposals:

  - Proposal 1. RAN4 to further discuss conditions for NPUSCH format 2 in SAN 36.108 with respect to band 249 for better adaptation to 8 ms TDD Tx time (corresponding to D/U=8). Reference channel A7-2 could be used (A7-1 does not seem applicable).

#### 8.5.2.1	DTX to ACK performance

The DTX to ACK probability for NPUSCH format 2 case denotes the probability that ACK is detected when nothing is sent on the wanted signal and only the noise is present per NPUSCH format 2 transmission.

An NB-IoT Base Station supports 15 KHz sub-carrier spacing requirements for both TDD and FDD, or 3.75 KHz sub-carrier spacing requirements for FDD only, or both in the case of FDD.

##### 8.5.2.1.1	Minimum requirement

The DTX to ACK probability, i.e. the probability that ACK is detected when nothing was sent, shall not exceed 1% per NPUSCH format 2 transmission. Where the performance measure definition is as follows:

where:

-	#(false ACK bits) denotes the number of detected ACK bits.

-	#(ACK/NACK bits) denotes the number of HARQ-ACK information bit per NPUSCH format 2 transmission.

-	#( NPUSCH format 2 DTX) denotes the number of DTX occasions.

#### 8.5.2.2	ACK missed detection requirements

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent per NPUSCH format 2 transmission.

- Recommended WF

  - Agree with Proposal 1 if no controversial discussion.

Issue 3-5-4: NTN NPUSCH format 2 demodulation requirements in TS 36.108

- Proposals:

  - Proposal 1. Revisit demodulation requirements of NPUSCH format 2 by taking into account specific NTN NB-IoT TDD system constraints with D/U=8 in TS 36.108:

Table 8.5.2.2.1-3: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, 1Tx, for TDD

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN TDLA100-1 | 1 | 15KHz | 16 Hz | 32 TBD | 16 TBD | 5.1 TBD |
| 1 | 2 | NTN TDLA100-1 | 1 | 15KHz | 16 Hz | 32 | 16 | -3.3 |

- Recommended WF

  - Agree with Proposal 1 if no controversial discussion.

Issue 3-5-5: Update the NTN NPRACH performance for TDD in TS 36.108

- Proposals (P13/R4-2502027):

  - Proposal 1. Update the NPRACH performance accordingly for TDD 249 band:

### 8.5.3	Performance requirements for NPRACH

#### 8.5.3.1	NPRACH False alarm probability

The false alarm requirement is valid for any number of receive antennas, for all repetition numbers and for any number of subcarriers.

The false alarm probability is the conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

##### 8.5.3.1.1	Minimum requirement

The false alarm probability shall be less than or equal to 0.1%.

#### 8.5.3.2	NPRACH detection requirements

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. For NTN-TDLA100 and NTN-TDLC5, a timing estimation errors occurs if the estimation error of the timing of the largest delay path is larger than the time error tolerance given in Table 8.5.3.2-2.

The requirements for TDD are optional and only valid for SAN supporting TDD.

Table 8.5.3.2-1: Test preambles for NPRACH

| Parameter | Value |
| --- | --- |
| Narrowband physical layer cell identity | 0 |
| Initial subcarrier index | 0 |
| Uplink-downlink configuration for TDD | N=9 ; D/U=8 TBD (currently 1 in the TN) |
| Special subframe configuration for TDD | TBD (currently 7 in the TN) |

Table 8.5.3.2-2: Time error tolerance for AWGN, NTN-TDLA100-1

| PRACH | PRACH SCS | Time error tolerance |
| --- | --- | --- |
| preamble | (kHz) | NTN-TDLA100-1 |
| 0 | 3.75KHz | 3.646us |
| 1 | 3.75KHz | 3.646us |

- Recommended WF

  - Agree to update the NPRACH performance accordingly for TDD 249 band with specific NTN NB-IoT TDD format (U/D=8, N=9) in TS 38.108.

Issue 3-5-6: NTN NPRACH missed detection requirements for TDD in TS 36.108

- Proposals (P14/R4-2502027):

  - Proposal 1. Add a new table for NPRACH missed detection requirements for TDD. Consider 2 preamble formats: Preamble format 0 and Preamble format 1.

Table 8.5.3.2.1-2: NPRACH missed detection requirements for TDD

| Number of TX antennas | Number of RX antennas | TxDuration | Repetition number | Propagation conditions and correlation matrix (Annex D) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | TBD (currently 8) | TBD (currently 8) | NTN-TDLA100-1 | 200 Hz | TBD | TBD |
| 1 | 1 | TBD (currently 16) | TBD (currently 16) | NTN-TDLA100-1 | 200 Hz | TBD | TBD |
| 1 | 2 | TBD (currently 8) | TBD (currently 8) | NTN-TDLA100-1 | 200 Hz | TBD | TBD |
| 1 | 2 | TBD (currently 16) | TBD (currently 16) | NTN-TDLA100-1 | 200 Hz | TBD | TBD |

- Recommended WF:

  - Add a new table for NPRACH missed detection requirements for NTN TDD in TS 36.108.

  - Agree to consider Preamble format 0 and Preamble format 1 for NTN TDD 249 band in TS 36.108.

  - Repetition number and TxDuration to take into account specific NTN TDD NB-IoT specific format format (U/D=8, N=9). Other parameters (propagation conditions) could be reused from FDD NB-IoT NTN transmission mode in TS 36.108.

Issue 3-5-7: Number of SAN Rx antennas for demodulation requirements

  - Proposal 1. Do not consider 2 Rx antennas for SAN demodulation performance in NTN TDD 249 band

    - NOTE: the intention is to have both TDLA and TDLC configuration tested, which will increase the number of demodulation requirements, therefore we propose only 1 Rx SAN to be used.

- Recommended WF

  - Agree with Proposal 1 if no controversial discussion.

Issue 3-5-8: NPUSCH single-tone SCS

  - Proposal 1. Do not consider single-tone NPUSCH transmission with 3.75 kHz for demodulation performance in NTN TDD 249 band

- Recommended WF

  - Agree with Proposal 1 if no controversial discussion.

# Conclusion

See above issues and associated proposals.