3GPP TSG-RAN WG4 Meeting #116bis	R4-2514452

Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	On BS OTA test scope reduction

Agenda item:	6.8.4.2.1 - Identification and reduction of BS OTA test scope (TxIM, RX OoB blocking)

Document for:	Approval

Introduction

During last RAN4#116 meeting in Bangalore discussion on OTA test scope reduction was continued. A way forward was agreed in [1] which provides more conclusions on some detailed issues regarding the OTA test enhancements for this objective for BS RF evolution work item. This contribution continues further Nokia’s views on the topic.

Discussion

  - BS OTA test scope reduction

Following agreements on OTA scope reductions are included in the agreed Way forward after RAN4#116 meeting [1]:

Issue 1-1-1: test scope reduction for OBUE - Agreement: - use 3xCBW offset to check OBUE limits when the smallest CBW of the band is less than or equal to 5 MHz, otherwise 1xCBW offset. Issue 1-2-1: Out of band blocking - Agreement on step size of OOBB requirement, keep the following options for further discussion: - Option 1: 1/3 of the minimum supported bandwidth - Option 1a: · Frequency range (MHz); Minimum supported BS channel bandwidth (MHz); Measurement step size (MHz). FR1; 5; 1. 10; 3. 20; 6. - Option 2: use 2/3 of the test signal configuration channel bandwidths Frequency range (MHz); Minimum supported BS channel bandwidth (MHz); Measurement step size (MHz). FR1; 5; 3. 10; 6. 20; 13.

OTA out-of-band blocking

The OTA out-of-band blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the RIB at its assigned channel in the presence of an unwanted interferer.

The test stresses the ability of the receiver unit associated with the RIB under test to withstand high-level interference from unwanted signals at specified frequency bands, without undue degradation of its sensitivity.

The OTA general blocking interferer is injected via test antenna, with the blocking interferer producing specified interferer field strength level for each supported polarization. CW interfering signal is swept with a step size of 1 MHz within the frequency range corresponding to 30 MHz to FUL_low - ΔfOOB and from FUL_high + ΔfOOB up to 12750 MHz for FR1. Whilst for FR2, 1 MHz step size applies for frequency range 30 MHz to 6000 MHz, and above 6000MHz bigger step sizes is defined.

The OTA co-location blocking interferer for FR1 is injected via the CLTA. The CLTA is fed with the specified co-location blocking interferer power per supported polarization. CW interfering signal is swept with a step size of 1 MHz within the frequency range corresponding to downlink operating bands related to co-located systems

Observation 1: Interfering signal step size is 1 MHz in frequency range 30-12750MHz for FR1 and in frequency range 30-6000MHz for FR2.

Observation 2: Out of all test requirements of radiated conformance test specification TS 38.141-2, the OTA out-of-band blocking case is one of the cases which requires the largest test efforts.

This is largely due to 1 MHz step size used and, therefore, the number of measurements points is significantly large. Increase of step size from 1 MHz to X MHz effects test efforts proportionally.

Observation 3: Increase of interferer step size from 1 MHz to X MHz effects test efforts proportionally.

To reduce the amount of time used for the time-consuming OTA conformance testing, while continue to ensure that any spurious response within the receive bandwidth will still be verified with sufficient granularity, the measurement step size for interfering signal can be set larger than 1 MHz.

In 116 WF we have 2 options defined, in here we discuss a bit about those:

Option 1: 1/3 of the minimum supported bandwidth. For certain bands, the minimum bandwidth is 3 MHz for NR which leads to step size of 1 MHz. Therefore, instead of the minimum supported bandwidth, it is more relevant to use the test signal configuration channel bandwidths. This 1/3 of the minimum supported bandwidth was chosen based on similar topic in TR 38.817-2 for FR2, which was indeed based on assumption of 3rd harmonic of blocking interferer. Whether or not that 3rd harmonic argument is still valid, is hard to conclude or argue.

Observation 4: Whether or not that 3rd harmonic argument is still valid, is hard to conclude or argue.

Observation 5: instead of the minimum supported bandwidth, it is more relevant to use the test signal configuration channel bandwidths

In addition, other than 1/3 fraction will also provide the same or similar coverage e.g. 2/3 of the test signal configuration channel bandwidths.

It should be noted that in specification TS 37.145-2 defines test signal configuration ATCR7/ANTCR7 for OTA blocking. Furthermore, subclauses 4.11.2.10.1 and 4.11.2.11.1 text states:

“Unless otherwise stated, for all test configurations in this section, the narrowest supported NR channel bandwidth and lowest SCS for that bandwidth for the operating band shall be used in the test configuration.“

However, in TS 37.142-2 clause 4.11.2.10.2 and 4.11.2.11.2, there is reference to clause 4.11.1A, which says 20MHz for operating bands higher that 100MHz.

“Unless otherwise stated, the E-UTRA bandwidth shall be 5 MHz unless the BS does not support 5 MHz E-UTRA, in which case the E-UTRA bandwidth shall be the lowest supported bandwidth for the operating band.”

For 5 MHz channel bandwidths, 1/3 leads back to 1 MHz, which is not meeting the test effort reduction target.

Observation 6: For 5 MHz or 3 MHz supported channel bandwidths, 1/3 leads back to 1 MHz, which is not meeting the test effort reduction target.

“Option 2: use 2/3 of the test signal configuration channel bandwidths“. For this option, we see that test effort will be reduced compare to option 1 with minimum step size of 3 MHz for 5 MHz channel bandwidth up to 13 MHz for 20 MHz channel bandwidth.

Taking into account WF [1] For out of band blocking we support option 2.

Proposal 1: We support option 2: use 2/3 of the test signal configuration channel bandwidths

Proposal 2: If 3rd harmonic argument is still valid, then 1/3 of the test signal configuration channel bandwidths

Conclusion

This document shares Nokia’s views on BS OTA test scope reduction objective. The following observations and proposals have been made:

Observation 1: Interfering signal step size is 1 MHz in frequency range 30-12750MHz for FR1 and in frequency range 30-6000MHz for FR2.

Observation 2: Out of all test requirements of radiated conformance test specification TS 38.141-2, the OTA out-of-band blocking case is one of the cases which requires the largest test efforts.

Observation 3: Increase of interferer step size from 1 MHz to X MHz effects test efforts proportionally.

Observation 4: Whether or not that 3rd harmonic argument is still valid, is hard to conclude or argue.

Observation 5: instead of the minimum supported bandwidth, it is more relevant to use the test signal configuration channel bandwidths

Observation 6: For 5 MHz or 3 MHz supported channel bandwidths, 1/3 leads back to 1 MHz, which is not meeting the test effort reduction target.

Proposal 1: We support option 2: use 2/3 of the test signal configuration channel bandwidths

Proposal 2: If 3rd harmonic argument is still valid, then 1/3 of the test signal configuration channel bandwidths

- References

- R4-2512580 Way Forward for [116][304] NR_BS_RF_Part3_OTA_TRP, ZTE Corporation