3GPP TSG-RAN4 Meeting #116bis	R4-2514459

# Prague, Czech Republic, Oct 13th – Oct 17th 2025

Source: 	Qualcomm Incorporated

Title: 	RRM requirements of Rel-19 low band carrier aggregation via switching

Agenda item:	4.8.2

Document for:	Discussion

# Introduction

3GPP introduced a new work item on low band carrier aggregation via switching in RAN #106 [1]. We discuss some relevant RRM requirements for this topic.

# Discussion

According to [2], low band spectrum carries significant traffic volume in urban indoor and rural areas. However, SDL remains under-utilized since they cannot be paired with UL. That’s why, [2] proposed to introduce a network configured switching pattern in low band carrier aggregation via switching

Figure 1: RRC configured switching pattern in low band carrier aggregation via switching [2]

Figure 1 shows how network could configure this switching pattern to enable this low band carrier aggregation. This switching pattern would instruct UE to alternate between FDD DL+UL band and SDL band in a periodic manner. We discuss different RRM requirements of SCells and PCells in this scenario.

## Activation of Switching Pattern

RAN1 made the following agreement during the last meeting:

Issue 1-3-5: Activation of Switching Pattern [Agreement]: - Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options. - For MAC CE based SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length. - Other options are not precluded. - For RRC based direct SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length. - Other options are not precluded. Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline. Issue 1-3-5a: Deactivation of Switching Pattern [Agreement]: - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length). - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - FFS in maintenance stage: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot.

Figure 1: Timeline of Activation of Switching Pattern during MAC-CE based SCell Activation

Figure 1 shows the two options of activating switch patterns and the impact of T_{LBCA}.

If UE must switch immediately after THARQ + 3ms, i.e., TLBCA = 0 ms, then UE will have to switch at the beginning of a slot. RAN1 has already agreed that the UE is expected to switch at the end of the slot while following the RRC configured switching pattern. That same principle should hold during the activation of SCell and switching pattern. Hence, TLBCA should be equal to 1ms during both SCell activation and deactivation.

During SCell deactivation, UE cannot be scheduled with DL or UL if UE does not switch to PCell immediately after completion of SCell deactivation. Hence, during SCell deactivation, UE should not wait until the next switching period to switch to PCell from deactivated SCell. UE behavior regarding this should be consistent during SCell activation and deactivation. That means, UE should switch to SCell immediately after the RRC switching pattern configuration is applied.

Observation 1: If UE must switch immediately after THARQ + 3ms, i.e., TLBCA = 0 ms, then UE will have to switch at the beginning of a slot

Observation 2: RAN1 has already agreed that the UE is expected to switch at the end of the slot while following the RRC configured switching pattern.

Observation 3: During SCell deactivation, UE cannot be scheduled with DL or UL if UE does not switch to PCell immediately from deactivated SCell after completion of SCell deactivation.

Observation 4: UE should maintain consistent behaviour, in terms of applying switching pattern and performing switching, during both Scel activation and deactivation.

Proposal 1: RAN4 defines TLBCA to be 1ms during SCell activation and deactivation.

Proposal 2: UE is required to follow the switching pattern and perform switching, if needed, immediately after n+(THARQ+3ms+ 1ms)/slot-length and n+(TRRC_Process + 1ms)/slot-length during MAC-CE based SCell activation and RRC based direct SCell activation respectively. Switching is placed at the end of the slot.

Proposal 3: If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after TLBCA. Switching is placed at the end of the slot.

## UE Feature regarding RRC based Direct SCell Addition in NR LB-CA

RAN4 made the following agreements during the past few meetings [4] [3]

Online agreement on Tuesday: - PUCCH SCell activation, Rel-19 fast SCell activation through EMR, and activation of multiple SCells is not in the scope of the WI. Agreement on Thursday: - The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature. - R16 Direct SCell activation with HO is not considered for R19 LB CA. - R17 PUCCH SCell activation is not considered for R19 LB CA. - R18 SSB-less SCell activation for NES is not considered for R19 LB CA. - R17 Fast SCell activation is not considered for R19 LB CA. - Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation. - Known/unknown conditions are reused from existing SCell activation requirements

- For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms

These agreements suggest that RRC based direct SCell addition feature is allowed in Rel-19 NR LB-CA. RAN4 has not, so far, introduced any separate UE capability for RRC based direct SCell addition in Rel-19 NR LB-CA. That means, according to current status, UE is expected to support RRC based direct SCell addition feature if it signals the capability to support RRC based direct SCell addition and Rel-19 NR LB-CA feature.

It is safer for UE to signal support for a feature only after it completes IODT with infra vendor regarding that feature. Since Rel-19 NR LB-CA is a new feature, infra vendors are expected to first bring up this feature with MAC-CE based SCell activation. Infra vendors may bring up Rel-19 NR LB-CA with direct SCell activation later. With the current status of UE features, UE vendors may have to pass conformance test of NR LB-CA with direct SCell activation without going through IODT. This is not desirable. This issue can be resolved by introducing a UE capability regarding RRC based direct SCell addition in NR LB-CA.

Observation 5: RRC based direct SCell addition feature is allowed in Rel-19 NR LB-CA.

Observation 6: RAN4 has not introduced any separate UE capability for RRC based direct SCell addition in Rel-19 NR LB-CA. UE is expected to support RRC based direct SCell addition feature if it signals the capability to support RRC based direct SCell addition and Rel-19 NR LB-CA features.

Observation 7: It is safer for UE to signal support for a feature only after it completes IODT with infra vendor regarding that feature.

Observation 8: Infra vendors are expected to first bring up this feature with MAC-CE based SCell activation. Infra vendors may bring up Rel-19 NR LB-CA with direct SCell activation later.

Observation 9: According to current status, UE vendors may have to pass conformance test of NR LB-CA with direct SCell activation without going through IODT.

Proposal 4: RAN4 introduces a new capability that allows UE to signal whether it supports RRC based direct SCell activation in Rel-19 NR LB-CA.

- Potential details of the capability are shown in Table 1

Table 1: Details of UE capability for RRC based Direct SCell Activation in Rel-19 NR LB-CA

| Features | Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 55. NR_LBCA_Sw | [55-x] | RRC based Direct SCell Activation in Rel-19 NR LB-CA | [UE supports RRC based direct SCell activation in Rel-19 NR LB-CA] | 55-1 | [UE will not support RRC based SCell activation in Rel-19 NR LB-CA] | [Optional with capability signaling] |

# Conclusions

Observation 1: If UE must switch immediately after THARQ + 3ms, i.e., TLBCA = 0 ms, then UE will have to switch at the beginning of a slot

Observation 2: RAN1 has already agreed that the UE is expected to switch at the end of the slot while following the RRC configured switching pattern.

Observation 3: During SCell deactivation, UE cannot be scheduled with DL or UL if UE does not switch to PCell immediately from deactivated SCell after completion of SCell deactivation.

Observation 4: UE should maintain consistent behaviour, in terms of applying switching pattern and performing switching, during both Scel activation and deactivation.

Observation 5: RRC based direct SCell addition feature is allowed in Rel-19 NR LB-CA.

Observation 6: RAN4 has not introduced any separate UE capability for RRC based direct SCell addition in Rel-19 NR LB-CA. UE is expected to support RRC based direct SCell addition feature if it signals the capability to support RRC based direct SCell addition and Rel-19 NR LB-CA features.

Observation 7: It is safer for UE to signal support for a feature only after it completes IODT with infra vendor regarding that feature.

Observation 8: Infra vendors are expected to first bring up this feature with MAC-CE based SCell activation. Infra vendors may bring up Rel-19 NR LB-CA with direct SCell activation later.

Observation 9: According to current status, UE vendors may have to pass conformance test of NR LB-CA with direct SCell activation without going through IODT.

Proposal 1: RAN4 defines TLBCA to be 1ms during SCell activation and deactivation.

Proposal 2: UE is required to follow the switching pattern and perform switching, if needed, immediately after n+(THARQ+3ms+ 1ms)/slot-length and n+(TRRC_Process + 1ms)/slot-length during MAC-CE based SCell activation and RRC based direct SCell activation respectively. Switching is placed at the end of the slot.

Proposal 3: If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after TLBCA. Switching is placed at the end of the slot.

Proposal 4: RAN4 introduces a new capability that allows UE to signal whether it supports RRC based direct SCell activation in Rel-19 NR LB-CA.

- Potential details of the capability are shown in Table 1

Table 1: Details of UE capability for RRC based Direct SCell Activation in Rel-19 NR LB-CA

| Features | Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 55. NR_LBCA_Sw | [55-x] | RRC based Direct SCell Activation in Rel-19 NR LB-CA | [UE supports RRC based direct SCell activation in Rel-19 NR LB-CA] | 55-1 | [UE will not support RRC based SCell activation in Rel-19 NR LB-CA] | [Optional with capability signaling] |

# References

[1] RP-243317, New WID on low band carrier aggregation via switching, 3GPP TSG RAN #106.

[2] RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19, 3GPP TSG RAN #106.

[3] R4-2512119, WF on RRM requirements for NR_LBCA_Sw, RAN4 115.

[4] R4-2508259, “WF on RRM requirements for NR_LBCA_Sw”, RAN4 #115