3GPP TSG-RAN4 Meeting #116bis	R4-2514464

# Prague, Czech Republic, 13th – 17th October, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for AI-ML based Positioning

Agenda item:	6.11.5

Document for:	Discussion

# Introduction

In this paper, we present our views regarding RRM core requirements for case 1.

# Discussion

## Case 1 Evaluation Period

Figure 1 shows different cases of 5G AI-ML based positioning. RAN4 made the following agreement regarding case 1 during the last meeting [3]

Issue 3-1: Reporting Delay Requirements for case 1 Agreement: Take the framework of the existing reporting delay requirements - Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report - Check until next meeting on the number of samples needed for the measurements

RAN4 agreed to take the framework of the existing reporting delay requirements to define reporting delay requirements for case 1. Case 1 allows UE to directly calculate its co-ordinates via AI-ML. Legacy RRM spec does not have any requirement regarding evaluation period of UE co-ordinate calculation. On the other hand, legacy RRM spec has defined measurement period requirements for several UE assisted positioning metrics, e.g.: RSTD, RX-TX time difference, PRS-RSRP, etc. RAN1’s most recent agreement does not clarify which legacy measurement delay requirement will be used to define the measurement period requirement of case 1.

UE needs to calculate channel impulse response of different TRPs to run AI-ML based inference and estimate its coordinates. Hence, while looking for the baseline of evaluation period of case 1, RAN4 needs to select a UE assisted positioning metric that also requires UE to measure channel impulse response of different TRPs. RSTD and RX-TX time difference metrics fit this category. PRS-RSRP metric does not fit this category because UE needs to calculate RSRP of PRS resources, and not the channel impulse response of TRPs, during PRS-RSRP measurement.

The difference in pre-processing between PRS-RSRP and RSTD also shows up in the evaluation period requirements of these metrics. The eval period of RSTD in each positioning frequency layer has a scaling factor for measurement of same PRS resource with multiple RX TEGs (kmultiTEG,i). This scaling factor does not show up in the measurement period of PRS-RSRP.

UE does not need to transmit to estimate its co-ordinates via AI-ML based inference. On the other hand, the estimation of RX-TX time difference requires UE to receive and transmit. Hence, UE’s required pre-processing to estimate its co-

Figure: Different cases of AI-ML based positioning in AI-ML air interface

ordinate via AI-ML is closer to UE’s required pre-processing to estimate RSTD. Hence, the eval period of RSTD is a suitable option to act as the baseline of the eval period of case 1, i.e., AI-ML based direct calculation of UE co-ordinates.

The eval period of Rel-17 RSTD (shown in appendix section) assumes the number of PRS RSTD measurement samples to be 1, 2 or 4 depending on UE capability. Due to the potential similarity in pre-processing between RSTD and case 1 positioning, the same number of samples can be assumed in the measurement period of case 1 positioning.

Observation 1: Legacy RRM spec does not have any requirement regarding evaluation period of UE’s direct calculation of its co-ordinates.

Observation 2:  Legacy RRM spec has defined measurement period requirements for several UE assisted positioning metrics, e.g.: RSTD, RX-TX time difference, PRS-RSRP, etc.

Observation 3: UE needs to calculate channel impulse response of different TRPs to run AI-ML based inference and estimate its coordinates.

Observation 4: UE needs to calculate channel impulse response of different TRPs to calculate RSTD or RX-TX time difference metrics.

Observation 5: UE needs to calculate RSRP of PRS resources, and not the channel impulse response of TRPs, during PRS-RSRP measurement.

Observation 6: The eval period of RSTD in each positioning frequency layer has a scaling factor for measurement of same PRS resource with multiple RX TEGs (kmultiTEG,i). This scaling factor does not show up in the measurement period of PRS-RSRP.

Observation 7: UE does not need to transmit to estimate its co-ordinates via AI-ML based inference. On the other hand, the estimation of RX-TX time difference requires UE to receive and transmit.

Observation 8: The eval period of Rel-17 RSTD (shown in appendix section) assumes the number of PRS RSTD measurement samples to be 1, 2 or 4 depending on UE capability.

Proposal 1: RAN4 uses the measurement delay of 5G NR RSTD as the baseline of measurement delay of AI-ML based positioning case 1. The number of samples needed for the measurement for positioning case 1 also follows the required number of measurement samples for 5G NR RSTD.

# Conclusions

Observation 1: Legacy RRM spec does not have any requirement regarding evaluation period of UE’s direct calculation of its co-ordinates.

Observation 2:  Legacy RRM spec has defined measurement period requirements for several UE assisted positioning metrics, e.g.: RSTD, RX-TX time difference, PRS-RSRP, etc.

Observation 3: UE needs to calculate channel impulse response of different TRPs to run AI-ML based inference and estimate its coordinates.

Observation 4: UE needs to calculate channel impulse response of different TRPs to calculate RSTD or RX-TX time difference metrics.

Observation 5: UE needs to calculate RSRP of PRS resources, and not the channel impulse response of TRPs, during PRS-RSRP measurement.

Observation 6: The eval period of RSTD in each positioning frequency layer has a scaling factor for measurement of same PRS resource with multiple RX TEGs (kmultiTEG,i). This scaling factor does not show up in the measurement period of PRS-RSRP.

Observation 7: UE does not need to transmit to estimate its co-ordinates via AI-ML based inference. On the other hand, the estimation of RX-TX time difference requires UE to receive and transmit.

Observation 8: The eval period of Rel-17 RSTD (shown in appendix section) assumes the number of PRS RSTD measurement samples to be 1, 2 or 4 depending on UE capability.

Proposal 1: RAN4 uses the measurement delay of 5G NR RSTD as the baseline of measurement delay of AI-ML based positioning case 1. The number of samples needed for the measurement for positioning case 1 also follows the required number of measurement samples for 5G NR RSTD.

# References

[1] RP-2423999, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN 105, Sep 2024.

[2] RP-250792, “Revised WID for NR_AIML_air”, 3GPP TSG RAN meeting #107.

[3] R4-2511890, “WF on AI-ML air interface”, 3GPP TSG-RAN WG4 Meeting #116.

[4] 3GPP TS 38.133, v 17.11.0

[5] 3GPP TS 37.355, v 19.0.0

# Appendix A: Rel-17 Measurement Period Requirement of RSTD [4]

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

Where ,

i is the index of positioning frequency layer,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSTD,i=kmultiTEG,i*CSSFPRS,i*ceil( Kp,PRS,i)*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1;

and in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

CSSFPRS,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in clause 9.1.5.2.

kmultiTEG,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i=1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation;

otherwise,

kmultiTEG=NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

-	For a window W of duration max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioningfrequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable =0.

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the

sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe, Tlast,i = Ti +MGL. Otherwise, Tlast,i = Ti + Tavailable_PRS,i ,

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Where,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi.

MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the PRS frequency layer i.	TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where

TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Note: For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [34].