3GPP TSG-RAN WG4 Meeting #116bis	R4-2514489

Prague, Czech Republic, October 13th - 17th, 2025

Source:	Nokia

Title:	RRM performance requirements for NR NTN RedCap

Agenda item:	6.19.3.1

Document for:	Discussion

Introduction

RAN plenary has revised the WI for NR NTN enhancements in [1], which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_Ph3).

In this document we provide our views on the 5th objective, namely the support of Rel-17 and Rel-18 RedCap and (e)RedCap UEs in NTN systems, related to RRM performance requirements considering the open issues identified in the latest WF from RAN4 #116 [2] and further detailed in the Topic summary [3].

Discussion

In the present document we discuss the open issues presented in the WF [2] and the Topic summary [3], as listed below:

- Issue 2-1-1: Measurement accuracy requirement

- Issue 2-1-2: Test principle and setups

- Issue 2-1-3: How and where to capture the test cases for (e)RedCap UE with FR1-NTN

- Issue 2-1-4: Test configuration and parameters for (e)RedCap UE with FR1-NTN

- Issue 2-1-5-1: Whether to define separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN

- Issue 2-1-5-2: Test case list

RAN4 has discussed general aspects for RRM performance requirements for (e)RedCap UE with FR1-NTN.

  - Measurement accuracy

At RAN4 #116, following agreement was achieved [2].

Issue 2-1-1: Measurement accuracy requirement Agreement: - FFS: RAN4 to define accuracy requirements for following measurements for RedCap in NTN. - Intra- and inter-frequency RSRP - Intra- and inter-frequency RSRQ - Intra- and inter-frequency SINR - L1-RSRP - Rx-Tx for NW verified location - For L1/L3 measurement accuracy, RAN4 to follow the agreements reached in RAN4 #114-bis meeting. - For measurement accuracy for NW verified location, RAN4 to follow the agreements reached in RAN4 #114 meeting. - RAN4 to define new sections for measurement accuracy requirements with suffix ‘10.X’ for (e)RedCap UE with NTN. - Further to discuss the side conditions.

One open aspect is the definition of side conditions. We propose to reuse side conditions from RedCap in TN.

For 2Rx (e)RedCap UE with NTN, the same side conditions are applied as for normal UE with NTN, i.e. following  side conditions are applicable:

- Intra- and inter-frequency RSRP: clauses 10.1.2C and 10.1.4C

- Intra- and inter-frequency RSRQ: clauses 10.1.7C and 10.1.9C

- Intra- and inter-frequency SINR: clauses 10.1.12C and 10.1.14C

- L1-RSRP: clause 10.1.19C

- Rx-Tx for NW verified location: clause 10.1.25C

For 1 Rx (e)RedCap UE with NTN, the following side conditions are applicable:

- Intra- and inter-frequency RSRP: clauses 10.1A.2 and 10.1A.4

- Intra- and inter-frequency RSRQ: clauses 10.1A.6 and 10.1A.8

- Intra- and inter-frequency SINR: clauses 10.1A.10 and 10.1A.12

- L1-RSRP: clause 10.1A.14

- Rx-Tx for NW verified location: clause 10.1A.18

Following side conditions are applicable for 2Rx and 1 Rx UE with FR1-NTN:

| Measurement type | 1Rx (e)RedCap UE | 2Rx (e)RedCap UE |
| --- | --- | --- |
| Intra- and inter-frequency RSRP | clauses 10.1A.2 and 10.1A.4 | clauses 10.1.2C and 10.1.4C |
| Intra- and inter-frequency RSRQ | clauses 10.1A.6 and 10.1A.8 | clauses 10.1.7C and 10.1.9C |
| Intra- and inter-frequency SINR | clauses 10.1A.10 and 10.1A.12 | clauses 10.1.12C and 10.1.14C |
| L1-RSRP | clause 10.1A.14 | clause 10.1.19C |
| Rx-Tx for NW verified location | clause 10.1A.18 | clause 10.1.25C |

Test principle

At RAN4#116, the following way forward was provided by Moderator [2].

#### Issue 2-1-2: Test principle and setups <Way Forward>: - Proposal 1: For test principle and setups, the testing related to satellite access and RedCap defined in A.3.35 and A.3.36 can be a reference. - Proposal 2: Take the NTN test setups as the reference, except both FDD and HD-FDD shall be involved in test configuration.

Both proposals serve well for identifying the test principle for (e)RedCap UE with FR1-NTN. While for satellite access, FDD configurations have been assumed for cell reselection in RRC_IDLE state and TDD configurations only for E-UTRAN or NR TN target cell, TDD configurations are also assumed for tests in RRC_CONNECTED state. HD-FDD configurations need also to be tested for RRC_IDLE and RRC_CONNECTED state regarding reporting delay tests. For measurement accuracy tests, both 1Rx and 2Rx (e)RedCap UEs need to be addressed.

For the test principle and setups, NTN tests in A.3.36 and RedCap tests in A.3.35 need to be merged to allow, in addition to FDD and TDD configurations, testing of HD-FDD configurations both in RRC_IDLE and RRC_CONNECTED state. Aside 2Rx (e)RedCap UE, 1 Rx (e)RedCap UE tests are needed for procedures in RRC_IDLE and RRC_CONNECTED state.

Place of (e)RedCap test cases in spec

At RAN4#116, the following way forward was provided by Moderator [2].

#### Issue 2-1-3: How and where to capture the test cases for (e)RedCap UE with FR1-NTN <Way Forward>: - Proposal 1: The existing test cases defined for FR1 NTN in A.14 can be reused as baseline for RedCap UEs with FR1-NTN. - Proposal 1a: RAN4 will further adapt the test cases for RedCap UE, e.g. test configuration, 1Rx/2Rx and so on. - Proposal 1b: Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values - Proposal 2: RAN4 to use the ‘refer to’ method when drafting the performance spec to avoid duplicated contents, as in core requirements. - Proposal 2a: RAN4 only clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs. - Proposal 3: - RAN4 to define new sections for test cases with suffix ‘A.X’. - Proposal 4: For RedCap UEs in NTN scenario, - To incorporate clauses for RedCap in NTN within Clause A.14, - Some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

We agree with the Proposals 1 and 2. The test cases for 1Rx and 2Rx (e)RedCap UE with FR1-NTN should be included in Annex A.14. New test cases are needed for HD-FDD for 1Rx and 2 Rx (e)RedCap UE and for FDD for 1Rx (e)RedCap UE, whilst 2Rx FDD (e)RedCap UE is expected to reuse the legacy NTN RRM test cases in FR1. One open issue is the support of NCD-SSB configurations, which is discussed in section 2.4 below.

RAN4 to include test cases for (e)RedCap UE with FR1-NTN in Annex A.14, referring to legacy NTN test cases where appropriate. New test cases in A.14 are needed for NCD-SSB configurations for all duplex modes, if agreed to be tested.

Test configurations and parameters

At RAN4#116, the following way forward was provided by Moderator [2].

#### Issue 2-1-4: Test configuration and parameters for (e)RedCap UE with FR1-NTN <Way Forward>: - Proposal 1: - Both FDD and HD-FDD duplex mode should be considered in test configuration for (e)RedCap UE with FR1-NTN. - 15 kHz SCS and 10MHz bandwidth can be configured in test cases. - Both GSO and NGSO are configured in test cases for RedCap UE with FR1-NTN. - If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. - Proposal 1a: Take the NTN test setups as the reference, except both FDD and HD-FDD shall be involved in test configuration. - Proposal 1b: RAN4 to use following supported test configurations for (e)RedCap with FR1-NTN. Table A.14.5.x.y.z: Supported test configurations Configuration; Description. 1; GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW. 2; NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW. 3; GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW. 4; NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW. NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both.. - Proposal 1c: RAN4 shall consider the following general test configurations as follows: Configuration; Description. 1; GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz. 2; NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz. 3; GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz. 4; NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz. NOTE: The UE is only required to be tested in one of the supported test configurations. - Proposal 2: - For the proposed test configuration, the existing RMC related parameters, Io values, SSB configuration and SMTC configurations can be reused for RedCap UE with FR1-NTN. - RAN4 not to consider NCD-SSB related configuration in test cases for (e)RedCap UE with NTN. - RAN4 not to consider RedCap specific UL/DL BWP configuration for (e)RedCap UE with NTN.

We agree with Proposals 1, 1a and 1b. Proposal 1c is clearly not aligned to tests for NTN normal UEs.

RAN4 to adopt Proposals 1, 1a and 1b in issue 2-1-4 in R4-2512146 related to test case configurations.

The above Proposal 2 requires more consideration. We propose to consider each of the three bullets in separate issues.

Regarding the first bullet, further study is needed, if existing RMC related parameters for terrestrial RedCap, Io values, SSB configurations and SMTC configurations can be reused.

RAN4 to further study is needed, if existing RMC related parameters for terrestrial RedCap, Io values, SSB configurations and SMTC configurations can be reused.

Regarding the second bullet, further discussion is needed if NCD-SSB related configurations should be tested for (e)RedCap UE with FR1-NTN or not. It is noted, that NCD-SSB configurations are used in RedCap TN test cases in Annex A.16 for handover, intra-frequency measurements and RSTD measurements. While the last one is not relevant for (e)RedCap with FR1-NTN, the two first ones are relevant. Also, the work item [1] stresses the importance of the global coverage of RedCap UEs, i.e. the support of the (e)RedCap feature independent of the TN or NTN scenario (“Global coverage would clearly benefit RedCap devices.”). Thus we support the test of NCD-SSB configuration(s) for (e)RedCap with FR1-NTN to keep the same level of functionality as for TN.

RAN4 to consider test of NCD-SSB configuration(s) for (e)RedCap with FR1-NTN to keep the same level of functionality as for TN, reusing test configurations for (e)RedCap in TN as baseline.

Regarding the third bullet, further discussion is needed if RedCap specific DL/UL BWP configuration(s) for (e)RedCap should be tested for (e)RedCap UE with FR1-NTN or not. It is noted, that for NCD-SSB configurations such dedicated DL and UL BWP have been specified in RedCap TN test cases, and these could be reused here. Again the same level of functionality as for TN should be targeted.

RAN4 to consider test of RedCap specific DL/UL BWP configuration(s) for (e)RedCap with FR1-NTN to keep the same level of functionality as for TN, reusing test configurations for (e)RedCap in TN as baseline.

Separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN

At RAN4#116, the following way forward was provided by Moderator [2].

##### Issue 2-1-5-1: Whether to define separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN, <Way Forward>: - Proposal 1: RAN4 to define the separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN. - Proposal 2: Further discuss whether to define separate test cases for 1Rx and 2Rx RedCap UEs - Proposal 3: Discuss case by case.

In our view, Proposal 1 can serve as baseline. For RedCap UE test cases, specified in Annex A.16, test cases are distinguished between 1Rx UE and 2 Rx UE: for cell reselection, for CG-SDT, for handover from FR1 to FR1, for handover to E-UTRAN, for RRC Re-establishment, for Tx timing, TA adjustment, RLM, BFD, active BWP switch, UE specific CBW change and for RRM measurement types SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP. Thus, similar structuring of the test cases should be done for (e)RedCap UE with FR1-NTN.

RAN4 to adopt separate test cases for 1Rx and 2Rx (e)RedCap with FR1-NTN as baseline and evaluate thereafter, if test cases can be merged to reduce the extent of changes to spec.

Test case list

At RAN4#116, the following way forward was provided by Moderator [2].

##### Issue 2-1-5-2: Test case list <Way Forward>: - Try to summarize based on the input from companies and RAN4 need to further discuss which specific test cases with configuration can be deleted to reduce test effort. Test categories/configurations; Company comments; Test cases (as baseline). RRC_IDLE state mobility; 4.2E Cell Re-selection for NR RedCap UE with Satellite Access - Intra/inter-f - 1Rx/2Rx - with the feature for enhanced requirements (without differentiating 1Rx and 2Rx) - time-based/location-based - fulfilling low mobility/ not-at-cell edge relaxed measurement criterion - inter-RAT - with TN carrier; Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC, Nokia supports these tests. On the FFS items, they should be tested as well, except the location-based measurement.; - Cell reselection to FR1 intra-frequency NR case for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements - Time-based measurement initiation to FR1 intra-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN - FFS: Location-based measurement initiation to FR1 intra-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-frequency NR case for 1Rx/2Rx RedCap UE with NTN - FFS: Cell re-selection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements - FFS: Time-based measurement initiation to FR1 inter-frequency cell reselection for 1Rx/2Rx RedCap UE with NTN - Location-based measurement initiation to FR1 inter-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-RAT for NR NTN carrier for 1Rx/2Rx RedCap UE with NTN - Cell re-selection to FR1 inter-frequency NR case with TN carrier for 1Rx/2Rx RedCap UE with NTN. RRC_IDLE state mobility; 5.5E Configured Grant based Small Data Transmissions (CG-SDT) for RedCap UE with satellite access - 1Rx/2Rx; Support: CATT Not support: OPPO Nokia supports this test, as specified for RedCap in TN.; - NR UE CG-SDT Test in FR1 for 1Rx/2Rx RedCap UE with NTN. RRC_CONNECTED state mobility; 6.1F.1 NR SAN Handover - Intra/inter-f - 1Rx/2Rx - RACH-based/RACH-less; Support: CATT, Samsung, Ericsson, ZTE, CMCC Not support: OPPO Nokia supports these tests.; - Intra-frequency SAN Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - Inter-frequency SAN Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - Intra-frequency SAN RACH-less Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN. 6.1F.2 NR SAN Conditional Handover - Intra/inter-f - 1Rx/2Rx - time-based/distance-based CHO - CHO without L3 measurement criteria; Support: CATT, OPPO (choose one from time-based and distance-based CHO), Samsung, Ericsson, ZTE, CMCC Nokia supports these tests. On the FFS items, they may not be specified to reduce test effort.; - Intra-frequency SAN time-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Inter-frequency SAN time-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - Inter-frequency SAN distance-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN. 6.1F.3 NR SAN Satellite switching with re-synchronization - 1Rx/2Rx - RACH-based/RACH-less - Hard/soft; Support: CATT, Samsung, Ericsson, ZTE, CMCC Not support: OPPO Nokia supports these tests.; - RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN for 1Rx/2Rx RedCap UE with NTN - RACH-less Soft Satellite switching with re-synchronization from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN for 1Rx/2Rx RedCap UE with NTN. 6.2E RRC Connection Mobility Control for RedCap UE with Satellite Access - Intra/inter-f - contention based/ non-contention based - only 4-step RA type (follow NTN); Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC Nokia supports these tests.; SA: RRC Re-establishment for SAN - Intra-frequency RRC Re-establishment in FR1 for 1Rx/2Rx RedCap UE with NTN - Inter-frequency RRC Re-establishment in FR1 for 1Rx/2Rx RedCap UE with NTN Random Access - 4-step RA type contention based random access test in FR1 for NR standalone for 1Rx/2Rx RedCap UE with NTN - 4-step RA type non-contention based random access test in FR1 for NR standalone for 1Rx/2Rx RedCap UE with NTN RRC Connection Release with Redirection - Redirection from NR in FR1 to NR in FR1 for 1Rx/2Rx RedCap UE with NTN. Timing; 7.1E UE transmit timing for Redcap with Satellite Access - FFS: without differentiating 1Rx and 2Rx; Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, CMCC Nokia supports this test with removing FFS.; - NR UE Transmit Timing Test for FR1. 7.3E Timing advance for Redcap with Satellite Access - FFS: without differentiating 1Rx and 2Rx; Support: CATT, Samsung, Ericsson, CMCC Not support: OPPO Nokia supports this test with removing FFS.; - SA FR1 timing advance adjustment accuracy. Signalling characteristics; 8.1E Radio Link Monitoring for RedCap UE with Satellite Access - 1Rx/2Rx - Other aspect will follow legacy NTN test cases; Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC Nokia supports these tests.; - Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode for 1Rx/2Rx RedCap UE with NTN - Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode for 1Rx/2Rx RedCap UE with NTN - Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode for 1Rx/2Rx RedCap UE with NTN - Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS based RLM in DRX mode for 1Rx/2Rx RedCap UE with NTN. 8.5E Link Recovery Procedures for RedCap UE with Satellite Access - 1Rx/2Rx - Other aspect will follow legacy NTN test cases; Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC Nokia supports these tests.; - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode for 1Rx/2Rx RedCap UE with NTN - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS based BFD and LR in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS based BFD and LR in DRX mode for 1Rx/2Rx RedCap UE with NTN. 8.6E Active BWP switch delay for RedCap UE with satellite access - FFS: without differentiating 1Rx and 2Rx; Support: CATT, Samsung, Ericsson, CMCC Not support: OPPO Nokia supports this test. It is then aligned to normal NTN UE..; - DCI-based and Timer-based Active BWP Switch - RRC-based Active BWP Switch. 8.13E UE-specific CBW change for RedCap UE with satellite access - FFS: without differentiating 1Rx and 2Rx; Support: CATT, Samsung, Ericsson, CMCC Not support: OPPO Nokia supports this test. It is then aligned to normal NTN UE.; - UE specific CBW change on PCell in FR1 in non-DRX. 8.14E Pathloss reference signal switching delay for RedCap UE with satellite access - FFS: without differentiating 1Rx and 2Rx; Support: CATT, Samsung, Ericsson Not support: OPPO Nokia supports this test. It is then aligned to normal NTN UE.; - MAC-CE based pathloss reference signal switch delay. Measurement procedure; 9.2E NR intra-frequency measurements for RedCap with satellite access - 1Rx/2Rx - Other aspect will follow legacy NTN test cases; Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC Nokia supports these tests; - SA event triggered reporting tests without gap under non-DRX for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests without gap under DRX for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests with single measurement gap under non-DRX for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests with FNO concurrent gaps under DRX for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for 1Rx/2Rx RedCap UE with NTN. 9.3E NR inter-frequency measurements for Redcap UEs with satellite access - 1Rx/2Rx - Other aspect will follow legacy NTN test cases; Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC Nokia supports these tests; - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overlapping for 1Rx/2Rx RedCap UE with NTN - Event triggered reporting test without gap under non-DRX for 1Rx/2Rx RedCap UE with NTN - Event triggered reporting tests without gap under DRX for 1Rx/2Rx RedCap UE with NTN. 9.5E L1-RSRP measurements for Reporting for RedCap UE with satellite access - SSB/ CSI-RS based - DRX/non-DRX - 1Rx/2Rx; Support: CATT, OPPO (only test the delta part), Samsung, Ericsson, ZTE, CMCC Nokia supports these tests.; - SSB based L1-RSRP measurement for satellite access when DRX is not used for 1Rx/2Rx RedCap UE with NTN - SSB based L1-RSRP measurement for satellite access when DRX is used for 1Rx/2Rx RedCap UE with NTN - CSI-RS based L1-RSRP measurement for satellite access when DRX is not used for 1Rx/2Rx RedCap UE with NTN - CSI-RS based L1-RSRP measurement for satellite access when DRX is used for 1Rx/2Rx RedCap UE with NTN. 9.9E NR measurements for positioning; FFS: Nokia Not support: OPPO Nokia: Not support this test. It is then aligned to normal NTN UE.. Measurement Performance; SS-RSRP/SS-RSRQ/SS-SINR Measurement Performance - Intra/inter-f - 1Rx/2Rx; Support: CATT, OPPO (only test the delta part), Ericsson, CMCC Nokia supports these tests.; SS-RSRP - SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN - SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN SS-RSRQ - SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN - SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN SS-SINR - SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN - SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN. L1-RSRP measurement Performance for beam reporting - SSB/ CSI-RS based - 1Rx/2Rx; Support: CATT, OPPO (only test the delta part), Ericsson, CMCC Nokia supports these tests.; - SSB based L1-RSRP measurement for 1Rx/2Rx RedCap UE with NTN - CSI-RS based L1-RSRP measurement on resource set with repetition off for 1Rx/2Rx RedCap UE with NTN.

We agree with the above test case list with comments marked in yellow. In addition, the open issue whether NCD-SSB configurations should be tested as well, as depicted in section 2.4, needs to be addressed.

Specify the tests in the test case list, taking into account our comments in section 2.6 marked in yellow and achieve agreement in RAN4 whether NCD-SSB configurations as well need to be tested.

Conclusion

This paper has analysed the open issues related to performance requirements for (e)RedCap UE with FR1-NTN and for DL coverage enhancement, provided in the latest WF for this WI [2], and the following set of proposals are made.

- Following side conditions are applicable for 2Rx and 1 Rx UE with FR1-NTN:

| Measurement type | 1Rx (e)RedCap UE | 2Rx (e)RedCap UE |
| --- | --- | --- |
| Intra- and inter-frequency RSRP | clauses 10.1A.2 and 10.1A.4 | clauses 10.1.2C and 10.1.4C |
| Intra- and inter-frequency RSRQ | clauses 10.1A.6 and 10.1A.8 | clauses 10.1.7C and 10.1.9C |
| Intra- and inter-frequency SINR | clauses 10.1A.10 and 10.1A.12 | clauses 10.1.12C and 10.1.14C |
| L1-RSRP | clause 10.1A.14 | clause 10.1.19C |
| Rx-Tx for NW verified location | clause 10.1A.18 | clause 10.1.25C |

For the test principle and setups, NTN tests in A.3.36 and RedCap tests in A.3.35 need to be merged to allow, in addition to FDD and TDD configurations, testing of HD-FDD configurations both in RRC_IDLE and RRC_CONNECTED state. Aside 2Rx (e)RedCap UE, 1 Rx (e)RedCap UE tests are needed for procedures in RRC_IDLE and RRC_CONNECTED state.

RAN4 to include test cases for (e)RedCap UE with FR1-NTN in Annex A.14, referring to legacy NTN test cases where appropriate. New test cases in A.14 are needed for NCD-SSB configurations for all duplex modes, if agreed to be tested.

RAN4 to adopt Proposals 1, 1a and 1b in issue 2-1-4 in R4-2512146 related to test case configurations.

RAN4 to further study is needed, if existing RMC related parameters for terrestrial RedCap, Io values, SSB configurations and SMTC configurations can be reused.

RAN4 to consider test of NCD-SSB configuration(s) for (e)RedCap with FR1-NTN to keep the same level of functionality as for TN, reusing test configurations for (e)RedCap in TN as baseline.

RAN4 to consider test of RedCap specific DL/UL BWP configuration(s) for (e)RedCap with FR1-NTN to keep the same level of functionality as for TN, reusing test configurations for (e)RedCap in TN as baseline.

RAN4 to adopt separate test cases for 1Rx and 2Rx (e)RedCap with FR1-NTN as baseline and evaluate thereafter, if test cases can be merged to reduce the extent of changes to spec.

Specify the tests in the test case list, taking into account our comments in section 2.6 marked in yellow and achieve agreement in RAN4 whether NCD-SSB configurations as well need to be tested.

- References

- RP-251954, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT, RAN #109

- R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT), RAN4 #116

- R4-2509069, Topic summary for [116][226] NR_NTN_Ph3_Part1, Moderator (CATT), RAN4 #116