3GPP TSG-RAN WG4 Meeting #116-bis								                        R4-2514501

Prague, Czech Republic, October 13 – 17, 2025

Agenda item:			7.1.3

Source:	Ericsson

Title:	On HPUE 4Tx PC1 coexistence study aspects

Document for:	Approval

# Introduction

In RAN#109, a new WID has been approved [1] for “UE RF enhancements for NR FR1, Phase 5” WI. According to RAN#109 discussion, for “High power UE (HPUE) for NR single carrier operation” part, only the work on the following two objectives will begin in RAN4#114-bis meeting:

The objectives of core part for UE RF enhancements for NR FR1 Phase 5 include: High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25

In the following, we will focus on the co-existence study for ACLR part.

# Discussion

In Rel-15, the NR ACLR requirements for PC3 and PC2 were derived based on the co-existence study. For PC3, the ACLR requirement is equal to 30 dBc, meaning that the total leaked power over the entire adjacent channel bandwidth should not be larger than -7 dBm.

The ACLR requirements for other power classes were derived based on the ACLR for PC3, initially considering increasing the ACLR requirement by the exact same value as it is the difference between the maximum output powers of the two power classes, for example, for PC1 the ACLR of 38 dBc (as 8 dB is the difference between the MOPs of PC3 and PC1). In the end, the final value was slightly relaxed (37 dBc for PC1) as a compromise due to implementation challenges to achieve it.

The purpose of ACLR is to ensure the co-existence and we don’t see why would the 4 Tx implementation assumption necessitate relaxing the ACLR. If anything, if the MOP is assumed to be equal to 32 dBm, the ACLR requirement for PC1 should potentially be increased by 1 dB.

Proposal: There is no need for co-existence study to relax the ACLR requirement since its purpose is to ensure the co-existence between the different UEs and 4 Tx implementation assumption doesn’t justify the reason to relax it. Since the assumed maximum output power is equal to32 dBm, tightening of PC1 ACLR by 1 dB should be considered.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal: There is no need for co-existence study to relax the ACLR requirement since its purpose is to ensure the co-existence between the different UEs and 4 Tx implementation assumption doesn’t justify the reason to relax it. Since the assumed maximum output power is equal to32 dBm, tightening of PC1 ACLR by 1 dB should be considered.

# References

- RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, RAN#109.