3GPP TSG-RAN WG4 Meeting #117	R4-2520017

Dallas, USA, 17th – 21st November 2025

Source:	Nokia

Title:	Discussion on 6Rx Remaining Issues

Agenda item:	6.1.2

Document for:	Discussion

Introduction

There are a few remaining issues for 6Rx which need to be clairifed during RAN4#117, these have been articulated in the Way Forward from RAN4#116-bis [1].

UE demodulation performance requirements

Test Applicability Rules

The following is captured in the previous WF [1].

Issue 1-2: Test applicability rule for 6Rx 6 layers performance requirements - Agreement: - Agree to define the applicability rules that 6 MIMO layers performance requirements are applicable for UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6. - Proposals: - Discuss whether to explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4

Nokia would like to clarify that it is not required to specifically state that a 6 layer requirement is only applicable to FWA UE, and that in adding this information it is unnecessary, and perhaps may be perceived to imply that a handheld UE for some reason does not require to conduct these requirements even if it supports 6 layers.

If a capability rule is introduced to state “6 layers requirements are only applicable for FWA UE” then it could be perceived that the requirements do not apply to handheld UEs, which is not the intention.

RAN4 shall not explicitly capture that requirements are only applicable for FWA

SDR Cases

The following is captured in the previous WF [1].

Issue 3-1-2: MCS value for 256QAM/ 6 layers with scaling factor 1 - Proposals - Option 1: 27 - Option 2: 24 - Option 3: 25 Issue 3-1-3: MCS value for 64QAM/ 6 layers with scaling factor 1 - Proposals - Option 1: 28 - Option 2: 26 Issue 3-1-4: MCS value for 1024QAM/ 2 layers with scaling factor 1 - Proposals - Option 1: 23 - Option 2: 24

For these options, Nokia has no strong view, but for testability, it may be easier to choose the lower of all options

The MCS value for 256QAM/ 6 layers with scaling factor 1 shall be 24

The MCS value for 64QAM/ 6 layers with scaling factor 1 shall be 26

The MCS value for 1024QAM/ 6 layers with scaling factor 1 shall be 23

Conclusion

This paper highlights the discussion on the remaining issues for 6Rx

The following Observations and Proposals were made:

- Observation 1: If a capability rule is introduced to state “6 layers requirements are only applicable for FWA UE” then it could be perceived that the requirements do not apply to handheld UEs, which is not the intention.

- Proposal 1: RAN4 shall not explicitly capture that requirements are only applicable for FWA

- Proposal 2: The MCS value for 256QAM/ 6 layers with scaling factor 1 shall be 24

- Proposal 3: The MCS value for 64QAM/ 6 layers with scaling factor 1 shall be 26

- Proposal 4: The MCS value for 1024QAM/ 6 layers with scaling factor 1 shall be 23

- References

- R4-2514788 “WF for Rel-19 6Rx UE performance requirements”, Huawei, RAN4#116-bis, October 2025