3GPP TSG-RAN WG4 Meeting #117		   				 		  							R4-2520032

Dallas, USA, 17th – 21th Nov., 2025

Agenda Item:	6.6.3.1.2

Source:	MediaTek Inc.

Title:	Discussion on Applicability of dynamic channel model for demodulation and RRM

Document for:	Discussion

# Introduction

The applicability of dynamic channel model for demodulation requirement and RRM test cases are addressed in this paper.

- Applicability of dynamic channel model for demodulation requirements

The applicability of dynamic channel model for demodulation requirements was addressed in WF [1] as below.

Way forward: Applicability of Rel-19 tests with time-varying Doppler shift and propagation delay model Background In Rel-17 RAN4 defined four PDSCH demodulation requirements for NR NTN (1-1, 1-2, 1-3, and 1-4). In this WI, RAN4 has agreed to add 2 new test cases (1-5 and 1-6) by applying the time-varying Doppler shift and propagation delay model for NGSO to tests 1-1 and 1-2, as follows. Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 0.3. 1-2; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; 7.6. 1-3; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLC5-200; 1x2, ULA Low; 70; -0.4. 1-4; R.PDSCH.1-1.1 FDD(1); 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 1.1. 1-5; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; [0.3+0.5]. 1-6; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; [7.6+0.5]. Note 1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. Note 2: For Tests 1-5 and 1-6, the time-varying Doppler shift and propagation delay model, specified in Annex E, is applied... For UE declaring Rel-19 UE, - Option 1 - Applicable tests are 1-5, 1-6, 1-3 and 1-4. - Option 2 - Applicable tests are 1-5, 1-6. For UE declaring Rel-17/18 UE, - Option 1a: - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6, 1-3 and 1-4. - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4. - Option 1b: - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6. - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4. - Option 2: - Applicable tests are 1-1, 1-2, 1-3 and 1-4. Note the same test applicability rule is used for IoT-NTN UE demodulation requirements.

For UEs declaring as Rel-19, we prefer Option 1. Although tests 1-3 and 1-4 are optional features, no testing will be conducted for Rel-19 UEs supporting 32 HARQ processes and HARQ disabled. For UEs declaring as Rel-17/18, we prefer Option 2, which limits the channel model change to Rel-19. This approach helps mitigate potential adverse effects on pre-Rel-19 UEs, especially since testing activities for Rel-17/18 requirements are still ongoing. Additionally, applying this channel model to pre-Rel-19 releases would require further specification efforts to clearly define its applicability and scope for earlier releases.

Observation 1: If applicable tests are 1-5 and 1-6 for UE declaring Rel-19, there will be no tests for UE supporting 32 HARQ processes and HARQ disabled

Observation 2: Applying time varying channel model to pre-Rel-19 releases would require further specification efforts in TS38.101-5 and TS38.102 to clearly define its applicability and scope for earlier releases

Proposal 1: For applicability of dynamic channel model for NR-NTN demodulation requirements, we support

- For UE declaring Rel-19 UE: Applicable tests are 1-5, 1-6, 1-3 and 1-4. (Option 1)

- For UE declaring Rel-17/18 UE: Applicable tests are 1-1, 1-2, 1-3 and 1-4. (Option 2)

# Applicability of dynamic channel model for RRM test cases

#### Issue 2-2-1: Extend the applicability of the new channel model for the existing test cases

WayForward

- Further discuss the options.

  - Option 1: apply to only UL transmit timing test cases.

  - Option 2: apply to all NTN FR1 RRM tests.

  - Companies are encouraged to provide a more concrete analysis of the impact of RRM test cases.

#### Issue 2-3-1: Applicability of the new channel model for pre-Rel-19 NTN UEs

WayForward

- Further discuss the options

  - Option 1: not apply to pre-Rel-19 test cases

  - Option 2: apply to pre-Rel-19 test cases subject to DUT declaration

The applicability of dynamic channel model for RRM test cases were addressed in RRM WF R4-2515055 [2] as below.

Regarding Issue 2-2-1, according to WID RP-241281, the scope is to specify UE RRM performance test of uplink timing in the FR1-NTN bands, and other RRM tests are not included.

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

Observation 3: According to WID RP-241281, the scope is to specify UE RRM performance test of uplink timing in the FR1-NTN bands, and other RRM tests are not included.

In our view, a single test using the new dynamic channel is sufficient to verify UE pre-compensation of timing and doppler based on ephemeris.

Applying it to all RRM cases would complicate the test procedure. For example, for the test case with 2 satellites, it will require multiple ephemeris data sets and the corresponding time-varying timing shift and doppler shift. Additionally, the test procedure needs to be revised to continuously update the SMTC/MG configurations during the test, and the update frequency and appropriate timing offset need to be further specified. This will increase either RAN4 or RAN5 testing effort.

Observation 4: Applying dynamic channel model to all RRM cases would complicate the test procedure. For example, the test procedure needs to be revised to continuously update the SMTC/MG configurations during the test, and the update frequency and appropriate timing offset need to be further specified.

Proposal 2: Specify UE RRM performance test of uplink timing for NGSO only and do not extend it to other RRM tests. If needed, raise WID upscoping at RANP.

Regarding Issue 2-3-1, we would prefer to retain this change of channel model within R19 in order to mitigate any potential adverse effects on pre-R19 UEs, especially considering that testing activities for these earlier versions are still in progress. Furthermore, implementing this channel model in pre-R19 would require additional specification efforts to clearly define its applicability and scope for earlier releases.

Observation 5: Applying dynamic channel model to pre-Rel-19 test cases will require additional RRM specification efforts to clearly define its applicability and scope for earlier releases.

Proposal 3: Not apply to pre-Rel-19 test cases in RRM.

# Summary

Our observations and proposals in this paper are summarized as below:

Demod

Observation 1: If applicable tests are 1-5 and 1-6 for UE declaring Rel-19, there will be no tests for UE supporting 32 HARQ processes and HARQ disabled

Observation 2: Applying time varying channel model to pre-Rel-19 releases would require further specification efforts in TS38.101-5 and TS38.102 to clearly define its applicability and scope for earlier releases

Proposal 1: For applicability of dynamic channel model for NR-NTN demodulation requirements, we support

- For UE declaring Rel-19 UE: Applicable tests are 1-5, 1-6, 1-3 and 1-4. (Option 1)

- For UE declaring Rel-17/18 UE: Applicable tests are 1-1, 1-2, 1-3 and 1-4. (Option 2)

RRM

Observation 3: According to WID RP-241281, the scope is to specify UE RRM performance test of uplink timing in the FR1-NTN bands, and other RRM tests are not included.

Observation 4: Applying dynamic channel model to all RRM cases would complicate the test procedure. For example, the test procedure needs to be revised to continuously update the SMTC/MG configurations during the test, and the update frequency and appropriate timing offset need to be further specified.

Proposal 2: Specify UE RRM performance test of uplink timing for NGSO only and do not extend it to other RRM tests. If needed, raise WID upscoping at RANP.

Observation 5: Applying dynamic channel model to pre-Rel-19 test cases will require additional RRM specification efforts to clearly define its applicability and scope for earlier releases.

Proposal 3: Not apply to pre-Rel-19 test cases in RRM.

# References

[1] R4-2515053, Way Forward for [116bis][328] Rel-19 Demodulation_Part3, Moderator (Ericsson), RAN4#116bis, Oct. 2025.

[2] R4-2515055, Way Forward for [116bis][310] Rel-19 RRM performance_Part2, Moderator (MediaTek inc.), RAN4#116bis, Oct. 2025.