3GPP TSG-RAN WG4 Meeting #117		   				 		  								 	   R4-2520035

Dallas, USA, 17th – 21th Nov., 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test cases of Radio Link Monitoring for RedCap UE with Satellite Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Introduction of test cases of Radio Link Monitoring for RedCap UE with Satellite Access |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following RLM test cases - Radio Link Monitoring Out-of-syncTest for FR1 SAN PCell configured with SSB-based RLM RS in non-DRXmode for 1Rx/2Rx RedCap UE with NTN - Radio Link Monitoring In-syncTest for FR1 SAN PCell configured with SSB-based RLM RS in DRX modefor 1Rx/2Rx RedCap UE with NTN - Radio Link Monitoring Out-of-syncTest for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRXmode for 1Rx/2Rx RedCap UE with NTN - Radio Link Monitoring In-syncTest for FR1 SAN PCell configured with CSI-RS basedRLM in DRX modefor 1Rx/2Rx RedCap UE with NTN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test case of Radio Link Monitoring for RedCap UE with Satellite Access |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A.20.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | NA |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

### A.20.4.1	Radio link Monitoring

#### A.20.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode for 2Rx RedCap UE with NTN

##### A.20.4.1.1.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.1.1.1 shall apply for 2RX RedCap UE except that:

-	Table A.14.4.1.1.1-1 is replaced with A.20.4.1.1.1-1, and

- 	Table A.14.4.1.1.1-2, Table A.14.4.1.1.1-3 shall apply to configuration 1,2,3,4

Table A.20.4.1.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

##### A.20.4.1.1.2	Test Requirements

The test requirement in clause A.14.4.1.1.2 shall apply for RedCap.

#### A.20.4.1.2	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode for 1Rx RedCap UE with NTN

##### A.20.4.1.2.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.1.1.1 shall apply for 1RX RedCap UE except that:

-	Table A.14.4.1.1.1-1 is replaced with A.20.4.1.1.1-1, and

-	Table A.14.4.1.1.1-2 is replaced with A.20.4.1.2.1-2, and

-	Table A.14.4.1.1.1-3 shall apply to configuration 1,2,3,4.

Table A.20.4.1.2.1-2: General test parameters for FR1 out-of-sync testing in non-DRX mode for 1 Rx UE

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| BWchannel |  | Config 1, 2, 3, 4 | MHz | 10: NPRB,c = 52 |
| DL initial BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  | Config 1, 2, 3, 4 |  | CR.1.1 FDD |
| Dedicated CORESET Reference Channel |  | Config 1, 2, 3, 4 |  | CCR.3.2 FDD |
| SSB Configuration |  | Config 1, 2, 3, 4 |  | SSB.1 FR1 |
| SMTC Configuration |  | Config 1, 2, 3, 4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 3, 4 |  | 15 kHz |
| PRACH Configuration |  | Config 1, 2, 3, 4 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1, 2, 3, 4 |  | CSI-RS.1.1 FDD |
| CSI-RS for tracking |  | Config 1, 2, 3, 4 |  | TRS.1.1 FDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.88 |
| T3 |  |  | s | 0.88 |
| D1 |  |  | s | 0.84 |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

##### A.20.4.1.2.2	Test Requirements

The test requirement in clause A.14.4.1.1.2 shall apply for RedCap.

#### A.20.4.1.3	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode for 2Rx RedCap UE with NTN

##### A.20.4.1.3.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.1.4.1 shall apply for 2RX RedCap UE except that:

-	Table A.14.4.1.4.1-1 is replaced with A.20.4.1.1.1-1, and

- 	Table A.14.4.1.4.1-2, Table A.14.4.1.1.1-3 shall apply to configuration 1,2,3,4.

##### A.20.4.1.3.2	Test Requirements

The test requirement in clause A.14.4.1.4.2 shall apply for RedCap.

#### A.20.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode for 1Rx RedCap UE with NTN

##### A.20.4.1.4.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.1.4.1 shall apply for 2RX RedCap UE except that:

-	Table A.14.4.1.4.1-1 is replaced with A.20.4.1.1.1-1, and

- 	Table A.14.4.1.4.1-2 is replaced with A.20.4.1.4.1-2, and

-	Table A.14.4.1.1.1-3 shall apply to configuration 1,2,3,4.

Table A.20.4.1.4.1-2: General test parameters for FR1 in-sync testing in DRX mode

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active PCell |  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  |  | 1 |
| BWchannel |  |  | Config 1, 2, 3, 4 | MHz | 10: NPRB,c = 52 |
| DL initial BWP configuration |  |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  |  | Config 1, 2, 3, 4 |  | CR.1.1 FDD |
| Dedicated CORESET Reference Channel |  |  | Config 1, 2, 3, 4 |  | CCR.3.1 FDD |
| SSB Configuration |  |  | Config 1, 2, 3, 4 |  | SSB.1 FR1 |
| SMTC Configuration |  |  | Config 1, 2, 3, 4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2, 3, 4 |  | 15 kHz |
| PRACH Configuration |  |  | Config 1, 2, 3, 4 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  |  | 0 |
| OCNG parameters |  |  |  |  | OP.1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x1 Low |
| In sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX Configuration |  |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  |  | N.A. |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | 2000 |
| T311 timer |  |  |  | ms | 1000 |
| N310 |  |  |  |  | 1 |
| N311 |  |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1, 2, 3, 4 |  |  | CSI-RS.1.1 FDD |
| CSI-RS for tracking |  | Config 1, 2, 3, 4 |  |  | TRS.1.1 FDD |
| T1 |  |  |  | s | 0.2 |
| T2 |  |  |  | s | 0.2 |
| T3 |  |  |  | s | 0.44 |
| T4 |  |  |  | s | 0.2 |
| T5 |  |  |  | s | 0.88 |
| D1 |  |  |  | s | 0.84 |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

##### A.20.4.1.4.2	Test Requirements

The test requirement in clause A.14.4.1.4.2 shall apply for RedCap.

#### A.20.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode for 2Rx RedCap UE with NTN

##### A.20.4.1.5.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.1.5.1 shall apply for 2RX RedCap UE except that:

-	Table A.14.4.1.5.1-1 is replaced with A.20.4.1.1.1-1, and

- 	Table A.14.4.1.5.1-2, Table A.14.4.1.5.1-3 shall apply to configuration 1,2,3,4.

##### A.20.4.1.5.2	Test Requirements

The test requirement in clause A.14.4.1.5.2 shall apply for RedCap.

#### A.20.4.1.6	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode for 1Rx RedCap UE with NTN

##### A.20.4.1.6.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.1.5.1 shall apply for 1RX RedCap UE except that:

-	Table A.14.4.1.5.1-1 is replaced with A.20.4.1.1.1-1, and

- 	Table A.14.4.1.5.1-2 is replaced with A.20.4.1.6.1-2, and

- 	Table A.14.4.1.5.1-3 shall apply to configuration 1,2,3,4.

Table A.20.4.1.6.1-2: General test parameters for FR1 PCell for CSI-RS out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Satellite information | Config 1,3 |  | SSC.1 |
|  | Config 2,4 |  | SSC.2 |
| DL initial BWP configuration | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1, 2, 3, 4 |  | CR.1.1 FDD |
| Dedicated CORESET Reference Channel | Config 1, 2, 3, 4 |  | CCR.3.2 FDD |
| SSB Configuration | Config 1, 2, 3, 4 |  | SSB.1 FR1 |
| SMTC Configuration | Config 1, 2, 3, 4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2, 3, 4 |  | 15 kHz |
| TRS configuration | Config 1, 2, 3, 4 |  | TRS.1.1 FDD |
| CSI-RS for RLM | Config 1, 2, 3, 4 |  | Resource #4 in TRS.1.1 FDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State. 2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1, 2, 3, 4 |  | CSI-RS.1.1 FDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.88 |
| T3 |  | s | 0.88 |
| D1 |  | s | 0.84 |
| NOTE 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

##### A.20.4.1.6.2	Test Requirements

The test requirement in clause A.14.4.1.5.2 shall apply for RedCap.

#### A.20.4.1.7	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode for 2Rx RedCap UE with NTN

##### A.20.4.1.7.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.1.8.1 shall apply for 2RX RedCap UE except that:

-	Table A.14.4.1.8.1-1 is replaced with A.20.4.1.1.1-1, and

- 	Table A.14.4.1.5.1-2, Table A.14.4.1.5.1-3 shall apply to configuration 1,2,3,4.

##### A.20.4.1.7.2	Test Requirements

The test requirement in clause A.14.4.1.8.2 shall apply for RedCap.

#### A.20.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode for 1Rx RedCap UE with NTN

##### A.20.4.1.8.1	Test Purpose and Environment

The test purpose and environment in clause A.14.4.1.8.1 shall apply for 1RX RedCap UE except that:

-	Table A.14.4.1.8.1-1 is replaced with A.20.4.1.1.1-1, and

- 	Table A.14.4.1.8.1-2 is replaced with A.20.4.1.6.1-2, and

- 	Table A.14.4.1.8.1-3 shall apply to configuration 1,2,3,4.

Table A.20.4.1.8.1-2: General test parameters for FR1 PCell for CSI-RS in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Satellite information | Config 1, 3 |  | SSC.1 |
|  | Config 2, 4 |  | SSC.2 |
| DL initial BWP configuration | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1, 2, 3, 4 |  | CR.1.1 FDD |
| Dedicated CORESET Reference Channel | Config 1, 2, 3, 4 |  | CCR.3.1 FDD |
| SSB Configuration | Config 1, 2, 3, 4 |  | SSB.1 FR1 |
| SMTC Configuration | Config 1, 2, 3, 4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2, 3, 4 |  | 15 kHz |
| TRS configuration | Config 1, 2, 3, 4 |  | TRS.1.1 FDD |
| CSI-RS for RLM | Config 1, 2, 3, 4 |  | Resource #4 in TRS.1.1 FDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State. 2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 2000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1, 2, 3, 4 |  | CSI-RS.1.1 FDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 0.488 |
| T4 |  | s | 0.2 |
| T5 |  | s | 1.88 |
| T6 |  | s | 1.84 |
| NOTE 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

##### A.20.4.1.8.2	Test Requirements

The test requirement in clause A.14.4.1.8.2 shall apply for RedCap.

## <<< END OF CHANGES 1>>>