3GPP TSG-RAN WG4 #117	R4-2520044

Dallas, US, November 17-21, 2025

Agenda item:	8.8.2		RAN4-driven AI/ML use cases

Source:	Qualcomm Incorporated

Title:	Further comments on RAN4-driven AI/ML use cases of 6GR

Document for:	Discussion

# Introduction

The 6G study item was approved in RAN #108 [1]. It also includes a study on AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework. In addition, a study on the 6G scenarios and requirements is conducted in RAN plenary [2]. Its outcome is summarized in the technical report 38.914 [3].

In RAN4 #116bis the work on 6G AI/ML was started. Meeting minutes of the discussion and the way forward are provided in [4], [5] and [6].

In this contribution, we discuss our views of the goals of the RAN4-driven AI/ML use cases of the 6G study item.

# Views on the RAN4-driven AI/ML Use Cases for 6GR

## AI/ML Use Case for RF Issues

### Use Case#1: AI-nonlinearity compensation

#### Issue 4-1b: Sub use case definition and selection for AI-DPD/DPoD

Observation 1: In RAN4, non-AI based DPD at the UE is not standardized.

Proposal 1: We propose not to study AI based DPD at the UE in the context of Issue 4-1b.

The study item description (SID) agreed in RAN#109 for UE Tx EVM relaxation is provided below [9].

Power enhancement with relaxed UE Tx EVM and advanced BS receiver - Note: Discuss both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement as part of 6G study. RAN4 will further discuss and finalize (if agreed) the scope and other necessary details (e.g. NW control of this feature) in 2025.Q4 under 6G agenda items. It will be further decided in RAN#113 (Sep, 2026) whether or not there will be a follow up Rel-20 WI under 5G-Adv. For 5G-Adv, solution should not be AI based and there should not be any impact to RAN1.

Observation 2: Based on SID, power enhancement with relaxed UE Tx EVM and advanced BS receiver is defined for UL.

Proposal 2: Power enhancement on the DL i.e AI based DPD at the NW and AI based DPoD at the UE is out of scope of this study item.

#### Issue 4-1c: Evaluation for AI-DPoD

Given the broad reach of relaxed UE Tx EVM feature and based on discussions in RAN4#116bis, it was decided to split the study in 3 separate sessions listed below.

- 6G System Parameter : PA models

- 6G General RF and UE RF : Non-linearity model(s) of transmission signals

- 6G AI : AI-DPD, AI-DPoD, AI and Non-AI based non-linearity compensation

Observation 3: Non-linearity model(s) of transmission signals is discussed in 6G General RF and UE RF session.

Proposal 3: Evaluation of AI based DPoD should remain on hold until the model of UE TX non-linearity model(s) gets agreed in the 6G General RF and UE RF session.

#### Issue 4-1d: Evaluation for AI-DPD

Observation 4: In RAN4, non-AI based DPD at the UE is not standardized.

Proposal 4: We propose not to study AI based DPD at the UE in the context of Issue 4-1d.

## AI/ML Use case for Demod

### SRS Assisted Channel Reconstruction

RAN4 made the following agreement during the last meeting

Agreement: - The scope of potential RAN4-led AI use case of AI-SRS assisted channel reconstruction, will be further discussed in the next meeting.

The proponents of AI-SRS assisted channel reconstruction [7][8] mention two primary motivations regarding this topic. 1) Compensation of SRS insertion loss indued imbalance and 2) Tackling of SRS aging.

SRS insertion loss indued imbalance reduces UE’s power during SRS transmission. UE can compensate for insertion loss by boosting its transmit power as long as it has power headroom. UE cannot boost its SRS Tx power and compensate for insertion loss when it does not have power headroom. This can only happen at the cell edge where the role of SRS is somewhat diminished since the smaller link coverage of SRS may not be able to sustain cell edge UEs.

Observation 5: UE can compensate for insertion loss by boosting its transmit power as long as it has power headroom.

Observation 6: UE cannot boost its SRS Tx power and compensate for insertion loss when it does not have power headroom. This can only happen at the cell edge where the role of SRS is somewhat diminished due to the smaller link coverage.

Insertion loss induces imbalance is one relatively minor aspect of channel estimation. Other potential issues are: imbalance between RX and TX and its impact on channel estimation through the combination of CSI-RS and SRS. [7] proposes to use a two-sided model to investigate SRS assisted channel reconstruction. [8] proposes to train the network sided model to estimate SRS insertion loss by comparing reconstructed channel of SRS and CSI-RS. RAN1 is a suitable WG to lead this topic. For the example, RAN1 is discussing how network can share parameter regarding encoder input and encoder output to train the UE encoder in the two-sided CSF compression topic of 5G.  If 3GPP decides to study CSI-RS and SRS based two-sided channel reconstruction, 3GPP will have to potentially discuss how network can share parameters regarding encoder input and decoder input to train UE encoder. Here, decoder input contains both encoder output and channel obtained from SRS. The details of this parameter sharing can only be done in RAN1.

Observation 7: Insertion loss induced imbalance is one relatively minor aspect of the overall topic of channel reconstruction through the combination of CSI-RS and SRS.

Observation 8: If 3GPP decides to study CSI-RS and SRS based two-sided channel reconstruction, 3GPP will have to discuss how network can share parameters regarding encoder input and decoder input to train UE encoder. Here, decoder input contains both encoder output and channel obtained from SRS. The details of this parameter sharing can only be done in RAN1.

Proposal 5: SRS assisted channel reconstruction, if needed, should be a RAN1 led AI-ML topic.

### AI-ML Based PRACH Receiver

RAN4 made the following agreement during the last meeting

Agreement: - The scope of potential RAN4-led AI use case of AI-enabled PRACH will be further discussed in the next meeting.

[7] proposed AI enabled PRACH. The motivation is to allow network to detect multiple UEs per PRACH preamble and ensure early contention resolution. The proponents of [7] describe following RACH related procedure to achieve the motivation. First, network detects multiple UEs per PRACH preamble in the same PRACH resource. Second, network sends Msg2 where one random access response includes multiple grants where the number of grants corresponds to the base station’s estimation of how many UEs transmitted the same PRACH preamble in the same PRACH resource. Third, the contented UE chooses the corresponding TA and UL grant for Msg3 transmission to solve the preamble collision in early stage.

Several questions arise regarding the above-mentioned procedure. They are mentioned below.

- It is not clear how a UE would know its corresponding TA and UL grant if multiple UL grants are tied with the same random access preamble index (RAPID) within one random access response (RAR).

- If the network can detect multiple UEs within a PRACH preamble, it may mean that the spacing between two consecutive PRACH preambles, in terms of cycle shifts, is large enough to contain more UEs. If that is the case, network can configure smaller zeroCorrelationZoneConfig (i.e., smaller cyclic shift) and include more PRACH preambles within the same time-frequency resource. 5G limited the total number of preambles per PRACH time-frequency resource to 64. 6G can simply allow network to configure a higher number of PRACH preambles (>64) per same time-frequency resource. This would allow network to support higher number of UEs’ RACH transmission per time-frequency resource but tie one random access preamble ID in random access response with only one UL grant.

Observation 9: In AI based PRACH, it is not clear how a UE would know its corresponding TA and UL grant if multiple UL grants are tied with the same random access preamble index (RAPID) within one random access response.

Observation 10: If network can detect multiple UEs within a PRACH preamble, it may mean that the spacing between two consecutive PRACH preambles, in terms of cycle shifts, is large enough to contain more UEs. If that is the case, network can configure smaller cyclic shift and include more PRACH preambles within the same time-frequency resource.

The evaluation of multi-user detection within same PRACH preamble sequence and time-frequency resource completely depends on PRACH sequence. RAN4 cannot start studying this topic until RAN1 has made more progress regarding PRACH preamble sequence design. This topic, if required, is more suitable to be RAN1 led.

Observation 11: The evaluation of multi-user detection within same PRACH preamble sequence and time-frequency resource completely depends on PRACH sequence of 6G, which is expected to be studied in RAN1.

Proposal 6: Multi-user detection within the same PRACH preamble via AI-based PRACH receiver, if required, is more suitable to be a RAN1 led topic. The proponents should clarify how UE could resolve its ambiguities while selecting Msg3 UL grant and why network cannot instead use smaller cyclic shift to increase the number of users per PRACH time-frequence resources before 3GPP starts studying this topic.

### Channel Model for Defining AI-ML based Demod Requirements

During the last meeting, it was proposed to consider CDL model to define AI-ML based demod requirements. In our understanding, existing CDL models as defined currently in Rel-19/Rel-20 in RAN4 are deterministic after initialization. Hence, existing CDL models are not suitable models in case of AI/ML based receivers since such a receiver can learn the determinism of the channel easily and results are too optimistic. In our view the applicability of CDL models for AI/ML receivers and potential enhancements of CDL, if possible at all, should be discussed in the demod session first.

Observation 12: Existing CDL models are not suitable in case of AI/ML based receivers since such a receiver can learn the determinism of the channel easily and generate too optimistic results.

Proposal 7: The applicability of CDL models for AI/ML receivers and potential enhancements of CDL, if possible, at all, should be discussed in the demod session first.

### Model Driven Approach for AI Receiver

In the last meeting it was proposed to study model driven methods for the AI receiver. The idea behind the proposal is to use a deep learning network to replace certain modules like e.g., DMRS based channel estimation. In our opinion it is not possible to study model driven methods for any module as long as the signal structure like waveform, reference signals, modulation and channel codes has not been defined precisely by RAN1. Hence, we propose to postpone any discussion on model driven approaches for the AI receiver till RAN1 has defined the physical layer for 6GR.

Observation 13: It is not possible to study model driven methods for any module as long as the signal structure like waveform, reference signals, modulation and channel codes has not been defined precisely by RAN1

Proposal 8: We propose to postpone any discussion on model driven approaches for the AI receiver till RAN1 has defined the physical layer for 6GR.

## AI/ML Use Case for RRM

Following different AI-ML RRM use cases came up during the last meeting.

| Index | use cases | Model location (Training method) |
| --- | --- | --- |
|  | L1 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Spatial domain (inter-cell) - Time and spatial domain (inter-cell) | NW and UE sided model (Offline training) |
|  | L3 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Time and freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) - Freq. domain prediction for N carriers single cell | NW and UE sided model (Offline training) |
|  | L3 cell measurement: - Time domain (intra-cell) - Freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) | NW and UE sided model (Offline training) |
|  | Multi-Domain L3 level Prediction for Measurement-Gap Reduction | UE sided model (Offline training) |
| 5a (5+6 combined) | Dynamic Adaptation of Measurement Procedure | UE sided model |
| 7a (7+8 combined) | Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model |
|  | AI/ML based RLM/BFD/CBD, LTM by measurement prediction | UE-sided model |
|  | AI/ML based SCell related enhancement by measurement prediction | UE-sided model |
|  | AI-based prediction for enhanced s-measure mechanism | UE-sided model |
|  | AI/ML based AGC through temporal/frequency domain prediction | UE-sided model |
|  | AI/ML based DL sync through temporal/frequency domain prediction | UE-sided model |
|  | AI abstraction of the wireless environment to facilitate data scheduling and network power saving | NW-sided model |

We provide our views regarding some of the AI-ML RRM use cases in this section.

### Spatial Domain RX Beam Sweeping Reduction (L1/L3)

Figure 3-1: Rx Beam Prediction via AI/ML

RAN4 has assumed the RX beam sweeping factor to be eight in many FR2 measurement periods of Rel-15. RAN4 assumed that UE needs to sweep eight RX beams in FR2 to cover angle of arrivals in relevant azimuth and elevation domain and detect or measure reference signals. This scaling factor of eight significantly increases many evaluation periods in FR2. This long evaluation period slows down the mobility procedures significantly.

Observation 14: RAN4 has assumed the RX beam sweep factor to be eight in many FR2 measurement periods of Rel-15. This scaling factor of eight significantly increases evaluation periods in FR2.

The work item of 5G AI/ML beam management allowed UE to measure the properties of one set (setB) of reference signals and predict the properties of a different set (setA) of reference signals. 5G AI/ML assumed UE to sweep eight RX beams, too, to measure the reference signals of setB. The measurement period of 5G spatio-temporal beam prediction scenario gets additionally extended by the scaling factor of the minimum number of observation samples. However, the presence of two scaling factors (FR2 RX beam sweeping factor and the minimum number of observation samples) will make the evaluation period of 5G spatio-temporal beam prediction excessively long.

Observation 15: 5G AI/ML beam management work item assumed FR2 UE to sweep eight RX beams to measure setB beams.

Observation 16: The measurement period of 5G spatio-temporal beam prediction gets scaled by both legacy RX beam sweeping factor (eight) and the minimum number of observation samples. The presence of these two factors may make the measurement period of 5G AI/ML based spatio-temporal beam prediction excessively long.

RAN4 has introduced new UE capabilities in Rel-18 and Rel-19 that allow UE to signal support of reduced beam sweeping factors for serving and neighbor cells. These UE capabilities do not assume any use of AI/ML. RAN4 should study if and how 6G UE can support reduced RX beam sweeping factor in FR2.

Observation 17: RAN4 has introduced new UE capabilities in Rel-18 and Rel-19 that allow UE to signal support of reduced FR2 RX beam sweeping factors for serving and neighbor cells respectively.

An AI/ML based model, developed based on prior training, can help a UE to measure a reference signal with a reduced set of RX beams and infer the properties of full set of RX beams for that reference signal. Figure 3-1 shows that UE can measure a set of gNB beams with a subset of RX beams and predict the properties of the same set of gNB beams with the full set of RX beams. RAN4 should discuss whether 6G UE’s support of reduced beam sweeping factor should depend on the availability of AI/ML model and prior training.

Table 3-1 gives an overview of these cases. This topic can use the framework of the simulation assumptions of Rel-19 AI-ML beam management. However, it is not dependent on RAN1/RAN2 led AI-ML use cases, including Rel-19 AI-ML beam management, since the FR2 RX beam sweeping factor of RAN4 RRM spec is transparent to RAN1 and RAN2 specs.

Table 3-2 displays one set of examples to evaluate spatial domain to evaluate “spatial domain RX beam sweeping reduction”. Other examples should also be considered to evaluate this topic, especially regarding spatial domain RX beam sweeping reduction for L3 measurement.

Observation 18: AI/ML based training and inference can help UE to measure a reference signal with reduced set of RX beams and infer the properties of full set of RX beams for that reference signal. This would significantly reduce the evaluation period in FR2.

Proposal 9: RAN4 should study if and how AI/ML based training and inference can help a 6G FR2 UE to support reduced RX beam sweeping factor.

Observation 19: Table 3-1 shows an overview of the “spatial domain RX beam sweeping reduction” use case.

Table 3-1: Overview of “Spatial Domain RX Beam Sweeping Reduction”

| use cases | Model location (Training method) | Expected benefits | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- |
| Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model | Measurement delay reduction | Minor dependence on other WG-led use cases. FR2 RX beam sweep factor is transparent to RAN1/RAN2 spec. | Benefits and feasibility of the use of AI-ML to reduce spatial domain RX beam sweeping factor |

Observation 20: Table 3-2 shows one set of examples to evaluate “spatial domain RX beam sweeping reduction”. Other examples should also be considered to evaluate this topic, especially regarding spatial domain RX beam sweeping reduction for L3 measurement.

Table 3-2: Different aspects of evaluation for “Spatial Domain RX Beam Sweeping Reduction”

| Aspects |  | Example |
| --- | --- | --- |
| AI model input | Input in training | L1-RSRP of a subset (<8) of RX beams |
|  | Input in inference | L1-RSRP of a subset (<8) of RX beams |
| AI model output | Label in training (if applicable) | L1-RSRP of “full set” (=8) of RX beams |
|  | Output in inference | L1-RSRP of “full set” (=8) of RX beams |
| Assumption on training | Training type | Offline training |
|  | Label construction | Measurement data |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | UE informs network the required number of RX beams it needs to measure to predict the properties of full set of RX beams. |
| Evaluation methodology |  | system level simulation |
| Non-AI-ML benchmark |  | Radial basis function interpolation |
| Evaluation KPI |  | Accuracy of predicted RSRP |
| Evaluation assumption |  | As an example, the framework of Rel-18 and Rel-19’s evaluation assumptions for AI-ML beam management (mentioned in table 6.3.1-1 of 38.843) can be used. This framework needs to be slightly modified to focus on RX beam prediction, instead of TX beam prediction. Other examples should also be considered, especially to evaluate RX beam sweeping factor reduction in L3. |
| High level potential specification impact |  | UE capability regarding the required number of RX beams. |
| Feasibility issues, including complexity, and other aspects related to implementation |  | No big concern compared to other AI-ML use cases |

### Dynamic Adaptation of Measurement Procedure

The existing RAN4 requirements mandate that the UE perform RRM measurements strictly according to the network configuration, even when some cells are not detectable or measurable. In addition, these requirements assume that the UE measures cells in the configured measurement objects with equal priority in a TDD manner. Although different weighting factors are considered in CSSF and in FR2 measurements for the frequency and spatial domains, respectively, these weighting factors are hard-coded in the specification, as shown in the following snippets.

Considering the growing demand for AI-native UE implementations, it is anticipated that mobile UEs will adopt AI-based measurement adaptation solutions, which may or may not be transparent to the network. While the UE may be able to prudently adjust its measurement behavior to enhance user experience without compromising performance, RRM requirements may limit the applicability of such intelligent implementations since these requirements are typically defined in a scenario- and context-agnostic manner. As AI-based solutions are expected to mature by the 6G timeline, RAN4 should investigate and explore how to ensure such implementations are not unfairly restricted by the requirement specifications.

Observation 21: Although AI-native UE implementations have gained significant traction and are expected to mature by the 6G timeframe, scenario-agnostic RRM requirements could unintentionally constrain such intelligent behaviors.

It would be prudent for 6G to allow Rel-21 UE to dynamically adapt the measurement period of both serving and neighbor cell based on channel condition, along with network configuration. AI/ML based model, developed via prior training, and subsequent inference may allow UE to judiciously adjust measurement procedure.

Figure 3-2: Influence of gNB beam shape on beam failure experienced at UE

In scenario 1 of Figure 1, UE would experience beam failure quite quickly in its trajectory since gNB is using a narrow beam shape. On the other hand, in scenario 2 of Figure 2, UE would experience beam failure a bit late in its trajectory since gNB is using a wider beam shape. If this UE were pre-trained with this gNB’s beam configuration, it could have developed an AI/ML model to predict quality of serving cell soon. That AI/ML model, along with recent measurements, would have allowed UE to judiciously and dynamically adapt the measurement period of serving cell.

Observation 22: AI/ML based training and inference may allow 6G UE to judiciously adjust measurement periods.

Proposal 10: RAN4 should include ‘Dynamic Adaptation of Measurement Procedure’ as one of the use cases for the 6G AI study and investigate whether and how to accommodate AI-based UE dynamic measurement adaptation within the 6G RRM specification, as well as identify any required signaling support from other working groups.

Observation 23: Table 3-3 outlines an overview of the ‘Dynamic Adaptation of Measurement Procedure’ use case.

Table 3-3: Overview of ‘Dynamic Adaptation of Measurement Procedure’

| use cases | Model location (Training method) | Proposed by companies | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- |
| Dynamic Adaptation of Measurement Procedure | UE sided model | MediaTek, Qualcomm, Apple | UE performs measurements on more relevant cells potentially more aggressively than required by RRM requirements, which could lead to improved mobility performance. | Prediction performance will be evaluated within Rel-20 AI/ML-based mobility WI in 5GA. Potentially, RAN2 signalling support. | How to define UE measurement requirements in a way that AI/ML based UE measurement adaptation can be spec-compliant. |

Observation 24: Table 3-4 shows one set of examples to evaluate ‘Dynamic Adaptation of Measurement Procedure’.

Table 3-4: Different aspects of evaluation for ‘Dynamic Adaptation of Measurement Procedure’

| Sub-use case 5a |  | Example |
| --- | --- | --- |
| AI model input | Input in training | Beam and/or cell level RSRP measurements from serving cell and/or neighbor cells |
|  | Input in inference | Beam and/or cell level RSRP measurements from serving cell and/or neighbor cells |
| AI model output | Label in training (if applicable) | Beam and/or cell level RSRP measurements from serving cell and neighbor cells which may or may not be co-located with the measurement cells |
|  | Output in inference | Beam and/or cell level RSRP measurements from serving cell and neighbor cells which may or may not be co-located with the measurement cells |
| Assumption on training | Training type | offline training, potentially site-specific online finetuning |
|  | Label construction (if applicable) | Collected measurement data |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | With or without collaboration/interaction, depending on whether and to what extent the UE measurement adaptation decision needs to be communicated to the NW. |
| Evaluation methodology |  | System level simulation (a new simulation might not be necessary, depending on how much out of Rel-20 AI/ML-based mobility can be leveraged) |
| Evaluation assumption |  | May or may not be needed, depending on how much out of Rel-20 AI/ML-based mobility can be leveraged |
| Evaluation KPI |  | Measurement period optimization for more relevant cells (in terms of RSRP), compared to the RAN4 minimum requirements, which mandate TDM-based measurements across all cells and frequencies in the configured measurement object. |
| Evaluation benchmark |  | RAN4 minimum requirements, which mandate TDM-based measurements across all cells and frequencies in the configured measurement object. |
| Preliminary evaluation results |  | TR 38.744 “Technical Report on Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR” |
| High level potential specification impact |  | Overall measurement period requirements in RAN4 specification. Potentially, RAN2 signalling support (depending on whether and to what extent the UE measurement adaptation decision needs to be communicated to the NW) |
| Feasibility issues, including complexity, and other aspects related to implementation |  | No big concern compared to other AI/ML use cases. |

### Comments regarding L1/L3 Beam Measurement and L3 Cell Measurement

At RAN4#116bis, we observed strong alignment among companies on the following use cases, which significantly overlap with the ongoing 5G Rel-20 work item ‘AI/ML-based mobility.’ These use cases aim to extend and merge Rel-19 AI/ML-based beam management with Rel-20 AI/ML-based mobility. It is evident that these items will require intensive simulation efforts. Given that the ongoing work item’s observations and conclusions could impact the effectiveness and practicality of these proposals, we believe RAN4 should wait for the Rel-20 AI/ML-based mobility WI outcome rather than repeating similar discussions in multiple places. This would not be an efficient use of the limited TU for the 6G study.

- 1) L1 beam measurement:

  - Time domain (intra-cell)

  - Time and spatial domain (intra-cell)

  - Freq. and spatial domain (inter-cell)

  - Spatial domain (inter-cell)

  - Time and spatial domain (inter-cell)

- 2) L3 beam measurement:

  - Time domain (intra-cell)

  - Time and spatial domain (intra-cell)

  - Freq. and spatial domain (inter-cell)

  - Time and freq. domain (inter-cell)

  - Freq.  domain prediction for non-collocated cells (inter-cell)

  - Freq.  domain prediction for N carriers single cell

- 3) L3 cell measurement:

  - Time domain (intra-cell)

  - Freq. domain (inter-cell)

  - Freq.  domain prediction for non-collocated cells (inter-cell)

Proposal 11: RAN4 should avoid overlapping study efforts on use case indices 1–3 with the ongoing Rel-20 AI/ML-based mobility WI and wait for the outcome of the Rel-20 WI before initiating studies on use case indices 1–3.

# Conclusions

In this paper we provided our views for the RAN4-driven AI/ML use cases in the 6GR study item. Our observations and proposals are summarized as follows:

AI/ML Use Case for RF Issues

Observation 1: In RAN4, non-AI based DPD at the UE is not standardized.

Proposal 1: We propose not to study AI based DPD at the UE in the context of Issue 4-1b.

Observation 2: Based on SID, power enhancement with relaxed UE Tx EVM and advanced BS receiver is defined for UL.

Proposal 2: Power enhancement on the DL i.e AI based DPD at the NW and AI based DPoD at the UE is out of scope of this study item.

Observation 3: Non-linearity model(s) of transmission signals is discussed in 6G General RF and UE RF session.

Proposal 3: Evaluation of AI based DPoD should remain on hold until the model of UE TX non-linearity model(s) gets agreed in the 6G General RF and UE RF session.

Observation 4: In RAN4, non-AI based DPD at the UE is not standardized.

Proposal 4: We propose not to study AI based DPD at the UE in the context of Issue 4-1d.

AI/ML Use case for Demod

Observation 5: UE can compensate for insertion loss by boosting its transmit power as long as it has power headroom.

Observation 6: UE cannot boost its SRS Tx power and compensate for insertion loss when it does not have power headroom. This can only happen at the cell edge where the role of SRS is somewhat diminished due to the smaller link coverage.

Observation 7: Insertion loss induced imbalance is one relatively minor aspect of the overall topic of channel reconstruction through the combination of CSI-RS and SRS.

Observation 8: If 3GPP decides to study CSI-RS and SRS based two-sided channel reconstruction, 3GPP will have to discuss how network can share parameters regarding encoder input and decoder input to train UE encoder. Here, decoder input contains both encoder output and channel obtained from SRS. The details of this parameter sharing can only be done in RAN1.

Proposal 5: SRS assisted channel reconstruction, if needed, should be a RAN1 led AI-ML topic.

Observation 9: In AI based PRACH, it is not clear how a UE would know its corresponding TA and UL grant if multiple UL grants are tied with the same random access preamble index (RAPID) within one random access response.

Observation 10: If network can detect multiple UEs within a PRACH preamble, it may mean that the spacing between two consecutive PRACH preambles, in terms of cycle shifts, is large enough to contain more UEs. If that is the case, network can configure smaller cyclic shift and include more PRACH preambles within the same time-frequency resource.

Observation 11: The evaluation of multi-user detection within same PRACH preamble sequence and time-frequency resource completely depends on PRACH sequence of 6G, which is expected to be studied in RAN1.

Proposal 6: Multi-user detection within the same PRACH preamble via AI-based PRACH receiver, if required, is more suitable to be a RAN1 led topic. The proponents should clarify how UE could resolve its ambiguities while selecting Msg3 UL grant and why network cannot instead use smaller cyclic shift to increase the number of users per PRACH time-frequence resources before 3GPP starts studying this topic.

Observation 12: Existing CDL models are not suitable in case of AI/ML based receivers since such a receiver can learn the determinism of the channel easily and generate too optimistic results.

Proposal 7: The applicability of CDL models for AI/ML receivers and potential enhancements of CDL, if possible, at all, should be discussed in the demod session first.

Observation 13: It is not possible to study model driven methods for any module as long as the signal structure like waveform, reference signals, modulation and channel codes has not been defined precisely by RAN1

Proposal 8: We propose to postpone any discussion on model driven approaches for the AI receiver till RAN1 has defined the physical layer for 6GR.

AI/ML Use Case for RRM

Observation 14: RAN4 has assumed the RX beam sweep factor to be eight in many FR2 measurement periods of Rel-15. This scaling factor of eight significantly increases evaluation periods in FR2.

Observation 15: 5G AI/ML beam management work item assumed FR2 UE to sweep eight RX beams to measure setB beams.

Observation 16: The measurement period of 5G spatio-temporal beam prediction gets scaled by both legacy RX beam sweeping factor (eight) and the minimum number of observation samples. The presence of these two factors may make the measurement period of 5G AI/ML based spatio-temporal beam prediction excessively long.

Observation 17: RAN4 has introduced new UE capabilities in Rel-18 and Rel-19 that allow UE to signal support of reduced FR2 RX beam sweeping factors for serving and neighbor cells respectively.

Observation 18: AI/ML based training and inference can help UE to measure a reference signal with reduced set of RX beams and infer the properties of full set of RX beams for that reference signal. This would significantly reduce the evaluation period in FR2.

Proposal 9: RAN4 should study if and how AI/ML based training and inference can help a 6G FR2 UE to support reduced RX beam sweeping factor.

Observation 19: Table 3-1 shows an overview of the “spatial domain RX beam sweeping reduction” use case.

Table 3-1: Overview of “Spatial Domain RX Beam Sweeping Reduction”

| use cases | Model location (Training method) | Expected benefits | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- |
| Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model | Measurement delay reduction | Minor dependence on other WG-led use cases. FR2 RX beam sweep factor is transparent to RAN1/RAN2 spec. | Benefits and feasibility of the use of AI-ML to reduce spatial domain RX beam sweeping factor |

Observation 20: Table 3-2 shows one set of examples to evaluate “spatial domain RX beam sweeping reduction”. Other examples should also be considered to evaluate this topic, especially regarding spatial domain RX beam sweeping reduction for L3 measurement.

Table 3-2: Different aspects of evaluation for “Spatial Domain RX Beam Sweeping Reduction”

| Aspects |  | Example |
| --- | --- | --- |
| AI model input | Input in training | L1-RSRP of a subset (<8) of RX beams |
|  | Input in inference | L1-RSRP of a subset (<8) of RX beams |
| AI model output | Label in training (if applicable) | L1-RSRP of “full set” (=8) of RX beams |
|  | Output in inference | L1-RSRP of “full set” (=8) of RX beams |
| Assumption on training | Training type | Offline training |
|  | Label construction | Measurement data |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | UE informs network the required number of RX beams it needs to measure to predict the properties of full set of RX beams. |
| Evaluation methodology |  | system level simulation |
| Non-AI-ML benchmark |  | Radial basis function interpolation |
| Evaluation KPI |  | Accuracy of predicted RSRP |
| Evaluation assumption |  | As an example, the framework of Rel-18 and Rel-19’s evaluation assumptions for AI-ML beam management (mentioned in table 6.3.1-1 of 38.843) can be used. This framework needs to be slightly modified to focus on RX beam prediction, instead of TX beam prediction. Other examples should also be considered, especially to evaluate RX beam sweeping factor reduction in L3. |
| High level potential specification impact |  | UE capability regarding the required number of RX beams. |
| Feasibility issues, including complexity, and other aspects related to implementation |  | No big concern compared to other AI-ML use cases |

Observation 21: Although AI-native UE implementations have gained significant traction and are expected to mature by the 6G timeframe, scenario-agnostic RRM requirements could unintentionally constrain such intelligent behaviors.

Observation 22: AI/ML based training and inference may allow 6G UE to judiciously adjust measurement periods.

Proposal 10: RAN4 should include ‘Dynamic Adaptation of Measurement Procedure’ as one of the use cases for the 6G AI study and investigate whether and how to accommodate AI-based UE dynamic measurement adaptation within the 6G RRM specification, as well as identify any required signaling support from other working groups.

Observation 23: Table 3-3 outlines an overview of the ‘Dynamic Adaptation of Measurement Procedure’ use case.

Table 3-3: Overview of ‘Dynamic Adaptation of Measurement Procedure’

| use cases | Model location (Training method) | Proposed by companies | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- |
| Dynamic Adaptation of Measurement Procedure | UE sided model | MediaTek, Qualcomm, Apple | UE performs measurements on more relevant cells potentially more aggressively than required by RRM requirements, which could lead to improved mobility performance. | Prediction performance will be evaluated within Rel-20 AI/ML-based mobility WI in 5GA. Potentially, RAN2 signalling support. | How to define UE measurement requirements in a way that AI/ML based UE measurement adaptation can be spec-compliant. |

Observation 24: Table 3-4 shows one set of examples to evaluate ‘Dynamic Adaptation of Measurement Procedure’.

Table 3-4: Different aspects of evaluation for ‘Dynamic Adaptation of Measurement Procedure’

| Sub-use case 5a |  | Example |
| --- | --- | --- |
| AI model input | Input in training | Beam and/or cell level RSRP measurements from serving cell and/or neighbor cells |
|  | Input in inference | Beam and/or cell level RSRP measurements from serving cell and/or neighbor cells |
| AI model output | Label in training (if applicable) | Beam and/or cell level RSRP measurements from serving cell and neighbor cells which may or may not be co-located with the measurement cells |
|  | Output in inference | Beam and/or cell level RSRP measurements from serving cell and neighbor cells which may or may not be co-located with the measurement cells |
| Assumption on training | Training type | offline training, potentially site-specific online finetuning |
|  | Label construction (if applicable) | Collected measurement data |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | With or without collaboration/interaction, depending on whether and to what extent the UE measurement adaptation decision needs to be communicated to the NW. |
| Evaluation methodology |  | System level simulation (a new simulation might not be necessary, depending on how much out of Rel-20 AI/ML-based mobility can be leveraged) |
| Evaluation assumption |  | May or may not be needed, depending on how much out of Rel-20 AI/ML-based mobility can be leveraged |
| Evaluation KPI |  | Measurement period optimization for more relevant cells (in terms of RSRP), compared to the RAN4 minimum requirements, which mandate TDM-based measurements across all cells and frequencies in the configured measurement object. |
| Evaluation benchmark |  | RAN4 minimum requirements, which mandate TDM-based measurements across all cells and frequencies in the configured measurement object. |
| Preliminary evaluation results |  | TR 38.744 “Technical Report on Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR” |
| High level potential specification impact |  | Overall measurement period requirements in RAN4 specification. Potentially, RAN2 signalling support (depending on whether and to what extent the UE measurement adaptation decision needs to be communicated to the NW) |
| Feasibility issues, including complexity, and other aspects related to implementation |  | No big concern compared to other AI/ML use cases. |

Proposal 11: RAN4 should avoid overlapping study efforts on use case indices 1–3 with the ongoing Rel-20 AI/ML-based mobility WI and wait for the outcome of the Rel-20 WI before initiating studies on use case indices 1–3.

# References

- RP-251881, New SID: Study on 6G Radio, RAN #108, June 2025

- RP-251395, Revised SID: Study on 6G Scenarios and requirements, RAN #108, June 2025

- TR 38.914, Study on 6G Scenarios and Requirements, v0.1.0, June 2025

- R4-2514636, Topic Summary for [116bis][107] 6G AI, RAN4 #116bis, October 2025

- R4-2514539, Topic Summary for [116bis][107] 6G AI, RAN4 #116bis, October 2025

- R4-2514587, WF on 6G AI, RAN4 #116bis, October 2025

- R4-2513049, Discussion on AI Framework and Use Cases for 6GR, 3GPP TSG RAN WG4 #116bis.

- R4-2513310, Consideration on RAN4 AI Study for 6G, 3GPP TSG RAN WG4 #116bis

- RP-252942, New WID: UE RF enhancements for NR FR1 Phase 5, 3GPP TSG-RAN Meeting #109.