3GPP TSG-RAN WG4 Meeting #117                        R4-2520054                                                                                                       Dallas, Texas, USA, 17 – 21 November 2025

Source:	China Telecom

Title:	Discussion on demodulation aspects for 6G study

Agenda Item:	8.7

Document for:	Discussion

Introduction

In RAN #108 meeting, the new SI on 6G Radio was approved [1], with the following objectives for the demodulation aspect:

Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

The RAN4#116bis is the first meeting to discuss the demodulation aspects for 6G and a WF is approved in [2].

In this contribution, our views on the open issues are provided.

Discussion

From our point of view, the demodulation part of the 6G study should mainly focus on receiver and channel modeling.

General

RAN4 demod study timeline

- Option 1: For RAN4 6G Demodulation, RAN4 establish a more realistic and structured timeline to ensure adequate depth in technical deliberation and a well-paced progression toward completion. - Option 2: RAN4 closely follow RAN1's discussion progress on other parameters related to 6G physical layer structure. Once relevant conclusions are available, RAN4 could consider adopting them for RAN4 6G Demodulation study. - Option 3: Postpone RAN4 discussion on demodulation requirements related to physical layer channel and procedure design until sufficient progress reached in RAN1 i.e., no early than Q2’26. - Option 4: 5GA Topics that were studied and proven useful should be introduced from day 1 in 6G, and RAN4 needs start discussions on corresponding test specifications to enable early adoption.

We agree it is important to have a work plan and timeline for 6G demodulation study. As per the WF, there are totally 9 sub-topics 30+ and only 3-4 issues were discussed in the last meeting. Therefore, we believe it is also important to have a priority list for the sub-topics.

From our point of view, we propose to first prioritize the issues require no or minimal inputs from other WGs (e.g., channel model, EVM, interference modelling, receiver structure and implementation). For the other issues, we recommend starting after there are sufficient progress from RAN1.

Proposal 1: It is important to have a work plan, timeline and a priority list for the sub-topics for 6G demodulation study

Proposal 2: Prioritize the issues that require no or minimal inputs from other WGs (e.g., channel model, EVM, interference modelling, receiver structure and implementation). For the other issues, we recommend starting after there are sufficient progress from RAN1.

Channel model

In 5G, currently, all UE and BS demodulation tests are using TDL channel modeling with some simplification compared with the original TDL captured in TR38.901, which is somehow sufficient for basic R15 5G performance testing. However, with the evolution of 5G, more features are introduced and many of them require to be tested under spatial channel model (SCM), e.g., rank 8 transmission with 2 CWs, MU-MIMO with e-Type II PMI feedback, PRACH repetition with different beams, etc.

In Rel-19, RAN4 has performed studies for SCM for DL SU-MIMO scenario, PDSCH 4T4R rank4 and 8T8R rank8 cases and PMI type I and eType II cases are selected. As a result, simplified CDL channel model (namely rCDL-C1 in TR38.753) is derived and will be used for some example test requirement definition in Rel-20.

In addition, in Rel-20, RAN4 will do further studies for MU-MIMO based on simplified CDL channel model under SI FS_NR_demod_SCM_Ph2, and both DL and UL will be covered. According to the SID [3], this study will be applicable for both 5G-A and 6G.

To avoid duplicate studies, for this 6G SI thread, we propose to study the CDL performance for more SU-MIMO cases such as inter-cell interference, advanced receiver, CSI reporting for DL and PUSCH for the UL.

Observation 1: In Rel-19, RAN4 has performed studies for SCM for SU-MIMO scenario, PDSCH 4T4R rank4 and 8T8R rank8 cases and PMI type I and eType II cases are selected.

Observation 2: In Rel-20, RAN4 will do further studies based on simplified CDL channel model for MU-MIMO and both DL and UL will be covered under SI FS_NR_demod_SCM_Ph2, will be applicable for both 5G-A and 6G.

Proposal 3: For this 6G SI thread, RAN4 to study the simplified CDL performance for more SU-MIMO cases such as inter-cell interference, advanced receiver, CSI reporting for DL and PUSCH for the UL.

Receiver

In 5G, from Rel-15 to Rel-20, RAN4 has defined requirements for the following receivers/receiving features for UE and BS.

|  | UE | BS |
| --- | --- | --- |
| Rel-15 | - MMSE-IRC for 2/4Rx SU-MIMO - R-ML for 2/4Rx SU-MIMO |  |
| Rel-17 | - MMSE-IRC for 2/4Rx with inter-cell and intra-cell inter-user (MU-MIMO) interference - CRS-IM for inter-cell 4G CRS interference for dynamic spectrum sharing (DSS) assuming LLR-weighting processing |  |
| Rel-18 | - MMSE-IRC for 8Rx SU-MIMO (cover both baseline and simplified 8Rx structures) - R-ML for 2/4Rx with MU-MIMO interference |  |
| Rel-19 | - MMSE-IRC for 8Rx with inter-cell and MU-MIMO interference (cover both baseline and simplified 8Rx structures) - MMSE-IRC for 6Rx SU-MIMO | - MMSE-IRC for 2/4/8Rx with inter-cell interference |
| Rel-20 | - MMSE-IRC for 6Rx with inter-cell and MU-MIMO interference |  |

We have the following observations based on the above background:

- RAN4 has defined performance requirements for 1/2/4/6/8Rx UEs.

- MMSE-IRC receiver with DMRS-based interference covariance estimation is widely used for 5G UEs and BSs.

- Given that RAN4 has limited time to perform system-level simulations based on the 5G network structure in the WI phase, while defining the requirements for inter-cell interference scenarios for both DL and UL, RAN4 had to reuse the same interference power value as 4G LTE. This means some important 5G features (e.g., massive MIMO, HPUE) are ignored.

- For the receiving features (i.e., MU-MIMO R-ML) that requires additional information of the interference source, RAN4 had to introduce some default assumptions to balance the additional NW signaling overhead and the UE complexity.

Based on the above observations, we have the following proposals for the 6G demodulation study:

Proposal 4: The 6G SI should cover 1/2/4/6/8Rx UE and 2/4/8Rx BS. For 8Rx UE, RAN4 to discuss whether to consider different receiver structures (e.g., baseline and simplified)

Proposal 5: The MMSE-IRC should be the baseline receiver for both UE and BS.

Proposal 6: As a preparation for 6G requirement with interference definition, RAN4 should make use of this study phase, to perform system level simulation and derive inter-cell interference model for the state-of-the-art network.

We also propose to cover advanced receivers (R-ML, soft-IC, …) in the 6G study. The performance gain should be re-evaluated since UE processing capability is much improved since 5G Rel-15, which could enable higher algorithm complexity (e.g., larger radius for sphere decoding of R-ML). In addition, the required information for advanced receivers for MU-MIMO could also be re-visited to avoid many RAN4 default assumptions. While studying the performance of advanced receivers, the UE computation time should be considered.

Proposal 7: Cover advanced receivers (R-ML, soft-IC, …) in the 6G study, to re-evaluate the performance gain and to re-visit the required information for MU-MIMO scenario.

Proposal 8: The UE computation time should be considered while studying the performance of advanced receivers.

Moreover, we also notice that Multi-RAT Spectrum Sharing (MRSS) is a possible scenario for 6G in [1].

- Physical Layer structure for 6GR, - … - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

RAN4 should also be prepared to deal with the possible interference caused by MRSS, by interference cancellation or mitigation, to avoid the severe interference from the 4G neighbor cell CRS in 5G DSS.

Proposal 9: RAN4 should also be prepared to deal with the possible interference caused by MRSS, by interference cancellation or mitigation.

New TE functionalities

OLLA with link adaptation

- Feature Lead notes - Option 1 discussion has been captured in Chairman and Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on OLLA in the next meeting as several companies see value to study OLLA as part of 6G demod SI. - Proposals - Option 1: Study to include OLLA in ATP requirements. - Option 1A: Use proposed OLLA model from R4-2300703 as a starting point. - Option 1B: Study the feasibility to include OLLA in ATP requirements.

SRS based precoding

- Feature Lead notes - Option 1 discussion has been captured in Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on SRS based precoding in the next meeting as several companies see value to study SRS based precoding as part of 6G demod SI. - Proposals - Option 1: Study whether and how to define baseline SRS based precoding procedure in TE to enable aligned simulation assumptions.

In 5G, RAN4 has introduced tests for PDSCH absolute physical layer throughput with link adaptation, where the UE reports CSI (RI/PMI/CQI) based on CSI-RS from the BS and the BS schedules rank, precoder and MCS based on the UE reported CSI.

From our point of view, the main purpose of the ATP test is to verify the UE CSI calculation accuracy based on CSI-RS. That is the reason why OLLA was not considered in 5G testing: even if the UE reports an instantaneous incorrect CSI due to implementation issues, OLLA can correct it based on historical reported values, making it hard for the test to verify the UE CSI calculation functionality.

In our understanding, the additional values of introducing OLLA in the ATP test is to ensure the best DL throughput in case some UEs prefer reporting lower CSI. From that point, in addition to the legacy ATP test with OLLA, we are supportive to study the new ATP test functionality with OLLA.

Proposal 10: It is necessary to have an ATP test without OLLA to verify the UE CSI calculation accuracy. And RAN4 can study the new ATP test functionality with OLLA, as an additional test to ensure the best DL throughput.

As for SRS based precoding, we do see the value to consider SRS transmission, which is widely used in the current TDD bands. However, it is now unclear is this new TE functionality is used for UE or BS testing.

We are supportive to consider SRS based precoding as a new BS test to verify the BS DL SRS-based precoder calculation accuracy (same test purpose as the existing ATP test based on CSI-RS). But in that case, no new TE functionality is needed since the precoder generation is up to BS implementation.

For the UE side, as in 5G, all existing PDSCH performance requirements use random Type I PMI precoding, we are supportive for 6G to study more practical and optimal precoder based on SRS calculation.

Proposal 11: Consider SRS based precoding as a new BS test to verify the BS DL SRS-based precoder calculation accuracy. In that case, no new TE functionality may not be needed since the precoder generation is up to BS implementation.

Observation 3: In 5G, all existing PDSCH performance requirements use random Type I PMI precoding.

Proposal 12: 6G can study more practical and optimal precoder based on SRS calculation for UE PDSCH testing.

Conclusion

Proposal 1: It is important to have a work plan, timeline and a priority list for the sub-topics for 6G demodulation study

Proposal 2: Prioritize the issues that require no or minimal inputs from other WGs (e.g., channel model, EVM, interference modelling, receiver structure and implementation). For the other issues, we recommend starting after there are sufficient progress from RAN1.

Observation 1: In Rel-19, RAN4 has performed studies for SCM for SU-MIMO scenario, PDSCH 4T4R rank4 and 8T8R rank8 cases and PMI type I and eType II cases are selected.

Observation 2: In Rel-20, RAN4 will do further studies based on simplified CDL channel model for MU-MIMO and both DL and UL will be covered under SI FS_NR_demod_SCM_Ph2, will be applicable for both 5G-A and 6G.

Proposal 3: For this 6G SI thread, RAN4 to study the simplified CDL performance for more SU-MIMO cases such as inter-cell interference, advanced receiver, CSI reporting for DL and PUSCH for the UL.

Proposal 4: The 6G SI should cover 1/2/4/6/8Rx UE and 2/4/8Rx BS. For 8Rx UE, RAN4 to discuss whether to consider different receiver structures (e.g., baseline and simplified)

Proposal 5: The MMSE-IRC should be the baseline receiver for both UE and BS.

Proposal 6: As a preparation for 6G requirement with interference definition, RAN4 should make use of this study phase, to perform system level simulation and derive inter-cell interference model for the state-of-the-art network.

Proposal 7: Cover advanced receivers (R-ML, soft-IC, …) in the 6G study, to re-evaluate the performance gain and to re-visit the required information for MU-MIMO scenario.

Proposal 8: The UE computation time should be considered while studying the performance of advanced receivers.

Proposal 9: RAN4 should also be prepared to deal with the possible interference caused by MRSS, by interference cancellation or mitigation.

Proposal 10: It is necessary to have an ATP test without OLLA to verify the UE CSI calculation accuracy. And RAN4 can study the new ATP test functionality with OLLA, as an additional test to ensure the best DL throughput.

Proposal 11: Consider SRS based precoding as a new BS test to verify the BS DL SRS-based precoder calculation accuracy. In that case, no new TE functionality may not be needed since the precoder generation is up to BS implementation.

Observation 3: In 5G, all existing PDSCH performance requirements use random Type I PMI precoding.

Proposal 12: 6G can study more practical and optimal precoder based on SRS calculation for UE PDSCH testing.

Reference

RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator), RAN #108, June 2025

R4-2514645, WF on [116bis][106] 6G Demod, Feature lead (MediaTek inc.), RAN4 #116bis, Oct 2025

RP-252956, New SID: Study on spatial channel model for demodulation performance requirements for NR: Phase 2, Apple, RAN#109, Sep 2025