3GPP TSG-RAN WG4 Meeting #117                                                                                                      R4-2520063

Dallas, USA, 17th –21st November, 2025

Source: 	CATT

Title: 	Discussion on angle and coordinate system for Expected EIRP mask for upper 6GHz

Agenda item:	6.8.4.1

Document for:	Approval

- Introduction

In this contribution, we provide our views on angle and coordinate system for Expected EIRP mask for upper 6GHz.

- Discussion

The angle symbol in test procedure and averaging process are unclear

The text for theta sign inversion and coordinate translation in clause 6.9.5.1 for EEIRP test requirement is copied as below:

NOTE OTA spatial emissions in table 6.9.5.1-1 are defined with respect to the horizon with angles above the horizon positive and below the horizon negative. The reference BS coordinate system in sub-clause 4.14 system is defined with respect to the BS enclosure with angles above the boresight negative and below the boresight positive. If the BS is aligned with the horizon (mechanical tilt is zero) then the coordinate systems θ sign is inverted. If the EUT is mounted at the positioner with mechanical down-tilt set to zero and non-zero mechanical tilt is applied, then coordinates are translated as described in Annex N2. In case the EEIRP measurement is for non-zero mechanical tilt and mechanical tilt of the BS is arranged at the positioner with respect to horizon, then θ sign is inverted and other coordinate translation in Annex N2 is not needed.

The above text indicates that angle used for measured EIRP defined based on reference coordinate system in clause 4.14 is different from angle for EEIRP calculation defined based on coordinate system in Figure 6.9.5.1-1 for EEIRP test requirement. So the symbol for angle used for measured EIRP shall be different from that for angle used for EEIRP calculation, since theta sign inversion or coordinate translation is required.

But in current conformance test procedure and averaging process, the symbol  θn,φm was used for angle for measured EIRP in test procedure in clause 6.9.4.2 copied as below:

10. Calculate EEIRPi from measured EIRPkθn,φm for bins i=1..7, as described in Annex N.

At same time, the same symbol  θn,φm was used for angle used for EEIRP calculation in last averaging process after coordinate translation in annex N.2.2 copied as below:

## N.2.2 Averaging over horizontal and vertical angles The second averaging process is over a spherical strip. This EEIRP for a spherical strip is therefore: EEIRPθHLi , θHHi=(θHHi-θHLi)×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn

As above, it is incorrect that the same angle symbol θn,φm was used for angle for measured EIRP in clause 6.9.4.2 and angle for EEIRP calculation in annex N.2.2.

Observation 1: it is incorrect that the same angle symbol θn,φm was used for angle for measured EIRP in clause 6.9.4.2 and angle for EEIRP calculation in annex N.2.2.

So, the angle symbol used for measured EIRP in clause 6.9.2 should differ from angle symbol θn,φm used for EEIRP calculation in annex N.2.2. It is sufficient to change angle symbol form θn,φm to   θn',φm' for measured EIRP in test procedure in clause 6.9.4.2.

Proposal 1: Change angle symbol form θn,φm to   θn',φm' for measured EIRP in test procedure in clause 6.9.4.2.

The angle symbol used in first step (test procedure) for measured EIRP is  θn,φm, the angle symbols used in second step(coordinate translation) are θ,φ and θH, φH, and the angle symbol used in last step (EEIRP calculation) for EEIRP calculation is θn,φm. The angle symbols θ,φ and θH, φH in coordinate translation are unrelated with

It is also incorect that angle symbols θ,φ and θH, φH used in second step(coordinate translation) are unrelated with angle symbol used in first step (test procedure) for measured EIRP and angle symbol used in last step (EEIRP calculation) for EEIRP calculation, since the angle in reference(local) coordinate system for measured EIRP need to be inverted to or coordinate-translated to angle in global coordinate system for EEIRP calculation.

Observation 2: It is also incorrect that angle symbols θ,φ and θH, φH used in second step(coordinate translation) are unrelated with angle symbol used in first step (test procedure) for measured EIRP and angle symbol used in last step (EEIRP calculation) for EEIRP calculation.

To solve the above issue, it is reasonable to use  θn',φm' in first step (test procedure) for measured EIRP, use θn',φm' and θn,φm in second step (coordinate translation) and use θn,φm in last step (EEIRP calculation) for EEIRP calculation. The θn',φm' is defined based on reference coordinate system in clause 4.14, and θn,φm is defined based on coordinate system in Figure 6.9.5.1-1 for EEIRP test requirement.

So, we propose,

Proposal 2: Replace θ,φ and θH, φH with  θn',φm' and  θn,φm in coordinate translation, where  θn',φm' is defined based on reference coordinate system in clause 4.14, and θn,φm is defined based on coordinate system in Figure 6.9.5.1-1 for EEIRP test requirement.

About theta sign inversion and coordinate translation

The text for theta sign inversion and coordinate translation in clause 6.9.5.1 for EEIRP test requirement is copied as below:

NOTE OTA spatial emissions in table 6.9.5.1-1 are defined with respect to the horizon with angles above the horizon positive and below the horizon negative. The reference BS coordinate system in sub-clause 4.14 system is defined with respect to the BS enclosure with angles above the boresight negative and below the boresight positive. If the BS is aligned with the horizon (mechanical tilt is zero) then the coordinate systems θ sign is inverted. If the EUT is mounted at the positioner with mechanical down-tilt set to zero and non-zero mechanical tilt is applied, then coordinates are translated as described in Annex N2. In case the EEIRP measurement is for non-zero mechanical tilt and mechanical tilt of the BS is arranged at the positioner with respect to horizon, then θ sign is inverted and other coordinate translation in Annex N2 is not needed.

The above text indicates the measured EIRP can’t be directly to be used for EEIRP calculation, and it requires theta sign inversion or coordinate-translation. And theta sign inversion or coordinate translation depends on whether the mechanical down-tilt is set. The above text indicates the following process for EEIRP calculation is needed:

If EIRPk(θn',φm') is measured when the BS is mounted at the positioner with mechanical down-tilt set to non-zero,

θn=-θn'

φm=φm'

If EIRPk(θn',φm') is measured when the BS is mounted at the positioner with mechanical down-tilt set to zero, and when zero or non-zero mechanical tilt θMT is applied,

θn=90-arccoscosφm'sinθn'+90sin-θMT+cosθn'+90cos-θMT

φm=argcosφm'sinθn'+90cos-θMT-cosθn'+90sin-θMT+jsinφm'sinθn'+90

Where, θn',φm' is defined based on reference coordinate system in clause 4.14, and θn,φm is defined based on coordinate system in Figure 6.9.5.1-1 for EEIRP test requirement.

But above theta sign inversion or coordinate translation is not captured in averaging process, and this missing will cause how to calculate EEIRP using measured EIRP is unclear.

Observation 3: Theta sign inversion or coordinate translation is not captured in averaging process, and this missing will cause how to calculate EEIRP using measured EIRP is unclear.

It is reasonable to add theta sign inversion or coordinate translation in averaging process since angle of measured EIRP is different from the angle for EEIRP test requirement.

Proposal 3: Add theta sign inversion or coordinate translation in averaging process as following:

If EIRPk(θn',φm') is measured when the BS is mounted at the positioner with mechanical down-tilt set to non-zero,

θn=-θn'

φm=φm'

If EIRPk(θn',φm') is measured when the BS is mounted at the positioner with mechanical down-tilt set to zero, and when zero or non-zero mechanical tilt θMT is applied,

θn=90-arccoscosφm'sinθn'+90sin-θMT+cosθn'+90cos-θMT

φm=argcosφm'sinθn'+90cos-θMT-cosθn'+90sin-θMT+jsinφm'sinθn'+90

Where, θn',φm' is defined based on reference coordinate system in clause 4.14, and θn,φm is defined based on coordinate system in Figure 6.9.5.1-1 for EEIRP test requirement.

Based on above proposal, A CR for TS 38.141-2 on conformance testing requirement for EEIRP for upper 6GHz is provided for approval.

- Conclusion

This contribution provides analysis on angle and coordinate system for Excepted EIRP mask for upper 6GHz. The following observation and proposal is concluded as follows:

Observation 1: It is incorrect that the same angle symbol θn,φm was used for angle for measured EIRP in clause 6.9.4.2 and angle for EEIRP calculation in annex N.2.2.

Proposal 1: Change angle symbol form θn,φm to   θn',φm' for measured EIRP in test procedure in clause 6.9.4.2.

Observation 2: It is also incorrect that angle symbols θ,φ and θH, φH used in second step(coordinate translation) are unrelated with angle symbol used in first step (test procedure) for measured EIRP and angle symbol used in last step (EEIRP calculation) for EEIRP calculation.

Proposal 2: Replace θ,φ and θH, φH with  θn',φm' and  θn,φm in coordinate translation, where  θn',φm' is defined based on reference coordinate system in clause 4.14, and θn,φm is defined based on coordinate system in Figure 6.9.5.1-1 for EEIRP test requirement.

Observation 3: Theta sign inversion or coordinate translation is not captured in averaging process, and this missing will cause how to calculate EEIRP using measured EIRP is unclear.

Proposal 3: Add theta sign inversion or coordinate translation in averaging process as following:

If EIRPk(θn',φm') is measured when the BS is mounted at the positioner with mechanical down-tilt set to non-zero,

θn=-θn'

φm=φm'

If EIRPk(θn',φm') is measured when the BS is mounted at the positioner with mechanical down-tilt set to zero, and when zero or non-zero mechanical tilt θMT is applied,

θn=90-arccoscosφm'sinθn'+90sin-θMT+cosθn'+90cos-θMT

φm=argcosφm'sinθn'+90cos-θMT-cosθn'+90sin-θMT+jsinφm'sinθn'+90

Where, θn',φm' is defined based on reference coordinate system in clause 4.14, and θn,φm is defined based on coordinate system in Figure 6.9.5.1-1 for EEIRP test requirement.

Based on above proposal, A CR[2] for TS 38.141-2 on conformance testing requirement for EEIRP for upper 6GHz is provided for approval.

- References

- R4-209356, Discussion on angle and coordinate system for Expected EIRP mask for upper 6GHz, CATT, RAN4#116

- R4-2520066, (NR_BS_RF_req_evo-Core)CR for TS 38.141-2, On conformance testing requirement for EEIRP for upper 6GHz, CATT, RAN4#117