3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2520096

Dallas, US, 17th –21th Nov, 2025

Source: 	CATT

Title: 	Discussion on 6GR co-existence

Agenda item:	8.4.3

Document for:	Discussion

- Introduction

In RAN4#116bis meeting, 6GR BS RF and co-existence began to be discussed and the WF is captured in [1]. In this contribution, we focus on co-existence issues for 6GR.

- Discussion

Below is the 6GR co-existence related content in [1].

- Coexistence studies - Issue 2-1-1: Coexistence studies Way Forward: List of candidate frequencies: 700 MHz, 2 GHz, 4 GHz, 7 GHz, 15 GHz and 30 GHz. For each exemplary frequency, companies proposing to redo coexistence studies should: - Identify the key assumptions differences (comparing to the assumptions used when RAN4 did the coex study e.g. the work done in TR 38.921 for 6.425-7.125GHz - Identify the potential impacts on the conclusions of previous coex studies (e.g. how much ACLR/ACS difference is expected). - Identify the next steps (update existing requirements? New requirements/new type? Regulation impacts?).

Frequency bands

The first priority band for 6G deployment is possible around 7GHz, either current band n104 or new band between 7GHz~8GHz. Although n104 is already specified in NR, it is rarely deployed till now and it is still possible to have big change in co-existence study as well as the RF requirements definition in 6GR.

For FR1 low band, as it is already commercial deployed for NR, improvement of BS hardware or architecture cannot be achieved overnight, so it is proposed to deprioritize these bands and leave the RF requirements as much the same as possible with NR. For higher bands 15GHz and 30GHz, as it is not well-prepared in the first day, changes may still exist when it is closed to deployment, so it can be holding on till the industry is clear.

Proposal 1: It is proposed 7GHz band as the highest priority for both co-existence and RF requirements study.

Deployment scenario

For deployment scenario, we think WID [1] has already given a guidance as below:

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

With this guidance, Sub-Urban and Urban deployment scenario should be the highest priority in co-existence study at least for 7GHz.

Proposal 2: It is proposed Sub-Urban and Urban deployment scenario as the highest priority in co-existence study at least for 7GHz.

Simulation assumptions

Some simulation assumptions in previous co-existence study need to be further considered as below.

The first thing is the BS antenna model, it was assumed antenna array (Mg, Ng, M, N, P) is (3, 1, 8, 16, 2) with 768 antenna elements in TR 38.922. Antenna model has impact on the co-existence study, the more of antennas make the narrow of antenna beams, and the expected less interference in the system. There will be different antenna models in the real deployment, we need to choose a typical one in the simulation, so the co-existence results and ACLR/ACS requirements will not be optimistic or pessimistic.

The second thing is the UE maximum output power. As high power UE increased in the field and it will impact on the simulation results, we should consider it in the simulation, at least some of UE should be high power UE.

The next thing is the channel bandwidth. Although there is no conclusion whether 200MHz or 400MHz maximum channel bandwidth should be defined, co-existence study should consider it as larger channel bandwidth may have different impact in simulation.

The last is antenna array correlation factor roll-off model with the frequency. A model in given in section 7.3.2.1.2 in TR 38.922, this model can derive a more realistic simulation results and RF requirements, such as ACLR, ACS, IBB and co-existence spurious emission requirements.

Proposal 3: BS antenna model, UE maximum output power, maximum channel bandwidth and correlation factor roll-off model need to be reconsidered in co-existence study.

- Conclusion

This contribution provides 6GR co-existence analysis.

Proposal 1: It is proposed 7GHz band as the highest priority for both co-existence and RF requirements study.

Proposal 2: It is proposed Sub-Urban and Urban deployment scenario as the highest priority in co-existence study at least for 7GHz.

Proposal 3: BS antenna model, UE maximum output power, maximum channel bandwidth and correlation factor roll-off model need to be reconsidered in co-existence study.

- References

- R4-2514638 WF on 6G BS RF and co-existence BS RF requirements

- RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone