3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2520101

Dallas, US, 17th –21th Nov, 2025

Source: 	CATT

Title: 	BS RF requirements for 4.9GHz new band

Agenda item:	5.11.3

Document for:	Approval

- Introduction

In RAN4#116bis meeting, 4.9GHz BS requirements are discussed. In this contribution, we give the proposals to capture additional BS RF requirements on this new band based on WF and agreement in [1].

## <Sub-topic 3-1> BS RF requirements <Way forward>: BS max output power - Encourage companies to investigate if/which maximum output power limit(s) should be specified. <Agreement>: OBUE requirement - Specify an additional OBUE requirement for the US 4.9 GHz band based on FCC mask M. <Agreement>: Spurious requirement - Specify an additional spurious requirement for the US 4.9 GHz band. <Agreement>: Co-location and co-existence requirements - Refrain from explicitly introducing co-location and co-existence requirements for the 4.9GHz band into the legacy co-location and co-existence tables. WID revision is necessary.

- Discussion

  - BS max output power

Additional BS max output power requirement come from FCC title 47 § 90.1215 Power limits. According to § 90.1215, additional BS output power requirements can be captured as below. For 25MHz which is not listed in § 90.1215, the output power is calculated based on the same PSD of other channel bandwidth and 15kHz SCS spectrum utilization.

-----------------------------------TS 38.104---------------------------------

6.2.4	Additional requirements (regional)

For Band n114 operation in US subject to FCC title 47, the rated output power Prated,c.sys for BS type 1-H or sum of Prated,c,AC over all antenna connectors for BS type 1-C declared by the manufacturer for each channel bandwidth shall be as specified in table 6.2.4-1.

Table 6.2.4-1 BS type 1-C and BS type 1-H rated output power limits for n114

| Channel bandwidth (MHz) | Lower limit for Prated,c,sys or sum of Prated,c,AC over all antenna connectors (dBm) | Upper limit for Prated,c,sys or sum of Prated,c,AC over all antenna connectors(dBm) |
| --- | --- | --- |
| 10 | 17 | 30 |
| 15 | 18.8 | 31.8 |
| 20 | 20 | 33 |
| 25 | 20.8 | 33.8 |
| 30 | 21.8 | 34.8 |
| 40 | 23 | 36 |
| 50 | 24 | 37 |

-----------------------------------TS 38.104---------------------------------

  - OBUE

2.2.1 OBUE general requirement

4.9GHz new band should be added to the list of bands for Wide Area cat A and cat B option 1 limits for bands above 1GHz, in clauses 6.6.4.2.1 and 6.6.4.2.2, as n79.

-----------------------------------TS 38.104---------------------------------

For BS operating in Bands n1, n2, n3, n7, n24, n25, n30, n34, n38, n39, n40, n41, n48, n50, n54, n65, n66, n70, n74, n75, n77, n78, n79, n90, n92, n94, n109, n110, n114, basic limits are specified in table 6.6.4.2.1-2 and 6.6.4.2.1-2a:

Table 6.6.4.2.1-2: Wide Area BS operating band unwanted emission limits for above 3 MHz channel bandwidth (NR bands above 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -13 dBm (Note 3) | 1MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -13 dBm/1 MHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

-----------------------------------TS 38.104---------------------------------

For BS operating in Bands n1, n2, n3, n7, n25, n34, n38, n39, n40, n41, n48, n50, n65, n66, n70, n75, n77, n78, n79, n90, n92, n94, n109, n114, basic limits are specified in tables 6.6.4.2.2.1-2:

Table 6.6.4.2.2.1-2: Wide Area BS operating band unwanted emission limits 
(NR bands above 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 3) | 1MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -15 dBm/1 MHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

-----------------------------------TS 38.104---------------------------------

2.2.2 OBUE additional requirement

Additional BS OBUE requirement come from FCC title 47 § 90.210 Emission Mask M. According to § 90.210, additional BS output power requirements can be captured as below:

-----------------------------------TS 38.104---------------------------------

6.6.4.2.5.7	Additional operating band unwanted emissions limits for Band n114

The following requirement may apply to BS operating in Band n114 in certain regions. Emissions shall not exceed the maximum levels specified in table 6.6.4.2.5.7-1.

Table 6.6.4.2.5.7-1: Additional operating band unwanted emission limits for Band n114

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note) |
| --- | --- | --- | --- |
| -0.05*CBW MHz f < 0 MHz | -0.05*CBW+0.05 MHz f_offset < 0.05 MHz |  | 100 kHz |
| 0 MHz f < 0.05*CBW MHz | 0.05 MHz f_offset < 0.05*CBW+0.05 MHz |  | 100 kHz |
| 0.05*CBW MHz f < 0.5*CBW MHz | 0.05*CBW+0.05 MHz f_offset < 0.5*CBW+0.05 MHz |  | 100 kHz |
| 0.5*CBW MHz f < CBW MHz | 0.5*CBW+0.05 MHz f_offset < CBW+0.05 MHz |  | 100 kHz |
| CBW MHz f fmax | 10.05+0.05 MHz f_offset < f_offsetmax |  | 100 kHz |
| NOTE 1: TBWC is the Transmission bandwidth configuration specified in Table 5.3.2-1 in MHz. |  |  |  |

NOTE:	The resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

-----------------------------------TS 38.104---------------------------------

2.3 Spurious emission

Additional BS OBUE requirement come from FCC title 47 § 90.210 Emission Mask M. According to § 90.210, additional BS output power requirements can be captured as below:

-----------------------------------TS 38.104---------------------------------

6.6.5.2.3	Additional spurious emissions requirements

The following requirement shall apply to BS operating in Band n114 in certain regions. The power of any spurious emission shall not exceed:

Table 6.6.5.2.3-15: Additional BS Spurious emissions limits for Band n114

| Frequency range | Maximum Level | Measurement Bandwidth |
| --- | --- | --- |
| f ≥ fmax |  | 100 kHz |
| NOTE 1: TBWC is the Transmission bandwidth configuration specified in Table 5.3.2-1 in MHz. |  |  |

-----------------------------------TS 38.104---------------------------------

- Conclusion

This contribution provides BS RF requirements for 4.9GHz new band and the proposal is as below:

Proposal: It is proposed above changes listed in this contribution.

- References

- R4-2515103, Way Forward for [116bis][322] NR_TDD_4.9GHz_band,	Fujitsu