3GPP TSG-RAN WG4 Meeting #117	R4-2520110

# Dallas, US, 17th – 21st Nov, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on test case for SA event triggered reporting tests without gap under non DRX based on OD-SSB (TC10) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case for SA event triggered reporting tests without gap for SCell under non DRX based on OD-SSB needs to be introduced for R19 NES. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the test case for SA event triggered reporting tests without gap for SCell under non DRX based on OD-SSB in clause A.7.6.1.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for SA event triggered reporting tests without gap for SCell under non DRX based on OD-SSB would be missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new)A.7.6.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

A.7.6.1.x	SA event triggered reporting test without gap for SCell under non-DRX based on OD-SSB

A.7.6.1.x.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements for OD-SSB based measurement in clauses 9.17.5.1 and 9.17.5.2. Supported test configurations are shown in table A.7.6.1.x.1-1.

Table A.7.6.1.x.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: Same configuration applies to both PCell and SCell |  |

There are three cells in the test, which are PCell (Cell 1), FR2 neighbour cell (Cell 2) and a FR2 SCell (Cell 3) on the same frequency as the PCell(Cell 1). The test parameters for the Cell 1, Cell 2 and Cell 3 are given in tables A.7.6.1.x.1-2, A.7.6.1.x.1-3 and A.7.6.1.x.1-4 below.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.7.6.1.x.1-2: General test parameters for intra-frequency event triggered reporting for SA with both TDD PCell and SCell in FR2 without gap without DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |
| Neighbour cell |  | 1, 2 | Cell 2 |  |
| SCell |  | 1, 2 | Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 and 3 | One TDD carrier frequency is used for the NR cells. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| OD-SMTC Configuration |  | 1, 2 | SMTC.1 |  |
| A3-Offset | dB | 1, 2 | -11 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  |
| Time offset between Cell 2 and Cell 3 |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

Table A.7.6.1.x.1-3: NR Cell specific test parameters for intra-frequency event triggered reporting for SA with both TDD PCell and SCell in FR2 without gap without DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NPRB,c = 66 |  |  |  |  |  |
| Data PRBs allocated |  |  | 1 | 24 |  | 24 |  | 24 |  |
|  |  |  | 2 | 48 |  | 48 |  | 48 |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1 | SR.3.2 TDD |  | N/A |  | SR.3.2 TDD |  |
|  |  | 2 | SR.3.3 TDD |  |  |  | SR.3.3 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | N/A |  | - |  |
|  |  | 2 | CR.3.2 TDD |  | N/A |  | - |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | N/A |  | CCR.3.1 TDD |  |
|  |  | 2 | CCR.3.7 TDD |  | N/A |  | CCR.3.7 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  | TRS.2.1 TDD |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.State.2 |  | N/A |  | TCI.State.2 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 120 |  | 120 |  | 120 |  |
| OCNG Patterns |  | 1, 2 | OP.5 |  | N/A |  | OP.5 |  |
| cellIndividualOffset | dB | 1~2 | N/A |  | 16 |  | N/A |  |
| SSB |  | 1 | SSB.1 FR2 |  | SSB.7 FR2 |  | SSB.1 FR2 |  |
|  |  | 2 | SSB.2 FR2 |  | SSB.8 FR2 |  | SSB.2 FR2 |  |
| OD-SSB Configuration for NCD-SSB |  | 1 | SSB.x2 FR2 |  | SSB.x2 FR2 |  | SSB.x2 FR2 |  |
|  |  | 2 | SSB.x4 FR2 |  | SSB.x4 FR2 |  | SSB.x4 FR2 |  |
| Propagation Condition |  | 1, 2 | No external noise (Note 1) |  | No external noise (Note 1) |  | No external noise (Note 1) |  |
| NOTE 1: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |  |  |

Table A.7.6.1.x.1-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with both TDD PCell and SCell in FR2 without gap without DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 | Setup 3 defined in A.3.15.3 |  |  |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  | AoA1 |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  | Rough |  |
| Es | dBm/SCS | 1 | -89 | -89 | -Infinity | -89 | -89 | -89 |
|  |  | 2 | -86 | -86 | -Infinity | -86 | -86 | -86 |
| BB Note 5 | dB | 1, 2 | -0.12 | -0.12 | -Infinity | -0.12 | -0.12 | -0.12 |
| SSB_RP | dBm/SCS | 1 | -89 | -89 | -Infinity | -89 | -89 | -89 |
|  |  | 2 | -86 | -86 | -Infinity | -86 | -86 | -86 |
|  | dBm/95.04 MHz | 1 | -64.41 | -64.41 | -Infinity | -64.41 | -64.41 | -64.41 |
|  |  | 2 | -61.41 | -61.41 | -Infinity | -61.41 | -61.41 | -61.41 |
| Time multiplexing of the downlink transmissions from each AoA |  | 1, 2 | Defined in figure A.7.6.1.x.1-1 |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Void NOTE 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

Figure A.7.6.1.x.1-1: Time multiplexed downlink transmissions (Config 1 example)

A.7.6.1.x.2	Test Requirements

In the test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	2.4 s for a UE supporting power class 1,

-	1.44 s for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

==============End of change==============