3GPP TSG RAN WG4 Meeting #117		R4-2520176

Dallas, Texas, United States, Nov 17th – 21st , 2025

Title: 	Further discussion on waveform for 6GR

Agenda Item:	8.2.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

At RAN4#116-bis, the first meeting where RAN4 initiated its 6GR studies, extensive discussions were held on system parameters, including the waveform for 6GR, leading to the following agreements:

## Framework study for waveform - Agreement - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1 - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects - Tx assumption including PA model - Related RF requirements which should be taken into consideration - Both UL and DL are considered - Take the candidate waveforms identified/discussed in RAN1 as the baseline - Identify the evaluation metric, e.g. net gain - Implementation constraints - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs - Evaluation cases could be selected step-by-step, based on RAN1 progress - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement ## PA model - Agreement - Consider PA modelling at least for the following aspects for RAN4 discussion - Memory effects for UE supporting larger CBW - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3) - Calibration conditions - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc. - PA models for different device types, handheld UE is prioritized - Applicable requirements - 5G-A requirements as starting point - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

In this contribution, we present our views on waveform studies, especially PA modelling from RAN4 perspective.

# 2 Discussion

In RAN1#122bis, some agreements have been reached regarding the 6GR waveform study [2]:

Agreement - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding Agreement - Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

A suitable PA model is needed to enable further evaluations; therefore, an LS requesting such PA models has been sent from RAN1 to RAN4 [3], and timely feedback from RAN4 to RAN1 is anticipated.

Indeed, it is a challenging task to model a real PA, where many effects may exist at the same time. In fact, PA modelling was extensively investigated already at the beginning of the 5G era, and those studies provide a solid foundation for further work, especially for the much wider bandwidths and new frequency ranges envisioned for 6G. TR 38.803 Annex A documented several PA models, such as GMP (Generalized Memory Polynomials model), Rapp model, Saleh model, Ghorbani Model etc [4].

A mathematical expression for GMP model is denoted as the following equation [4]:

yGMPn=k∈Kal∈Laaklxn-lxn-l2k+k∈Kbl∈Lbm∈Mbklmxn-lxn-m2k

where parameters should be tuned according to the working bandwidth, frequencies, etc.

Similarly, Rapp model is expressed as the following equation [4]:

Vout=Vin1+VinVsat2P12P

And Saleh model for AM-PM is described as:

frAM-PM=αφr21+βφr2

To accurately represent a real PA, a single model type may not be sufficient. Therefore, an initial approach could be to combine multiple model types, after which RAN4 can focus on tuning the corresponding parameters of the composite PA model.

Proposal 1: As a general methodology for developing a realistic PA model, RAN4 to first consider a combination of multiple model types, i.e., a composite PA model, and then focus on tuning the corresponding parameters.

Since around 7 GHz is expected to be one of the most promising frequency ranges for 6GR deployment, where larger channel bandwidths (e.g., 200 MHz or 400 MHz) are under discussion, RAN4 could begin by developing a composite PA model tailored for this frequency range.

Proposal 2: RAN4 to start by developing a composite PA model for 7GHz assuming a larger channel bandwidth e.g., 200MHz or 400MHz.

Actually, each PA model may serve a different purpose, with each focusing on capturing nonlinearity mechanism observed in practical transmitters.

In our view, an effective starting point is a three-stage composite PA model composed of:

- GMP model – capturing dynamic nonlinearities and memory effects due to biasing, thermal, and electrical feedback phenomena. The GMP stage provides amplitude and phase distortion correlated over time, which becomes increasingly significant for wideband signals (e.g., 200 MHz or 400 MHz at around 7 GHz).

- Rapp model (AM/AM) – representing the static nonlinear compression characteristic of the amplifier with a controllable smoothness parameter p, emulating soft saturation behavior typically observed in class-AB or Doherty-type PAs.

- Saleh model (AM/PM) – introducing amplitude-dependent phase rotation, which is important for accurate EVM and spectral regrowth estimation in realistic uplink scenarios.

This composite structure enables separate yet complementary control over:

- Memory effects (via GMP coefficients and memory depth),

- Amplitude compression (via Rapp saturation level and smoothness),

- Phase distortion (via Saleh AM/PM coefficients).

This approach provides both flexibility and realism, forming a solid foundation for subsequent parameter refinement and validation across different device types and deployment scenarios.

Proposal 3: RAN4 to consider, as a starting point, a composite PA model for 7GHz consisting of a GMP model followed by a Rapp model and Saleh AM-PM model.

# 3 Conclusion

In this contribution, we have the following proposals for PA modelling for 6GR waveform evaluations:

Proposal 1: As a general methodology for developing a realistic PA model, RAN4 to first consider a combination of multiple model types, i.e., a composite PA model, and then focus on tuning the corresponding parameters.

Proposal 2: RAN4 to start by developing a composite PA model for 7GHz assuming a larger channel bandwidth e.g., 200MHz or 400MHz.

Proposal 3: RAN4 to consider, as a starting point, a composite PA model for 7GHz consisting of a GMP model followed by a Rapp model and Saleh AM-PM model.

# Reference

[1] R4-2514641, “WF for 6GR system parameter”, Huawei, HiSilicon

[2] R1-25xxxxx, “Minutes_report_RAN1#122bis_v104”, RAN1

[3] R1-2508069, “LS on PA models in 6GR waveform evaluations”, RAN1

[4] 3GPP TR 38.803 v14.4.0