3GPP TSG-RAN WG4 Meeting #117	                                 R4-2520224

Dallas, USA, November 17 – November 21, 2024

Source: 	Murata Manufacturing Co Ltd.

Title: 	Discussion on general methodology for FDD PC1.5 2Tx RSD

Agenda item:	7.1.4

Document for:	Discussion

Introduction

In the last RAN#109 plenary meeting in September, a new WID on UE RF requirements for Rel-20 was approved [1]. One of the objectives for HPUE is to specify RF requirements of PC1.5 with 2Tx for FDD bands including maximum output power, MPR as well as reference sensitivity degradation (RSD). The baseline PA configuration is to use 2 x 26 dBm PAs as in PC1.5 TDD band and the example bands are n1 and n25.

In the last RAN4#116-bis meeting in October, MOP and ACLR requirements, some NS scenarios to be evaluated and 0dB RSD of n1 were agreed but RSD of n25 was not discussed. Instead, it was agreed to encourage companies to provide the general methodology for deriving the RSD in next meeting in WF [2].

In this contribution	we propose the general PC1.5 2Tx RSD calculation methodology based on the existing PC3 REFSENS and PC2 1Tx RSD which can apply to not only example bands but also other FDD bands.

Discussion

Motivation of general methodology

The reference sensitivity degradation (RSD) is a term to describe how much the reference sensitivity of FDD band can be relaxed when Tx power class is changed from PC3 to PC2 (and PC1.5). There are two tables of RSD of PC2 in the specification: 1Tx and 2Tx (supporting Tx diversity) depending on the PA capability. In general, the RSD of 2Tx is higher than 1Tx since diversity gain is not applied to 2Tx.

Since PC2 FDD was introduced, 3GPP RAN4 working group has spent much time and efforts specifying RSD. Several companies analysed those based on their own RF architecture and performance assumptions. In the end, the requirements are averaged between different proposals from different companies.

Even though only n1 and n25 are example bands for PC1.5 in Rel-20, it is expected that other FDD bands for PC1.5 are to be introduced later from operator requests. Therefore, it is not efficient to follow the previous method of determining PC1.5 RSD for many FDD bands which is time consuming in terms of RAN4 activity.

Observation 1: RAN4 spent much time and efforts to determine PC2 RSD and it is expected that many FDD bands supporting PC1.5 are to be introduced later.

Moreover, if RAN4 iterates the procedure of deriving PC2 RSD, there might be some inconsistency between PC2 1Tx RSD and PC1.5 2Tx RSD. For example, PC2 1Tx RSD of one band (A) is higher than another band (B) but PC1.5 2Tx RSD of band A might be smaller than band B. This is because the conventional way of determining RSD is to average the different company’s inputs based on different RF assumptions. In other words, the PC1.5 2Tx RSD might not follow the trend of existing PC2 1Tx RSD requirements.

Observation 2: There might be some inconsistency between the existing PC2 1Tx RSD and newly specified PC1.5 2Tx RSD if RAN4 follows the traditional way of determining PC1.5 2Tx RSD due to different inputs from different companies.

Based on the above observations, it is highly recommended to have a general methodology for deriving the PC1.5 2Tx RSD not only for RAN4 working efficiency but also for specification consistency.

Proposal 1: It is highly recommended to have a general methodology for deriving the PC1.5 2Tx RSD not only for RAN4 working efficiency but also for specification consistency.

General methodology procedure

In the last few meetings, the HPUE MSD simplification rules using LUT was agreed and this would be reflected to TS document. This achivement is very encourging since RAN4 does not need to spend time and efforts in deriving HPUE MSD and TS document can be simplified. Therefore, we referred HPUE MSD rule analysis as a starting point [3].

It should be noted that the relationship between PC1.5 2Tx RSD and PC2 1Tx RSD is very similar to relationship between PC1.5 2Tx MSD and PC2 1Tx MSD for 1UL 1CC case. In other words, the PC1.5 2Tx FDD architectue uses two PC2 FDD PAs in both primary and secondary paths. Therefore, the Tx self inteferer noise for both paths would be same as disussed in [3] because the RF front end architecture is symmetrical between primary and secondary paths.

However, there is one different point compared to HPUE MSD rule. There is only one channel bandwidth test point in MSD calculation where as REFSENS and corresponding RSD is dependent on channel bandwidth. Moreover, the bandwidth scaling rule does not apply to large bandwidth in calculating REFSENS because the additional nonlinearity noise from PA and Tx chain is falling into DL bandwidth as Tx channel bandwidth gets large. Therefore modifications for RSD calculation are needed.

The most important modifcation is to calculate the thermal noise level without any other inteferer for each channel bandwidth. It is directly calculated from the REFSENS in case of small channel bandwidth such as 5MHz since there is no self Tx inteferer noise. However, it is not straighforward to derive the thermal noise for large channel bandwidth from REFSENS because the self Tx inteferer noise is taken into account for the REFSENS derivation. So below equation was used for thermal noise calculation of each DL channel bandwidth.

Nth = RefSens_5MHz + 4 + 10*log10(DL_BW_FRB/4.5)

Where DL_BW_FRB is occuped channel bandwidth

After that, we can calculate the self Tx inteferer noise at primary Rx path (Pint, A) by reverse engineering method using the existing REFSENS at each DL CBW, calculated thermal noise at DL CBW (Nth) and the specified PC2 1Tx RSD with 6dB inteferer imbalance between PRx and DRx as shown in Table 1.

Note that “Log[N1 + N2]” means noise power summation using 10*log10(10^(N1/10) + 10^(N2/10)).

Table 1 PC2 1Tx RSD calculation (or Pint calculation by reverse engineering)

|  | PRX | DRX |
| --- | --- | --- |
| REFSENS at 5MHz CBW | RefSens_5MHz |  |
| REFSENS at DL CBW | RefSens_DL_CBW |  |
| Thermal at DL CBW | Nth | Nth |
| Interferer | Pint (A) | A-6 (B) |
| Composite noise | Log[Nth + A] | Log[Nth + B] |
| MRC sensitivity (cor.) | MRC composite noise -1 |  |
| RSD after MRC | MRC sensitivity - RefSens_DL_CBW |  |

Finally, the Tx inteferer noise at PRX and DRX are reused to calculate PC1.5 2Tx SRD as shown in Table 2. It should be noted that both PRX and DRX see the same Tx self interferer noise (power sum of A and B).

Table 2 PC1.5 2Tx RSD calculation using Pint result in Table 1

|  | PRX | DRX |
| --- | --- | --- |
| REFSENS at 5MHz CBW | RefSens_5MHz |  |
| REFSENS at DL CBW | RefSens_DL_CBW |  |
| Thermal at DL CBW | Nth | Nth |
| Interferer due to Tx1 | A | B |
| Interferer due to Tx2 | B | A |
| Interferer sum | Log[A + B] (C) | Log[A + B] (C) |
| Composite noise | Log[Nth + C] | Log[Nth + C] |
| MRC sensitivity (cor.) | MRC composite noise -1 |  |
| RSD after MRC | MRC sensitivity - RefSens_DL_CBW |  |

This way, there is no need to evaluate the PC1.5 2Tx RSD using the conventional method RAN4 has done. Moreover, the PC1.5 2Tx RSD is calculated as general methodology based on the exising PC2 1Tx RSD requirement, the consistency between them can be guranteed.

Observation 3: The general PC1.5 2Tx RSD calculation methodology in Table 1 and 2 can be used not only for PC1.5 2Tx RSD calculation procedure simplification but also for consistency of spec

Proposal 2: RAN4 to use the general PC1.5 2Tx RSD calculation methodology in Table 1 and 2 for PC1.5 2Tx RSD calculation

Application to n25 PC1.5 2Tx RSD

Table 3 shows the summary of PC1.5 2Tx RSD calculation result of n25 following the format from Table 2. The Tx self inteferer noise of PC2 1Tx RSD was firstly calculated and used for the PC1.5 2Tx RSD calculation as in Table 2.

Table 3 PC1.5 2Tx RSD calculation of n25

| CH BW | 5MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | 25 MHz |  | 30 MHz |  | 35 MHz |  | 40 MHz |  | 45 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRX | DRX | PRX | DRX | PRX | DRX | PRX | DRX | PRX | DRX | PRX | DRX | PRX | DRX | PRX | DRX | PRX | DRX |
| REFSENS at 5MHz CBW | -96.5 |  | -96.5 |  | -96.5 |  | -96.5 |  | -96.5 |  | -96.5 |  | -96.5 |  | -96.5 |  | -96.5 |  |
| REFSENS at DL CBW | -96.5 |  | -93.3 |  | -91.5 |  | -90.3 |  | -89.3 |  | -82.2 |  | -81.7 |  | -79.5 |  | -77.6 |  |
| Thermal at DL CBW | -92.5 | -92.5 | -89.3 | -89.3 | -87.5 | -87.5 | -86.2 | -86.2 | -85.2 | -85.2 | -84.4 | -84.4 | -83.7 | -83.7 | -83.1 | -83.1 | -82.6 | -82.6 |
| Interferer due to Tx1 | -99.8 | -105.8 | -96.5 | -102.5 | -94.2 | -100.2 | -92.3 | -98.3 | -90.4 | -96.4 | -75.2 | -81.2 | -74.6 | -80.6 | -71.2 | -77.2 | -68.9 | -74.9 |
| Interferer due to Tx2 | -105.8 | -99.8 | -102.5 | -96.5 | -100.2 | -94.2 | -98.3 | -92.3 | -96.4 | -90.4 | -81.2 | -75.2 | -80.6 | -74.6 | -77.2 | -71.2 | -74.9 | -68.9 |
| Interferer sum | -98.8 | -98.8 | -95.5 | -95.5 | -93.2 | -93.2 | -91.3 | -91.3 | -89.4 | -89.4 | -74.2 | -74.2 | -73.6 | -73.6 | -70.2 | -70.2 | -67.9 | -67.9 |
| Composite Noise | -91.6 | -91.6 | -88.4 | -88.4 | -86.5 | -86.5 | -85.1 | -85.1 | -83.8 | -83.8 | -73.8 | -73.8 | -73.2 | -73.2 | -70.0 | -70.0 | -67.8 | -67.8 |
| MRC sensitivity | -94.8 |  | -91.6 |  | -89.7 |  | -88.1 |  | -86.8 |  | -75.0 |  | -74.4 |  | -71.1 |  | -68.9 |  |
| RSD after MRC | 1.7 |  | 1.7 |  | 1.8 |  | 2.2 |  | 2.5 |  | 7.2 |  | 7.3 |  | 8.4 |  | 8.7 |  |

It is highly recommended that more general methodology, e.g. LUT basis should be developed considering the different REFSENS and PC2 1Tx RSD for each DL CBW not only for n25 but also other FDD bands. Unfortunately, this approach is not discussed in this contribution due to lack of time. However, it is worth discussing further within RAN4 depending on company’s inputs.

Proposal 3: Use Table 3 for PC1.5 2Tx RSD of n25. Additionally, RAN4 to discuss further on the more general methodology, e.g. LUT basis.

Conclusion

In this contribution, we analyzed and provided the general methodology for FDD PC1.5 2Tx RSD based on the existing PC3 REFSENS and PC2 1Tx RSD. We believe that this can simplify the process of specifying the PC1.5 2Tx RSD for all FDD bands and reduce the burden of RAN4 workload and guarantee the consistency with the current PC2 1Tx RSD requirement. With this analysis result, we make the following proposals.

Observation 1: RAN4 spent much time and efforts to determine PC2 RSD and it is expected that many FDD bands supporting PC1.5 are to be introduced later

Observation 2: There might be some inconsistency between the existing PC2 1Tx RSD and newly specified PC1.5 2Tx RSD if RAN4 follows the traditional way of determining PC1.5 2Tx RSD due to different inputs from different companies.

Proposal 1: It is highly recommended to have a general methodology for deriving the PC1.5 2Tx RSD not only for RAN4 working efficiency but also for specification consistency.

Observation 3: The general PC1.5 2Tx RSD calculation methodology in Table 1 and 2 can be used not only for PC1.5 2Tx RSD calculation procedure simplification but also for consistency of spec

Proposal 2: RAN4 to use the general PC1.5 2Tx RSD calculation methodology in Table 1 and 2 for PC1.5 2Tx RSD calculation

Proposal 3: Use Table 3 for PC1.5 2Tx RSD of n25. Additionally, RAN4 to discuss further on the more general methodology, e.g. LUT basis.

References

[1]	RP-252962, “New WID: UE RF enhancements for NR FR1, Phase 5”, 3GPP TSG RAN Plenary Meeting #109, Beijing, China, September 15th – 18th, 2025

[2]	R4-2515087, “Way Forward for [116bis][316] NR_UE_RF_Ph5”, Nokia, Murata, 3GPP TSG RAN WG4 Meeting #116-bis, Prague, Czech Republic, October 13th – 17th, 2025

[3]	R4-2417784, “Further discussion on HPUE MSD rules”, Murata Manufacturing Co Ltd., 3GPP TSG RAN WG4 Meeting #113, Orlando, USA, November 18th – 22nd, 2024