# 3GPP TSG-RAN WG4 Meeting #117                                                                    R4-2520319

Dallas, USA, Nov. 17-21, 2025

Source:                Tejas Networks

Title:                    Spectrum sharing for migration, mobility between 5G NR and 6G

Agenda Item:      8.9

Document for:    Discussion

# Introduction

Dynamic Spectrum Sharing (DSS) has supported 5G NR deployment since Release 15, evolving through Releases 17 and 18 with enhancements for coexistence and resource optimization. As 3GPP initiates 6G studies under 6GR, MRSS between NR and 6G will be critical for migration, mobility, and efficient use of existing bands. This contribution outlines key RAN4 considerations for MRSS, reflecting discussions from the recent RAN4 meeting and the approved 6G plan. Views on future spectrum sharing for 5G NR and 6G are presented.

# Discussion

RAN4#116bis addressed key aspects of numerology and inter-RAT measurement for 5G–6G MRSS. The WF agreed in [1] is reproduced below for reference.

#### Numerology - Proposals from companies - P1 (CMCC): the numerology discussion for 6GR in legacy band has impact on MRSS. It is proposed to follow RAN1 agreements to take 15KHz SCS for FDD and 30KHz SCS for TDD. - P2 (OPPO): For MRSS operation via FDD sharing, 6G should adopt the same SCS with 5G when sharing carrier/channel - P3 (ZTE): for the MRSS BS, the commercialized NR subcarrier spacing (e.g. FR1 FDD with 15kHz, FR1 TDD with 30kHz and FR2-1 with 120kHz SCS) should be the baseline for 6G BS - P4 (Ericsson): During the discussion on system parameters, take into account that alignment between 5G and 6G slot length and numerology will facilitate MRSS - P5 (Xiaomi): On 5G migration spectrum/bands, harmonize the numerology between 5G and 6G by taking existing NR commercialization deployment choice into account. - Way forward - Further discuss numerology impacted by MRSS

#### Inter-RAT measurement - Proposal from companies - Option 1 (CMCC): it is proposed to support inter-RAT measurements without gaps, including inter-RAT NR measurement without gap and inter-RAT 6GR measurement without gap, from 6G day-1. - Option 2 (Xiaomi): RAN4 shall study potential inter-RAT RRM measurement impact including w/o and with gap under MRSS scenario. - Way forward - Further discuss

## Numerology Considerations for MRSS

Mismatched subcarrier spacing, for example 15 kHz in NR versus 30 kHz in 6GR, introduces symbol boundary offsets every 0.5 ms. This misalignment requires cyclic prefix extensions or guard symbols to prevent inter-symbol interference, resulting in added overhead for dynamic TDD sharing. Aligning subcarrier spacing ends these inefficiencies by ensuring symbol-level orthogonality across RATs, enabling coordinated scheduling in frequency division multiplexing partitions, and allowing reuse of the NR frame structure with 10 ms frames and scaling factors μ = 0 for 15 kHz or μ = 1 for 30 kHz. For FDD-based MRSS, such as in bands n1 or n3, adopting 15 kHz for 6GR aligns with existing NR deployments, supporting uplink and downlink separation, avoiding cross-link interference, and preserving phase continuity for channel estimation. For TDD-based MRSS, such as band n78, using 30 kHz for 6GR corresponds to most NR configurations, reducing guard band requirements in partial bandwidth overlaps and enabling efficient resource block packing. Harmonized numerology also provides added benefits. Identical slot durations simplify synchronization signal block placement and enable gapless inter-RAT measurements. Base station implementation is streamlined through shared clock references and processing chains, reducing hardware complexity. User equipment benefits by avoiding parallel baseband paths for different scaling factors, minimizing power consumption during MRSS operation.

Observation1: Per-band subcarrier spacing harmonization ends most multiplexing losses seen in misaligned configurations.

Proposal 1: Consider 15 kHz SCS for 6GR in FR1 FDD and 30 kHz SCS for 6GR in FR1 TDD under MRSS, aligned with NR deployments.

Proposal 2: Explore supporting identical SCS between NR and 6GR within the same shared MRSS carrier to ensure orthogonality.

## Inter-RAT measurement

Release 18 introduced gapless inter-RAT measurements primarily for LTE–NR in EN-DC configurations, where UEs monitor LTE CRS within NR bandwidth parts without retuning. For 5G–6G MRSS, shared carriers enable similar overlap; however, dynamic partitioning introduces risks due to unaligned measurement occasions. A hybrid approach is proposed, establishing gapless measurement as the default for MRSS in FR1, enabling concurrent detection of NR and 6GR synchronization signal blocks within the active bandwidth part. The network can configure measurement objects to cover both RATs without requiring retuning. For scenarios with high interference or limited overlap, selective gaps may be triggered based on UE reports or network prediction. This framework positions gapless inter-RAT measurement as the baseline, with selective gaps under study, supporting seamless operation during early 6G phases.

Observation 2: Gapless inter-RAT measurement is set as the baseline for MRSS in FR1.

Proposal 3: Support gapless inter-RAT measurements for NR and 6GR in MRSS FR1 from initial deployments.

# Conclusion

Observation1: Per-band subcarrier spacing harmonization ends most multiplexing losses seen in misaligned configurations.

Observation 2: Gapless inter-RAT measurement is set as the baseline for MRSS in FR1.

Proposal 1: Consider 15 kHz SCS for 6GR in FR1 FDD and 30 kHz SCS for 6GR in FR1 TDD under MRSS, aligned with NR deployments.

Proposal 2: Explore supporting identical SCS between NR and 6GR within the same shared MRSS carrier to ensure orthogonality.

Proposal 3: Support gapless inter-RAT measurements for NR and 6GR in MRSS FR1 from initial deployments.

# Reference

- R4-2514646, “WF for [116bis][108] 6G spectrum sharing”, CMCC