3GPP TSG-RAN WG4 Meeting #117	R4-2520329

Dallas, USA, 17 – 21 November, 2025

Title: 	Further consideration on RRM study for 6G

Source: 	Huawei, HiSilicon

Agenda item:	8.6

Document for:	Discussion

Introduction

RAN WG level study for 6GR was approved in RAN#108, and the latest SID can be found in [1]. The scope for RRM study were discussed in RAN4#116-bis, and the outcomes are captured in WF [2]. Based on WF [2], RAN4 is expected to discuss the following topics for 6G RRM in this meeting.

- Measurement gap (MG) and interruption

- RRM framework

- Mobility related RRM framework

- Other

In this paper, we will provide our further considerations on several aspects for 6GR RRM study.

Discussion

6GR is a new generation of RAT, and the SID [1] lists a lot of aspects to be studied in different RAN WGs, including the following RAN4 RRM related aspect.

- 6GR core and performance requirements - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles

As in 5G NR and earlier generations, some aspects of RRM requirements depends on the signal, channel and procedure design from other WGs, mainly RAN1 and RAN2. On the other hand, some aspects are driven by RAN4, and can be studied in parallel with RAN1/2. Per RAN4 Chair guidance, we will discuss topics that are driven by RAN4 RRM and require study in SI, and provide our views on the enhancements in those topics based on learnings from 5G NR and earlier generations.

Measurement gap (MG) and interruption

Agreement in main session (Tue): Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion.

Measurement gap has been a pain point for RRM measurement since 4G LTE. Considering mandatory gap pattern with 6ms MGL and 80ms MGRP, the throughput loss due to use of measurement gap would be 15% from UE perspective. From NW perspective, measurement gap also causes restrictions in scheduling and configuration thus adds implementation complexity.

The need of measurement gap comes from the fact that UE has limited RF resource, so it needs to borrow the RF resource for serving carriers to measure target carriers. Measurement without gap has been defined in 4G LTE and 5G NR based on spare RF resource. In the time of 6GR, UE capable of aggregating spectrum over multiple frequency bands with multiple RF chains would be typical, at least for smartphones. For such UEs, it is straightforward to support measurement without gap to fully utilize their RF capability.

In our view, RAN4 should study solution to support measurement without gap for the first release of 6GR, based on use the spare RF chain as shown in Figure 1.

Figure 1: Example of measurement without gap for 6G

As we know, measurement without gap comes with short interruptions. In 5G NR, 2 solutions are defined to support measurement without gap based on spare RF chain, namely NFG and NCSG. NCSG allows NW to control the location of interruption thus leads to better reliability from system point of view. On the other hand, NFG allow more flexible UE implementation and simpler NW implementation, and has the potential possibility to achieve less interruption than NCSG (which allows worst case interruption). As the 2 solutions have its own pros and cons, UE vendor and NW vendor may have distinctly different interests in the two directions. This may cause segmentation and impact commercial application of measurement without gap in 5G NR. In 6GR, RAN4 should strive for a unified solution which preferably retains advantages of both NFG and NCSG.

Proposal 1: RAN4 to study solutions to support measurement without gap, and strive for a unified solution the first release of 6GR.

Besides measurement without gap, we also suggest to study simultaneous measurements within MG. As discussed in section 2.2, measurement delay is a critical requirement. In NR, baseline requirements for measurement outside MG is defined with the assumption of 2 searchers, but the number of searchers for measurement within MG is only 1.

Technically, it is feasible for UE to do simultaneous measurement within MG if its RF capability allows because the searcher number is same for measurement within or outside MG. It is also noted that in 4G LTE simultaneous measurement within gap is already supported in R14.

Proposal 2: RAN4 to study how to support simultaneous measurement within MG.

We also suggest RAN4 to study interruption requirements in combination with measurement without gap. For measurement without gap, RAN4 will study whether and how to improve the interruption performance, including the interruption length. The study on interruption length can cover all scenarios where interruption is caused by RF re-tuning, e.g. those resulted from spectrum aggregation, and the study can also cover MGL.

For the location of the interruption in other procedure, e.g. those resulted from spectrum aggregation, our view is that it is too early to study because the interruption location should be discussed case by case for each procedure, but the procedure (like SCell activation or Tx switching in NR) itself has not been defined.

Proposal 3: RAN4 to study the interruption length requirements in combination with measurement without gap.

For other sub-topics under the topic of MG and interruption,

- MG pattern reduction: our view is that this sub-topic can be postponed because we do not have the baseline pattern for the mobility reference signal, e.g. periodicity and duration of SSB burst, from RAN1, so RAN4 cannot even define any single MG pattern at this stage. Of course, we understand RAN4 will study MG pattern later, and the number of MG patterns should be considered carefully.

- Unified MG: the scope is not fully clear to us. It is too early to discuss unification of MG for RRM and gap for other features because we do not know which other features would require gap. On unifying MG and scheduling restriction, our view is that we can consider it if per-CC gap is supported, otherwise it can be deprioritized because there is no clear technical benefit.

- Adaptative MG operation: we understand this can be studied. Different solutions have been defined in NR to support adaptive MG operation, including RRC based, MAC CE based, DCI based and rule based. RAN4 could study the use cases and solutions based on NR experience.

- MG granularity: we understand per CC gap can be studied. NR supports per UE gap and per FR gap, but still per CC gap has its specific use case, i.e. when measurement of an MO requires MG on one CC but not on another CC. Also, per CC has been supported in LTE in R14. On the other hand, per CC gap will add complexity to both spec and NW/UE implementation, which should also be part of the study.

Proposal 4: Besides the sub-topics in Proposal 1-3, RAN4 can also study following sub-topics under MG and interruption topic.

- Adaptative MG operation

- Per CC MG

RRM framework

Agreement: RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. Note: the title of the main topic can be revised after selection of the sub-topics.

RRM measurement is the cornerstone for the timeliness and correctness of mobility decisions, e.g. late HO resulted from large measurement delay is a common mobility issue observed in real field.

It is noted that the measurement delay performance is worse in 5G NR compared to 4G LTE. In 4G LTE, the measurement of intra-frequency carriers can be performed in parallel. In 5G NR the RS (SSB) is transmitted in burst manner with larger periodicity than LTE RS, so UE has to do computationally demanding cell search in a time concentrated manner. In order to keep reasonable UE complexity, parallel measurement capability is limited by the number of so-called “searchers”. In R15 2-searcher is taken as baseline, i.e. UE can measure up to 2 carriers in parallel. In R19 3-searcher is supported as an add-on feature.

In our view, 6GR would likely adopt burst RS pattern similar as 5G NR, so RAN4 should study enhanced capability on simultaneous measurements to achieve better measurement delay performance compared to 5G NR as shown in Figure 2.

Figure 2: Example of measurement delay reduction for 6G

The first priority is to consider certain enhancement for the measurement outside gap because 6GR UE is supposed to aggregate more serving carriers and support measurement without gap in many configurations.

Proposal 5: RAN4 to study RRM measurement delay reduction for the first release of 6GR by considering enhanced simultaneous measurements.

For other sub-topics under the topic of RRM framework,

- Measurement capability: our view is that this sub-topic can be merged into other sub-topics or postponed. Capability in terms of number of cells, beams and carriers heavily depends on the UE implementation, and it more proper to be discussed during WI phase when the implementation is approaching. Capability in terms of number of searchers can be discussed in the sub-topic of measurement delay reduction. RF related capability can be discussed in the sub-topic of measurement without gap as discussed in section 2.1.

- Unified measurement: our view is that study of this sub-topic needs to wait for RAN1/2 conclusions. On the unification of L1 and L3 measurements, RAN2 is already discussing potential unification of the two measurements from procedure and signaling perspective, and UE implementation will depend on the RAN1/2 conclusions about the supported measurement framework. It may be too early for RAN4 to discuss the unification of requirements based on measurement framework and requirements in NR. For unification of measurements for different functions, it is similar, the MIMO discussion has not started in RAN1, and it is not clear which functions, besides the inter-cell mobility discussed in RAN2, would require measurement and whether those measurements will be based on single or separate framework.

Proposal 6: RAN4 to wait for RAN1/2 conclusions on measurement framework, procedure and signalling before discussing unified measurement requirements.

Mobility related RRM framework

Mobility is fundamental for wireless networks, and HO is the most basic and important procedure mobility. The delay and interruption during HO procedure also have clear impact on the user experience in the field.  A few solutions have been standardized in 5G NR to reduce the HO delay and interruption, but commercial deployment was not that much.

In basic HO requirements in 5G NR, the HO delay is defined as RRC procedure delay plus the interruption time. The interruption time further includes:

- Time for UE processing e.g. SW processing and RF warmup

- Time for cell search or coarse sync, if the target cell has not been measured before HO is commanded

- Time for fine time and frequency sync in DL to get prepared for data scheduling in the target cell

- Time for obtaining RACH resource to complete HO procedure (and also get UL sync)

From RRM requirements, the typical HO delay in 5G NR would be 60ms with 50ms interruption time, assuming known target cell and 20ms SSB period) where the ending point of interruption is first PRACH transmission. In reality, RACH procedure may take tens of milli-second, so the total time duration without data scheduling would be around 100ms.

5G NR introduced several enhanced HO mechanisms to reduce the HO delay and interruption:

- DAPS: UE supports simultaneous Tx/Rx with source and target cells, so the interruption time can be minimized. From UE implementation complexity perspective, some enhancements of BB and RF resources may be required to support simultaneous Tx/Rx.

- CHO: UE executes HO based on pre-configured target cell configurations when configured conditions are met. It can help to avoid the RRC procedure delay for the HO command transmission, but it does not reduce the HO interruption time.

- RACH-less HO: UE directly transmits HO complete message to the target cell on PUSCH based on specified mechanisms to determine TA. It can reduce the HO delay for the RA procedure (which is not visible in RAN4 requirements), but it does not reduce the HO interruption time.

- LTM: UE performs cell switch based on switch command in MAC CE, and UE may perform pre-sync in both DL and UL. It can reduce both delay and interruption in HO. UE may also pre-obtain the beam and CSI measurement for the target cell to enable efficient data scheduling immediately after the HO.

In our view, RAN4 should study how to reduce delay and interruption during HO, and strive for a unified solution for the first release of 6GR. Some principles in 5G NR LTM can be used as starting point, e.g. UE can do pre-sync for the candidate target cells while maintaining data connection with the source cell. This will leave the processing time as the main component of interruption time during HO and the interruption is largely reduced, as shown in Figure 3. This can be done with reasonable UE complexity as UE is required to maintain sync rather than whole data link with the two cells.

Proposal 7: RAN4 to study how to reduce delay and interruption during HO, and strive for a unified solution for the first release of 6GR.

Figure 3: Example of HO enhancement for 6G

Other

For Issue 7 – 12 in WF [1], it is noted that the discussions in RAN have not started except for Issue 7 energy efficiency. In our view, RAN4 should defer the study on topics in Issue 8 – 12.

For energy efficiency, RAN1#122-bis reached some agreements, and some energy saving mechanisms will be evaluated in RAN1, including large SSB periodicity, on-demand SSB/SIB1, SSB-less operation and LP-WUS. While most of the energy saving mechanisms discussed in RAN1 have impacts to RRM requirements, it can be seen that RAN1 is studying those mechanisms, including the user scenarios, solutions, power saving gain, and impact to performances including RRM performances. Clearly, RAN4 should not repeat same studies as done in RAN1. Based on the RAN1 agreements so far, we do not see clear aspects that need to be studied in RAN4 at this stage.

Proposal 8: RAN4 to wait for more RAN1/2 conclusions before studying the following topics.

- Energy efficiency

- Spectrum aggregation

- MIMO and mTRP

- NTN

- initial access

- Other PHY procedure

Another sub-topic that was highlighted in RAN4#116-bis is the RRM-specific Categories.

- RRM-specific Categories: - Proposal 1(QC): - Study how to establish a more intuitive and practical framework for differentiating UE RRM performance beyond baseline RRM requirements and individual capabilities: - The framework aims to reflect real UE implementation-based RRM performance, while RRM requirements continue to serve as the baseline for minimum performance. - The framework aims to cover a limited set of scenarios and cases with field/deployment relevance, within those defined in the RRM requirement specification. - The framework aims to focus on the most important RRM KPIs, such as measurement latency, mobility latency, carrier aggregation latency, interruption, etc. - The framework aims to accommodate different UE implementations across vendors without requiring disclosure of implementation details, unless such details must be known to the network. - The framework aims to minimize impacts on existing 3GPP specifications. - The framework aims to provide relationships to UE capabilities, where applicable. - The framework aims to be maintainable and verifiable with minimal effort.

In our understanding, this issue is not related to introduction or enhancement of particular RRM requirement, rather it is a principle for defining RRM requirements, either mandatory or optional. In our view, it may be too early to discuss this principle without knowing which requirements would be defined and the framework of the requirements. Even it is proposed to first consider “measurement latency, mobility latency, carrier aggregation latency, interruption”, RAN4 still needs to know the basic framework of these requirements, e.g. the applicable scenario, the side condition, and the min or mandatory requirements etc., before RAN4 can discuss whether to define enhanced or optional requirements, and whether to use the capability approach (as in NR) or the proposed category approach.

Proposal 9: RAN4 to discuss the RRM specific category when RAN4 has clear view on the basic framework of the RRM requirements to be defined for 6GR, e.g. the applicable scenario, the side condition, and the min or mandatory requirements.

Conclusions

In this paper we provided our further considerations on several aspects for 6GR RRM study.

Proposal 1: RAN4 to study solutions to support measurement without gap, and strive for a unified solution the first release of 6GR.

Proposal 2: RAN4 to study how to support simultaneous measurement within MG.

Proposal 3: RAN4 to study the interruption length requirements in combination with measurement without gap.

Proposal 4: Besides the sub-topics in Proposal 1-3, RAN4 can also study following sub-topics under MG and interruption topic.

- Adaptative MG operation

- Per CC MG

Proposal 5: RAN4 to study RRM measurement delay reduction for the first release of 6GR by considering enhanced simultaneous measurements.

Proposal 6: RAN4 to wait for RAN1/2 conclusions on measurement framework, procedure and signalling before discussing unified measurement requirements.

Proposal 7: RAN4 to study how to reduce delay and interruption during HO, and strive for a unified solution for the first release of 6GR.

Proposal 8: RAN4 to wait for more RAN1/2 conclusions before studying the following topics.

- Energy efficiency

- Spectrum aggregation

- MIMO and mTRP

- NTN

- initial access

- Other PHY procedure

Proposal 9: RAN4 to discuss the RRM specific category when RAN4 has clear view on the basic framework of the RRM requirements to be defined for 6GR, e.g. the applicable scenario, the side condition, and the min or mandatory requirements.

Reference

- RP‑252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

- R4-2514644, WF on 6G RRM, Feature lead (Apple)