3GPP TSG-RAN WG4 #117	R4-2520337

Dallas, US, November 17-21, 2025

Agenda item:	7.10.3		RRM core requirement

Source:	Qualcomm Incorporated

Title:	Further comments on the RRM impact of NR MIMO Phase 6

Document for:	Discussion

# Introduction

In RAN4 #116bis RAN4 started working on the RAN1-led work item for MIMO Phase 6 [1]. First agreements has been made and a way forward has been provided in [2].

In this paper, we share our views on the major topics of NR-MIMO Phase 6 and provide a preliminary assessment of the RRM impact.

# Discussion

The work item has two main topics, i.e., improvement of SRS capacity and coverage and enhancing the downlink CSI acquisition. In the following we share our view on the impact of core requirements.

## Improvement of SRS Capacity and Coverage

Regarding multiple frequency-domain starting positions for SRS frequency hopping it has been agreed that there is no RRM impact. Until further impact has been identified by new RAN1 decisions, no further discussions about RRM impact are needed.

Regarding cross-slot SRS, two open issues have been identified, namely Issue 2-1-1 and Issue 2-1-2.

Issue 2-1-1: Whether to introduce additional RRM requirements of SRS antenna port switching for cross-slot SRS

< Way forward > FFS the following options:

  - Option 1 (QC, Huawei, Ericsson): No RRM impact.

  - Option 2 (MTK, LG): Wait for further progress in RAN1.

    - MTK: wait for RAN1 conclusions and check whether the legacy interruption requirements are still applicable.

    - LG: Regarding cross-slot SRS, RAN4 to postpone the discussion until further progress in RAN1

  - Option 3 (vivo):

    - RAN4 to define SRS AS requirements for the case for cross-slot SRS resource.

    - RAN4 start from Scenario 1 as agreed in RAN1, and whether to consider other scenarios depends on further conclusion in RAN1.

  - Option 4 (Samsung)

    - FFS on whether to extend the applicability of SRS antenna port switching requirements to the case of SRS resource(s) not only in last 6 symbols of a slot.

Regarding the SRS transmission across two adjacent slots the following scenario has been agreed so far in RAN1

Figure 2-1: SRS resource transmitted across two slots (Scenario 1)

Scenario 1 in RAN1 is defined as an SRS resource set which includes at least one SRS resource with time-domain resource transmitted across a first S-slot and a second consecutive U-slot. That means that the SRS in the U-slot can be transmitted in the beginning of the slot. However, this is already supported since Rel-16 as an optional feature. Requirements in TS 38.133 for SRS antenna port switching always assume that the SRS is in the last six symbols of a slot. Hence, there are no requirements if the SRS comes earlier in a slot than in the last six symbols. We also do not think that new requirements for this case should be introduced now.

Observation 1: 5G NR only supports SRS antenna port switching in case SRS is in the last six symbols of a slot.

Proposal 1: RAN4 should not introduce additional RRM requirements for SRS antenna port switching in case of cross-slot SRS.

Another question is whether cross-slot SRS can be applied for SRS carrier switching.

Issue 2-1-2: Whether to introduce additional RRM requirements of SRS carrier switching for cross-slot SRS

< Way forward > FFS the following options:

  - Option 1 (QC, MTK, LG):

    - RAN4 should wait for further RAN1 progress before RRM requirements for cross-slot SRS are discussed.

  - Option 2 (Huawei):

    - No RRM impact.

  - Option 3 (vivo):

    - RAN4 to define SRS AS requirements for the case for cross-slot SRS resource.

    - RAN4 start from Scenario 1 as agreed in RAN1, and whether to consider other scenarios depends on further conclusion in RAN1.

RAN1 #122 and RAN1 #122bis made the following agreements.

RAN1 #122 Agreement:

When there is a cross-slot SRS resource in the end of a S slot and the beginning of a U slot in a serving cell, support transmitting PUSCH with a priority index 0 and corresponding DMRS after this cross-slot SRS in the U slot in the serving cell.

RAN1 #122bis Agreement

- Support cross-slot SRS for all SRS resource types, i.e., for periodic, semi-persistent, and aperiodic SRS.

- Support cross-slot SRS for usages of codebook, non-codebook, antenna switching, beam management

The first agreement is related to the question whether PUSCH can be transmitted after SRS in the U-slot and says that SRS in the S-slot and the U-slot and PUSCH in the U-slot are in the same serving cell. Hence, cross-slot SRS is not attractive for SRS carrier switching. Based on this RAN1 agreement, we also do not think that RAN4 should start discussing requirements for SRS carrier switching. This is in line with the second agreement that does not explicitly mention SRS carrier switching to be supported by cross-slot SRS.

Observation 2: Based on current RAN1 agreements, cross-slot SRS is not an attractive feature for SRS carrier switching since SRS and PUSCH in both slots are transmitted in the same serving cell.

RAN4 should wait for further RAN1 progress before RRM requirements for cross-slot SRS are discussed.

Proposal 2: RAN4 should wait for further RAN1 progress before RRM requirements for cross-slot SRS are discussed.

## Enhancing Downlink CSI Acquisition

One main topic under downlink CSI acquisition is about providing early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED, from SCell deactivation to activation and when switching out from SCell dormancy. In the following we discuss each topic separately.

### Early SRS/CSI-RS/CSI for Transition from IDLE to CONNECTED

Figure 2-2 illustrates the early triggering of aperiodic SRS, aperiodic CSI-RS and aperiodic CSI report in the transition from IDLE/INACTIVE to CONNECTED mode.

Figure 2-2: Timeline for early SRS/CSI/CSI-RS triggering in IDLE/INACTIVE mode

The UE sends a capability report through msg3 of a 4-step RACH procedure and the network triggers SRS/CSI-RS/CSI through msg4. Our assumption is that any timelines of msg3 and mg4 follow legacy timelines since msg3 and msg4 belong to legacy RACH.

One open question in RAN4 #116bis was whether the UE can transmit or receive during RRC setup or resume delay. No signal can be transmitted or received if msg4 triggers a switch to a new BWP that is different from the initial BWP. This time is TRRCprocessingdelay + TBWPswitchDelayRRC = 10 ms + 6 ms = 16 ms.

Observation 3: No signal can be transmitted or received if msg4 triggers a switch to a new BWP that is different from the initial BWP. This time is TRRCprocessingdelay + TBWPswitchDelayRRC = 10 ms + 6 ms = 16 ms.

For the case that msg4 triggers a BWP switch, RAN4 should not define any requirements for early early SRS/CSI/CSI-RS triggering. For the case that msg4 does not trigger a BWP switch the question of interest is how quickly the UE can process CSI-RS and send the aperiodic CSI report to the network. That depends on the trigger mechanism itself and, in case of CSI reporting, how the PUSCH for the CQI report is scheduled. Relevant RAN1 agreements are copied below.

RAN1 #122bis Agreement

For UE transition from IDLE/INACTIVE to CONNECTED mode, using MAC-CE in the PDSCH for MSG4 to trigger early aperiodic SRS-AS/CSI-RS/CSI

- Note: How to determine the PUSCH for aperiodic CSI reporting is separately discussed.

RAN1 #122bis Agreement:

On PUSCH allocation for MSG4-triggered aperiodic CSI reporting associated with aperiodic CSI-RS for CSI, when UE transition from IDLE/INACTIVE to CONNECTED mode, down select one from the following alternatives to determine PUSCH for carrying the aperiodic CSI report by RAN1#123:

- Alt-1: The PUSCH is scheduled by MAC CE in MSG4 along with the aperiodic CSI report triggering.

- Alt-2: The PUSCH is scheduled by a legacy DCI after MSG4

- Alt-3: The PUSCH is configured by SIBx providing resource/reporting configuration for early triggering of aperiodic CSI reporting.

Note: The timeline of PUSCH for aperiodic CSI reporting is separately discussed.

Since the triggering and reporting mechanism is not yet fully specified in RAN1, RAN4 should wait till RAN1 has fully specified the timeline for SRS-AS transmission, aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI.

Observation 4: The triggering and reporting mechanism for SRS-AS transmission, aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI is not yet fully specified in RAN1.

Proposal 3: RAN4 should wait till RAN1 has fully specified the timeline for SRS-AS transmission, aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI.

### Early SRS/CSI-RS/CSI Triggering for SCell Activation

Regarding the early trigger of SRS-AS/CSI-RS/CSI, RAN1 made the agreement that the definition of the requirement is on RAN4. That means that the timeline of the aperiodic SRS-AS transmission and the timeline of the aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI must be defined by RAN4.

Proposal 4: RAN4 to define the timeline of the aperiodic SRS-AS transmission and the timeline of the aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI after Scell activation command is received.

RAN1 #122 Agreement:

For early triggering of SRS-AS when SCell transition from deactivation to activation, support at least aperiodic SRS-AS transmission on a SCell, triggered based on legacy SCell activation command activating the SCell.

- FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4)

- FFS: How to trigger the aperiodic SRS-AS transmission based on legacy SCell activation command

- FFS: Whether/what new information is provided in SCell activation command?

RAN1 #122 Agreement:

For early triggering of CSI/CSI-RS when SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell triggered based on legacy SCell activation command activating the SCell.

- The aperiodic CSI reporting is assosicated with at least aperiodic CSI-RS for CSI on the SCell

- FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4)

- FFS: How to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI based on legacy SCell activation command?

- FFS: Whether/what new information is provided in SCell activation command?

However, details are not yet clear in RAN1 that will impact the timeline. The triggering of the aperiodic SRS-AS and the aperiodic CSI-RS and aperiodic CSI report is either implicit or explicit according to the discussion in RAN1 #112bis.

Observation 5: It is still open in RAN1 how to trigger the aperiodic SRS-AS and the aperiodic CSI-RS and aperiodic CSI report based on legacy Scell activation command.

Figure 2-3 illustrates the current timeline of Scell activation for FR1 in case the Scell is known and the SCell measurement cycle is less than or equal to 160 ms.

Figure 2-3: Timeline for early SRS/CSI/CSI/-RS triggering in SCell activation procedure

It is seen that there is a fixed time of TFirstSSB + 5 ms that includes MAC CE processing and SSB processing times. This first SSB is required for fine time and frequency tracking. This time and frequency tracking impacts both uplink transmission as well as downlink reception. Hence, aperiodic SRS-AS and aperiodic CSI-RS must come after the first SSB. The figure also shows the P-CSI-RS after the first SSB. Legacy SCell activation makes use of this P-CSI-RS and its associated CSI report. Based on the figure it seems that the gain in activation delay is mainly coming from the fact that now an aperiodic CSI-RS is used instead of a periodic CSI-RS. However, an aperiodic CSI-RS can already be used in Rel-15, and it is a simple task for the network to accelerate SCell activation by Rel-15 features. Hence, the benefit of this feature is not clear and should be discussed in RAN4.

Observation 6: The gain in activation delay is mainly coming from the fact that an aperiodic CSI-RS is used instead of a periodic CSI-RS. However, an aperiodic CSI-RS can already be used in Rel-15. The same holds for aperiodic SRS-AS. Also, this can be done already today.

Proposal 5: RAN4 should discuss the use case and benefit of early triggering of aperiodic CSI-RS and aperiodic SRS-AS in SCell activation procedure.

Proposal 6: RAN4 should discuss sending an LS to RAN1 asking for clarification of the use case.

### Early SRS/CSI-RS/CSI Triggering for Switching from SCell Dormany

In RAN1 #122bis the agreement was made that early triggering of SRS-AS and CSI/CSI-RS is achieved via a DCI indicating switching out of SCell dormany, i.e., legacy DCI formats are reused. That also means, on the other side, that RAN1 has not introduced any new DCI for switching out of SCell dormancy. Hence, the switching delay out of SCell dormancy is not impacted.

Observation 7: Based on current RAN1 agreements, the switching delay out of SCell dormancy is not impacted by earlier SRS/CSI/CSI-RS triggering.

Proposal 7: RAN4 should assume to reuse the switching delay TdormantBWPswitchdelay until RAN1 makes decisions that impact this assumption.

# Conclusions

In this paper we provided our views on the major topics of NR MIMO Phase 6 and a preliminary assessment of the RRM impact. Our observations and proposals are summarized as follows:

Improvement of SRS Capacity and Coverage

Observation 1: 5G NR only supports SRS antenna port switching in case SRS is in the last six symbols of a slot.

Proposal 1: RAN4 should not introduce additional RRM requirements for SRS antenna port switching in case of cross-slot SRS.

Observation 2: Based on current RAN1 agreements, cross-slot SRS is not an attractive feature for SRS carrier switching since SRS and PUSCH in both slots are transmitted in the same serving cell.

Proposal 2: RAN4 should wait for further RAN1 progress before RRM requirements for cross-slot SRS are discussed.

Enhancing Downlink CSI Acquisition

Observation 3: No signal can be transmitted or received if msg4 triggers a switch to a new BWP that is different from the initial BWP. This time is TRRCprocessingdelay + TBWPswitchDelayRRC = 10 ms + 6 ms = 16 ms.

Observation 4: The triggering and reporting mechanism for SRS-AS transmission, aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI is not yet fully specified in RAN1.

Proposal 3: RAN4 should wait till RAN1 has fully specified the timeline for SRS-AS transmission, aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI.

Proposal 4: RAN4 to define the timeline of the aperiodic SRS-AS transmission and the timeline of the aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI after Scell activation command is received.

Observation 5: It is still open in RAN1 how to trigger the aperiodic SRS-AS and the aperiodic CSI-RS and aperiodic CSI report based on legacy Scell activation command.

Observation 6: The gain in activation delay is mainly coming from the fact that an aperiodic CSI-RS is used instead of a periodic CSI-RS. However, an aperiodic CSI-RS can already be used in Rel-15. The same holds for aperiodic SRS-AS. Also, this can be done already today.

Proposal 5: RAN4 should discuss the use case and benefit of early triggering of aperiodic CSI-RS and aperiodic SRS-AS in SCell activation procedure.

Proposal 6: RAN4 should discuss sending an LS to RAN1 asking for clarification of the use case.

Observation 7: Based on current RAN1 agreements, the switching delay out of SCell dormancy is not impacted by earlier SRS/CSI/CSI-RS triggering.

Proposal 7: RAN4 should assume to reuse the switching delay TdormantBWPswitchdelay until RAN1 makes decisions that impact this assumption.

# References

- RP-252936, Revised WID: NR MIMO Phase 6, RAN #109, September 2025

- R4-2514825, WF on NR_MIMO_Ph6_RRM, RAN4 #116bis, October 2025