3GPP TSG-RAN4 #117	R4-2520393

Dallas, USA, 17th Nov. – 21st Nov. 2025

Title:		Discussion on UE types for 6GR

Source:			Anritsu Limited

Agenda Item:			8.2.5

Document for:			Discussion

# 1.	Introduction

RAN4 started a discussion on 6GR system parameter during RAN4 #116-bis meeting in Prague and an associated WF [1] was approved. In this contribution, we would like to discuss topic #7 (“Device types”).

The agreement regarding “Device types” from the WF [1] is extracted below for reference.

# Topic #7: Device types - Agreement - The following aspects could be further studied - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range. - Study on the NW impact should also be considered - Study how to better support variety devices from RAN4 perspective - E.g. differentiate devices, use cases, features - Other aspects are not precluded - Collaborate on the framework with other WGs: - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

# 2.	Discussion

# 2.1	An example of different UE types from 5G NR

We would like to take the example of 5G NR to question at which degree it is necessary to classify UEs to different UE types. In the TS 38.101-1 V19.3.1 [1], there are the following requirements (reference sensitivity adjustment due to support for 4 antenna ports) in Table 7.3.2-2.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R Operating band; ΔRIB,4R (dB). n5, n8, n13, n26, n28, n71, n85, n105; -2.71. n5, n8, n13, n20, n26, n28, n71, n85; -2.42. n1, n2, n3, n7, n25, n30, n34, n38, n39, n40, n41, n66, n70; -2.7. n48, n77, n78, n79, n104; -2.2. NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE..

As the difference is just 0.3dB in terms of allowance and as the requirement for handheld UE is already significantly better compare to two antenna port (though not 3dB but 2.4dB), it could be considered that having a common requirement for “FWA form factor” and “handheld UE” based on that later one could be good enough.

Observation 1: The requirements for “handheld UE” (smartphones, laptops mainly) are set reasonably high and they are devices that can be potentially be using very high data throughput during very long duration, their requirements can be considered as satisfactory by operators and infrastructure vendors such that can be considered also reasonable for FWA CPEs.

Proposal 1a: Study if in 6GR, smartphones, laptops, FWA CPEs should be considered as belonging to the same UE type and have the same 3GPP RAN4 requirements for the same power class/number of antennas, but whatever their size/form factor and power consumption differences.

Proposal 1b: Study if 6GR, fixed outdoor FWA CPEs should be considered as belonging to a different UE type than other FWA CPEs (not fixed and not outdoor) which them should belong to the same UE type than smartphones and laptops for the same power class/number of antennas, but whatever their size/form factor and power consumption.

# 2.2	Should power consumption instead of size/form factor be considered as parameter for determining 6GR UE types?

Size and form factor can be very subjective particularly if the UEs have a display or not, have a battery (and its size, performance) or operates on mains. Power consumption maybe a better differentiator to categorize UE types like between smartphone and a wearable (RedCap).

Observation 2: Size/form factor can be subjective while power consumption maybe a better parameter to differentiate UE types and more aligned with expectations from 6GR IMT-2030.

Proposal 3: Study if RAN4 can use significant power consumption (like between smartphones and wearables using RedCap) differences between UEs to define UE types, and if it is more relevant than size/form factor.

Proposal 4: Study if RAN4 can totally disregard UE size/form factor in defining UE types.

# 3.	Conclusion

Based on this contribution, the following observations and proposals are shared:

Observation 1: The requirements for “handheld UE” (smartphones, laptops mainly) are set reasonably high and they are devices that can be potentially be using very high data throughput during very long duration, their requirements can be considered as satisfactory by operators and infrastructure vendors such that can be considered also reasonable for FWA CPEs.

Proposal 1a: Study if in 6GR, smartphones, laptops, FWA CPEs should be considered as belonging to the same UE type and have the same 3GPP RAN4 requirements for the same power class/number of antennas, but whatever their size/form factor and power consumption differences.

Proposal 1b: Study if 6GR, fixed outdoor FWA CPEs should be considered as belonging to a different UE type than other FWA CPEs (not fixed and not outdoor) which them should belong to the same UE type than smartphones and laptops for the same power class/number of antennas, but whatever their size/form factor and power consumption.

Observation 2: Size/form factor can be subjective while power consumption maybe a better parameter to differentiate UE types and more aligned with expectations from 6GR IMT-2030.

Proposal 3: Study if RAN4 can use significant power consumption (like between smartphones and wearables using RedCap) differences between UEs to define UE types, and if it is more relevant than size/form factor.

Proposal 4: Study if RAN4 can totally disregard UE size/form factor in defining UE types.

# 4.	References

[1] R4-2514650, “WF on 6GR system parameter”, Huawei, HiSilicon (Feature Lead), RAN4 #116-bis, Prague, Czech Republic

[2] TS 38-101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, version 19.3.1

[3] R4-2514190, “Discussion on the usage of the terminology “FWA form factor” in 6Rx, 4Rx, 8Rx requirements and possible lessons for 6GR”, Anritsu Limited, RAN4 #116-bis, Prague, Czech Republic