# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2520428

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.2.2

Source:	CMCC

Title:	Discussion on 6GR modulation

Document for:	Discussion

Introduction

In RAN#108, the technology study item for 6G radio (6GR) has been approved [1]. The anticipated services of 6GR range from immersive communication and massive communication to integrated sensing and communication. Furthermore, 6G services can be provided by both terrestrial network (TN) and non-terrestrial network (NTN). in last meeting, WF[2] and LS[3] are approved which show the high level guideline for further discussion.

- In this contribution, we show our initial analysis on modulation.

Discussion

- Agreement - RAN4 evaluation work focus on the feasibility study and RF requirements impact - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1 - Identify the main affected requirements for modulation evaluations - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions. - Align on the evaluation assumptions. - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation. - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed. - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies. - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

Agreement For 6GR constellation shaping evaluation for DFT-s-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. In addition to what has been agreed for CP-OFDM in earlier agreement, the evaluation and comparison should further consider at least the following: - PAPR/CM of the resulting waveform - EVM, MPR/A-MPR Agreement For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model). - FFS: How to involve RAN4 early - FFS: Shaping of higher order modulation - System level evaluation can be done after link level evaluation.

The EVM value comes from R8 for QPSK and 16 QAM and comes from R15 for 256QAM. Since then, there have been no more updates. At the beginning of 6G, it’s suggested to re-evaluate the EVM for all existing modulation schemes which can be done in RAN4 if there is no concrete LS from RAN1 to establish the foundational evaluation framework in RAN4.

EVM requirement is related to two aspects. One is the performance which benefit from lower EVM by expanding the operating SNR range such that higher MCS and higher data rate can be supported, such performance can be evaluated by system level and link level simulation. The other aspect is the limitation of UE RF implementation which depends on RA non-linearity, the phase noise, CFR distortion…the lower EVM means the increase of cost. So the proper EVM value for requirement is a trade-off between them. Such RF implementation can be evaluated by EVM budget based on RF non-linearity.

For the system level performance, some companies use 5% throughput loss based on averaged MCS as performance metric[7,8] while others use their own performance metric, e.g. the throughput loss compared with lower order modulation scheme with the same EVM [6]. for 6G, at the beginning of study, it’s suggested to unify performance metric for the system level simulation, e.g. use 5% throughput loss based on averaged MCS.

For the link level performance, some companies use marginal performance loss at certain relative throughput e.g. 70% - 90% compared to the performance when EVM is 0% just like what is used for demodulation simulation. Such methodology can be used as baseline.

Proposal 1: it’s suggested to implement unified performance metric for EVM system level and link level simulation.

- For system level, the metric should be 5% throughput loss across all MCS

- For link level, the metric should be the marginal performance loss (e.g. 1dB) at a relative throughput range (e.g. 70% -90%) compared to the performance with 0% EVM.

For the EVM budget based on RF non-linearity, following list the value as summarized in [5] for the EVM budget.

Table 1 EVM budget for 256QAM and 64QAM

| Non-linearity source | 256QAM |  | 64QAM |  |
| --- | --- | --- | --- | --- |
|  | % | C/N | % | C/N |
| PA | 1.50% | 36.5 | 4.00% | 28.0 |
| Transmitter nonlinearity | 1.34% | 37.4 | 3.37% | 29.5 |
| LO Phase noise | 1.78% | 35.0 | 2.24% | 33.0 |
| IQ Image | 2.24% | 33.0 | 5.62% | 25.0 |
| Transmitter total | 3.50% | 29.1 | 8.00% | 21.9 |

RAN4 is suggested to use above table as the baseline for further updates. The detailed EVM analysis of all non-linearity source can be further studies.

Higher-order modulation is essential to meet the 6G peak data rate targets, e.g., 50Gbps outlined in the ITU recommendation document [2]. However, a performance gap of approximately 1.53dB persists between current high-order QAM modulation and the theoretical bound. Constellation shaping is a promising technique to bridge this gap, primarily through two approaches: geometric shaping and probabilistic shaping. For the constellation shaping, RAN1 has approved to at least use following as performance: PAPR/CM of the resulting waveform, EVM, MPR/A-MPR. Besides, the ACLR may also needed to evaluate signal quality.

Observation 1: the main affected requirements for constellation shaping modulation evaluation include the EVM, MPR/A-MPR requirements.

For the uniform 4096QAM for DL and uniform 1024QAM for UL, it’s FFS how to involve RAN4 early. The issue is how to split workload between RAN1 and RAN4. From our understanding, RAN4 can be responsible for the MPR/A-MPR and EVM increase evaluation for no shaping case while for shaping case, RAN4 can be involved to provide more RF impairment analysis.

- Proposal 2: it’s suggested to further study the workload split between RAN1 and RAN4 is listed as below:

- For the uniform higher order modulation without shaping, RAN4 can evaluate the MPR/A-MPR and EVM as RAN4 has done previously

- For the constellation shaping case, RAN4 can be involved to provide more RF impairment analysis.

Conclusion

- This contribution makes the following observations and proposals.

Proposal 1: it’s suggested to implement unified performance metric for EVM system level and link level simulation.

- For system level, the metric should be 5% throughput loss across all MCS

- For link level, the metric should be the marginal performance loss at a relative throughput range (e.g. 70% -90%) compared to the performance with 0% EVM.

Observation 1: the main affected requirements for constellation shaping modulation evaluation include the EVM, MPR/A-MPR requirements.

- Proposal 2: it’s suggested to further study the workload split between RAN1 and RAN4 is listed as below:

- For the uniform higher order modulation without shaping, RAN4 can evaluate the MPR/A-MPR and EVM as RAN4 has done previously

- For the constellation shaping case, RAN4 can be involved to provide more RF impairment analysis.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO

- R4-2514641, WF on system parameter, HW

- R4-2514643, LS on 6G parameter, HW

- R1-2508069, LS on PA models in 6GR waveform evaluations, HW

- R4-165195, TX Challenges for 256 QAM, Qualcomm

- R4-151647, show detailed link level and system level simulation results for UL 64QAM, HW

- R4-152014, show detailed system level simulation results for UL 64QAM, Ericsson

- R4-070368, show detailed system level simulation results for all modulations, Qualcomm