# 3GPP TSG-RAN WG4 Meeting #1167                              R4-2520450

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.9.3

Source: 	CMCC

Title: 			Discussion on Inter-vendor training collaboration and interoperability for two-sided AI/ML models

Document for:	Discussion

- Introduction

In RAN #108 meeting, the new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements was approved [1]. The objectives for RAN4 are duplicated as following.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In last meeting, there was discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, and a WF was approved [2]. This contribution provides further discussion on this topic.

- Discussion

For inter-vendor training collaboration for two-sided AI/ML models, the target is to fully define/specify reference model. Direction C refer to fully standardized reference model (structure + parameters). Direction A, sub-option 3a-1 refer to specification of standardized encoder model structure plus parameter exchange. The agreements on inter-vendor training collaboration are duplicated as below.

Agreement: Definitions: Fully specified the model is defined as a specified structure with parameter in [TS]. Reference model means UE/NW is not mandated to implement it. Test decoder means TE vendor needs to implement it for testing purpose Direct C in RAN1: fully specified reference encoder and decoder Option 3 in RAN4: fully specified test decoder and the reference encoder is under discussion (it is agreed to have a fully specified reference encoder) Direction A 3a-1 in RAN1: specify the reference encoder structure. Presumably, NW will send the parameters to the UE. Relation between RAN1 and RAN4: • The test decoder of option 3 is the same as the reference decoder of direction C - Test decoder of option 3 will be specified by RAN4 - If RAN1 develops the reference decoder for direction C which is different from RAN4’s, further discussion across RAN4 and RAN1 is needed. • The reference encoder is the same for both option 3 and direction C • The option 3 reference encoder structure is the same as the encoder structure for direction A option 3a-1. - Reference encoder of option 3 will be specified in RAN4. How to respond to the RAN checkpoint • Option 3 is feasible for RAN4 to complete. RAN4 will take into account the conclusions in RAN1 on scalability in order to create a scalable model. - RAN4 agrees that the test decoder of option 3 is the same as the reference decoder of direction C and the reference encoder of option 3 is the same as the reference encoder of direction C. If so, RAN4 believes that direction C should be feasible to complete too. • Direction A 3a-1 is feasible for RAN4 to complete from reference encoder perspective. • FFS on detailed steps to develop the model • RAN4 will send LS on RAN4’s agreement regarding the RAN checkpoint.

The steps to develop the test decoder and reference encoder are agreed as following:

- Step 1:  Create dataset

  - Step 1-1: Align on configurations for mixed dataset construction, taking factors such as scalability over Tx port number and subband size and payload size into account

  - Step 1-2 Decide how align and on which metrics can be used for evaluating dataset alignment, if any

  - Step 1-3 Companies create datasets

  - Step 1-4 Identify which company datasets to include in the mixed dataset(s)

  - Step 1-5 Create new dataset(s)

- Step 2: Align on the model backbone, model hyperparameters and the scalability solution.

  - Step 2-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution.

    - This can be in parallel with step 1.

    - Transformer is agreed as baseline for backbone

    - Same backbone for test decoder and reference encoder

  - Step 2-2: Preliminary selection of structures, taking into account complexity

  - [Step 2-2a Align on training parameters]

  - Step 2-3: Agree on metrics and steps for evaluation

  - Step 2-4: Check on performance alignment -> see simulation results from contributing companies.

  - Step 2-5: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed.

  - Step 2-6: Select one or more model structures considering complexity and robustness and decide how many models.

  - Step 2-7: Select one or more scalability solutions. This step can be done in parallel with Steps 2-2 to 2-4.

- Step 3: Specify the model for the following work in WID. Where and how to specify may need further discussion.

Note: At this stage, models using the agreed structures, dataset, scalability have already been created by most companies

  - Step 3-1 Decide how to select from the trained model(s) from each company

  - Step 3-2: Freeze model(s)

  - Step 3-3: Agree on how and where to capture the models, the dataset and relevant information

In last meeting, it was agreed that transformer is assumed as the baseline and only Quantized parameters are specified. According to the steps to develop the test decoder and reference encoder, step 2 is to align the model backbone, model hyperparameters and the scalability solution. In our view, RAN1 agreements on transformer can be used as baseline.

In RAN1 #119 meeting, it was agreed to adopt Transformer as the backbone structure for temporal domain Case 0 in case of take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output. Following Figure is the encoder structure considered in RAN1 as the example in the study for Case 0 and taking spatial-frequency domain precoding matrix as input.

Figure 1: Encoder structure example

Encoder description in RAN1 study is elaborated in Table 1. The decoder has a mirroring design as the encoder.

Table 1: Encoder description for CSI feedback using two-sided model

| Domain | Parameters |
| --- | --- |
| Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer) | Number of tokens Feature dimension of each token Output dimension dmodel per token |
| Positional encoding | Number of tokens positions Dimension of positional encoding for each token position: dmodel |
| Transformer blocks | Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nℎead Dimension of attention head dℎead Dimension of latent space inside feedforward module dFF Activation function choice |
| Output linear layer | Reshape the matrix to a vector Output dimension zdim |
| Quantization | Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit∗Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit∗Nseg - Number of segments = Zdim/Nseg - Total payload size for Vector quantization = Number of segments * bits per segment |

Since we agreed to take Transformer as baseline, it is proposed that the example structure and parameters in RAN1 can be used as baseline.

Proposal 1: for model backbone and model hyperparameters, it is proposed that the example structure and parameters for transformer agreed in RAN1 are used as baseline.

- Conclusion

This contribution provides discussion on inter-vendor training collaboration and interoperability for two-sided AI/ML models. The proposals are:

Proposal 1: for model backbone and model hyperparameters, it is proposed that the example structure and parameters for transformer agreed in RAN1 are used as baseline.

- Reference

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

- R4-2514614, WF on R20 NR_AIML_air_Ph2, Ericsson

[3] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface